8.3GPP TSG-SA WG1 Meeting #113 	draft S1-261xxx

9-13 February 2026, Goa, India	(revision of S1-26xxxx)

Source:	6G Study Rapporteurs

pCR Title:	Table 14.1.11-1 (Satellite-based communication) Consolidation

Draft Spec:	3GPP TR 22.870 v1.1.0

Agenda item:	x.x

Document for:	Approval

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Abstract: This PCR provides the initial Table 14.1.11-1 for consolidation discussions.

1. Introduction

The remaining potential requirements need to be consolidated prior to sending to SA for approval and publication. This pCR provides the latest version of the CPR table as the basis for discussions at this meeting. As agreed in SA1 #112 Ad Hoc-e, Table14.1.11-1 will include CPRs about the communication with satellite access and/or satellite backhaul.

2. Reason for Change

Previous versions of this pCR used S1-254020 as the initial basis for this pCR. With modifications from SA1 #112 (Dallas) and #112 ad hoc (January e-meeting).

In particular, the following TDOCs were used to update this table:

- S1-254412, the output of the initial consolidation discussions for Ubiquitous Connectivity (SA1 #112)

- S1-254120  and S1-254121 (CATT)

- S1-254163 (ZTE Corporation, CSCN)

- S1-254250 (Qualcomm)

- S1-254300r1 (Huawei)

This pCR was updated to reflect the endorcsement of S1-254410.

Orig PRs were added (shaded in grey) for information and rapporteur notes added to provide additional information.

There are some CPRs/PRs that are included in both Table 14.1.11-1 (Satellite-based comms) and Table 14.1.11-3 (Other aspects). This needs to be resolved, and I tried to mark those entries.

3. Proposal

It is proposed to agree the following changes to 3GPP  TR 22.870 v1.1.0.

* * * First Change * * *

Table 14.1.11-1: Satellite-based communication

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.11-1-1 | The 6G system with satellite access shall be able to support flexible configurations (e.g. parameters for satellite access) for both sparse satellite constellation and dense satellite constellation and optimise the coverage per LEO satellite for sparse satellite constellation. | PR 8.3.6-1 PR 8.3.6-3 | [Satellite access] LEO Coverage Flex configs |
| CPR 14.1.11-1-2 | The 6G system with satellite access shall support service continuity when UE is within the coverage of the LEO constellation, e.g. in the sparse LEO scenarios. | PR 8.3.6-2 | [Satellite access] Service continuity LEO |
| CPR 14.1.11-1-3 | Subject to regulatory requirements, the 6G system with satellite access, shall support text messaging services and PWS for UEs in adverse propagation conditions e.g. light indoor conditions, dense forest. | PR 8.4.6-3 PR 8.3.6-3 | [Satellite access] PWS Text Messaging |
| CPR 14.1.11-1-4 | The 6G system shall support communication between UEs under the coverage of HAPS platforms, without the user traffic going through the ground network. NOTE: it is assumed there is a 6G core network deployed on the HAPS platform. | PR 8.13.6-1 | Other non-terrestrial access] UE-HAPS-UE direct communications [CATT: move to Table 14.11.1-3, due to irrelevant to satellite] Huawei: wording is modified slightly. Remove the “base stations onboard”, and add a note: it is assumed there is a 6G core network deployed on the HAPS platform. [QC-NEW]: agree with CATT suggestion] |
| CPR 14.1.11-1-5 | The 6G system with satellite backhaul links between a 6G base station on board of UAV and the 6G CN, shall support the selection and switching between satellite links, each having different characteristics, based on e.g. traffic load, quality of the link, satellite availability. NOTE: The following terms 6G base station/CN do not imply any architectural assumption, e.g. whether 6G CN/base station is a new or evolved CN/base station (compared to 5G). | PR 8.12.6-1 | Satellite backhaul links selection/switching Should be moved out of this table (about SAT access), e.g. can go in “others” [CATT: Keep in this table. Satellite backhaul is also satellite-based communication.] [QC-NEW]: given the new title, ok to keep it here |
|  |  |  | Moved to Table 14.1.11-3: Other aspects (ubiquitous) [CATT: move to Table14.1.11-2, because it’s relevant to satellite-based positioning.] |
| CPR 14.1.11-1-7 | Subject to operator’s policy, the 6G network with satellite access shall support onboard Service Hosting Environment(s) and mechanisms to minimise the necessary bandwidth of the inter-satellite and feeder links, and modify the path for routing data traffic between a UE and the Service Hosting Environment to minimize service interruption considering the movement of UE and/or satellite, provide a computing service via a to UEs (e.g. UAV) using only satellite access e.g. considering the latency and satellite capabilities, and subject to user consent, support data sharing among multiple UEs (e.g. UAV). NOTE: The shared data is the processed results based on non-3GPP sensing data from the UEs (e.g. UAV) provided by Service Hosting Environment on aboard satellite. | PR 8.15.6-1 PR 8.9.6-1 | Onboard Service Hosting Environment Modification of data routing Computing service UE Data Sharing Non-3GPP sensing It is suggested to seperate which are targeting different aspects [CATT]: Our proposal is to move CPR for PR8.15.6-1 and PR8.9.6-1 to Table14.1.11-3. Leave PR8.9.6-3, PPr8.9.6-4 in this table and consolidated separately. Both PRs are in this table and Table 14.1.11-3: Other aspects (ubiquitous) and PR 8.9.6-1 is also in Table 14.1.9-1 – General Computing requirements Resolve to which table these PRs/CPRs belong Huawei: this CPR combines too many aspects. We need to discuss what services are exactly provided by the Onboard Service Hosting Environment. Regarding computing service provided by the Service Hosting Environment on aboard satellite, 6G RAN should be excluded. [QC-NEW]: split PRs are much clearer/better, suggest NOT to merge |
| Orig PR | The 6G network using satellite access shall be able to provide the service hosting environment onboard satellite minimising the necessary bandwidth of the inter satellite links and feeder links. | PR 8.15.6-1 | Provided for info |
| Orig PR | Subject to operator’s policy and agreement with 3rd party, the 6G system with satellite access shall be able to provide a computing service via a suitable Service Hosting Environment on board satellite to a UE (e.g. UAV) using only satellite access e.g. considering the latency and satellite capabilities | PR 8.9.6-1 | Provided for info |
| CPR 14.1.11-1-8 | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G system with satellite access shall be able to support data sharing among multiple UEs (e.g. UAV). | PR 8.9.6-3 | CATT proposed to move to Table 14.1.11-3: Other aspects In both Table 14.1.11-1: Satellite-based communication and Table 14.1.11-3: Other aspects (ubiquitous). Resolve to which table these PRs/CPRs belong Huawei: what is the relationship between 14.1.11-1-7 and 14.1.11-1-8? There is some overlapping about the requirement wording. In addition, suggest to remove the note [QC-NEW]: this requirement is very confusing (who is transmitting/receiving data, is it UE-SAT-UE communication?) suggest to remove |
| CPR 14.1.11-1-9 | Subject to operator’s policy, the 6G network with satellite access shall be able to support modifying the path for routing data traffic between a UE and Service Hosting Environment (excluding RAN) on board satellites to minimize service interruption considering the movement of UE and/or satellite. | PR 8.9.6-4 | In both Table 14.1.11-1: Satellite-based communication and Table 14.1.11-3: Other aspects (ubiquitous). Resolve to which table these PRs/CPRs belong Huawei: exclude RAN from the Service Hosting Environment |
| CPR 14.1.11-1-10 | The 6G system using satellite access shall support mechanisms to improve communication QoS and capacity under constrained satellite link conditions, | PR 8.18.6-1 | QoS Improvement |
|  |  |  | Text Msg Service Not included in previous table. Could go into enh legacy? Huawei: this one is already covered in the CPR 14.1.11-1-3. Should be removed. [QC-NEW]: agree with Huawei |
|  |  |  | Service continuity TN-NTN Not in the Table in S1-254410, so verify that these requirements are considered “not consolidated” (covered by 5G requirements) Huawei: this one is already covered in the TS 22.261. See clause 6.46.3 Service continuity. Should be removed. [QC-NEW]: agree with Huawei |
| CPR 14.1.11-1-NEW (CATT proposed move) | Subject to operator’s policy, the 6G network shall support mechanisms to connect the base stations (onboard satellite) and the ground core networks using 3GPP technology. | PR 8.17.6-1 | New connection method Previous EN was resolved CATT: proposed move from Table 14.1.11-2: Satellite-based positioning and Table 14.1.11-3: Other aspects (ubiquitous) Huawei: open to which table this requirement should belong to. [QC-NEW]: we have concerns about this, i.e. on the impacts of defining new feeder link interface and protocols in 3GPP. This should be a stage-2/3 architecture discussion, not a service requirement. Prefer to remove |
| NEW CPR b (CATT proposed move) | The 6G system with satellite backhaul links between 6G base station on board of UAV and the 6G CN, shall be able to minimise service interruption when the satellite backhaul link changes. | PR 8.12.6-2 | Sat Backhaul Not included in previous table CATT: proposed move from Table 14.1.11-3: Other aspects (ubiquitous) |
| NEW CPR c (CATT proposed move) | Subject to operator’s policy and the regulatory requirements and operator’s policy, the 6G network shall support a mechanism to establish an efficient connection between the base station (onboard satellite) and the ground core network considering the characteristics (e.g. reliability, latency, data rate) of the carried traffic. NOTE: Non-3GPP satellite access technologies are not precluded for the feeder link (i.e. radio link between ground station and satellite). | PR 8.17.6-2 | Sat Backhaul Not included in previous table CATT: proposed move from Table 14.1.11-3: Other aspects (ubiquitous) [QC-NEW]:aligned with the comment above, prefer to remove the note |

* * * End of Changes * * *

Note (provided for information): the following initial table entries in S1-254020 were moved to the KPI discussions.

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| 14.1.11-1-4 | The 6G system with satellite access, shall support SMS delivery to a high density of UEs i.e. up to [1000] UE per km2. | PR 8.4.6-2 | KPI (Y.2) high density # of UEs SMS CATT, QC, ZTE/CSCN all proposed to move to KPI |
| 14.1.11-1-6 | The 6G system with satellite access, shall be able to provide eMBB service to UE mounted on SWAP constrained moving platforms (e.g. flying drone, small vessels mounted). | PR 8.6.6-1 | eMBB [ZTE] It is suggested to merge it with KPI table 8.6.6-1 |
| 14.1.11-1-7 | The 6G system with satellite access shall be able to support communication service for UEs (e.g. UAV) with variant altitudes (e.g. from 0 to 3 km [99]) with satellite access and/or terrestrial access. | PR 8.9.6-2 | Communication service UEs at variant altitudes [QC]: These values are already part of the merged KPI table. |
| 14.1.11-1-14 | The 6G system with satellite access shall be able to provide eMBB service with KPIs from Table 8.6.6-1: KPIs of the 6G satellite access network to support the use case disaster relief [112]. | PR 8.6.6-2 | KPI eMBB [ZTE]: This is part of KPI table. |
| 14.1.11-1-15 | The KPIs in Table 8.8.6-1: KPIs for global mobile video should be met for global mobile video service. | PR 8.8.6-2 | KPI Global mobile video service |

Note (provided for information): the following initial table entries in S1-254020 were moved during SA1 discussions.

| CPR # | Consolidated Potential Requirement |  | Original PR # |  | Comment |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | In both Table 14.1.11-3: Other aspects (ubiquitous) and Table 14.1.9-1 – General Computing requirements |