3GPP TSG-SA WG1 Meeting #113 	S1-261xxx

9-13 February 2026, Goa, India	(revision of S1-26xxxx)

Source:	6G Study Rapporteurs

pCR Title:	Table 14.1.11-3 (Other Aspects) Consolidation

Draft Spec:	3GPP TR 22.870 v1.1.0

Agenda item:	x.x

Document for:	Approval

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Abstract: This PCR provides the initial Table 14.1.11-3 for consolidation discussions.

1. Introduction

The remaining potential requirements need to be consolidated prior to sending to SA for approval and publication. This pCR provides the latest version of the CPR table as the basis for discussions at this meeting. As agreed in SA1 #112 Ad Hoc-e, Table14.1.11-3 will include CPRs relevant to other NTN such as HAPS, or other non-communication capabilities of satellite, etc.

2. Reason for Change

Previous versions of this pCR used S1-254020 as the initial basis for the consolidation discussions on Ubiquitous Connectivity.

In particular, the following TDOCs were used to update this table:

- S1-254412, the output of the initial consolidation discussions for Ubiquitous Connectivity (SA1 #112)

  - Both CATT and Huawei proposed a new Table to capture “ Other capabilities”

- S1-254120  and S1-254121 (CATT)

- S1-254163 (ZTE Corporation, CSCN)

- S1-254250 (Qualcomm)

- S1-254300r1 (Huawei)

This pCR was updated to reflect the endorcsement of S1-254410.

Orig PRs were added (shaded in grey) for information and rapporteur notes added to provide additional information.

There are some CPRs/PRs that are included in both Table 14.1.11-1 (Satellite-based comms) and Table 14.1.11-3 (Other aspects). This needs to be resolved, and I tried to mark those entries.

3. Proposal

It is proposed to agree the following changes to 3GPP  TR 22.870 v1.1.0.

* * * First Change * * *

Table 14.1.11-3: Other aspects

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.11-3-1 | The 6G system with satellite access shall be able to provide time synchronization to UEs and applications using 3GPP technologies, independently of non-3GPP technologies (e.g. GNSS). | PR 8.14.6-1 | Time synchronization [CATT: Consider to move it to Table14.1.11-2? The timing is based on satellite-based positioning technology.] QC-New: we can live with the CPR as is |
| Alt New CPR Used to be CPR 14.1.11-1-11 (CATT) | The 6G system with satellite access shall be able to support the transfer of computing information (e.g. pre-processed data) of a computing task between Service Hosting Environments on board satellites over a given area. | PR 8.15.6-2 | CATT proposed this entry Data transfer between onboard computing resources |
| CPR 14.1.11-3-2 | The 6G system using with satellite access based on regenerative satellites shall be able to support the transfer of computing information (e.g. pre-processed data) of a computing task between S within the Service Hosting Environments on board ) between satellites over a given area. | PR 8.15.6-2 | Satellite-Satellite Data transfer between onboardof computing inforesources |
| New CPR Used to be CPR 14.1.11-1-11 | The 6G system using satellite access based on regenerative satellites shall be able to support the transfer of computing information (e.g. pre-processed data within the service hosting environment) between satellites over a given area. | PR 8.15.6-2 | Satellite-Satellite transfer of computing info Moved from Table 14.1.11-1 (Sat based comms) QC-New: I guess you can remove the 2 previous rows (as this was green) |
| CPR 14.1.11-3-3 (CATT) | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G system with satellite access shall be able to share data (e.g. the processed result based on the non-3GPP sensing data from multiple UEs) provided by the Service Hosting Environment on board satellite among multiple UEs (e.g. UAV). | PR 8.9.6-3 | Computing data sharing CATT proposed this entry be moved from Table 14.1.11-1 (Sat-based comms) Computing data sharing Revised as part of SA1 112 Ad Hoc-e QC-New: see comments in the SAT-comm part |
| CPR 14.1.11-3-b | Subject to operator’s policy and agreement with 3rd party, the 6G network with satellite access shall be able to provide a computing service via a Service Hosting Environment on board (s) satellite to a UE (e.g. UAV) using only satellite access e.g. considering the latency and satellite capabilities |  | Onboard Computing Marked in yellow because of the controversial definition of SHE QC-New: ok with this version, with some edits |
| CPR 14.1.11-3-c | Subject to operator’s policy, the 6G network with satellite access may support onboard Service Hosting Environment(s) and mechanisms to - Minimise the necessary bandwidth of the inter-satellite and feeder links, - Modify the path for routing data traffic between a UE and the Service Hosting Environment to minimize service interruption considering the movement of UE and/or satellite, - provide a computing service via a UE (e.g. UAV) using only satellite access e.g. considering the latency and satellite capabilities, and - subject to regulatory requirements and subscriber permission,, support data sharing among multiple UEs (e.g. UAV). NOTE: The shared data is the processed results based on non-3GPP sensing data from the UEs (e.g. UAV) provided by Service Hosting Environment on aboard satellite. This implies some need to transfer computing information in/related to Service Hosting Environment (e.g. pre-processed data within the Service Hosting Environment) between satellites over a given area. | PR 8.15.6-1 PR 8.9.6-3 PR 8.9.6-4 PR 8.15.6-2 | Onboard Service Hosting Environment Modification of data routing Computing service UE Data Sharing Non-3GPP sensing and transfer of computing service related info related to SHE [Huawei]: Spaceborne SHE, how compelling? PRs 8.9.6-3, 8.9.6-4 and 8.15.6-1 are also in Table 14.1.11-1: Satellite-based communication. PR 8.15.6-2 is also in Table 14.1.9-1 – General Computing requirements. Also in Table 14.1.11-1: Satellite-based communication and Table 14.1.9-1 – General Computing requirements Need to resolve in which table to consolidate. QC-New: prefer to split, see also comments in the SAT-comm part |
| Orig PR | The 6G network using satellite access shall be able to provide the service hosting environment onboard satellite minimising the necessary bandwidth of the inter satellite links and feeder links. | PR 8.15.6-1 | Provided for info [CATT: This PR can be included in Table14.1.11-3. To merge with PR8.9.6-1 or not will be discussed later.] |
| Orig PR | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G system with satellite access shall be able to support data sharing among multiple UEs (e.g. UAV). NOTE: The shared data is the processed results based on non-3GPP sensing data from the UEs (e.g. UAV) provided by Service Hosting Environment on aboard satellite. | PR 8.9.6-3 | Provided for info |
| Orig PR | Subject to operator’s policy, the 6G network with satellite access shall be able to support modifying the path for routing data traffic between a UE and Service Hosting Environment on board satellites to minimise service interruption considering the movement of UE and/or satellite. | PR 8.9.6-4 | Provided for info |
| Orig PR | The 6G system using satellite access based on regenerative satellites shall be able to support the transfer of computing information (e.g. pre-processed data within the service hosting environment) between satellites over a given area. | PR 8.15.6-2 | Provided for info |
| Alt CPR 14.1.11-3-d (Huawei) | Subject to operator’s policy and agreement with 3rd party, the 6G system with satellite access may be able to provide a computing service in a given area via a Service Hosting Environment on board satellite to a UE (e.g. UAV) using only satellite access e.g. considering the latency and satellite capabilities. |  | [Huawei]: Spaceborne SHE, how compelling? Also in Table 14.1.11-1: Satellite-based communication and Table 14.1.9-1 – General Computing requirements Need to resolve in which table to consolidate. QC-New: also ok with this version (instead of 3-b), with some edits |
| Orig PR | Subject to operator’s policy and agreement with 3rd party, the 6G system with satellite access shall be able to provide a computing service via a suitable Service Hosting Environment on board satellite to a UE (e.g. UAV) using only satellite access e.g. considering the latency and satellite capabilities. | PR 8.9.6-1 | Provided for info |
| CPR 14.1.11-1-NEW | Subject to operator’s policy, the 6G network shall support mechanisms to connect the base stations (onboard satellite) and the ground core networks using 3GPP technology. | PR 8.17.6-1 | New connection method Previous EN was resolved [CATT: Suggest to move to Table14.1.11-1] QC-New: see comments in the SAT-comm part |
| New CPR a | Subject to operator’s policy, the 6G system shall support Femtocell deployments. | PR 5.9.1.3-1 | Coverage Not included in previous table. Could also go into enh legacy? QC-New: better in enh. Legacy in my view. Made some edits |
| New CPR b | The 6G system with satellite backhaul links between 6G base station on board of UAV and the 6G CN, shall be able to minimise service interruption when the satellite backhaul link changes. | PR 8.12.6-2 | Sat Backhaul Not included in previous table QC-New: see comments in the SAT-comm part |
| New CPR c | Subject to operator’s policy and the regulatory requirements and operator’s policy, the 6G network shall support a mechanism to establish an efficient connection between the base station (onboard satellite) and the ground core network considering the characteristics (e.g. reliability, latency, data rate) of the carried traffic. NOTE: Non-3GPP satellite access technologies are not precluded for the feeder link (i.e. radio link between ground station and satellite). | PR 8.17.6-2 | Not included in previous table [CATT: Suggest to move to Table14.1.11-1] QC-New: see comments in the SAT-comm part |
| New CPR d | Subject to operator’s policy and regulatory requirements, the 6G network shall support UE mobility between a base station on the ground and a base station on board a HAPS platform connected to the same core network, taking into account network load and the availability of the HAPS platform (e.g. due to varying power between night and day). | PR 8.19.6-1 | Not included in previous table QC-New: suggest few edits |
| Alt CPR 14.1.11-1-8 (CATT) | The 6G system shall support communication between UEs under the coverage of base stations onboard HAPS platforms, without the user traffic going through the ground network. | PR 8.13.6-1 | [CATT: Suggest to move from Table 14.11.1-1, for HAPS based communication] |

* * * End of Changes * * *