3GPP TSG-SA WG1 Meeting #113 	draft S1-261xxx

9-13 February 2026, Goa, India	(revision of S1-26xxxx)

Source:	6G Study Rapporteurs

pCR Title:	Table 14.1.4-1 (Energy-related Aspects) Consolidation

Draft Spec:	3GPP TR 22.870 v1.1.0

Agenda item:	x.x

Document for:	Approval

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Abstract: This PCR provides the initial Table 14.1.4-1 for consolidation discussions.

1. Introduction

The remaining potential requirements need to be consolidated prior to sending to SA for approval and publication. This pCR provides the latest version of the CPR table as the basis for discussions at this meeting.

2. Reason for Change

S1-254020 was the initial basis for this pCR. With modifications from SA1 #112 (Dallas) and #112 ad hoc (January e-meeting).

In particular, the following TDOCs were used to update this table:

- S1-254411, Output of intial consolidation discussions for Basic Serices & Capabilities (SA1 #112)

- S1-254160 (ZTE)

- S1-254250 (Qualcomm)

- November 2025 reflector comments

Orig PRs were added (shaded in grey) for information and rapporteur notes added to provide additional information.

The EN associated with PR 5.8.2.6-2 (concerning energy consumption information) was cleared so the PR was added to this Table.

3. Proposal

It is proposed to agree the following changes to 3GPP  TR 22.870 v1.1.0.

* * * First Change * * *

Table 14.1.4-1: Energy-related Aspects

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.4-1-1 | The 6G system shall improve energy efficiency of 6G system compared to 5G system. | PR 5.8.1.6-1 | System Energy Efficiency |
| NEW Alt CPR 14.1.4-1-1 | The 6G system shall improve both the network and UE energy efficiency compared to the 5G system. | PR 5.8.1.6-1 PR 5.8.1.6-4 | System Energy Efficiency PR 5.8.1.6-4 was not included in the previous version of the table. |
| Orig PR | The 6G system shall provide means to improve the joint network and UE energy efficiency when providing services to the subscribers. | PR 5.8.1.6-4 | Provided for info |
| Orig PR | The 6G network shall be able to support mechanisms to improve UE energy efficiency when providing services to the subscribers. | PR 5.8.1.6-2 | Provided for info |
| Orig PR | The 6G system shall provide means for a user to provide a preference for UE energy efficiency. NOTE: The 6G network takes into account this preference when improving UE energy efficiency. Examples of preference are user preference on prioritizing service performance, prioritizing UE energy efficiency, or prioritizing essential service (e.g. voice call) while improving UE energy efficiency. | PR 5.8.1.6-3 | Provided for info |
| Orig PR | Subject to regulatory requirements and subscriber permission, the 6G system shall be able to optimize the UE energy saving considering the current traffic conditions of the UE. | PR 5.8.4.6-2 | Provided for info Revised as part of SA1 112 Ad Hoc-e |
| Orig PR | Subject to regulatory requirements and subscriber permission, the 6G system shall support mechanisms to optimize the energy consumption of a UE via enabling task offloading (e.g. XR rendering/AI inference) from the UE to the Service Hosting Environment, considering battery life and thermal issue of UE. | PR 5.8.5.6-1 | Provided for info Revised as part of SA1 112 Ad Hoc-e |
| CPR 14.1.4-1-2 | The 6G system shall support mechanisms to improve UE energy efficiency when providing services to the subscriber, optimizing the UE energy saving considering the current traffic conditions of the UE (subject to regulatory requirements and subscriber permissions), enabling task offloading from the UE to the Service Hosting Environment, considering battery life and thermal issue of UE, and provide means for a subscriber to provide a preference for UE energy efficiency. NOTE: The 6G network uses this preference when improving UE energy efficiency. Examples of preference include: - subscriber preference on prioritizing service performance, - prioritizing UE energy efficiency, or - prioritizing essential service (e.g. voice call) while improving UE energy efficiency. | PR 5.8.1.6-2 PR 5.8.1.6-3 PR 5.8.4.6-2 PR 5.8.5.6-1 | UE Energy Efficiency Revised as part of SA1 112 Ad Hoc-e |
| Alt 1 CPR 14.1.4-1-2 (S1-254160) | The 6G system shall support mechanisms to improve UE energy efficiency when providing services to the subscriber, optimizing the UE energy saving considering the current traffic conditions of the UE (regulatory requirements and subscriber permission), enabling task offloading from the UE to the Service Hosting Environment, considering battery life and thermal issue of UE. | PR 5.8.1.6-2 PR 5.8.4.6-2 PR 5.8.5.6-1 | UE Energy Efficiency [ZTE] move PR5.8.1.6-3 to seperate CPR Revised as part of SA1 112 Ad Hoc-e |
| New CPR 14.1.4-1-ZTE (S1-254160) | The 6G system shall provide means for a user to provide a preference for UE energy efficiency. NOTE: Examples of preference are user preference on prioritizing service performance, prioritizing UE energy efficiency, or prioritizing essential service (e.g. voice call) while improving UE energy efficiency. | PR 5.8.1.6-3 | [ZTE] it is for user’s preference, better to seperate. Should “user preference” be “subscriber preference? How does this relate to subscriber permission? |
| Alt 2 CPR 14.1.4-1-2 (10 Nov reflector comments) | The 6G system shall be able to support mechanisms (including a mechanism to consider user preference) to improve UE energy efficiency when providing services to subscribers. Examples of such preferences - prioritizing service performance, - prioritizing UE energy efficiency, or - prioritizing essential service (e.g. voice call) while improving UE energy efficiency. | PR 5.8.1.6-2 PR 5.8.1.6-3 | UE Energy Efficiency Should “user preference” be “subscriber preference? How does this relate to subscriber permission? |
| New CPR 14.1.4-1-Nokia | Subject to regulatory requirements and subscriber permission, the 6G system shall optimize UE energy consumption by considering the UE's current traffic conditions and by enabling task offloading (e.g. XR rendering/AI inference) from the UE to the Service Hosting Environment, considering the UE's battery life and thermal issues. | PR 5.8.4.6-2 PR 5.8.5.6-1 | UE Energy Efficiency Revised as part of SA1 112 Ad Hoc-e |
| Alt 3 CPR 14.1.4-1-2 (13 Nov reflector comments) | The 6G system shall support mechanisms to improve UE energy efficiency and saving when providing services to the subscriber, at least considering the current traffic conditions of the UE (subject to regulatory requirements and subscriber permission), by enabling task offloading from the UE to the Service Hosting Environment, also based on battery life and thermal issue of UE. | PR 5.8.1.6-2 PR 5.8.4.6-2 PR 5.8.5.6-1 | UE Energy Efficiency ZTE] move PR5.8.1.6-3 to seperated CPR Nokia: rewording to: - Factorise efficiency & saving - use “at least by” instead of “ie” (exact list) or “eg” (examples only) - change “considering” to “also based on” (also removing the comma) to avoid another verb in -ing which may be confused with another “by”, and better link it with the offloading only Revised as part of SA1 112 Ad Hoc-e |
| New CPR 14.1.4-1-reflect | The 6G system shall provide means for a user to provide a preference for UE energy efficiency. NOTE: Examples of preference are user preference on prioritizing service performance, prioritizing UE energy efficiency, or prioritizing essential service (e.g. voice call) while improving UE energy efficiency. | PR 5.8.1.6-3 | [ZTE] it is for user’s preference. UE Energy Efficiency Preference |
| CPR 14.1.4-1-3 | The 6G network with multiple access networks (e.g. terrestrial, satellite) shall provide means to determine (e.g. through measurement and/or calculation) the energy consumed by the access networks for the provision of a one-to-many service (e.g. PWS [62], broadcast, multicast) characterised by at least the QoS/QoE, targeted service area and/or targeted set of users. | PR 5.8.2.6-1 | Access Energy Consumption [InterDigital: inline addition. QoE for the media services] |
| CPR 14.1.4-1-4 | Subject to operator’s policy, the 6G system shall support network slice-specific energy management policies that - define a maximum slice energy credit limit for services without QoS criteria, - support a mechanism to perform slice energy credit limit control for services without QoS criteria. NOTE: The result of the credit control is not specified by this requirement. | PR 5.8.3.6-1 PR 5.8.3.6-2 | Slice Energy Management |
| Orig PR | Subject to operator’s policy, the 6G system shall support network slice-specific energy management policies that define a maximum slice energy credit limit for services without QoS criteria. | PR 5.8.3.6-1 | Provided for info |
| Orig PR | Subject to operator’s policy, the 6G system shall support a mechanism to perform slice energy credit limit control for services without QoS criteria. | PR 5.8.3.6-2 | Provided for info |
| CPR 14.1.4-1-5 | Subject to regulatory requirements and subscriber permission, the 6G system shall be able to optimise network and UE energy saving jointly, while meeting the service performance requirements. | PR 5.8.4.6-3 | System Energy Saving Revised as part of SA1 112 Ad Hoc-e |
| Orig PR | Subject to operator’s policy and reglatory requirements, the 6G network shall provide suitable energy saving methods targeting different scenarios (e.g. different working time), with the respective information on predicted energy consumption and carbon equivalent emissions per scenario. NOTE: The following network energy information about energy consumption and carbon equivalent emissions can be calculated and/or estimated. | PR 5.8.6.6-1 | Provided for info Revised as part of SA1 112 Ad Hoc-e |
| Orig PR | Subject to regulatory requirements, the 6G system shall be able to optimise the network energy saving considering the current traffic conditions of the network. | PR 5.8.4.6-1 | Provided for info Revised as part of SA1 112 Ad Hoc-e |
| CPR 14.1.4-1-6 | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G network shall provide suitable energy saving methods targeting different scenarios (e.g. different working time) and optimise the network energy saving considering the current traffic conditions of the network (subject to regulatory requirements). | PR 5.8.6.6-1 PR 5.8.4.6-1 | Network Energy Saving Revised as part of SA1 112 Ad Hoc-e |
| Alt CPR 14.1.4-1-6 (S1-254160) | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G network shall provide suitable energy saving methods targeting different scenarios (e.g. different working time) and/or the current traffic conditions of the network (subject to local regulation). | PR 5.8.6.6-1 PR 5.8.4.6-1 | Network Energy Saving Revised as part of SA1 112 Ad Hoc-e |
| CPR 14.1.4-1-7 | Subject to operator’s policy and regulatory requirements, the 6G network shall be able to expose to a trusted third-party the network energy consumption information including the energy consumption related to sensing, AI, and computing services, over a specific time period (e.g. month etc.). | PR 5.8.6.6-2 | Network Energy Consumption Revised as part of SA1 112 Ad Hoc-e |
| Alt CPR 14.1.4-1-7 (S1-254160) | Subject to operator’s policy and regulatory requirements, the 6G network shall be able to monitor and expose to a trusted third-party the network energy consumption information including the energy consumption related to sensing, AI, and computing services, over a specific time period (e.g. month etc.). | PR 5.8.6.6-2 PR 6.37.6-1 PR 6.37.6-2 | Network (Sensing, AI, computing) related Energy Consumption Revised as part of SA1 112 Ad Hoc-e |
| NEW CPR AA Move from AI (S1-254160) | Based on operator's policy and agreement with 3rd party, the 6G network shall support monitoring energy consumption for an AI service (e.g. inference) requested by 3rd party. | PR 6.37.6-1 | [ZTE] move from AI section |
| New CPR BB Move from AI (S1-254160) | Based on operator's policy and agreement with 3rd party, the 6G network shall support exposing energy consumption information of an AI service to 3rd party. | PR 6.37.6-2 | [ZTE] move from AI section |
| CPR 14.1.4-1-8 | Subject to operator’s policy, regulatory requirements and subscriber permission, the 6G system shall support a means for a group of cooperating UEs to reduce the energy consumption for communication of the group of UEs whilst meeting requested service performance. | PR 5.8.8.6-1 | Cooperating UEs Revised as part of SA1 112 Ad Hoc-e |
| CPR 14.1.4-1-9 | The 6G system should support energy-efficient sensing operations. | PR 7.5.6-2 | Energy Efficient Sensing |
| Orig PR | The 6G network shall be able to collect energy related data of the Service Hosting Environment. | PR 6.24.6-1 | Provided for info |
| New CPR CC Moved from 14.1.4-1-11 (S1-254160) | The 6G network shall be able to collect energy related information of the Service Hosting Environment, from energy management system, which is outside of 3GPP networks. | PR 6.24.6-1 | [ZTE] SHE EE, move from AI section According to the use case of 6.24, revise the “energy related data” to “energy related information”: |
| Alt to above CPR 14.1.4-1-x (InterDigital) | The 6G network shall be able to collect energy related information exposed by Service Hosting Environments and/or by the energy management system, which is outside of 3GPP networks. | PR 6.24.6-1 | [InterDigital – new CPR wording for clarity and completeness. Receiving data from outside 3GPP is not in scope for us unless it can be correlated for use ] |
| New CPR EE Moved from 14.1.4-1-13 (S1-254160) | The 6G network shall support the use of AI for the OAM of the 6G network, to assist with its energy efficiency and carbon emissions reduction. | PR 6.16.6-1 | [ZTE] AI in OAM for EE |
| Orig PR | The 6G network shall be able to consider energy related information from energy management system, which is outside of 3GPP networks, to support mechanisms in order to effectively use the energy, e.g. efficiently use the supplied energy from the energy supplier. | PR 6.24.6-3 | Provided for info |
| New CPR FF Moved from 14.1.4-1-14 (S1-254160) | The 6G network shall be able to support mechanisms to effectively use the supplied energy from the energy supplier, considering energy related information from energy management system, which is outside of 3GPP networks. | PR 6.24.6-3 | [ZTE]: not sure what is the exact impact on 6G network According to the use case of 6.24, revise it as: |
| New CPR GG (S1-254160) | The 6G network shall be able to consider energy related information from energy management system, which is outside of 3GPP networks, to support mechanisms in order to effectively use the energy, e.g. efficiently use the supplied energy from the energy supplier. | PR 6.24.6-3 |  |
| New CPR HH | The 6G network shall be able to support means to provide the network operator an indication of the energy consumed by the available access networks for the provision of a one-to-many service. NOTE 1: The network operator may take into account this energy consumption information when deciding how to deliver the service. NOTE 2: The energy consumption information relates to the access network’s energy needed (taking into account the energy already consumed by the network over its life duration) for the delivery of a given one-to-many service. | PR 5.8.2.6-2 | EN cleared so added to the Table. Energy consumption of access networks |
| New CPR ## | Subject to operator’s policy, the 6G network shall provide means to consider energy consumption information and carbon equivalent emissions when configuring 3GPP services (e.g. sensing, AI, computing etc.) targeting different scenarios. | PR 5.8.6.6-3 | Not included in previous table |
| Alt to above NEW CPR ## (InterDigital) | Subject to operator’s policy, the 6G network shall provide means to consider energy consumption information and carbon equivalent emissions of energy sources when configuring 3GPP services (e.g. sensing, AI, computing etc.) targeting different scenarios. | PR 5.8.6.6-3 | [InterDigital: alternative adding clarification of emissions] |

* * * End of Changes * * *