3GPP TSG SA WG 1 Meeting #113 	S1-261145

9-13 February 2026, Goa, India	(revision of S1-261044)

Source:	6G Study Rapporteurs

pCR Title:	Pseudo-CR on Table 14.1.13-1 Massive Communication

Draft Spec:	3GPP TR 22.870 v 1.1.0

Agenda item:	8.1.7

Document for:	Approval

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Comments

This Table is the outcome of SA1 #112 that was captured in S1-254297. This table was not discussed during SA1 #112 but addressed the comments from SA1 #112 from companies’ emails and draft on wording proposals.

This pCR proposes to update Table 14.1.13-1 – Massive Communication in TR 22.870 with CPRs for inclusion into the draft TR.

* * * First Change (All New CPRs)  * * * *

Table 14.1.13-1 – Massive Communication

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.13-1-1 | The 6G system shall support UEs with both wide-area (i.e. rural area and deep indoor) coverage capabilities and broadband capabilities. NOTE: The wide-area coverage performance target will need to be confirmed by RAN working groups. | PR 10.2.6-1 | ZTE: Is it related KPI? |
| CPR 14.1.13-1-2 | The 6G system shall provide emergency services (e.g. emergency calls) support from rural area and deep indoor coverage scenarios, including leveraging wide-area coverage capabilities. | PR 10.2.6-2 |  |
| ZTE | The 6G system shall provide emergency services (e.g. emergency calls) from rural area to deep indoor coverage scenarios. | PR 10.2.6-2 |  |
| CPR 14.1.13-1-3 | The 6G system shall support MO and MT basic services with low-capacity demands (e.g. messaging, SMS, and small data). | PR 10.2.6-3 | ZTE: What is the low-capacity? Is it KPI related? |
| CPR 14.1.13-1-4 | The 6G system shall support low-complexity devices in massive communication scenarios. NOTE: Low-complexity devices are needed for massive deployment into next-generation smart meters. This could include low-power capabilities and limitations in the amount of antennas. | PR 10.3.6-2 |  |
| CPR 14.1.13-1-5 | Subject to operator’s policy, the 6G system shall enable an authorized 3rd party to set and configure parameters e.g., if the application depends on MO or MT traffic, small or high-data volumes. | PR 10.3.6-3 |  |