3GPP TSG SA WG 1 Meeting #113 	S1-261245

9-13 February 2026, Goa, India	(revision of S1-261202, 1040)

Source:	6G Study Rapporteurs

pCR Title:	Pseudo-CR on Table 14.1.8-4 IMS based AI services / AI related IMS services

Draft Spec:	3GPP TR 22.870 v 1.1.0

Agenda item:	8.1.3

Document for:	Approval

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Comments

This Table is the outcome of SA1 #112 that was captured in S1-254490. This table was not discussed during SA1 #112 but addressed the comments from SA1 #112 from companies’ emails and draft on wording proposals.

This pCR proposes to update Table 14.1.8-4 (IMS based AI services / AI related IMS services) in TR 22.870 with CPRs for inclusion into the draft TR.

Changes:

Clean up the green ones including deleting the red alternatives when there’s already green ones for a CPR, clean up the revision marks, delete the comments in the comment column.

* * * First Change (All new CPRs) * * * *

Table 14.1.8-4 – IMS based AI services / AI related IMS services/ intelligent MultiMedia TELephony (iMMTel)

This clause captures some requirements related to Intelligent Communication Assistant and the provided services can include Intelligent Calling Service…

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.8-4-1 | Subject to operator’s policy, regulatory requirements and subscriber permission, 6G network in conjunction with IMS shall be able to provide Intelligent Communication Assistant to users/subscribers via UE. The IMS shall be able to provide NOTE: Intelligent Communication Assistant: The virtual intelligent communication assistant locates in operator network and interacts with the users through voice, video, text, gestures or other modalities which may need to impact IMS. The assistant also may impact 6G network as it can be customized for each particular user by accessing user data and network data which are stored or collected in the network. It can ,and it can provide various communication services (e.g. 6G AI Services, 6G Computing Services) and support individual users based on user’s intention and requirement utilizing AI capability. One subscriber can have one or more Intelligent Communication Assistants. Proposal to separate in two requirements: | PR 6.11.6-1 PR 6.17.6-1 PR 6.11.6-7 | Proposed merged CPR on intelligent communication assistant service, general |
| CPR 14.1.8-4-2 | Subject to operator’s policy, regulatory requirements and subscriber permission, 6G network in conjunction with IMS shall be able to support the interaction, e.g. communication between different intelligent communication assistants, e.g. during an IMS session, where both calling and callee parties are using Intelligent Communication Assistant(s). | PR 6.11.6-3 | intelligent communication assistant service, interaction |
| CPR 14.1.8-4-3 | Subject to operator’s policy, regulatory requirements and subscriber permission, 6G network in conjunction with the IMS shall be able to support the interaction, e.g. communication between operator’s intelligent communication assistant and authorized third-party assistant during an IMS session between users utilizing the Intelligent Communication Assistant. NOTE 1: It is not expected that Intelligent Communication Assistants directly communicate with each other via IMS services (e.g. using audio/video media). | PR 6.11.6-4 | intelligent communication assistant service, interaction |
| CPR 14.1.8-4-4 | Subject to operator’s policy and subscriber permission, the 6G network in conjunction with the IMS shall be able to support the intelligent communication assistant to invoke operator’s network capabilities and 3GPP service (e.g. , SMS or voice, trigger QoS adjustment, Sensing) to meet user service requirements dynamically during an IMS session. | PR 6.11.6-5 | intelligent communication assistant service, invoke 3GPP service |
| CPR 14.1.8-4-5 | Subject to regulatory requirements, operator’s policy , the 6G network in conjunction with the IMS shall be able to support invoking authorized third-party capabilities (e.g. weather inquiry, language translation, takeout services and/or obtain various information). | PR 6.11.6-6 | intelligent communication assistant service, invoke 3rd party service |
| CPR 14.1.8-4-6 | Subject to operator’s policy and regulatory requirements, the 6G system shall provide secure exchange of personal data with the Intelligent communication assistant inside the network or (third party) service hosting environment. | PR 6.17.6-2 | AI assistant, data security Nokia: suggest to merge with QC: CPR 4.1.8-4-3 (implied by regulatory requirements) Huawei: Ok with the CPR provided RAN is excluded from SHE. We indeed prefer to keep the 2 different essence of this wording, e.g. Collecting subscriber permission, and invoking caps are different aspects. |
| CPR 14.1.8-4-7 | Subject to operator’s policy and subscriber permission, the 6G system in conjunction with IMS shall provide mechanisms for registration and discovery of AI-based services, e.g. Intelligent Communication Assistant provided by authorized 3rd party service providers. | PR 6.17.6-3 | registration and discovery |
| CPR 14.1.8-4-8 | Subject to operator’s policy and subscriber permission , 6G network in conjunction with IMS shall enable IMS services to provide intelligent calling service to users in two parties or multi-party call, when the user is unavailable or unwilling to take the calls, e.g. provide Intelligent Communication Assistant with usage of AI capability in case of user’s phone is in flight mode, powered-off or during busy time. Note: Intelligent answering service is an intelligent call-answering service provided by operators to users, based on the authorization from the user for user data. | PR 6.22.6-1 | intelligent calling service, general |
| CPR 14.1.8-4-9 | Subject to regulatory requirements, operator’s policy and subscriber permission, the 6G network in conjunction with IMS shall be able to support customizing intelligent calling , e.g. the call-answering tone, speech rate customization based on user’s voice, the response can be customized based on different caller. | PR 6.22.6-3 | intelligent calling service, customization Nokia:; consider merge with CPR 14.1.8-4-8 |
| CPR 14.1.8-4-10 | Subject to operator’s policy and subscriber permission , the 6G network in conjunction with IMS shall be able to provide the user with information related to the call as part of the intelligent calling ( e.g. send the conversation record or summary to users by SMS or voice mail after the call has ended). | PR 6.22.6-4 | intelligent calling service Nokia:; consider merge with CPR 14.1.8-4-8 |
| CPR 14.1.8-4-11 | Subject to operator’s policy,regulatory requirements and subscriber permission, when an intelligent calling is used on behalf of a subscriber, the 6G network in conjunction with IMS shall be able to identify and associate the specific intelligent calling being used and the subscriber on whose behalf the service is being used. | PR 6.22.6-5 | intelligent calling service Nokia: any link with supplementary services? |
| CPR 14.1.8-4-12 | Subject to operator’s policy and subscriber permission , the 6G system in conjunction with IMS should supportmechanisms (e.g. AI mechanisms) to enhance the audio and video stream for disability support. NOTE 1: The disability support can be e.g. describing and explaining the surroundings, audio aid (modification of the audio stream e.g., enhancements, clarifications, translations, speech to text, text to speech, etc), visual aid (modification of the visual stream). | PR 6.38.6-1 | IMS enhance |
| CPR 14.1.8-4-13 | Subject to operator’s policy, subscriber permission and regulatory requirements, the 6G system in conjunction with IMS may support mechanisms (e.g. AI mechanisms) to enhance the IMS emergency communication service with enhanced audio and video stream for disability support. | PR 6.38.6-2 | IMS enhance |
| CPR 14.1.8-4-14 | Subject to operator’s policy and subscriber permission , the 6G system in conjunction with IMS shall support media transformations in multi-modal communications, e.g. image to video, 2D video to 3D video/avatar media, text to video and vice-versa. NOTE 1: The media transformations could be provided via AI capabilities of 3rd party or 6G network (includingthe IMS. NOTE 2: For multi-modal communication service, refer to [14]. | PR 6.42.6-1 | media transformations |
| CPR 14.1.8-4-15 | Subject to operator policy, regulatory requirements and subscriber permission, the 6G network in conjunction with IMS shall be able to support to switch an ongoing session between the actual user and the intelligent Communication Assistant acting on behalf of that user and vice versa. | PR 6.22.6-6 | Switch ongoing service Nokia: Suggest to move after CPR #11 Any link with intelligent Communication Assistant? Any need for subscriber permission? |