3GPP TSG-SA2 Meeting #172	S2-2509975

17 - 21 November, 2025, Dallas, USA                                   (was )

Source:	Nokia, Qualcomm Incorporated

Title:	KI#2: proposed agreement updates of principles

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract: This document proposes to agree additional principles for KI#2

# 1.	Introduction

In the scope of KI#2, in the last meeting there was lesser amount of time spent on discussing aspects related to NG-RAN related aspects.

In reference to solution 4 and 7, the approach is to have SM PCF generated policies by having energy aspects into consideration. One solution proposes to use AQP which is limited to GBR, and other solution proposes to use Energy Saving QoS profiles which covers both GBR and non-GBR aspects.

The advantage of these approaches is that there is no need of a centralized decision and each gNB can trigger service adjustment based on operator defined policy for energy that apply to the UE based on gNB logic. SMF and then CHF should receive indication of change of QoS when it happens.

If AQP concept is used for GBR, then with different values considering energy aspects, would not really indicate to the gNB that these AQPs are defined due to energy aspects and additional indication that specific AQPs are generated based on energy inputs would be required.

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-67.

# * * * * First change * * * *

### 7.1.2	Agreed Principles for KI#2

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#2. This may be populated directly or e.g. also when a topic in clause 7.2.2 gets resolved and a principle is agreed.

The interim agreements on principles for KI#2 are as follows:

-	The PCF may subscribe to notifications of energy related information from the EIF and OAM.

NOTE 1:	Details of the notification reporting control (e.g. how to prevent too frequent notifications) will be decided during normative phase.

NOTE 2:	Renewable energy information can be considered based on SA5 feedback on supporting renewable energy.

-	The PCF may make the following policy control decisions for network energy saving based on subscription information, AF request (e.g. preferred or expected network energy saving behaviours such as QoS adjustment) and/or operator policy, and/or taking into account energy related information from the EIF/OAM:

-	SM policy control.

NOTE 3:	Based on operator policy, it should have the balance between energy saving and user experience.

-	PDTQ policy negotiation with the AF, e.g. adjusting the time window and/or QoS for data transfer.

-	Optionally UE policy control, i.e. generating URSP rules with the existing parameters based on energy related information from the OAM.

-	The AF may subscribe to be notified for the result of energy saving decisions made by PCF along with the request for preferred or expected energy saving behaviours.

-	The PCF may notify the SMF when making policy change decisions due to network energy saving. The SMF, in turn, may provide the reason for the service adjustment to the CHF.

Editor's note:	Whether to provide the reason (i.e. network energy saving) and related information for the service adjustment to the CHF depends on the feedback from SA WG5.

NOTE 4:	Existing AF services can be updated but no new AF services will be introduced for the AF request.

-	Energy Saving QoS - profile (GBR) / parameters (non-GBR) for energy saving operation handling can be provided to the NG-RAN by the SMF to cause service adjustment based on information locally available at the NG-RAN. These are generated by the PCF and provided to the SMF. The PCF can take AF input for the QoS parameters (non-GBR) and profiles (GBR) with energy saving service requirements. NG-RAN should provide indication of the applied QoS to the SMF.

Editor's note: 	Based on the RAN WG inputs, normative work would use Energy Saving QoS profiles for both GBR and non-GBR.

# * * * * End of change * * * *