SA WG2 Meeting #172	S2-2510005

17 – 21 November, 2025, Dallas, US

Source:	Apple

Title:	KI#2 Conclusion

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: KI#2 conclusion proposal based on agreed interim agreements.

# 1.	Discussion

This contribution proposes KI#2 conclusion proposal based on agreed interim agreements agreed at SA2#171.

Changes are highlighted in this section for clause 7 in track changes mode and proposed as all new text for clause 8 afterwards to avoid unnecessary discussion on two different clauses with the same contents.

### 7.1.2	Key Issue #2: Authorization and Revocation to Support Sensing Service

The following principles are agreed for KI#2:

a)	The Sensing Service authorization consists of two steps:

1)	Authorisation of the AF for Sensing Service request is performed by the NEF, if the AF is outside the trusted domain, as defined in TS 33.501 [9];

2)	Authorisation of the AF's Sensing Service Request performed by the Sensing control functionality (SCF);

b)	Sensing authorization determines whether the Sensing Service Request from the AF is authorised, considering the example of Sensing authorisation information described in Table 7.1.2-1. Additional parameters, if any and further details on Sensing authorization information can be determined during the normative phase.

Table 7.1.2-1: Sensing Authorization information for Sensing Service

| AF Authorization Data | Description |
| --- | --- |
| AF ID | Identifier used to identify the AF. |
| Allowed/Not allowed area for sensing | Indicate the allowed/not allowed area for the indicated AF to trigger the sensing services operations (NOTE 1). |
| Allowed/Not allowed time period for sensing | Indicate the allowed time period within which AF can trigger a particular sensing service operation. |
| (Allowed) sensing service type | Indicate the allowed sensing service type (e.g. object tracking, detection) for the indicated AF to trigger the sensing services operations. |
| NOTE 1: The Allowed area and the Not allowed area may be both present or only one of them can be present, for example if the AF is allowed to make a request for the majority of PLMN coverage excluding the not allowed area only the not allowed area can be present. If both are not present all area is allowed. |  |

c)	The Sensing Authorization information is pre-configured or configured by OAM and stored in SCF for authorisation of the sensing service request.

d)	SCF can revoke the Sensing Service Request when the authorisation criteria are no longer met.

e)	The AF can cancel the on-going Sensing Service Request when needed.

f)	There is no need for authorisation for the gNB as the Sensing Entity, as the gNB is deployed by operator.

# 2.	Proposal

It is proposed to agree the following changes in TR 23.700-14.

**** First Change (ALL NEW TEXT) ****

## 8.X	Key Issue #2: Authorization and Revocation to Support Sensing Service

The following principles are agreed for KI#2:

a)	The Sensing Service authorization consists of two steps:

1)	Authorisation of the AF for Sensing Service request is performed by the NEF, if the AF is outside the trusted domain, as defined in TS 33.501 [9];

2)	Authorisation of the AF's Sensing Service Request performed by the Sensing control functionality (SCF);

b)	Sensing authorization determines whether the Sensing Service Request from the AF is authorised, considering the example of Sensing authorisation information described in Table 7.1.2-1. Additional parameters, if any and further details on Sensing authorization information can be determined during the normative phase.

Table 7.1.2-1: Sensing Authorization information for Sensing Service

| AF Authorization Data | Description |
| --- | --- |
| AF ID | Identifier used to identify the AF. |
| Allowed/Not allowed area for sensing | Indicate the allowed/not allowed area for the indicated AF to trigger the sensing services operations (NOTE 1). |
| Allowed/Not allowed time period for sensing | Indicate the allowed time period within which AF can trigger a particular sensing service operation. |
| (Allowed) sensing service type | Indicate the allowed sensing service type (e.g. object tracking, detection) for the indicated AF to trigger the sensing services operations. |
| NOTE 1: The Allowed area and the Not allowed area may be both present or only one of them can be present, for example if the AF is allowed to make a request for the majority of PLMN coverage excluding the not allowed area only the not allowed area can be present. If both are not present all area is allowed. |  |

c)	The Sensing Authorization information is pre-configured or configured by OAM and stored in SCF for authorisation of the sensing service request.

d)	SCF can revoke the Sensing Service Request when the authorisation criteria are no longer met.

e)	The AF can cancel the on-going Sensing Service Request when needed.

f)	There is no need for authorisation for the gNB as the Sensing Entity, as the gNB is deployed by operator.

**** End of Changes ****