3GPP TSG-WG SA2 Meeting #172	S2-2510037

Dallas, USA, Oct 17 - 21, 2025

Source:	China Mobile

Title:	Update on Interim Agreements for KI#3

Document for:	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC/Rel-20

Abstract of the contribution: This paper proposes an update on Interim agreements for KI#3 for the FS_Sensing_ARC TR 23.700-14.

1	Discussion

Due to the mobility of the sensing objects, the SF/SE selection/re-selection procedure may be triggered based on the sensing results.

From the perspective of sensing network deployment and maintenance, we did not perceive the necessity to introduce an additional sensing gateway function. Therefore, the NEF is responsible for discovering the Sensing Function.

2	Proposal

It is proposed to include the following changes in TR 23.700-14 V1.x.0.

* * * * Start of Changes * * * *

# 7	Interim agreements

## 7.1	Agreed Principles

### 7.1.1	Agreed Principles for KI#1: System Architecture to Support Sensing

To support KI#1 System Architecture to Support Sensing, the following principles are concluded:

Principle 1: one new Network Function (i.e. Sensing Function, SF) is defined to support Sensing Service. the SF may contain Sensing control functionality (SCF) and Sensing Processing functionality (SPF).

NOTE 1:	There is no standardized interface between the SCF and SPF in Rel-20 5G-A.

NOTE 2:	How to capture the deployment options of SPF(s) functionality and SCF functionality of the same SF, e.g. co-located or separated, can be discussed during the normative phase.

NOTE 3:	The final name and Acronym for Sensing Function may need further update if necessary.

Principle 2: NO dedicated storage NF is needed to store the Sensing data and Sensing result.

Principle 3: In this study, the gNB is the only entity that acts as the Sensing Entity (SE).

NOTE  4:	Privacy protection and other security aspects will be coordinated with SA WG3 and the related impact to architecture enhancement will be based on SA WG3 during the normative phase.

NOTE 5:	Other KI's interim conclusions will be aligned with KI#1 conclusions.

### 7.1.2	Agreed Principles for KI#2: Authorization and Revocation to Support Sensing Service

The following principles are agreed for KI#2:

a)	The Sensing Service authorization consists of two steps:

1)	Authorisation of the AF for Sensing Service request is performed by the NEF, if the AF is outside the trusted domain, as defined in TS 33.501 [9];

2)	Authorisation of the AF's Sensing Service Request performed by the Sensing authorization functionality;

NOTE:	In this clause, Sensing authorization functionality may refer to Sensing Function, Sensing Control Function or Sensing Gateway and the wording may need alignment according to the conclusion of key issue 1.

b)	Sensing authorization functionality determines whether the Sensing Service Request from the AF is authorised, considering the example of Sensing authorisation information described in Table 7.1.2-1. Additional parameters, if any and further details on Sensing authorization information can be determined during the normative phase.

Table 7.1.2-1: Sensing Authorization information for Sensing Service

| AF Authorization Data | Description |
| --- | --- |
| AF ID | Identifier used to identify the AF. |
| Allowed/Not allowed area for sensing | Indicate the allowed/not allowed area for the indicated AF to trigger the sensing services operations (NOTE 1). |
| Allowed/Not allowed time period for sensing | Indicate the allowed time period within which AF can trigger a particular sensing service operation. |
| (Allowed) sensing service type | Indicate the allowed sensing service type (e.g. object tracking, detection) for the indicated AF to trigger the sensing services operations. |
| NOTE 1: The Allowed area and the Not allowed area may be both present or only one of them can be present, for example if the AF is allowed to make a request for the majority of PLMN coverage excluding the not allowed area only the not allowed area can be present. If both are not present all area is allowed. |  |

c)	The Sensing Authorization information is pre-configured or configured by OAM and stored in Sensing authorization functionality for authorisation of the sensing service request.

d)	Sensing authorization functionality can revoke the Sensing Service Request when the authorisation criteria are no longer met.

e)	The AF can cancel the on-going Sensing Service Request when needed.

f)	There is no need for authorisation for the gNB as the Sensing Entity, as the gNB is deployed by operator.

### 7.1.3	Agreed Principles for KI#3: Sensing Entity and Sensing Function Discovery and (Re-)Selection

The following principles are agreed in this release to address the key issue on Sensing Entity and Sensing Function Discovery and (Re-)Selection:

-	The Sensing Function selection is realized by the NEF.

-	The Sensing Function registers itself to the NRF, with its NF profile.

-	Sensing Function is selected based on at least its supported sensing area.

-	Sensing Function selects one or more Sensing Entities, i.e. gNBs, based on the sensing service request.

-	Sensing Function selects the Sensing Entities, i.e. gNBs, based on information about the Sensing Entities, e.g. its supported sensing area.

-  Due to the mobility of the sensing objects, the SF/SE selection/re-selection procedure may be triggered based on the sensing results.

NOTE:	The architecture aspect of this KI's interim conclusions will be aligned with KI#1 conclusions.

### 7.1.4	Agreed Principles for KI#4: Sensing Data and the Associated Information Collection and Transport

The following principles are agreed for KI#4:

-	Based on the received sensing service requirements from the Sensing Function, the Sensing Entity (gNB) can determine how to perform the relevant sensing measurement to collect the Sensing data.

-	Sensing Entity can support one time, periodical Sensing Data to the Sensing Function.

Editor's note:	The type (one time, periodical, etc.) and content of Sensing Data that will be provided by gNB needs to be coordinated with RAN WGs in the normative phase.

-	Sensing data can be collected from one or multiple gNBs by Sensing Function for the same sensing service request.

-	The Sensing Function can generate sensing results based on the information provided by the Sensing Entity (gNB) node(s).

NOTE:	The architecture aspect of this KI's interim conclusions will be aligned with KI#1 conclusions.

* * * * End of Changes * * * *