SA WG2 Meeting #S2-172	S2-2510068

17 – 21 November, 2025, Dallas, USA

Source: 	Interdigital

Title: 	Proposal for KI#1 conclusion

Document for: 	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: proposal for KI#1 conclusion.

# 1	Introduction

This contribution provides the update on interim agreements and conclusion on KI#1 on System Architecture to Support Sensing.

# 2 	Proposal

It is proposed to adopt the following changes into TR 23.700-14 V1.1.0.

# * * * * First change* * * *

# 7	Interim agreements

## 7.1	Agreed Principles

### 7.1.1	Agreed Principles for KI#1: System Architecture to Support Sensing

To support KI#1 System Architecture to Support Sensing, the following principles are concluded:

Principle 1: one new Network Function (i.e. Sensing Function, SF) is defined to support Sensing Service. the SF may contain Sensing control functionality (SCF) and Sensing Processing functionality (SPF).

NOTE 1:	There is no standardized interface between the SCF and SPF in Rel-20 5G-A.

NOTE 2:	How to capture the deployment options of SPF(s) functionality and SCF functionality of the same SF, e.g. co-located or separated, can be discussed during the normative phase.

NOTE 3:	The final name and Acronym for Sensing Function may need further update if necessary.

Principle 2: NO dedicated storage NF is needed to store the Sensing data and Sensing result.

Principle 3: In this study, the gNB is the only entity that acts as the Sensing Entity (SE).

NOTE  4:	Privacy protection and other security aspects will be coordinated with SA WG3 and the related impact to architecture enhancement will be based on SA WG3 during the normative phase.

NOTE 5:	Other KI's interim conclusions will be aligned with KI#1 conclusions.

Principle 4: For each SF, a sensing service area (SSA) is defined at which a sensing NF may manage sensing operation.  A sensing service area is at tracking area level.

Principle 5: A SF may be dedicated for authorization and result exposure for sensing service consumer. If a dedicated SF is deployed, the sensing service consumer communicates with the dedicated SF for service request and service response (via NEF, if needed.). The dedicated SF may be called as a Gateway Sensing Function.

Principle 6: A SF sets up a signalling connection with SE(s) via an AMF or directly for request and control of sensing operations of SE(s).

Principle 7: A SF obtains sensing data from SE(s) over a direct connection.

NOTE 6:	The details of signalling connection and data connection are decided by RAN.

# * * * * Second change * * * *

# 8	Conclusions

Editor's note:	This clause will capture conclusions for the study.

Where there is consensus, interim agreements (e.g. solution principles descriptions) should be documented in the TR as soon as possible during a study.

These can be documented in the TR as "7.1.Y Agreed Principles for KI#Y" in the "Interim Agreements" clause. If the interim agreement has impacts on another clause in the TR and if there is consensus, that TR clause can be updated.

By consensus interim agreements can become part of the final conclusions of the study.

The Overall Evaluation clause previously used in TR skeletons should not be used.

There should be a Topics for further consideration clause per Key Issue. It is recommended that this is used e.g. to capture common issues that need to be resolved for multiple solutions.

## 8.1 Conclusion on KI#1

Principle 1: one new Network Function (i.e. Sensing Function, SF) is defined to support Sensing Service. the SF may contain Sensing control functionality (SCF) and Sensing Processing functionality (SPF).

NOTE 1:	There is no standardized interface between the SCF and SPF in Rel-20 5G-A.

NOTE 2:	How to capture the deployment options of SPF(s) functionality and SCF functionality of the same SF, e.g. co-located or separated, can be discussed during the normative phase.

NOTE 3:	The final name and Acronym for Sensing Function may need further update if necessary.

Principle 2: NO dedicated storage NF is needed to store the Sensing data and Sensing result.

Principle 3: In this study, the gNB is the only entity that acts as the Sensing Entity (SE).

NOTE  4:	Privacy protection and other security aspects will be coordinated with SA WG3 and the related impact to architecture enhancement will be based on SA WG3 during the normative phase.

NOTE 5:	Other KI's interim conclusions will be aligned with KI#1 conclusions.

Principle 4: For each SF, a sensing service area (SSA) is defined at which a sensing NF may manage sensing operation.  A sensing service area is at tracking area level.

Principle 5: A SF may be dedicated for authorization and result exposure for sensing service consumer. If a dedicated SF is deployed, the sensing service consumer communicates with the dedicated SF for service request and service response (via NEF, if needed.). The dedicated SF may be called as a Gateway Sensing Function.

Principle 6: A SF sets up a signalling connection with SE(s) via an AMF or directly for requests and controls of sensing operations of SE(s).

Principle 7: A SF obtains sensing data from SE(s) over a direct connection.

NOTE 6:	The details of signalling connection and data connection are decided by RAN.

# * * * * End of change* * * *