3GPP TSG SA2 Meeting #172	S2-2510204

17 - 21 November 2025, Dallas, USA

Source:	Ericsson

Title:	KI#2 - Interim agreements

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2/Rel-20

Abstract of the contribution: This contribution proposes some updates to the interim agreements.

# 1. Discussion

This PCR discusses about the topic that are for further consideration for KI#2 then proposes some updates to the interim conclusions for KI#2.

The following topics are for further consideration for KI#2 in TR 23.700-04 v.1.1.0:

## 1.1	 The need and details of type/identification of traffic pattern is FFS

Only solution#26 adds an Editor´s Note on the need to include a traffic pattern when reporting pre-processed data from the UPF to NWDAF: “NOTE 2:	Contents of the pre-processed data (e.g. if the pre-processed data contain a traffic pattern ID) needs to be determined in the evaluation phase.”

Solution#26 includes the possibility that The UPF performs the local data processing and reports the locally pre-processed data to the NWDAF based on the received instructions.

Observation: UPF performing local data processing reduces the amount data and signalling that UPF needs to expose for training and inference at NWDAF, as such it is an efficient solution,

On the other hand, the UPF reports data to NWDAF using Nupf_EventExposure_Notify that contains already a list of traffic identifiers, as such if UPF reports to NWDAF e.g. that abnormal traffic is detected, the identification of that traffic may be Application ID, traffic filtering information, DNN and S-NSSAI, and corresponds to a PDR rules that is as such the need

Observation: The existing list identifiers that the UPF can use to refer to the traffic characteristics that are reported covers the reporting of pre-processed data, adding a new identifier such as traffic pattern id is not justified.

Proposal 1: Remove the open issue about the need of new identifiers.

## 1.2	Whether new UPF or SMF events are needed or not for NWDAF data collection

There is a very high level description of the UPF or SMF event that are required by NWDAF, and in particular what the traffic characteristics refer to:

-	Information to identify the traffic pattern that can contain:

-	Source of traffic (e.g. IP packet filter(s), IP Protocol (TCP, UDP, etc.), application ID, etc.)

-	traffic flow filters

-	identification of corresponding UEs

- 	Information of traffic characteristics.

It seems to be common to solution#23, 16, 31 and 34 that the traffic characteristics include both data volumes and data rates for a given application, for the NWDAF to determine if the traffic is abnormal or not and related statistical measurements that can be provided by the UserDataUsageTrends and UserDataUsageMeasurements.

Proposal 2: When possible, reusing the existing UPF or SMF events are preferred. If there is UPF EventID, i.e. UserDataUsageTrends and UserDataUsageMeasurements that already reports traffic characteristics such as volumes or bit rates those shall be reused.

## 1.3	How to reduce the reporting load of input data sources and control plane signalling overhead?

For example, by configuring appropriate Analytics Filter Information, combining notification to NWDAF at UPF for multiple events, instructions from NWDAF to UPF for pre-processing in the UPF, etc.

There are some actions to reduce the reporting load of input data source that are already standardized, see 23.288 and TS 29.564:

- The NWDAF provides a threshold value as part of the Event Reporting Instructions to the UPF. This indicates to the UPF that reporting of input data is performed when the threshold for the user plane traffic parameters such as data volume, number of malformed packets or number of retransmitted packets are reached. This indicates to the UPF that reporting of input data is performed when the threshold for the user plane traffic parameters such as data volume, number of malformed packets or number of retransmitted packets are reached.

- The NWDAF provides pre-processing instructions such as sampling of UEs or partitioning of UEs based on e.g. UE location, as part of the Event Reporting Instructions to the UPF.

- The UPF may bundle event reports of multiple subscriptions in a same Notify request, see TS 29.564 [x].

Proposal 3: To reduce reporting load, the existing Event Reporting Instructions can be used to provide a threshold to UPF, other pre-processing instructions can be requested by NWDAF at UPF, to aggregate or to provide statistics on data measurements with different level of granularity such as per PDU Session. Other actions defined in 29.564 applies.

23.501 5.8.2.17 reporting suggestion..

## 1.4 	Consumer actions are FFS

The mitigation actions that the consumer can take using the new Analytics ID are standardized, no new actions are proposed in the solutions. The existing list of actions are all examples, the actions depend on the operator local policies, as such this topic can be removed.

Proposal 4: Do not define more consumer actions, some examples are already provided. Define consumer actions per local operator policies.

## 1.5	Whether mitigation actions applicable to any UE can be provisioned by the SMF into the UPF on N4 level.

Interim conclusions state the UPF has operator-configured policies corresponding to the types of abnormal traffic, that can be activated for the received traffic descriptor. This does not apply on PDU-session level. This is applicable when UPF is the consumer of analytics. However, in case that SMF is the consumer of analytics, there is a need to provide similar level of functionality, i.e. to provision mitigation actions to UPF that are applicable on node level.

Proposal 5: Mitigation actions can be provisioned by the SMF into the UPF both on N4 level, a new PFCP per Node procedure as well as per PDU session level.

# 2. Text Proposal

It is proposed to capture the following proposal into TR 23.700-04:

Proposal 1: Remove the open issue about the need of new identifiers, such as traffic pattern ID.

Proposal 2: When possible, reusing the existing UPF or SMF events are preferred. If there is UPF EventID, i.e. UserDataUsageTrends and UserDataUsageMeasurements that already reports traffic characteristics such as volumes or bit rates those shall be reused. Alternatively, keep it open until the list of traffic characteristics is agreed.

Proposal 3: To reduce reporting load, the existing Event Reporting Instructions can be used to provide a threshold to UPF, other pre-processing instructions can be requested by NWDAF at UPF, to aggregate or to provide statistics on data measurements with different level of granularity such as per PDU Session. Other actions defined in 29.564 applies.

Proposal 4: Do not define more consumer actions, some examples are already provided. Define consumer actions per local operator policies.

Proposal 5: Mitigation actions can be provisioned by the SMF into the UPF both on N4 level, a new PFCP per Node procedure as well as per PDU session level.

*** Start of changes ***

### 7.1.2	Agreed Principles for KI#2

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#2. This may be populated directly or e.g. also when a topic in clause 7.2.2 gets resolved and a principle is agreed.

#### 7.1.2.1	General

No impact on RAN and UE operation.

To reduce the reporting load of input data sources (e.g. UPF), the NWDAF may provide Event Reporting Instruction as defined in TS 23.502. In particular, the NWDAF can provide sampling of UE or instructions on how to do partitioning of UEs, or pre-processing instructions to request UPF to generate statistical information per PDU Session.. Other actions that can be taken to reduce load when reporting input data to NWDAF are support the provisioning of the skip reporting instruction information in the subscription request, then the UPF do not send the event reports if they meet the skip reporting instruction, or UPF may bundle event reports of multiple subscriptions in a same Notify request, see TS 29.564 [x].

#### 7.1.2.2	Agreed Principles for Use Case #1

-	The service consumer of the analytics service may be SMF, PCF, OAM, UPF, and AF. The consumer may take the output Analytics into account.

-	To provide analytics to the consumers, the NWDAF may provide the output analytics, including the statistics and predictions of:

-	Type of abnormal traffic, e.g. abnormal traffic due to DDoS, abnormal data packets patterns, unexpected traffic volume or burst.

-	volume, rate or burst size of abnormal traffic,

-	Identifiers/addresses of affected UPF(s), UE(s), PDU session(s),

-	Identifiers/ information of the source, e.g. IP packet filter(s), IP Protocol (TCP, UDP, etc.), application ID, etc.

-	To derive the output analytics, the following input data may be collected from the UPF or SMF, to support the NWDAF-based analytics:

-	From UPF/SMF

-	Information to identify the traffic pattern that can contain:

-	Source of traffic (e.g. IP packet filter(s), IP Protocol (TCP, UDP, etc.), application ID, etc.)

-	traffic flow filters

-	identification of corresponding UEs

- 	Information of traffic characteristics, including measured UL/DL data volumes, measured UL/DL data rates, and statistics.

-	The SMF as consumer of NWDAF analytics may determine the mitigation actions based on local operator policies for example

-	UPF reselection to distribute the load across UPF instances (as defined in clause 6.3.3 of TS 23.501)

-	Configuring UPF to block or shape abnormal traffic, to enforce bandwidth limitations

- The mitigation actions per observed traffic anomaly and associated traffic identification reported by NWDAF to SMF are provisioned into SMF. Then, the SMF provides them to the UPF, as stated in NOTE 1, these are not actions on PDU session level, as such a new PFCP per Node procedure is defined.

-	UPF as consumer of NWDAF analytics may for instance take the following actions:

-	Downlink traffic suppression (e.g., selective packet dropping or rate limitations)

-	adjust packet processing resources,

NOTE 1:	UPF will only receive Analytics with reduced Output (i.e. type of abnormal traffic and traffic descriptor). UPF has operator-configured mitigation actions corresponding to the type of observed traffic anomalies reported by NWDAF and associated information for traffic identification . This does not apply on PDU-session level.

-	The PCF as consumer of NWDAF analytics determines the mitigation actions based on local operator policies for example::

- 	Policies creation or update and provisioning to SMF, e.g.  traffic gating or shaping, bandwidth parameters (e.g. rate limiting to 5Mbps) or adjusting QoS parameters.

-    	As stated in the architecture assumptions the UPF can detect abnormal traffic. When the abnormal traffic is detected the UPF can take actions to improve user plane performance using existing PDRs, and associated N4 rules such as QER or FAR, this is independent on the use of analytics.

NOTE 2:	This can be done either by detection of abnormal traffic at UPF with reporting to SMF that results in related N4 rules or directly detection and enforcement of N4 rules at the UPF.

- 	The NWDAF may generate Analytics on abnormal user plane traffic using UPF input, this input includes the option that UPF performs inference using a local ML model, then report the inference results to NWDAF, the output of the local ML model at UPF are used as input of the ML model in the NWDAF.

NOTE 3:	The local ML Model at UPF is an implementation option, how to train the ML Model or how to provision the ML Model is out of the scope of this study.

*** Next change***

### .2.2	Topics for further consideration for KI#2

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#2. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in clause 7.1.2) and can either be then marked as not pursued or postponed to a future release.

The following topics are for further consideration for KI#2:

-

-	Whether new UPF or SMF events are needed or not for NWDAF data collection.

-	.

-  For UC1, the input from AF, possibly via NEF, is FFS.

NOTE : How UPF subscribes to the new analytics is FFS.

*** End of changes ***