3GPP TSG-SA WG2 Meeting #172	S2-2510333

Dallas, US, 17 Nov – 21 Nov, 2025

Source:	OPPO

Title:	Conslusion for KI#1

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes the conclusion for KI#1.

- Discussion

The contribution proposes the conclusion for Key Issue #1: Transfer of data over UP for UE data collection.

- Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-04.

* * * Start of Change * * * *

8	Conclusions

The Conclusion for KI#1 is as follows:

Data transfer path

-	A 5GC NF that support the UE data collection functionality is defined as the UP path termination entity to support transferring the standardized UE data from the UE over UP. The 5GC NF that support the UE data collection functionality will further transfer the received standardized UE data to the AF for UE side model training. The 5GC NF that support UE data collection functionality can be standalone or co-located with an existing function (e.g., DCCF) in 5GC.

-	A UP path is used between the UE and a 5GC NF, for transferring standardized collected data from the UE using PDU connectivity service provided by a PDU session.

-	The UE establishes a new PDU Session, or modifies an existing PDU Session which has been established for UE data transfer, to transfer the standardized UE data, based on URSP rules.

-	The FQDN / IP address of the 5GC NF is self-constructed by the UE according to the URSP rule for UP path establishment. The trigger for UE establishing a UP path to the 5GC NF is based on AF’s request or UE side implementation, or

-	The 5GC NF that support the UE data collection functionality is discovered from NRF to support the UE data transfer to different AFs that need to receive data for UE side model training. The 5GC NF that support the UE data collection functionality sends itself the FQND / IP address to UE via NAS singalling.

Traffic differentiation

-	Dedicated S-NSSAI/DNN is used to differentiate the traffic of collected UE data from the UE regular traffic per PDU Session level or

-	The packet filter set (e.g., IP 5 tuples) is used in the QoS rule and the PDR to identify the QoS flow used for UE data transfer.

Data transfer UE selection

- The 5GC NF that support the UE data collection functionality selects the UEs to be used for the data transfer based on the UE side model training server request and user consent.

- The User anthorization is needed to permit the UE to perform the UE data collection and UE data transfer.

Data transfer procedure

-	The 5GC NF that support the UE data collection functionality performs the user consent check from UDM.

-	AF sends the data transfer request to 5GC NF with the event ID. The event ID is defined for corresponding feature ID that defined in RAN WGs.

-	The UE data transfer configuration (e.g., the requested event ID, the reporting period, time window, etc.) is sent from the 5GC NF that support the UE data collection functionality to UE.

-	The transferred data from UE to the 5GC NF corresponding to each event ID are decided by RAN WGs.

-	The UE is allowed to negotiate with the 5GC NF about transfer data (e.g., accept / reject / cancel data transfer request).

* * * End of Change * * * *