3GPP TSG-SA WG2 Meeting #172	S2-2510349

# November 17-21, 2025 Dallas, USA	(a revision of S2-2508528)

Source:	Skylo Technologies

Title:	FS_SMS2EC_ARC: New Sol to KI#3

Agenda item:	20.8.1

Document for:	Agreement

Work Item / Release: FS_SMS2EC_ARC/Rel-20

Abstract of the contribution: This paper proposes a new solution to KI#3

# 1. Discussion

The following aspects are to be studied in KI#3 of TS 23.700-65

-	how and when to trigger location determination and location data sharing when sending short messages using SMS over IP for routing to Emergency Response Centre, considering both 3GPP and non-3GPP access scenarios;

-	how to route short messages using SMS over IP to Emergency Response Centre serving the UE’s location, according to the local regulations (in roaming case, the regulations applicable to the UE’s location);

-	how to support PSAP callback capabilities for sending and routing regular short messages using SMS over IP.

The key issue covers SMS delivery via IP both in EPS and 5GS.

The key issue covers both roaming and non-roaming scenarios, and assuming the solutions for EPS and 5GS are based on same principles.

This paper proposes a solution for KI#3.

# 2 Proposal

It is proposed to include the following changes in TR 23.700-65 V0.2.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[6]	3GPP TS 22.101: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; Service aspects; Service principles".

[7]	3GPP TS 23.272: "Circuit Switched (CS) fallback in Evolved Packet System (EPS); Stage 2".

[8]	3GPP TS 23.204: "Support of Short Message Service (SMS) over generic 3GPP Internet Protocol (IP) access; Stage 2".

[9]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[X1]	NENA i3 Standard for Next Generation 9-1-1, available at https://www.nena.org/page/standards#NENA-STA-010

[X2]	SMS Text-to-9-1-1 Resources for PSAP & 9-1-1 Authorities, available at https://www.nena.org/page/textresources

[X3]	ATIS J-STD-110: “Joint ATIS/TIA Implementation Guideline for J-STD-110, Joint ATIS/TIA Native SMS/MMS Text to 9-1-1 Requirements and Architecture Specification

* * * Second Change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 |
| X |  |  | X |
| - |  |  |  |
| - |  |  |  |
| - |  |  |  |
| - |  |  |  |

## 6.X	Solution #X: Enhanced E-CSCF for SMS over IP Routing to Appropriate PSAP

### 6.X.0	High level principles

The E-CSCF functionality is expanded to support the SIP MESSAGE handling, routing decisions, policy control, and managing roaming messaging failures.

The SMS to EC message is primarily identified by the Emergency Service URN in the Request-URI. An explicit emergency service indication can be used in the signalling.

The messaging payload is expanded to include critical emergency metadata, such as the Service-Type (police/fire/ambulance), UE location, and a Callback ID.

The E-CSCF includes Emergency Services Query Key (ESQK), specified in TS 23.167, in signalling toward the PSAP to serve as a number for the PSAP to call back the caller and as a key for the PSAP to query the LRF for updated location or identity information.

The solution principles apply across all access scenarios, including both EPS and 5GS, as they share the same fundamental IP-based emergency architecture.

### 6.X.1	Description

The proposed solution integrates SMS to EC functionality into the IMS core by enhancing the E-CSCF to manage  SMS to EC handling, routing, location, and policy decisions. The entire architecture maintains consistency across both EPS and 5GS core networks.

The UE supports SMS over IP as specified in TS 23. 204, and is configured to use SMS over IP to send  SMS to EC.

When the UE detects a request for SMS to EC, it initiates the service using an IMS Emergency Registration as specified in clause 7.2 of TS 23.167.

The TP-UD in SMS-SUBMIT should contain specific Emergency Service-Type (police/fire/ambulance, etc.), UE location data (if available), and Callback Information (if available).

The UE then encapsulates the SMS to EC message contained in SMS-SUBMIT type PDU in a SIP MESSAGE method. The SIP message defined in RFC 3428 should be enhanced to include proper Emergency Service Indication. For example, the UE constructs an Emergency Service URN (e.g., urn:service:sos.text) in the Request-URI, which the P-CSCF is configured to recognize.

The UE then forward the SIP MESSAGE to the P-CSCF. The P-CSCF recognizes the incoming SIP MESSAGE contains an Emergency Service Indication. It then forwards the message to E-CSCF.

If the UE does not detect the emergency request, the session initially follows a normal establishment path to the P-CSCF. The P-CSCF may be configured to check the message destination against a database of local emergency numbers. If the destination matches a local or well-known emergency number, the P-CSCF treats the message as an emergency candidate and forwards it to the E-CSCF for emergency handling.

The E-CSCF is enhanced to support the routing of the received SIP MESSAGE. Upon receiving the SIP MESSAGE containing the Emergency Service Indication, the E-CSCF queries the LRF (Location Retrieval Function) using standardized interfaces to obtain the UE's validated location, following procedures outlined in TS 23.167.

The E-CSCF then utilizes the location and the requested Service-Type to consult its routing tables to proper PSAP, as described in clause 6.x.1.1 for the deployment with ESInet and clause 6.x.1.2 for the deployment without ESInet, respectively.

#### 6.X.1.1 Deployment with ESInet

The ESInet (Emergency Services IP Network) is defined by NENA i3 standard [X1]. The ESInet is the fundamental IP infrastructure for Next Generation 9-1-1 (NG9-1-1), enabling the direct reception of multimedia emergency communications (voice, text, video, and data) at the PSAP, often using SIP signalling. The ESInet is explicitly designed to serve as the secure, resilient backbone that connects the network provider's IMS architecture directly to the PSAP (Public Safety Answering Point).

ESRP (Emergency Service Routing Proxy) is an i3 functional element which is a SIP proxy server that selects the next-hop routing within the ESInet based on location and policy. There is an ESRP on the edge of the ESInet, usually at the entrance to an NG9-1-1 PSAP [X2]. There may be one or more intermediate ESRPs between them.

The Enhanced E-CSCF directly resolves and routes the SIP MESSAGE (using the specific Service URN) to the Emergency Service Routing Proxy (ESRP) or an equivalent SIP entity within the ESInet. The ESRP, which is compliant with NENA's i3 standards [X1], uses its own location and policy logic to guide the message to the proper PSAP instance.

#### 6.X.1.2 Deployment without ESInet

For deployments without an ESInet, the E-CSCF routes the SIP MESSAGE to the IP-SM-GW. The IP-SM-GW performs the essential SIP-to-MAP/Diameter conversion and delivers the converted message via the SMS-GMSC to the PSAP Gateway that supports the legacy Text-to-9-1-1 interface [X3].

#### 6.X.1.3 Callback

If the UE was already registered to the IMS network for normal services, that session has associated Public User Identity and TEL-URI would generally be known to the S-CSCF and the HSS/UDM. This prior binding can be used for callback for the purpose of messaging.

When the UE performs an IMS Emergency Registration, barring and roaming restrictions are ignored. The implicit registration set of the Public User Identifier used for emergency registrations shall contain an associated TEL-URI for callback messaging purposes, as specified in clause 4.2 of TS 23.167.

The LRF/RDF may allocate ESQK during routing. The E-CSCF receives this allocated ESQK and includes it in the signalling toward the PSAP. The ESQK is used as a temporary, routable telephone number for both messaging callback and querying location.

If the UE establishes an emergency session without registration (e.g., in an unauthenticated or "anonymous user" scenario), the E-CSCF shall derive a non-dialable callback number for the purpose of messaging where required by local regulation.

#### 6.X.1.4 Roaming

For roaming users, the VPLMN's Enhanced E-CSCF handles the entire routing process via local breakout, ensuring the message is routed to the local PSAP serving the UE’s current location, thereby satisfying local emergency regulation.

However, if the UE is unable to complete the SMS to EC request because the VPLMN's IMS core network is incapable of supporting the SMS to EC functions, which includes scenarios where the E-CSCF is absent or non-functional, or coordinated HPLMN forwarding fails, the signalling flow is reversed. The E-CSCF (if reachable) initiates the failure, or the P-CSCF detects the failure and returns a SIP error response to the UE.

Upon receiving this SIP failure response from the P-CSCF, the UE is explicitly advised to attempt an alternative, higher-priority emergency communication channel, as mandatory fallback must occur. This fallback may involve, for example, one of the following actions:

- The VPLMN, via the SIP response, advises the UE to initiate an Emergency Voice Call (SIP INVITE).

- The UE's domain selection logic may automatically trigger a fallback to the CS domain.

- The system may advise fallback to an alternative transport method, such as SMS over NAS, depending on network capabilities (e.g., if utilizing NTN or other access types).

### 6.X.2	Procedures

Figure 6.X.2-1 shows one example call flow for SMS over IP routing to appropriate PSAP.

Figure 6.X.2-1: Delivery of SMS to EC with SMS over IP

- For a UE detectable SMS to EC, the UE first detects the emergency intent. The UE initiates the service using an IMS Emergency Registration if one is required, as specified in clause 7.2 of TS 23.167.

The UE then encapsulates the SMS to EC short message contained in SMS-SUBMIT type PDU in a SIP MESSAGE method and forward it to the P-CSCF.

The TP-UD in SMS-SUBMIT should contain specific Emergency Service-Type (police/fire/ambulance, etc.), UE location data (if available), and Callback Information (if available).

The SIP message defined in RFC 3428 should be enhanced to include proper Emergency Service Indication. For example, the UE constructs an Emergency Service URN (e.g., urn:service:sos.text) in the Request-URI, which the P-CSCF is configured to recognize.

For Non UE Detectable SMS to EC, the session initially follows a normal establishment path to the P-CSCF.

- In step 2, the P-CSCF recognizes the incoming SIP MESSAGE contains an Emergency Service Indication. It then forwards the message to E-CSCF.

Otherwise, the P-CSCF checks the message destination against a database of local emergency numbers. If the destination matches a local or well-known emergency number, the P-CSCF treats the message as an emergency candidate and forwards it to the E-CSCF for emergency handling.

- The enhanced E-CSCF recognizes the incoming SIP MESSAGE contains the Emergency Service Indication. The E-CSCF immediately uses existing interfaces to the LRF to obtain the UE's location, as described in clause 7.6 of TS 23.167.

The E-CSCF then calls the LRF/RDF to resolve the final PSAP destination and obtain the necessary routing information for the UE's current location and requested Service-Type.

The LRF/RDF allocates the ESQK and determines the ESRN. And the LRF returns the PSAP address along with TEL-URI, ESRN/ESQK and other location/routing parameters to E-CSCF.

The E-CSCF then includes the allocated ESQK and ESRN in the outbound signalling, applying the same Service-Type identifier and semantics used for IMS emergency calls (TS 23.167). This ESQK serves as the mandatory key for the PSAP to query the LRF for location and identity information.

- E-CSCF determines, based on the information provided in step 3 to route the SIP MESSAGE to the proper PSAP.

In 4a, for deployment supporting ESInet, the Enhanced E-CSCF directly resolves and routes the SIP MESSAGE (using the specific Service URN) to the Emergency Service Routing Proxy (ESRP) or an equivalent SIP entity within the ESInet. The ESRP, which is compliant with NENA's i3 standards, uses its own location and policy logic to guide the message to the proper PSAP instance.

In 4b, for deployments without an ESInet, the E-CSCF routes the SIP MESSAGE to the IP-SM-GW. The IP-SM-GW performs the essential SIP-to-MAP/Diameter conversion and delivers the converted message via the SMS-IWMSC to the PSAP Gateway that supports SMS, e.g. Text-to-911 interface. Delivery acknowledgment returns via the reverse path: PSAP Gateway, SMS-GMSC, IP-SM-GW, E-CSCF, and to the UE.

### 6.X.3	Impacts to Services, Entities and Interfaces

P-CSCF:

- Recognize the SMS to EC messaging indication, e.g. Emergency Service URN in the SIP MESSAGE Request-URI, and forward the request to E-CSCF;

E-CSCF:

- Handle SIP MESSAGE with emergency indication, and route the message to the proper PSAP;

* * * End of Changes * * * *