3GPP TSG-WG SA2 Meeting #172	S2-2510424

17 - 21 November, 2025, Dallas, USA	(revision of S2-250xxxx)

Source:	Spreadtrum, UNISOC

Title:	Updated Sol#12 (Enable DO-A traffic with MICO like mechanism) to resolve EN

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract: a new solution for 5GC to support transferring DO-A traffic type.

1. Introduction/Discussion

An update of solution#12 is proposed to address the EN of Sol#12 related to the KI#2 (i.e. support of DO-A Capable AIoT devices) of the SID FS_AmbientIoT_Ph2_ARC.

2. Text Proposal

It is proposed to approve the following changes in the TR 23.700-30.

FIRST CHANGE (All text is new)

## 6.12	Solution #12: Enable DO-A traffic with MICO like mechanism

### 6.12.1	Description

#### 6.12.1.1	High-level solution principles

This solution applies to topology 1.

AIoT device can support inventory and command. In order to help the DO-A capable AIoT device with limited energy storage capability to save power, a DOO (device originated only) mode and DOO mode related Active Time is used.

The AF may request the network to configure a DO-A capable AIoT device to use the DOO mode and DOO mode related Active Time using enhanced command procedure with a new Command Type: Configure.

During the enhanced command procedure, the device could be configured to use the DOO mode and a DOO mode related Active Time value that is used to indicate the duration during which the device can send uplink AIoT traffic and receive the downlink AIoT traffic. During the enhanced command procedure the AIoTF ID and the AF routing information are sent from the AIoTF to the device to route the DO-A NAS message and DO-A data.

A DOO mode related Active Time duration is started or restarted by the AIoT device when the uplink AIoT traffic is sent out from the device. After the DOO mode related Active Time duration ends the DOO mode is activated in the device. The DOO mode is deactivated when there is DO-A data to be sent out from the device. A timer corresponding to the DOO mode related Active Time value is used in AIoTF.

NOTE 1:	The AIoT devices configured with DOO mode are assumed not applicable for Inventory service from AF perspective.

NOTE 2:	How time duration is maintained in AIoT device depends on RAN WG discussion (e.g. R2D synchronization signal).

#### 6.12.1.2	Definition

AF routing information: used to select NEF, for example AF ID, FQDN or IP address of the AF.

DO-A data: AIoT data related to the DO-A traffic.

DO-A NAS message: an AIOT NAS message that is sent from the AIoT device and includes the AIoT device ID and DO-A data.

DOO (Device Originated Only) mode: An AIoT device that has activated the DOO mode does not need to receive downlink AIoT traffic.

DOO mode related Active Time: An AIoT device that is in DOO mode related Active Time duration is able to receive downlink AIoT traffic and send uplink AIoT traffic.

downlink AIoT traffic: includes paging message related to the inventory procedure and NAS Command Request message.

uplink AIoT traffic: includes AIOT NAS message that responds to the paging message related to the inventory procedure, NAS Command Response message and DO-A NAS message.

### 6.12.2	Procedures

#### 6.12.2.1	Enhanced command procedure to configure AIoT device

Figure 6.12.2.1: Enhanced command procedure to configure AIoT device

0.	The AIoT device supports inventory and command procedure.

1.	The AF sends the Nnef_AIoT_Command message to NEF as specified in the step 1 of clause 6.2.3 TS 23.369 [3] with the following additional clarification:

The Nnef_AIoT_Command message additionally includes: a DO-A service indicator, a DOO mode indication and a DOO mode related Active time value. The DO-A service indicator is used to indicate that the AIoT device related with the device ID can initiate DO-A service. The DOO mode indication indicates the AF's preference for DOO mode for the AIoT device. The DOO mode related Active time value indicates how long the DOO mode related Active Time duration is.

The Command Type is set to Configure to request to negotiate and configure the DOO information (i.e. the DOO mode and the DOO mode related Active time value) for the AIoT device.

The NEF selects the AIOTF(s) as described in TS 23.369 [3].

The AIOTF authorize the AF, selects the AIoT reader(s) and response to the AF as described in TS 23.369 [3].

The AIOTF store the DO-A service indicator in the ADM, determine and store the DOO mode indication, the DOO mode related Active time value in the AIoT device profile related with device ID in the ADM based on the request from the AF, local configuration and the policy of the network.

2.	Step 7 of Command Procedure specified in clause 6.2.3 of TS 23.369 [3].

3.	Step 8 of Command Procedure in clause 6.2.3 of TS 23.369 [3] with the following additional clarification:

If the inventory reports include the device ID that corresponds to the device ID received by the AIOTF in step 1, the AIOTF sends Command Request message to the corresponding reader and the Command Request message additionally includes a DO-A service indication that is used to assist NG-RAN to provide DO-A service network resource information to the AIoT device.

The NAS Command Request includes a DOO mode indication, a DOO mode related Active time value, the AF routing information and the AIOTF ID.

4.	Step 9 of Command Procedure in clause 6.2.3 of TS 23.369 [3] with the following additional clarification:

If the DO-A service indication is received in step 3, the AIoT Reader provides DO-A network resource information to the device in the AS R2D message.

5.	After receiving the AS R2D message and the NAS Command Request in step 4:

The device stores DOO mode indication, the DOO mode related Active time value, the AIOTF ID and the AF routing information and the DO-A network resource information; and

The AIoT device shall set a DOO mode related Active Time duration corresponding to the DOO mode related Active Time value received.

6.	Step 10 of Command Procedure in clause 6.2.3 of TS 23.369 [3] with the following clarification:

After the NAS Command Response is sent out, the AIoT device shall start the DOO mode related Active Time duration if the DOO mode related Active Time value is not zero and the device stores DOO mode indication. If the Time value is zero and the device stores DOO mode indication the AIoT device activate DOO mode immediately.

When the DOO mode related Active Time duration ends the AIoT device activates DOO mode and is not available for receiving downlink AIoT traffic. A AIoT device in DOO mode may stop any access stratum procedures (i.e. the same principle as MICO) until the AIoT device needs to send a DO-A data to the network.

7.	Step 11~13 of Command procedure in clause 6.2.3 of TS 23.369 [3] with the following clarification:

After the NAS Command Response is received, the AIOTF shall start the timer corresponding to the DOO mode related Active Time value if the DOO mode related Active Time value is not zero and the device is indicated the DOO mode indication. If the Time value is zero and the device stores DOO mode indication the AIOTF can deduce that AIoT device has activated DOO mode. When the timer expires the AIOTF can deduce that the AIoT device has activated DOO mode and is not available for receiving downlink AIoT traffic.

#### 6.12.2.2	DO-A procedure

Figure 6.12.2.2-1: DO-A Procedure

0.	The AIoT device capable of DO-A is configured with DOO mode indication, stores a DOO mode related Active time value, AIOTF ID, AF routing information and the DO-A network resource information using the device configuration procedure. It is assumed that the AIoT device has activated DOO mode.

1.	When there is DO-A data needed to be sent to the network, the AIoT device deactivate the DOO mode and sends AS D2R message (AIOTF ID, DO-A NAS message) to the NG-RAN based on the stored DO-A network resource information. The AIOTF ID is used by NG-RAN to select AIOTF. DO-A NAS message includes AF routing information, device ID and DO-A data.

After the DO-A NAS message is sent out:

-	if the device stores DOO mode indication and if the stored DOO mode related Active Time value is not zero, the AIoT device shall start the DOO mode related Active Time duration;

-	if the device stores DOO mode indication and if the stored Time value is zero, the AIoT device shall activate the DOO mode immediately.

2.	NG-RAN selects the AIOTF based on the AIOTF ID received in step 1.

3.	NG-RAN forward the DO-A NAS message to the selected AIOTF together with the reader ID and RAN AIoT Device NGAP ID.

4.	The AIOTF authenticates the device corresponding to the device ID carried in the DO-A NAS message.

If the device is authenticated successfully:

-	The AIOTF shall start the timer corresponding to the DOO mode related Active Time value if the DOO mode related Active Time value is not zero and the device is indicated to use DOO mode; and

-	The AIOTF selects the NEF based on the AF routing information carried in the DO-A NAS message.

If the device failed the authentication, the DO-A NAS message is discarded by the AIoTF.

5.	The AIOTF send the DO-A data, the device ID and AF routing information received in step 3 to the NEF. And NEF forward the DO-A data and device ID to the AF based on the AF routing information.

6.	After receiving the DO-A data the AF may subsequently request AIoT service (e.g. Nnef_AIoT_Inventory, Nnef_AIoT_Command) for the corresponding device. The AIoT service request is sent to the AIOTF.

7.	If the device ID related timer corresponding to the DOO mode related Active Time value is running, the AIOTF determines the device could receive the DL AIoT traffic and proceed with the AIoT service. If the device ID related timer expires or is not running and the device has been indicated to use the DOO mode, the AIOTF considers the device has activated the DOO mode and is not able to receive downlink AIoT traffic and may reject the AIoT service and then the step 8~9 is skipped.

8.	The DL AIoT traffic is transferred to the device as described in the inventory and command procedure.

9.	When the DOO mode related Active Time duration ends the AIoT device activates DOO mode.

### 6.12.3	Impacts on Services, Entities and Interfaces

AIoT Device:

-	Support device configuration procedure.

-	Supports DO-A procedure.

NG-RAN(including AIoT Reader):

-	Support device configuration procedure.

-	Supports DO-A procedure.

AIOTF:

-	Support device configuration procedure.

-	Supports DO-A procedure.

ADM:

-	Support AIoT Device profile including: DO-A service indicator, DOO mode indication, the DOO mode related Active time value.

NEF:

-	Support device configuration procedure.

-	Support DO-A procedure.

END OF CHANGES