3GPP TSG-WG SA2 Meeting #172	S2-2510425

# Dallas, USA, 17 – 21 November 2025	(revision of)

Source: 	vivo

Title: 	Conclusions for KI#3

Document for: 	Approval

Agenda Item: 	20.4.1

Work Item / Release:	FS_EnergySys_Ph2/ Rel-20

Abstract of the contribution: This contribution proposes the conclusions for KI3.

- Discussion

The paper proposes the conclusion for KI#3 based on the agreed principles in clause 71.3, additional including:

1.	Propose to add the “Schedule of EnergySaving/not EnergySaving” back to the NF profile, schedule of EnergySaving/not EnergySaving reflects the availability of a NF in a time interval. according to the schedule info, the consumer NF can know the state of the NF in the future which can help consumer select a more proper NF. But for the Energy Priority Information, it only reflects the NF information of the current period of time, which cause NF selection more frequently.

2.	state that the extended energy related attributes in NF profile also is used for NF reselection.

2	Proposal

It is proposed to capture the following solution in TR 23.700-67.

Start of Changes

8	Conclusions

Editor's note:	This clause will capture conclusions for the study.

Where there is consensus, interim agreements (e.g. solution principles descriptions) should be documented in the TR as soon as possible during a study.

These can be documented in the TR as "7.1.Y Agreed Principles for KI#Y" in the "Interim Agreements" clause. If the interim agreement has impacts on another clause in the TR and if there is consensus, that TR clause can be updated.

By consensus interim agreements can become part of the final conclusions of the study.

The Overall Evaluation clause previously used in TR skeletons should not be used.

8.3	Conclusions for KI#3

The following conclusions are proposed for normative work.

-	For (re-)NF selection, the NF profile is enhanced to include the parameters in table 8.3-1.

Table 8.3-1

| Information | Description |
| --- | --- |
| Energy Priority Information | Value of Energy Priority Set by the operator (e.g. taking into account various aspects such as usage of renewable energy or other energy related information outside the scope of this document). It indicates the priority relative to other NFs of the same type. from an energy standpoint. For instance, if the Priority attribute is also used in the NF profile, the Energy Priority indicates a further priority based on energy among NFs with the same Priority attribute value in the NF profile. Whether the operator chooses the Energy Priority or the Priority attribute as primary evaluation criterion is up to the operator. |
| Schedule of EnergySaving/not EnergySaving | The schedule of the EnergySaving/not EnergySaving states (NOTE 1), includes the EnergySaving or not EnergySaving state which applies at each time interval in the schedule. |
| NOTE 1: EnergySaving/not EnergySaving refers to states of a network function which are defined in TS 28.310 [9]. |  |

-	For UP path selection and reselection, the UPFs are selected/reselected by using NF discovery and selection procedures which may be Energy-aware if NF profile is updated as per bullets above. Changes in NF profile of a UPF may trigger UPF reselection.

-	The NF Discovery procedure, based on operator policy, takes into account any Energy-related NF profile attributes query parameters. Both the NRF and the NF service consumer can take into account the Energy-related NF profile attributes if available, as part of the NF Discovery procedure.

-	A NF service consumer takes into account any Energy-related NF profile attributes from the NF profiles discovered from NRF using the NF Discovery procedure (and subsequent updates if the NF subscribed to NF profile updates), for the selection of a target NF.

End of Changes