3GPP TSG-WG SA2 Meeting #172	S2-2510426

# Dallas, USA, 17 – 21 November 2025	(revision of S2-2508378)

Source: 	vivo

Title: 	KI#2, Solution#16 update to solve the Editor’s notes

Document for: 	Approval

Agenda Item: 	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: This contribution tries to update Sol#16 for EN resolving.

- Introduction

This paper is trying to update Sol#16 with EN resolving.

For the following EN:

Editor's note:	It is FFS how to set and use the threshold.

The thresholds (which can be added in slice control policy data defined in 5.2.15 Resource: SlicePolicyControlData, of TS 29.519 [6]) can be set based on operator policy. So it is proposed to add the following NOTE for clarification.

NOTE 1:	The threshold is determined based on the operator policy or implementation.

For the following EN:

Editor's note:	Since it depends on UE implementation to re-evaluate the URSP; by the time the UE considers to evaluate the URSP the EC factors could be changed; which then questions the usability of the solution of changing the UE policies based on current EnC. This aspect is required to be addressed for the solution to be considered for evaluation.

The issue mentioned in this editor’s note is the inherent problem of UE using URSP. It can be understood that the CN have provided the recommended URSP, whether the UE uses it or not depending on the UE implementation. However, this contribution mainly proposes to update the URSP in the PCF based on the energy related information, e.g., energy consumption information from OAM. So, from network energy saving point of view, if certain slice consumes large amount of energy, the UE could access other network slices which may mainly consumes the renewable energy according to the URSP. Even if the UE may evaluate the URSP with some latency, it still helps for network energy saving. It is proposed to add a NOTE to further clarify this.

NOTE 2:	It depends on UE implementation to re-evaluate the URSP. It is the inherent issue that the latency may be caused for the UE to apply the URSP. However, considering the purpose of network energy saving, even if the UE may evaluate the URSP with some latency, it still helps for network energy saving.

2	Proposal

It is proposed to capture the following solution in TR 23.700-67.

Start of Changes

## 6.16	Solution #16: URSP generation/update considering the energy related information

### 6.16.0	High-level solution principles

This solution proposes that the PCF takes the energy related information into account for URSP generation. The energy related information in this solution can be from the OAM.

### 6.16.1	Description

The solution applies to Key Issue #2.

URSP generation currently doesn't consider the energy related information into account. Taking the network slice as an example, if the S-NSSAI-a has higher energy efficiency or lower energy consumption than S-NSSA-b's but with lower priority in the RSD, then the given priority will cause that the S-NSSA-b is selected with high priority for traffic routing, obviously, this does not conform to the consideration of energy saving, therefore the URSP generation taking the energy related information into account is necessary.

When there is local policy configuration for energy saving or received Energy Saving Indicator from AMF during the UE policy association establishment/modification procedure, the PCF will consider that the generation of URSP needs to take energy related information into account. Then, the PCF retrieves the energy related information from the OAM, e.g., the slice level EnC/EnE, renewable energy information for each slice in S-NSSAI subscription information of the UE in UDR from the OAM system (using SA5 defined services).

According to the above obtained energy related information, the PCF generates or updates the URSP, for examples:

-	For the obtained slice EnC/EnE, the PCF evaluates them and takes them value into account to set RSD priority for the slice, i.e. the PCF sets the higher priority of the RSD for the slice which has higher EnE or lower EnC;

-	According renewable energy information for the slice, the PCF can consider its applied time window and location in the RSD;

-	According to the slice level EnC/EnE, the PCF also can set some other elements, for example, if there is no appropriate slice satisfied to the thresholds (which can be added in slice control policy data defined in 5.2.15 Resource: SlicePolicyControlData, of TS 29.519 [6]), the PCF can improve the RSD priority with Non-Seamless Offload indication or the Access Type preference for non-3GPP access.

To avoid the URSP being frequently updated according to the slice EnC/EnE, some threholds can be set in the policy data, when the EnC exceeds the thresholds or the gap among the the EnC(s) exceeds the thresholds, the PCF initiates the URSP updates according the slice EnC/EnE.

If the URSP generation is also influenced by Application guidance as in clause 4.15.6.10 of TS 23.502 [3], when the PCF adjusts the URSP according the energy related information, the PCF may report in the interim status that URSP Rules have been changed by sending Npcf_EventExposure_Notify to NEF and then the NEF triggers the appropriate Nnef_ServiceParameter_Notify message to notify the AF as described in clause 4.15.6.10 of TS 23.502 [3].

NOTE 1:	The threshold is determined based on the operator policy or implementation.

NOTE 2:	It depends on UE implementation to re-evaluate the URSP. It is the inherent issue that the latency may be caused for the UE to apply the URSP. However, considering the purpose of network energy saving, even if the UE may evaluate the URSP with some latency, it still helps for network energy saving.

### 6.16.2	Procedures

The Figure 6.16.2.1 illustrates the UE Policy Association Establishment and Modification procedure taking the energy related information into account.

Figure 6.16.2.1: UE Policy Association Creation and Modification

1.	The AMF decide to initiate the UE Policy Association Establishment procedure as in clause 4.16.11 of TS 23.502 [3].

2.	The AMF sends a Npcf_UEPolicyControl Create Request additionally including the Energy Saving Indicator as defined in TS 23.501 [2].

3.	The PCF responds to AMF.

4.	After retrieving the S-NSSAI subscription information of the UE from the UDR as in 6.2.1.3 of TS 23.503 [4], if there is the Energy Saving Indicator received in step 2 from AMF, the PCF retrieves the slice EnC/EnE for each slice in the S-NSSAI subscription information periodically from the OAM.

5.	For the obtained slice EnC/EnE, the PCF evaluates them and takes them value into account to set RSD priority for the slice, i.e. the PCF sets the higher priority of the RSD for the slice which has higher EnE or lower EnC.

According to the slice EnC/EnE, the PCF may also consider some other elements in the RSD, for example, when no appropriate slice is satisfied to threshold (e.g., the threshold is pre-configured in PCF or which can be added in slice control policy data defined in clause 5.2.15 and retrieved by PCF as in Sol#2: SlicePolicyControlData, of TS 29.519 [6]), the PCF can improve the RSD priority with Non-Seamless Offload indication or Access Type preference for non-3GPP access.

6.	The PCF initiates the UCU procedure for policy to send the UE policy (URSP) to the UE as in clause 4.2.4.3 of TS 23.502 [3].

7.	The PCF monitors and evaluates the periodically received slice EnC/EnE as repeated of step4, the PCF may adjust the URSP according to the newly received slice EnC/EnE.

To avoid the URSP being frequently updated according to the slice EnC/EnE, some thresholds can be set in the policy data, when the EnC exceeds the thresholds or the gap among the the EnC(s) exceeds the thresholds, the PCF initiates the URSP updates according the slice EnC/EnE.

8.	if adjusted, the PCF updates the UE policy (URSP) by UCU procedure as in clause 4.2.4.3 of TS 23.502 [3].

### 6.16.3	Impacts on services, entities and interfaces

AMF:

-	Sends the Energy Saving Indicator to PCF during UE policy creation/modification procedure.

PCF:

-	Collect energy related info, e.g., the slice level EnC/EnE or re- renewable energy information from OAM (using SA5 defined services) for the slice in S-NSSAI subscription information of the UE in UDR if receiving Energy Saving Indicator or there is local energy saving policy.

-	Evaluate the slice level EnC/EnE and set the RSD priority for the slice according to the slice level EnC/EnE.

-	Notifies the AF of the URSP change info via NEF if the Application guidance for URSP is applied.

End of Changes