3GPP TSG-SA WG2 Meeting #172	S2-2510582

Dallas, USA, 17-21 November, 2025	                                               is a revision of S2-2509220

Source:	Samsung

Title:	TR 23.700-65 Update to Solution #11

Document for:	Approval

Agenda Item:	20.8.1

Work Item / Release:	FS_SMS2EC_ARC / Rel-20

Abstract of the contribution: This paper updates Solution #11 to resolve ENs.

1. Discussion

S2-2509220 was agreed during SA2#171 and was included in TR 23.700-65 V0.2.0 as Solution #11 with some ENs regarding the EPS procedure as well as exact modification to the existing procedure in TS 23.502.

In this document, we propose changes to resolve the ENs as well as some editorial corrections.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-65.

* * * Start of Change * * * *

## 6.11	Solution #11: Emergency SMS to EC over NAS

### 6.11.0	High level principles

The following principles are applied to this solution:

- The UE is normally registered with the network and emergency numbers are provisioned, as specified in TS 23.167 [9].

- SMS to EC via NAS is supported by both the UE and the network.

- The UE detects a MO SM addressed to an EC.

- The UE sending a SM to EC sends early indication via NAS type encapsulation.

- The SMSF and AMF (or MME in EPS case) send an acknowledgement to the UE.

- The SMSF/MME forwards the report to the UE.

### 6.11.1	Description

This solution provides and end-to-end procedure to support the SMS to Emergency Response Centre via NAS.

After having registered to the network, the UE is authorised for SMS to EC. Depending on location and local regulation, it may also be provisioned with emergency numbers and corresponding service types (details will be addressed by solutions to KI#1).

Upon detecting an originating SM to EC, the UE sends an UL NAS transport message to the AMF containing the SMS body addressed to an EC. The AMF forwards it to the SMSF that requests the NRF and UDM for emergency SM service.

When granted service for SMS to EC, the SMSF forwards the SM to the SMS-IWMSC or SMSC that sends it the the appropriate EC and then the SMSF sends an ACK back to the UE.

### 6.11.2	Procedures

### 6.11.2.1 MO SMS over NAS in the 5GS

The below figure shows the procedure to send an SMS to EC over the 5GS in clause 4.13.3 in TS 23.502 [3].

Figure 4.13.3.3-1 [3]: MO SMS over NAS

This solution proposes the following modifications (in bold) to the existing procedure (in italic).

1. The UE performs domain selection for UE originating SMS as defined in clause 5.16.3.8 of TS 23.501 [2] if

SMS delivery via non 3GPP access is allowed and possible. If an UE under CM-IDLE state is going to send

uplink SMS message, then UE and network perform the UE Triggered Service Request procedure firstly as

defined in clause 4.2.3.2 to establish a NAS signalling connection to AMF. During initial registration, if supported by the UE and the serving PLMN, SMS to EC is authorised.

2a. The UE builds the SMS message to be sent as defined in TS 23.040 [7] (i.e. the SMS message consists of CPDATA/

RP-DATA/TPDU/SMS-SUBMIT parts). The SMS message is encapsulated in an NAS message with an

indication indicating that the NAS message is for SMS transporting. The UE send the NAS message to the AMF. If a UE detects an SMS to EC and the UE is authorised for SMS to EC, the SM is encapsulated in a NAS message with an indication indicating that the NAS message is for SMS to EC. The RP-DATA shall indicate a RP-DA corresponding to the appropriate emergency service.

2b. The AMF forwards the SMS message and SUPI to the SMSF serving the UE over N20 message by invoking

Nsmsf_SMService_UplinkSMS service operation. In order to permit the SMSF to create an accurate charging

record, the AMF adds the IMEISV, the current UE Location Information (ULI) of the UE as defined in

clause 5.6.2 of TS 23.501 [2] and if the UE has sent the SMS via 3GPP access, the local time zone.

2c. The SMSF invokes Namf_Communication_N1N2MessageTransfer service operation to forward SMS ack

message to AMF.

2d. The AMF forwards the SMS ack message from the SMSF to the UE using downlink unit data message.

3-5. The SMSF checks the SMS management subscription data. If SMS delivery is allowed, the procedure

defined in TS 23.040 [7] or TS 23.540 [84] applies. If the SM is addressed to an EC, the SMSF forwards the SM to the appropriate serving SMSC, which then delivers the SMS to the EC corresponding to the emergency number included in the SM.

6a-6b. The SMSF forwards the submit report to AMF by invoking Namf_Communication_N1N2MessageTransfer

service operation which is forwarded to UE via Downlink NAS transport. If the SMSF knows the submit report

is the last message to be transferred for UE, the SMSF shall include a last message indication in the

Namf_Communication_N1N2MessageTransfer service operation so that the AMF knows no more SMS data is

to be forwarded to UE.

NOTE: The behaviour of AMF based on the "last message indication" is implementation specific.

If the UE has more than one SMS message to send, the AMF and SMSF forwards SMS /SMS ack/submit report

the same way as described in step 2a-6b.

6c-6d. When no more SMS is to be sent, UE returns a CP-ack as defined in TS 23.040 [7] to SMSF. The AMF

forwards the SMS ack message by invoking Nsmsf_SMService_UplinkSMS service operation to SMSF.

### 6.11.2.2 MO SMS over NAS in the EPS

### The procedure for SMS over NAS in the EPS is similar to the procedure in the 5GS where the MME plays the role of the AMF and SMSF.

### 6.11.3	Impacts to Services, Entities and Interfaces

The solution has the following impacts:

UE:

- Need to support indication to serving PLMN support for SMS to EC during Attach/Registration.

- Need to support a new NAS payload type for SMS to EC when initiating MO SMS to EC.

AMF/MME:

- Need to support indication to UE of support for SMS to EC during Attach/Registration.

- Need to support a new NAS payload type for SMS to EC.

SMSF/MME:

- Support routing to local SMS-IWMSC or SMSC for SMS to EC.

SMSC:

- Supports routing to EC.

* * * End of Change * * * *