3GPP TSG-SA WG2 Meeting #172	S2-2510606

Dallas, USA, 17 - 21 November, 2025

Source:	CATT

Title:	23.700-19: Discussion and Interim conclusion for KI#3

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This paper discusses the evaluations of KI#3, and proposes Interim conclusion for KI#3.

1 Summary of the solutions on KI#3

Six solutions have been proposed with different approaches:

| Sol# | Approach | Key Differentiators |
| --- | --- | --- |
| #4 | SCC AS-based emergency handling | SCC AS terminates I1 session and originates SIP emergency session toward E-CSCF; handles emergency over NIDD-UP via GEO; reuses existing location services |
| #21 | Basic emergency service support framework | Basic approach: MME/eNB indication, reuse TS 23.167, no new interfaces |
| #22 | Skip IMS emergency registration | Reduces call setup time; UE directly sends SIP INVITE to P-CSCF; P-CSCF validates via PCRF |
| #23 | Network-based emergency detection | Handles both UE detectable and non-UE detectable sessions; IMS core identifies emergency numbers |
| #24 | Using Gm* interface information | Handles both UE detectable and non-UE detectable sessions; Emergency Service Support indicator, enhanced interface Gm* |
| #25 | Location-based emergency number provision | Addresses GEO cross-border challenge; MME provides emergency numbers based on verified UE location |

2 Discussion

2.1 Basic approach for emergency service

The majority of solutions for KI#3 propose fundamental approaches to support emergency services over NB-IoT NTN via GEO satellite connected to the EPC. The main aspects are as follows:

-	Existing IMS emergency call procedures defined in TS 23.167 can be reused, including both UE detectable and non-UE detectable emergency sessions.

-	The only emergency service supported over NB-IoT NTN connected to EPC is for IMS voice.

-	The MME needs to include an indication for Emergency Services Support within the NAS Accept to the UE also for UE accessing over NB-IoT NTN which is currently not required in TS 23.401.

-	The eNB needs to support the related broadcast indicator that the cell supports Emergency Services over NB-IOT NTN.

Therefore, the following principles are proposed:

Principle 1: Existing IMS emergency call procedures defined in TS 23.167 can be reused.

Principle 2: Emergency service support for NB-IoT NTN via GEO satellite connecting to EPC shall be limited to IMS voice.

Principle 3: MME should provide an Emergency Services Support indication to the UE that initiates an emergency call over NB-IoT (GEO) access.

NOTE 1: Whether the eNB broadcasts the emergency service support indication is based on RAN decisions.

2.2 Signalling optimization

GEO satellite introduces significant propagation delay that impacts emergency call setup time. Multiple message exchanges over satellite links result in cumulative delays that are unacceptable for emergency scenarios. Several optimization approaches have been identified as follows:

-	Registration skip: UE skips IMS emergency registration and directly sends SIP INVITE to P-CSCF. P-CSCF verifies UE identity (e.g., via PCRF using IMS PDN connection information) before routing to E-CSCF.

-	Network-based emergency detection: For non-UE detectable emergency sessions, IMS core network identifies emergency numbers based on UE location and processes the session as emergency call without requiring UE re-initiation.

2.3 Other considerations

GEO satellite footprint can cover multiple countries or regions with different emergency numbers. Emergency service routing depends on the UE's actual geographic location to ensure correct PSAP routing and regulatory compliance. MME may perform location verification and provide emergency numbers corresponding to the jurisdiction where UE is located during Attach and TAU procedures.

3 Proposal

Based on the above observations, it is proposed to agree the interim conclusions as follows:

* First change (all New)*

8.1	Interim conclusions for Key Issue #3

To support KI#3 of IMS emergency call over NB-IoT NTN, the following conclusions are proposed:

-	Existing IMS emergency call procedures defined in TS 23.167 can be reused.

-	Emergency service support for NB-IoT NTN via GEO satellite connecting to EPC shall be limited to IMS voice.

-	MME should provide an Emergency Services Support indication to the UE that initiates an emergency call over NB-IoT (GEO) access.

NOTE 1: Whether the eNB broadcasts the emergency service support indication is based on RAN decisions.

* End of change (all New)*