3GPP TSG-SA WG2 Meeting #172	S2-2510654

# October 17-21, Dallas, US

Source:	Xiaomi

Title:	Conclusion for KI#1 System Architecture to Support Sensing

Agenda item:	20.2.1

Document for:	Agreement

Work Item / Release: FS_Sensing_ARC/Rel-20

Abstract of the contribution: This paper proposes the conclusion for KI#1 System Architecture to Support Sensing.

1. Discussion

TS 23.700-14 v1.1.0 has documented the interim agreements for KI#1 System Architecture to Support Sensing as following:

7.1.1 Agreed Principles for KI#1: System Architecture to Support Sensing To support KI#1 System Architecture to Support Sensing, the following principles are concluded: Principle 1: one new Network Function (i.e. Sensing Function, SF) is defined to support Sensing Service. the SF may contain Sensing control functionality (SCF) and Sensing Processing functionality (SPF). NOTE 1: There is no standardized interface between the SCF and SPF in Rel-20 5G-A. NOTE 2: How to capture the deployment options of SPF(s) functionality and SCF functionality of the same SF, e.g. co-located or separated, can be discussed during the normative phase. NOTE 3: The final name and Acronym for Sensing Function may need further update if necessary. Principle 2: NO dedicated storage NF is needed to store the Sensing data and Sensing result. Principle 3: In this study, the gNB is the only entity that acts as the Sensing Entity (SE). NOTE 4: Privacy protection and other security aspects will be coordinated with SA WG3 and the related impact to architecture enhancement will be based on SA WG3 during the normative phase. NOTE 5: Other KI's interim conclusions will be aligned with KI#1 conclusions.

Proposal 1: copy all the interim agreements for KI#1 in clause 7.1.1 into clause 8.1 conclusion for KI#1.

Observation 2: In last SA2#171 meeting, there was no interim agreement achieved regarding whether the AMF is involved for deliver the sensing data and/or sensing control signalling, with following status:

- For sensing data delivery, Majority companies prefer Principle 2Y “AMF is not involved to support Sensing data delivery”, only few companies prefer “AMF is involved to support Sensing data deliver”.

- For Sensing control signalling delivery, 1) some companies prefer “AMF involved to support sensing control signaling; 2) more companies prefer directly connection between SE and SF (i.e. AMF is not involved); 3) some companies strongly against the option to support both.

Principle 2X: Sensing control signalling is exchanged between SE and SF/SCF directly or via AMF, both.

Principle 2Y: AMF is NOT involved to support for Sensing data delivery.

For 5G-A Sensing, it is better to avoid to introduce additional and unnecessary impacts. Sensing data and sensing control signalling are exchanged directly between SE and SF, it can avoid to introduce additional impacts on AMF. And at the same time, it is also compatible for 6G sensing.

Proposal 2: Sensing data is delivered directly between SE and SF (i.e. without AMF involvement). Sensing control signalling is exchanged between SE and SF directly (i.e. without AMF involvement);

Observation 3: regarding to Whether to introduce a new Gateway Sensing Function (GSF)?), there are four potential functionalities that may be performed by new Gateway Sensing function in the TR solutions, as following:

- Authorization for Sensing service consumer.

- Authorization for Sensing service request

- Discovery and selection for SF

- Sensing result exposure to Sensing service consumer

For 5G-A Sensing, it is better to avoid to introduce unnecessary impacts. For 1) it was agreed in KI#2 interim agreement, that NEF performs the authorization service consumer. So, no need to introduce duplicated Gateway Sensing Function to perform the authorization service consumer.

In KI#1 interim agreement, it was agreed that Sensing Function is defined to support sensing service, there is no issue to perform the Authorization for Sensing service request. if GSF is introduced, the additional interface between GSF and SF should be also defined. So don’t see the need or advantage to define new Gateway Sensing Function to perform Authorization for Sensing service request

For discovery and selection for SF, it was also agreed in the interim agreement for KI#3, SF selection based on NEF’s local configuration or NRF query. It also indicates that NEF would perform the discovery the selection of SF. So there is also no need to introduce a new GSF to perform the same functionality as NEF.

For the sensing result exposure, NEF is defined to expose information to AF. There is no issue for NEF to exposure the sensing result for sensing service. So also don’t see the need to introduce a new GSF to expose the sensing result

Considering above all, the four functionalities can be performed by SF/NEF, don’t see the need to introduce one new Gateway Sensing Function.

Proposal 3: No Gateway Sensing Function is needed in this study.

- Principle 5: if AF is outside the trusted domain, NEF is enhanced to support Authorization for Sensing service consumer (as captured in clause 8.2), Discovery and selection for SF (as captured in clause 8.3), and Sensing result exposure to Sensing service consumer (as captured in clause 8.5).

- Principle 6: the functionality of Authorization for Sensing service request is performed by SF (as captured in clause 8.2)

Observation 4, during the discussion in SA3, it is controversial whether the sensing result may contain additional and un-expectation information, e.g., human information in the sensing area besides the requested aerial object.

Proposal 4: to make further clarification as following principle 7.

- Principle 7: Sensing architecture needs to support aerial object (e.g. drone) detection and tracking as defined in TS 22.137 [2]. And the exposed sensing result is restricted for aerial object (e.g. drone) detection and tracking in this study.

2 Proposal

It is proposed to include the following changes in TR 23.700-14 V1.1.0.

* * * First Change * * * *

8	Conclusions

8.1 conclusion for KI#1: System Architecture to Support Sensing

To support KI#1 System Architecture to Support Sensing, the following principles are concluded:

Principle 1: one new Network Function (i.e. Sensing Function, SF) is defined to support Sensing Service. the SF may contain Sensing control functionality (SCF) and Sensing Processing functionality (SPF).

NOTE 1:	There is no standardized interface between the SCF and SPF in Rel-20 5G-A.

NOTE 2:	How to capture the deployment options of SPF(s) functionality and SCF functionality of the same SF, e.g. co-located or separated, can be discussed during the normative phase.

NOTE 3:	The final name and Acronym for Sensing Function may need further update if necessary.

Principle 2: NO dedicated storage NF is needed to store the Sensing data and Sensing result.

Principle 3: In this study, the gNB is the only entity that acts as the Sensing Entity (SE).

Principle 4: Sensing data and Sensing control signalling are exchanged directly between SE and SF (i.e. without AMF involvement).

Principle 5: if AF is outside the trusted domain, NEF is enhanced to support Authorization for Sensing service consumer (as captured in clause 8.2), Discovery and selection for SF (as captured in clause 8.3), and Sensing result exposure to Sensing service consumer (as captured in clause 8.5).

Principle 6: the functionality of Authorization for Sensing service request is performed by SF (as captured in clause 8.2)

Principle 7: Sensing architecture needs to support aerial object (e.g. drone) detection and tracking as defined in TS 22.137 [2]. And the exposed sensing result is restricted for aerial object (e.g. drone) detection and tracking in this study.

* * * End of Changes * * * *