3GPP TSG-WG SA2 Meeting #172	S2-2510666

Dallas, US, 17th Nov – 21st Nov, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	[KI2, Sol 18 Update] Supported Registration Types

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract: Addition of deregistration and optional periodic registration procedures.

1. Introduction/Discussion

1.1	Overview

The following aspects are updated or added to solution 18:

-	Registration Updates, including types and deregistration.

-	Power Saving aspects.

-	DO-A Data Transfer enablement and disablement.

-	Generalisation of the configuration procedure.

1.2	Registration Updates

In solution 18 there is the following Editor’s Note:

Editor's note: It is FFS whether other registration type need to be supported.

Currently the solution provides an initial registration to enable setup of a context in the network, and a mobility registration so the network can know the (rough) location of the AIoT Device.

Normally there is a 3rd type of registration supported, the periodic registration. One of the main purposes of this type of registration is to allow the network to know a device (UE in 5GS case) is still present and reachable, even when there is no traffic flowing. If the device does not “check in” within the periodic registration period the network de-registers it. This also allows the clean-up of network resources etc when a device has not gracefully de-registered.

To enable clean-up and tracking of whether an AIoT Device is still available 2 additional aspects are provided:

1. 	Periodic Registration

2. 	Deregistration

For periodic registration, depending upon the requirements we place on an AioT Device, it may or may not be able to support maintaining a timer to trigger the periodic registration. A baseline of a network only timer is provided, with the optional ability of an AIoT Device to perform periodic registration.

In all cases it is expected the periodic timer in the network is reset when any NAS message is received from the AIoT Device. The network can be configured with a minimum timer based on expected intervals for UL data transmission from an AIoT Device, to prevent the timer expiring unexpectedly.

The AIoT Device is also provided with an AIoT Registration Area and the solution discusses different application requirements the AIoT Registration Area sizes. To enable the AIOTF to make the correct determination the AF can provide its requirements to the network, and the AIOTF can take these into account. This then means the network can generate mobility events towards the AF if/when an AIoT Device moves, triggered by the AIoT Device informing the network it has moved outside the AIoT Registration Area, as already described.

1.3	Power Saving

The current solution provides the locations in the procedure where power saving parameters can be provided to the AIoT Device. This is expanded to include information about power saving parameters, how they are used and how they can be applied to a group of devices.

The power saving parameters are used by an AIoT Device to determine when it should listen for paging, similar to eDRX in 5GS/EPS. The AIOTF takes when an AIoT Device is listening for paging into account during the Inventory procedure and may need to provide parameters to NG-RAN as part of the inventory procedures.

An AIoT Device can be assigned to a group of devices, and when the AIoT Device registers which is part of a group, if there is any power saving parameters for the group these are provided to the AIoT Device. As all AIoT Devices which are members of a group are provided with the same power saving parameters, then the network knows when it should perform inventory/paging for a specific group. This enables group operations as well as power saving in the AIoT Device.

1.4	Enabling and Disabling DO-A Data Transfer

To route DOA data from an AIoT Device the network needs to know which AF to send the data to. If there is no AF configuration then the network cannot route the DOA data. In order to save network resources and AIoT Device resources (e.g., power), if there is no routing configured then the AIoT Device can be instructed to not generate DOA uplink data.

If routing information is configured after the AIoT Device has registered it can then be updated to enable the DOA data transmission.

1.5	General Parameter Provisioning

The above updates can have configuration provided by an AF to the network, in addition to the provisioning DO-A data routing already described in the solution.

Instead of there being multiple configuration mechanisms, the existing DO-A configuration mechanism is made more general to handle the additional configuration aspects, and any future aspects.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-30.

# * * * * First change * * * *

6.18	Solution #18: AIoT Device Registration and data transmission Procedure

6.18.0	High-level solution Principles

DO-A capable AIoT Devices have the ability to send MO originating signalling and data. The AIoT Device actively registers to the network to inform the network of its presence autonomously and once the AIoT Device is registered it can actively send data to the network autonomously without triggers from the network. The transferred data is included in AIoT NAS messages. The network is configured with a route for uplink data from the AIOT Device and the route is used when traffic is sent.

6.18.1	Description

This solution addresses KI#2 and includes how to enable the AIoT Device to inform the network of its presence autonomously for Topology 1, how to enable the device to perform DO-A data transmission and Inventory and Command Procedures.

It is assumed that the AIoT Device actively initiates the registration process with the network and the data transmission procedure without any trigger from the network. The registration type can be an Initial Registration when not already registered, Mobility Registration Update when it to moves outside of an AIoT Registration Area, or Periodic Registration Update when a timer in the AIoT Device expires, if supported. When an AIoT Device initially registers a context is created in the network for the AIoT Device, the AIoT Device is provided with an AIoT Registration Area and, if supported, a periodic registration timer.

AIoT Areas may span multiple Readers (and NG-RAN nodes) and a Reader may belong to multiple AIoT Areas. The granularity of the AIoT Areas and Readers can mean that the AIoT Device has to be provided with and store a large amount of information, or would have to perform Mobility Registration Updates frequently when the AIoT Device moves relatively small distances. While in some cases knowledge that the AIoT Device has moved a small distance is desirable, it is not a universal desire and may cause significant overhead to AIoT Devices and networks. The AF can provision information to the network to assist with determining the AIoT Registration Area to provide to the AIoT Device.

Therefore, to enable the difference scenarios the network is configured with AIoT Registration Areas. An AIoT Registration Area is provided to the AIoT Device and is:

-	A Reader list, AIoT tracking area code (TAC), or an AIoT Area List. The AIoT Device performs Mobility Registration Update if it moves outside of the list, TAC or AIoT Area List.

-	Be hierarchical and be the first part of a TAC or a partial TAC. The AIoT Device performs Mobility Registration Update if the supplied part of the TAC no longer matches the TAC where the AIoT Device is.

The hierarchical AIoT Registration Area will reduce the amount of storage required by an AIoT Device.

NOTE 1:	The AIoT Registration Area is specific to Ambient IoT and does not need to be aligned with registration areas used by an AMF for NR. The AIoT tracking area code (TAC) is a different type of area identification from an AIoT Area List.

An AF can subscribe to events for AIoT Device mobility, similar to those supported in 5GS for UEs, to be informed when the AIoT Device moves outside of its AIoT Registration Area or an AF provided location or predefined movement path.

The AIoT Device, during registration, can indicate its support of Periodic Registration, and the AIOTF in this case can provide the AIoT Device with a Periodic Registration Timer value. The network always maintains a Periodic Registration Timer, and depending on configuration and AIoT Device support, determines the value of this timer. If the AIoT Device has a Periodic Registration Timer value from the AIOTF, when it expires the AIoT Device performs Periodic Registration. If the Periodic Registration Timer for an AIoT Device expires in the AIOTF then the AIOTF can start an implicit deregistration timer and if that expires it can deem the AIoT Device to be unregistered and discard the context. AIoT Device mobility events may be generated when the AIoT Device performs registration or is deregistered.

The AIoT Device is also able to perform a Deregistration procedure, for example, before being powered off. When this occurs the AIOTF can consider the device as unregistered and discard any AIoT Device context for the AIoT Device. AIoT Device mobility events can be generated when this occurs.

According to TS 23.369 [3] and TS 33.369 [11], the AIOTF is the supports termination point of the AIoT NAS protocol within the network and supports the authentication of the AIoT Device. For a DO-A capable AIoT Devices the AIOTF remains the NAS termination point, including for device authentication and additionally supports registration management and mobility management.

NOTE 2:	The Temporary ID used in the procedures for AIoT Device identification and AIOTF selection will need alignment with SA WG3.

An AIoT Device can be configured with power saving parameters which are used to determine when the AIoT Device will listen for paging/inventory, similar to eDRX configuration in EPS/5GS. The AIoT Device can also be assigned to a group and the power saving parameters for that group can be provided to the AIoT Device in addition to the individual AIoT Device power saving parameters.

6.18.2	Procedures

6.18.2.1	Registration Procedure

The Registration procedure is used by an AIoT Device for Initial Registration when not registered, Mobility Registration Update when it to moves outside of an AIoT Registration Area provided to it, or Periodic Registration Update when a timer in the AIoT Device expires, if supported.

Figure 6.15.2.1-1: Registration Procedure

1.	The AIoT Device sends a D2R message to NG-RAN, including D2R parameters and a NAS Registration Request (Registration Type, AIoT Device information, security parameters, periodic registration supported).

The D2R parameters contains, e.g. a Temporary ID (if available), the selected PLMN ID that allows NG-RAN to route the request to the appropriate AIOTF.

NOTE 1:	The details of the D2R parameters need to be coordinated with RAN WGs.

The Registration Type indicates if the AIoT Device is performing an Initial Registration, a Mobility Registration Update or a Periodic Registration Update. The AIoT Device performs an Initial Registration if it is not registered with the network, performs a Mobility Registration if it has moved outside of its AIoT Registration Area, or Periodic Registration Update if a previously provided Periodic Registration Timer expires.

If the AIoT Device supports Periodic Registration (i.e., running a Periodic Registration Timer), it indicates this in the NAS Registration Request.

2.	NG-RAN selects an AIOTF based on the D2R parameters or local configuration to send the NAS message received from the AIoT Device to. If Indirect Connectivity is used between NG-RAN and the AIOTF, NG-RAN sends the selected AIOTF ID to AMF which can then route the message to the selected AIOTF.

3.	NG-RAN sends the NAS Registration Request, Temporary ID and selected PLMN ID from the D2R message and Reader information to the selected AIOTF. If the Temporary ID indicates another AIOTF assigned it, the selected AIOTF fetches the AIoT Device context from that AIOTF.

4.	[Conditional] The new AIOTF may determine an old AIOTF using the Temporary ID and retrieves the AIoT Device context from the old AIOTF.

5.	The AIOTF determines the ADM for the AIoT Device based on the AIoT Device's permanent ID.

6.	The AIOTF performs authentication and security procedures and stores the security context.

NOTE 2:	Details of authentication and security procedures will be determined by SA WG3.

7.	The AIOTF registers itself with ADM as serving AIOTF for the AIoT Device and may retrieve the subscription data for the AIoT Device.

8.	[Conditional] The ADM informs the old AIOTF using Nadm_SDM_unsubscribe that it is no longer the serving AIOTF for the AIoT Device.

9.	The AIOTF created an AIOTF Context for the AIoT Device and sends the Registration Accept ([Temporary ID], [AIoT Registration Area], [Periodic Registration Timer], [power saving parameters], [DOA Data Enabled])to the AIoT Device.

The AIOTF always determines a Periodic Registration Timer for an AIoT Device and if the Periodic Registration Timer expires then the AIOTF starts an implicit deregistration timer. To determine the Periodic Registration Timer the AIOTF takes into account, for example, whether the AIoT Device supports the timer, subscription data and AF configured Expected UL Interval, and local configuration.

The Periodic Registration Timer is restarted, and any running implicit deregistration timer for the AIoT Device is stopped, whenever the AIoT Device sends NAS messages to the AIOTF.

If the implicit deregistration timer expires for an AIoT Device then the AIoT Device is implicitly deregistered and the actions in the Deregistration Procedure step 7 are performed to update the ADM, and the AIOTF discards the AIOTF context for the AIoT Device.

If there is no target AF information included in the subscription information then the AIoT Device is configured not to send DOA data using the DOA Data Enabled in Registration Accept.

The information provided to the AIoT Device, and other associated information is stored in the AIoT Device context.

6.18.2.2	Configuration provisioning for DOA

The procedure to configure the routing of uplink traffic from an AIoT Device is show in figure 6.18.2.2-1.

Figure 6.18.2.2-1: DO-A Target AF information configuration Procedure

1.	The AF sends an Nnef_DOAconfiguration_Create Request message (AIoT Device information, [AF Identifier], [validity time], [Expected UL Interval], [Location Information]) to the NEF.

The AF Identifier and validity time are used to indicate the routing for DOA data from the AIoT Device and how long network maintains the DO-A routing information. This subscribes the AF to notifications of received data, as described in clause 6.18.2.3.

The Expected UL Interval is used to indicate the how often the AF expects the AIoT Device to send UL, and is used for example in the Registration procedure.

The Location Information can be used to determine the AIoT Registration Area provided to the AIoT Device, and provide to device during registration or command procedure.

2.	The NEF selects an ADM based on the AIoT Device information received in step 1.

3.	The NEF sends a Nadm_DOAConfiguration_Create Request message (AIoT Device information, AF Identifier, validity time) to the ADM.

4.	The ADM validates the request from the NEF and stores the AF information for AIoT Devices' DO-A data transmission, i.e. AF Identifier and validity time. If the AIoT Device is already registered, the ADM notifies the registered AIOTF that the AIoT Device subscription data has been updated.

5-6.	If the AF information for an AIoT Device is added, then the AIOTF uses the Command Procedure to inform the AIoT Device that it can send UL DOA Data and updates the AIoT Device context.

If the validity time associated with the AF information expires, then the AIOTF uses the Command Procedure to configure the AIoT Device not to send UL DOA Data and updates the AIoT Device context

7.	The ADM sends the Nadm_DOAConfiguration_Create Response message to NEF to confirm the route has been stored.

8.	The NEF sends Nnef_DOAconfiguration_Create Response to the AF to acknowledge acceptance of the Nnef_DOAconfiguration_Create Request.

6.18.2.3	DO-A data transmission

The procedure uses the routing information stored in the ADM to route DO-A traffic from an AIoT Device and is shown in figure 6.18.2.3-1.

Figure 6.18.2.3-1: DO-A Data Transmission Procedure

1.	The AIoT Device sends D2R message to NG-RAN, including Temporary ID and AIoT UL NAS message (Temporary ID, DO-A Data).

2.	Based on the Temporary ID, NG-RAN selects an AIOTF and forwards the AIoT UL NAS message and the RAN AIoT Device NGAP ID to the selected AIOTF. If Indirect Connectivity is used between NG-RAN and the AIOTF, NG-RAN sends the selected AIOTF ID to AMF so it can route the messages to the AIOTF. The AIoT Device may provide NG-RAN with an indication that follow on NAS message(s) to or from the AIoT Device or AIOTF are expected. When NG-RAN receives a follow-on indication, it can ensure that subsequent messages can be sent from the AIoT Device.

The AIOTF has previously obtains the target AF information for the AIoT Device from the AIoT Devices subscription data from the the ADM.

3.	The AIOTF forwards the AIoT Device ID, DO-A Data and the target AF information an NEF. The NEF does not have to be the same NEF that that AF used to provide the routing information to the network.

4.	The NEF forwards the AIoT Device ID, DO-A Data to the AF based on the target AF information.

6.18.2.4	Inventory Procedures

The Inventory Procedure in clause 6.2.2 of TS 23.369 [3] is performed with following differences:

-	In step 1, the AF can provide a group identifier instead of complete AIoT Device Identifier(s) or Filtering Information in information about the target AIoT Device(s).

-	In step 2, if the information about the target AIoT Device(s) has been provided, the NEF determines the corresponding serving AIOTFs by querying the ADM for the serving AIOTF based on the AIoT Device ID and uses that AIOTF to perform the Inventory Procedure.

-	In step 7, if AIoT Device Identifier(s) or a group identifier is provided in information about the target AIoT Device(s) in step 1, then the AIOTF determines any power saving parameters provided to the AIoT Device. The AIOTF determines when to perform inventory such that the AIoT Device will receive the paging in RAN, and may also provide the power saving parameters to NG-RAN as part of the inventory procedure.

-	In step 11, the AIOTF may find the serving AIOTF based on the Temporary ID and retrieve the AIoT Device ID from the serving AIOTF. The AIoT Device context does not need to be transferred, if a new Temporary ID does not need to be provided to the AIoT Device.

6.18.2.5	Command Procedures

The Command Procedure in clause 6.2.3 of TS 23.369 [3] is performed with following differences:

-	In step 2, if the information about the target AIoT Device(s) has been provided, the NEF determines the corresponding serving AIOTFs by querying the ADM for the serving AIOTF based on the AIoT Device ID and uses that AIOTF to perform inventory procedure.

-	In step 7, the AIOTF may find the serving AIOTF based on routing information, e.g. from within the temporary ID and fetch AIoT Device contexts from the serving AIOTFs. It also registers/updates itself in the ADM as new Serving AIOTF.

If the serving AIOTF has changed, a new Temporary ID, needs be generated and provided to the AIoT Device so when it performs DO-A procedure the new AIOTF can be selected by NG-RAN.

-	In step 8, the new Temporary ID, is provided to the AIoT Device along with the NAS Command Request message.

NOTE:	Power consumption parameters might need to be provided to NG-RAN when performing the Command Procedure and details are expected to described other solutions.

6.18.2.6	Deregistration Procedure

The Deregistration procedure is used by an AIoT Device when it no longer wishes to communicate with the network.

Figure 6.18.2.6-1: Deregistration Procedure

1.	The AIoT Device sends a D2R message to NG-RAN, including D2R parameters and a NAS Deregistration Request (AIoT Device information, security parameters).

The D2R parameters contains, e.g., a Temporary ID (if available), the selected PLMN ID that allows NG-RAN to route the request to the appropriate AIOTF.

NOTE 1:	The details of the D2R parameters need to be coordinated with RAN WGs.

2.	NG-RAN selects an AIOTF based on the D2R parameters or local configuration to send the NAS message received from the AIoT Device to. If Indirect Connectivity is used between NG-RAN and the AIOTF, NG-RAN sends the selected AIOTF ID to AMF which can then route the message to the selected AIOTF.

3.	NG-RAN sends the NAS Deregistration Request, Temporary ID and selected PLMN ID from the D2R message and Reader information to the selected AIOTF. If the Temporary ID indicates another AIOTF assigned it, the selected AIOTF fetches the AIoT Device context from that AIOTF.

4.	[Conditional] The new AIOTF may determine an old AIOTF using the Temporary ID and retrieves the AIoT Device context from the old AIOTF.

5.	The AIOTF determines the ADM for the AIoT Device based on the AIoT Device's permanent ID.

6.	The AIOTF deregisters from the ADM as the serving AIOTF for the AIoT Device.

7.	[Conditional] The ADM informs the old AIOTF using Nadm_SDM_unsubscribe that it is no longer the serving AIOTF for the AIoT Device.

8.	The AIOTF discards the AIOTF Context for the AIoT Device and sends the Deregistration Accept to the AIoT Device.

6.18.3	Impacts on Services, Entities and Interfaces

AIOTF:

-	Support Registration, Mobility Management and Deregistration.

-	Support DO-A data routing, including retrieval of AF Identifier from ADM.

-	Register (and deregister) AIOTF ID into ADM as serving AIOTF of the AIoT Device.

NG-RAN:

-	Supports AIOTF selection and transferring NAS messages to and from the AIoT Device.

AIoT Device:

-	Support performing registration and deregistration procedure.

-	Optionally support a timer for Periodic Registration.

-	Support transmission of DOA data and its enablement.

ADM:

-	Support storage of AIOTF ID as serving AIOTF for AIoT Device.

-	Support storage of AF Identifier for AIoT Device data routing.

-	Support parameter provisioning for an AIoT Device

NEF:

-	Support exposure functionality for configuration and transmission of data to/from an AF.

# * * * * End of changes * * * *