SA WG2 Meeting #172	S2-2510748

Dallas, USA, 17 – 21 November 2025	(revision of S2-2509785)

Source:	[Futurewei (penholder),] Xiaomi

Title:	[WT#1.2, Enhanced User Plane] Scope and Key Issue for Enhanced User Plane

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: Enhanced User plane revision with updates to work task scope and key issue.

- Justifications

It is updated considering the CC#2 discussion, forward proposals of penholder, and agreements on UP achieved in RAN2#131bits, based on (S2-2509785) the postponed version of SA2#171.

Agreements on UP

- 1.	RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality.  Discussions should focus on standalone architecture, until told otherwise by plenary.  When discussing functionalities companies should focus on identifying the problems being address or new requirements.

2.  	UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer.

3.	6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB)

4.	Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic).  Understand the issues and shortcoming with the current NR QoS.   Aim to look at a general framework.

5.	Support at least the following scheduling schemes: dynamic grant and configured grant.  Further study configured grant.

6.	Study need for scheduling enhancements to address the SR/BSR/DSR latency

7.	Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ.  Study should identify enhancements needed for HARQ and L2 ARQ.

**** First Change ****

Annex A.X. WT #1.2 User Plane Architecture

In order to support 6G user plane for a diverse set of applications and traffic patterns, the following are studied taking the 5GS user plane framework as a starting point for discussion:

- Whether and how to enhance CP-UP functional split for better multi-vendor interoperability.

- Whether and how user plane flexibility (by user plane function (re)selection) for different service requirements, session continuity and mobility can be enhanced with consideration of path performance and capability between access network and data network.

- Study whether and how to improve UP architecture for resiliency, scalability, efficiency and load balancing, compared with 5G.

NOTE M: The study and outcome of bullet 3 should avoid the overlap with bullets 1 and 2.

NOTE N:	The handling of application traffic in the UP aspect of this work task may have dependency on WT#1.2 QoS.

NOTE z: The outcome of this study may include architectural requirements that could influence the design of user plane protocols in stage 3 working groups.

**** Second Change ****

5.X. Key Issue #X: User Plane Architecture

In order to support 6G user plane for a diverse set of applications and traffic patterns, the following are studied taking the 5GS user plane framework as a starting point for discussion:

- Whether and how to enhance CP-UP functional split for better multi-vendor interoperability.

- Whether and how user plane flexibility (by user plane function (re)selection) for different service requirements, session continuity and mobility can be enhanced with consideration of path performance and capability between access network and data network.

- Study whether and how to improve UP architecture for resiliency, scalability, efficiency and load balancing, compared with 5G.

NOTE M: The study and outcome of bullet 3 should avoid the overlap with bullets 1 and 2.

NOTE N:	The handling of application traffic in the UP aspect of this work task may have dependency on Work Task #1.2 QoS.

NOTE z: The outcome of this study may include architectural requirements that could influence the design of user plane protocols in stage 3 working groups.

**** End of Changes ****