3GPP TSG-WG SA2 Meeting #173	S2-260xxxx

Goa, IN, 9th Feb – 13th Feb, 2026	(revision of S2-260xxxx)

Source:	Huawei, HiSilicon

Title:	[KI#1 Conclusion] Dedicated Bearer Support

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract: Updated conclusions to clarify aspects about the EPS Bearers

# 1. Introduction/Discussion

The current conclusion for KI#1 does not describe how both IMS signalling and IMS voice within a single PDN connection is realised. At present this can be done with the traffic getting equal priority within the same EPS Bearer/DRB, or dedicated bearers could be supported.

In the status report to RAN (RP-253260) the following RAN2 assumption is documented:

3. In UP solution, RAN2 understands that voice data packets will be transmitted via UM DRB while the voice signalling packets will be transmitted via AM DRB (no intention to mandate this in the specification)

Given that a DRB cannot mix and match UM and AM modes, this implies that multiple DRBs will be used. In this case, coupled with the SA2 conclusion, a single PDN connection will be used, it means that dedicated bearers need to be supported.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-19.

# * * * * First change * * * *

## 8.2	Conclusions for Key Issue #1

Editor's note:	Other conclusions for Key Issue#1 are FFS.

The following conclusions for Key Issue #1 Support of IMS voice call over NB-IoT via GEO satellite connecting to EPC are made:

-	The voice packets shall be transported over the NB-IoT (GEO) user plane, i.e. using DRB and S1-U.

-	The IMS signalling shall be transported over the NB-IoT (GEO) user plane, i.e. using DRB and S1-U.

-	A single PDN connection shall be used to transport both IMS signalling and IMS voice:

-	A non-GBR default bearer for IMS signalling and a GBR dedicated bearer is used for IMS voice packets.

-	Dedicated bearer can be established for IMS voice packets by the MME by the UE using the MO Voice RRC establishment cause or UE requested bearer resource modification procedure.

-	The NB-IoT UE shall indicate support of multiple user plane radio bearers UE Core Network Capability, and shall indicate support dedicated bearers within the NB-IoT RAT(GEO) RAT.

# * * * * End of changes * * * *