3GPP TSG-WG SA2 Meeting #173	S2-2601455

Goa, IN, 9th Feb – 13th Feb, 2026	(revision of S2-2601190)

Source:	Huawei, HiSilicon, MediaTek Inc.

Title:	Conclusions for KI#2 IMS enhancement for GEO NB-IoT NTN access

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4-ARC / Rel-20

Abstract: This contribution provides conclusions for KI#2 IMS enhancement for GEO NB-IoT NTN access.

# 1. Introduction/Discussion

In TR 23.700-19 v1.1.0, a total of 13 solutions were proposed for Key Issue#2.

Among these 13 solutions only Sol#7 is CP based solution, based on the interim conclusion made in SA2#171 that “The voice packets shall be transported over the NB-IoT (GEO) user plane, i.e. using DRB and S1-U”, Sol#7 is not considered further in this paper.

Sol#27 focuses on P-CSCF re-selection with consideration of operator deployment, the other remaining 11 UP based solutions mainly aim to CST reduction and can be classified into two categories:

Cat1: CST reduction based on optimization of message size and call set up procedure (Sol#1, #12, #13, #14, #15, #16, #17, and #19).

Cat2: CST reduction based on I1（Sol#4，#18，#20）.

For Cat1 solutions, the solutions focus on the following aspects for what concerns the size and number of messages exchanged between P-CSCF and UE:

- Message size reduction

The following strategies have been proposed:

(i) Reduction of regular text-based SIP message: Sol#12, #15, #19

These solutions propose mechanisms to reduce SIP message composition by only including in the messages over Gm essential headers and enabling the P-CSCF to transition between complete message and simplified message by caching necessary information before call setup i.e. during IMS registration procedure.

(ii) SDP pre-configuration/pre-negotiation: Sol#16, #19

These two solutions propose doing SDP pre-configuration during IMS registration or by pre-call SIP messages for later SIP call to avoid SDP negotiation during call setup procedure.

Sol#19 in particular is moving the SDP negotiation (i.e. offer/answer) from the call setup procedure (i.e. SIP INVITE) to the IMS registration phase

(iii) Appling SigComp: Sol#12, #13, #16, #17

These solutions propose compression SIP messages by applying either SigComp alone or in combination with (i) and (ii)

(iv) Introducing binary encode format for messages between UE and P-CSCF: Sol#19

Sol#19 proposes Gm interface between UE and P-CSCF be enhanced to use a binary encoded format instead of clear text for SIP session management messages

(v) Non-IP PDN for IMS PDU session: Sol#19

Sol#19 proposes a solution without UDP/IP protocol from UE to PGW using a Non-IP PDN connection

- Call setup signalling messages reduction

(i) Disable precondition: Sol#12, #13, #14, #15, #17, #19

These solutions propose simplified call setup procedure without precondition as define in TS23.228, among which Sol#14 also propose SDP answer carried in 180 Ringing message based on baseline procedure without precondition.

(ii) Optimization on QoS authorization: Sol#1, #12, #13, #15, #17

Sol#1, #12, #13, #15 propose using either default bearer for both IMS signalling and voice data or pre-establishing dedicated bearer for voice data during PDN connection establishment procedure, while in Sol#17 P-CSCF supports early trigger QoS authorization and resource reservation to EPS upon receiving INVITE from/to UE.

Cat1 solutions also propose how to enable interworking between the UE engaged in voice over GEO over NB-IoT existing devices. The following approaches have been studied.

(i) P-CSCF acts as B2BUA: Sol#12, #14, #15, #19

In Sol#12, #14, #15, P-CSCF acts as B2BUA to support interoperation between simplified IMS signalling/shorten call setup procedure towards UE connected via NB-IoT GEO satellite and normal IMS signalling/complete call setup towards other UEs, In Sol#19, P-CSCF as SIP user agent performs translation between binary encoded SIP and text-based SIP.

(ii) IMS AS act as B2BUA: Sol#13, #17

In Sol#13, #17, IMS AS acts as B2BUA to support interoperation between simplified IMS signalling/shorten call setup procedure towards UE connected via NB-IoT GEO satellite and normal IMS signalling/complete call setup towards other UEs.

For Cat2: CST reduction based on I1（Sol#4，#18，#20）.

Sol#4 proposes to use IMS SCC-AS functionality defined in TS23.228 and TS23.292 and I1 protocol defined in TS24.294 for IMS Session Setup.

Sol#18 introduces extended I1 over IP/UDP between UE and AS for call setup signalling.

Sol#20 introduces binary encoding of SIP over UNI using the framework for I1 encoding, the solution is I1-like one and a UE proxy functionality in UE/P-CSCF support for interworking during IMS call related procedures to perform transit binary encoded SIP to text-based SIP encoding.

Based on majority of above solution summary, we propose below items as interim conclusions:

-	SigComp, SIP header reduction are the baseline for SIP message size reduction.

-	Shorten call setup procedure without pre-condition is applied between UE connected via NB-IoT GEO satellite and IMS.

-	B2BUA is introduced to support interoperation between simplified IMS signalling/shorten call setup procedure between UE connected via NB-IoT GEO satellite and normal IMS signalling/complete call setup for peer UE.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-19.

# * * * * First change (all new) * * * *

## 8.X	Conclusions for Key Issue #2

The following conclusions for Key Issue #2: IMS enhancement for GEO NB-IoT NTN access are made with regards to text based SIP signalling:

-	Text based SIP signalling support is mandatory in the UE and IMS

-	Compliance to RFC 3261 [18] shall be maintained for all SIP messages towards the remote party

-	Text-based SIP signalling transmission shall be based on IP/UDP.

-	When registering to IMS, the UE uses existing procedures specified in TS 23.228 [6]

-	For the reduction of SIP message size it is agreed that

- SigComp [20] may be used,

-	SDP fields which are static and not mandatory such as attribute for media session for IMS voice call over NB-IoT(GEO) are omitted from the SDP exchanged over NB-IoT(GEO) and restored based on the pre-defined values.

NOTE 1: Mandatory SDP fields are formatted in compliance to RFC 4566 [19].

NOTE 2: SDP fields that can be omitted and their pre-defined SDP value will be defined by SA4.

-	For the reduction of call setup time it is concluded to:

-	adopt a call setup procedure without pre-condition.

-	use a B2BUA to support interoperation with IMS peers.

NOTE 3: The Stage 3 details of optimised text based SIP signalling will be defined by CT1.

# * * * * End of changes * * * *