3GPP TSG-WG SA2 Meeting #173	S2-260xxxx

Goa, IN, 9th Feb – 13th Feb, 2026	(revision of S2-2510563)

Source:	Huawei, HiSilicon

Title:	Conclusions for KI#2 IMS enhancement for GEO NB-IoT NTN access

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4-ARC / Rel-20

Abstract: This contribution provides conclusions for KI#2 IMS enhancement for GEO NB-IoT NTN access.

1. Introduction/Discussion

In TR 23.700-19 v1.1.0, a total of 13 solutions were proposed for Key Issue#2.

Among these 13 solutions only Sol#7 is CP based solution, based on the interim conclusion made in SA2#171 that “The voice packets shall be transported over the NB-IoT (GEO) user plane, i.e. using DRB and S1-U”, Sol#7 is not involved in this paper.

Sol#27 focuses on P-CSCF re-selection with consideration of operator deployment, the other remaining 11 UP based solutions mainly aim to CST reduction and can be classified into two categories:

Cat1: CST reduction based on optimization of message size and call set up procedure (Sol#1, #12, #13, #14, #15, #16, #17, and #19).

Cat2: CST reduction based on I1（Sol#4，#18，#20）.

For Cat1 solutions, it can be summarized from below 3 aspects:

- How to reduce message size

(i) Slimming of regular text-based SIP message: Sol#12, #15, #19

These solutions propose mechanisms to reduce SIP message composition by only includes essential headers and P-CSCF performs transition between complete message and simplified message by caching necessary information before call setup i.e. during IMS registration procedure.

(ii) SDP pre-configuration/pre-negotiation: Sol#16, #19

These two solutions propose doing SDP pre-configuration during IMS registration or by pre-call SIP messages for later SIP call to avoid SDP negotiation during call setup procedure.

Sol#19: moving the SDP negotiation (i.e. offer/answer) from the call setup procedure (i.e. SIP INVITE) to the IMS registration phase

(iii) Appling SigComp: Sol#12, #13, #16, #17

These solutions propose compression SIP messages by applying either SigComp alone or in combination with (i) and (ii)

(iv) Introducing binary encode format for messages between UE and P-CSCF: Sol#19

Sol#19 proposes Gm interface between UE and P-CSCF be enhanced to use a binary encoded format instead of clear text for SIP session management messages

(v) Non-IP PDN for IMS PDU session: Sol#19

Sol#19 proposes a solution without UDP/IP protocol from UE to PGW using a Non-IP PDN connection

- How to reduce the number of call setup signalling

(i) Disable precondition: Sol#12, #13, #14, #15, #17, #19

These solutions propose simplified call setup procedure without precondition as define in TS23.228, among which Sol#14 also propose SDP answer carried in 180 Ringing message based on baseline procedure without precondition.

(ii) Optimization on QoS authorization: Sol#1, #12, #13, #15, #17

Sol#1, #12, #13, #15 propose using either default bearer for both IMS signalling and voice data or pre-establishing dedicated bearer for voice data during PDN connection establishment procedure, while in Sol#17 P-CSCF supports early trigger QoS authorization and resource reservation to EPS upon receiving INVITE from/to UE.

- How to interoperate with legacy mechanism

(i) P-CSCF acts as B2BUA: Sol#12, #14, #15, #19

In Sol#12, #14, #15, P-CSCF acts as B2BUA to support interoperation between simplified IMS signalling/shorten call setup procedure towards UE connected via NB-IoT GEO satellite and normal IMS signalling/complete call setup towards other UEs, In Sol#19, P-CSCF as SIP user agent performs translation between binary encoded SIP and text-based SIP.

(ii) IMS AS act as B2BUA: Sol#13, #17

In Sol#13, #17, IMS AS acts as B2BUA to support interoperation between simplified IMS signalling/shorten call setup procedure towards UE connected via NB-IoT GEO satellite and normal IMS signalling/complete call setup towards other UEs.

For Cat2: CST reduction based on I1（Sol#4，#18，#20）.

Sol#4 proposes to use IMS SCC-AS functionality defined in TS23.228 and TS23.292 and I1 protocol defined in TS24.294 for IMS Session Setup.

Sol#18 introduces extended I1 over IP/UDP between UE and AS for call setup signalling.

Sol#20 introduces binary encoding of SIP over UNI using the framework for I1 encoding, the solution is I1-like one and a UE proxy functionality in UE/P-CSCF support for interworking during IMS call related procedures to perform transit binary encoded SIP to text-based SIP encoding.

Based on majority of above solution summary, we propose below items as interim conclusions:

-	SigComp, SIP header slimming and SDP precondition are as the baseline for SIP message size reduction.

-	Shorten call setup procedure without pre-condition is applied between UE connected via NB-IoT GEO satellite and IMS.

-	B2BUA is introduced to support interoperation between simplified IMS signalling/shorten call setup procedure towards UE connected via NB-IoT GEO satellite and normal IMS signalling/complete call setup for peer UE.

-	Using dedicate bearer for IMS voice data and which can be pre-established before the call.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-19.

# * * * * First change (all new) * * * *

8.X	Conclusions for Key Issue #2

The following interim conclusions for Key Issue #2: IMS enhancement for GEO NB-IoT NTN access are made:

-	For the reduction of SIP message size it is agreed to enable SigComp [20], perform SIP header reduction as described in  (solution #12, #15, #19)as the baseline.

-	For the reduction of call setup time it is concluded to

-	adopt a call setup procedure without pre-condition.

-	introduce a B2BUA  to support interoperation with normal IMS signalling/complete call setup for peer UE

-	SDP fields for IMS voice call over NB-IoT NTN is pre-configured.

Editor’s Note: which SDP fields are pre-configured and how to restore the pre-configured value is FFS.

-

# * * * * End of changes * * * *