3GPP TSG-WG SA2#173	S2-2XXX

# Goa, India, 9 – 13 February, 2026

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | ? | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of User Plane Traffic Pattern Analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
|  | 5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN_Ph2 |  |  |  |  |  | Date: |  |  | 2026-01-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As documented in 8.1.3 of TR 23.700-04: the following conclusions for KI#2 Use Case#2 will be supported by normative work: - A new analytics ID will be defined to support UC#2. - The service consumer of the analytics service is PCF. - The NWDAF may provide the output analytics on traffic pattern information to the consumer. - To derive the output analytics, the following input data may be collected from the UPF/SMF - The analytics consumer may include the Analytics Filter Information in the request, such as application description, UE ID/address, DNN and S-NSSAI, etc. - To reduce UPF load of data reporting, the NWDAF may configure thresholds for traffic characteristics in event subscription. The UPF may only report data when the thresholds are exceeded. Also as documented in 8.1.1 of TR 23.700-04: To reduce the reporting load of input data sources (e.g. UPF): - The analytics consumer may include the Analytics Filter Information (as described in clause 6.1.3 of TS 23.288 [5]). The NWDAF may derive related parameters for the event subscription towards the UPF or SMF for input data filtering. - The NF consumer supports the provisioning of the skip reporting instruction information in the subscription request, then the UPF do not send the event reports if they meet the skip reporting instruction, or UPF may bundle event reports of multiple subscriptions in a same Notify request, see TS 29.564 [17]. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a new Analytics ID to support User Plane Traffic Pattern Analytics based on conclusions for KI#2 UC#2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The conclusions of SI cannot be supported by R20 specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x(new), 6.x.1 (new), 6.x.2 (new), 6.x.3 (new), 6.x.4 (new), 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * *Start of Changes (new text) * * *

## 6.x	User Plane Traffic Pattern Analytics

### 6.x.1	General

Clause 6.x describes how NWDAF can provide statistics and predictions of User Plane Traffic Pattern Analytics, based on the consumer request. The consumer can either subscribe to analytics notifications (i.e. a Subscribe-Notify model) or request a single notification (i.e. using a Request-Response model).

The service consumer may be a PCF.

The User Plane Traffic Pattern Analytics provides statistics and predictions of the User Plane Traffic Pattern, including the traffic pattern related information (e.g. Maximum Burst Size, Maximum Bitrate, Guaranteed Bitrate, 5GS delay, Packet Error Rate, Periodicity, etc.) of traffic flows, the associated information of the traffic flows, and the identifier(s) of the affected UE(s). The consumer (i.e. PCF) may take the statistics and prediction into account to generate or update PCC rules by adjusting QoS of the associated traffic flow to provide better service quality.

To reduce UPF load of data reporting for NWDAF data collection:

-	the analytics consumer may include the Analytics Filter Information (as described in clause 6.1.3 and detailed below), based which the NWDAF may derive the related parameters for the event subscription towards the UPF or SMF for input data filtering.

-	The NWDAF may configure the reporting thresholds for traffic characteristics in event subscription. Then the UPF may only report data when the thresholds are exceeded.

-	The consumer may also provision the skip and/or combine reporting instruction information (if supported) in the subscription request for NWDAF data collection. Then the UPF do not send the event reports if they meet the skip reporting instruction or UPF may bundle event reports of multiple subscriptions in a same Notify request (see TS 29.564 [51]).

The analytics consumer may indicate the following information in the request or subscription:

-	Analytics ID = " User Plane Traffic Pattern Analytics".

-	Target of Analytics Reporting as defined in clause 6.1.3.

-	Optional, instruction information for data sources (e.g. UPF) to skip and/or bundle event reports for reporting

-	Analytics Filter Information optionally includes:

-	DNN;

-	S-NSSAI;

-	Area of Interest (AOI);

-	UE IDs or UE IP addresses;

-	Application description information, e.g. Application ID, Packet Filter Set, PDR, identifier of the service data flow(s);

-	List of analytics subsets that are requested, (see clause 6.x.3), e.g. information related to User Plane Traffic Pattern.  .

-	An Analytics target period indicates the time period over which the analytics are requested.

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

-	Preferred order of results for the list of traffic pattern related information of traffic flow(s):

-	ordering criterion: any of Maximum Burst Size, Maximum Bitrate, Guaranteed Bitrate, 5GS delay, Packet Error Rate, Periodicity;

-	order: ascending or descending.

-	Reporting Thresholds, which apply only for subscriptions and indicate conditions on the levels to be reached for the respective analytics subsets (see clause 6.x.3).

-	Maximum number of objects, e.g. UEs, applications or traffic flows.

### 6.x.2	Input Data

The NWDAF supporting User Plane Traffic Pattern Analytics shall be able to collect information from data sources, e.g. UPF, SMF, OAM.

Table 6.x.2-1: Input data from 5GC NF related to User Plane Traffic Pattern information

| Information | Source | Description |
| --- | --- | --- |
| Application ID | SMF | Identifies the application service provider of a traffic flow. |
| Packet filter | SMF | The packet filter of the traffic flow associated to the above application ID. |
| UE identifier | SMF | The identifier of UE that is associated to traffic flow, e.g. SUPI, IP address, etc. |
| Characteristics of traffic flow |  | The information or parameters related to the characteristics of the corresponding traffic flow. |
| > N6 delay | UPF | The observed N6 delay between the UPF and DN, as defined in clause 5.8.2.23 of TS 23.501 [2]. |
| > Packet delay | SMF/ UPF | The delay between UE and UPF generated by QoS monitoring, e.g. UL, DL, round trip delay. |
| > Congestion information | SMF/ UPF/ OAM | Indicate the congestion status in the UPF or RAN, for example the information related to the size of buffered data, or the number of queued packets. |
| > Packet drop | SMF/ UPF/ OAM | Indicate the number of dropped/lost packet and/or packet loss/drop rate at UPF or RAN of the flow. |
| > UPF info | SMF | Address or identifier of the serving UPF of the UE or traffic flow. |
| > Data volume | SMF/ UPF | Measured data volume of the flow, e.g. UL, DL. |
| > Periodicity | SMF/ UPF | It refers to the time period between start of two data bursts, as defined in clause 5.27 of TS 23.501 [2]. |
| > Packet interval | SMF/ UPF | The time interval between data packet, e.g. average, minimum, maximum and/or variance value. |
| > Time stamp | SMF/ UPF/ OAM | A time stamp associated to the above measurement data. |

### 6.x.3	Output Analytics

The NWDAF shall be able to provide the statistics and predictions on User Plane Traffic Pattern Analytics to consumer NF, i.e. PCF, as defined in Table 6.x.3-1 and Table 6.x.3-2.

Table 6.x.3-1: User Plane Traffic Pattern statistics

| Information | Description |
| --- | --- |
| User Plane Traffic Pattern (1…max) (NOTE 1) | List of User Plane Traffic Pattern information. Max. is the number of the user plane traffic flows, if applicable. |
| > service data flow ID | Identifies the user plane data flow(s). |
| > Application ID(s) | Identifies the application(s) that is associated to the user plane traffic flow. |
| > Packet Filter Set | Identifies the Packet Filter Set of the associated user plane traffic flow. |
| > UE ID(s) | Identifies the UE(s) associated to the user plane traffic flow, e.g. SUPI, IP address. |
| > DNN | DNN for the PDU Session that contains the traffic flow. |
| > User Plane Traffic Pattern statistics (NOTE 1) | The statistics of the information/ parameters related to the characteristics of the associated traffic flow. |
| >> Maximum Burst Size/ Maximum Data Burst Volume (NOTE 2) | The Maximum Data Burst Volume (MDBV) or Maximum Burst Size of GBR QoS Flow with Delay-critical resource. |
| >> Maximum Bitrate (NOTE 3) | The Maximum Flow Bit Rate (MFBR), e.g. UL and DL. |
| >> Guaranteed Bitrate (NOTE 3) | The Guaranteed Flow Bit Rate (GFBR), .e.g. UL and D. |
| >> 5GS delay | The 5GS delay (UL, DL) as defined in clause 6.1.3.22 of TS 23.503 [4] of the associated application(s). |
| >> Packet Error Rate | The Packet Error Rate (UL, DL) as defined in clause 6.1.3.22 of TS 23.503 [4] of the associated application(s). |
| >> Periodicity | The time period between start of two data bursts, as defined in clause 5.27 of TS 23.501 [2]. |
| >> Packet interval | The time interval between data packet, e.g. average, minimum, maximum and/or variance value. |
| >> Validity period | The validity period within the time slot for the analytics in clause 6.1.3. |
| >> Spatial validity | Area where the analytics applies. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested" and "Reporting Thresholds". NOTE 2: this applies to QoS Flows use a Delay-critical GBR resource type only. NOTE 3: this applies to GBR QoS Flows only. |  |

Table 6.x.3-2: User Plane Traffic Pattern prediction

| Information | Description |
| --- | --- |
| User Plane Traffic Pattern (1…max) (NOTE 1) | List of User Plane Traffic Pattern information. Max. is the number of the user plane traffic flows, if applicable. |
| > Service data flow ID | Identifies the user plane data flow(s). |
| > Application ID(s) | Identifies the application(s) that is associated to the user plane traffic flow. |
| > Packet Filter Set | Identifies the Packet Filter Set of the associated user plane traffic flow. |
| > UE ID(s) | Identifies the UE(s) associated to the user plane traffic flow, e.g. SUPI, IP address. |
| > DNN | DNN for the PDU Session that contains the traffic flow. |
| > User Plane Traffic Pattern statistics (NOTE 1) | The predictions of the information/ parameters related to the characteristics of the associated traffic flow. |
| >> Maximum Burst Size/ Maximum Data Burst Volume (NOTE 2) | The Maximum Data Burst Volume (MDBV) or Maximum Burst Size of GBR QoS Flow with Delay-critical resource. |
| >> Maximum Bitrate (NOTE 3) | The Maximum Flow Bit Rate (MFBR), e.g. UL and DL. |
| >> Guaranteed Bitrate (NOTE 3) | The Guaranteed Flow Bit Rate (GFBR), .e.g. UL and D. |
| >> 5GS delay | The 5GS delay (UL, DL) as defined in clause 6.1.3.22 of TS 23.503 [4] of the associated application(s). |
| >> Packet Error Rate | The Packet Error Rate (UL, DL) as defined in clause 6.1.3.22 of TS 23.503 [4] of the associated application(s). |
| >> Periodicity | The time period between start of two data bursts, as defined in clause 5.27 of TS 23.501 [2]. |
| >> Packet interval | The time interval between data packet, e.g. average, minimum, maximum and/or variance value. |
| >> Validity period | The validity period within the time slot for the analytics in clause 6.1.3. |
| >> Spatial validity | Area where the analytics applies. |
| > Confidence | Confidence of this prediction. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested" and "Reporting Thresholds". NOTE 2: this applies to QoS Flows use a Delay-critical GBR resource type only. NOTE 3: this applies to GBR QoS Flows only. |  |

### 6.x.4	Procedures

Figure 6.x.4 shows the procedure for the NWDAF to derive and provide the QoS and policy assistance analytics to a consumer 5GC NF (i.e. PCF).

Figure 6.x.4-1: Procedure for User Plane Traffic Pattern Analytics

1.	The Consumer NF, i.e. PCF, requests or subscribes to User Plane Traffic Pattern Analytics and provides the input information as specified in clause 6.x.1 to NWDAF.

2a.	The NWDAF may subscribe to the service data from 5GC NFs, e.g. SMF, UPF, etc. as defined in clause 6.x.2 by invoking the corresponding NF event exposure service operation, e.g. using Nsmf_EventExposure_Subscribe service operation and/or Nupf_EventExposure_Subscribe (Event ID, parameters related to characteristics of traffic flow, packet filter information, SUPI(s) or Application ID), etc.

If individual UEs are targeted, the NWDAF subscribes at the SMF to obtain related information from the UPF.

2b.	The NWDAF collects data from the OAM, following the procedure captured in clause 6.2.3.2.

3.	The NWDAF derives the requested analytics on User Plane Traffic Pattern Analytics based on NWDAF internal logic and based on operator policies.

4.	The NWDAF provides the requested User Plane Traffic Pattern Analytics to the consumer NF, using either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service operation used in step 1.

5-7.	If the consumer NF subscribes to User Plane Traffic Pattern Analytics in step 1, once the NWDAF generates new analytics on User Plane Traffic Pattern, it provides notification using Nnwdaf_AnalyticsSubscription_Notify to the Consumer NF.

* * *Next Change * * *

## 7.1	General

Table 7.1-1 illustrates the NWDAF Services.

Table 7.1-1: NF services provided by NWDAF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnwdaf_AnalyticsSubscription | Subscribe | Subscribe / Notify | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, LMF |
|  | Unsubscribe |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, LMF |
|  | Notify |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, MFAF, LMF |
|  | Transfer | Request / Response | NWDAF |
| Nnwdaf_AnalyticsInfo | Request | Request / Response | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, LMF |
|  | ContextTransfer | Request / Response | NWDAF |
| Nnwdaf_DataManagement | Subscribe | Subscribe / Notify | NWDAF, DCCF |
|  | Notify |  | NWDAF, DCCF, MFAF, ADRF |
|  | Fetch | Request / Response | NWDAF, DCCF, MFAF, ADRF |
| Nnwdaf_MLModelProvision | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
| Nnwdaf_MLModelInfo | Request | Request / Response | NWDAF |
| Nnwdaf_MLModelMonitor | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
|  | Register | Request / Response | NWDAF |
|  | Request |  | NWDAF |
| Nnwdaf_MLModelTraining | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
| Nnwdaf_MLModelTrainingInfo | Request | Request / Response | NWDAF |
| Nnwdaf_RoamingAnalytics | Subscribe | Subscribe / Notify | H-RE-NWDAF, V-RE-NWDAF |
|  | Unsubscribe |  | H-RE-NWDAF, V-RE-NWDAF |
|  | Notify |  | H-RE-NWDAF, V-RE-NWDAF |
|  | Request | Request / Response | H-RE-NWDAF, V-RE-NWDAF |
| Nnwdaf_RoamingData | Subscribe | Subscribe / Notify | H-RE-NWDAF, V-RE-NWDAF |
|  | Unsubscribe |  | H-RE-NWDAF, V-RE-NWDAF |
|  | Notify |  | H-RE-NWDAF, V-RE-NWDAF |
| NOTE 1: How OAM consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.550 [7] Annex H and out of the scope of this TS. NOTE 2: How CEF consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.201 [21] and out of the scope of this TS. NOTE 3: The Nnwdaf_MLModelProvision service and the Nnwdaf_MLModelInfo service are provided by an NWDAF containing MTLF and consumed by an NWDAF containing AnLF or provided by an NWDAF containing MTLF supporting FL as a server and consumed by an NWDAF containing MTLF. |  |  |  |

Table 7.1-2 shows the analytics information provided by NWDAF service.

Table 7.1-2: Analytics information provided by NWDAF

| Analytics Information | Request Description | Response Description |
| --- | --- | --- |
| Slice Load level information | Analytics ID: load level information | Load level provided as number of UE registrations and number of PDU sessions for a Network Slice and Network Slice instances as well as resource utilization for Network Slice instances. |
| Observed Service experience information | Analytics ID: Service Experience | Observed Service experience statistics or predictions may be provided for a Network Slice or an Application. They may be derived from an individual UE, a group of UEs or any UE. For slice service experience, they may be derived from an Application, a set of Applications or all Applications on the Network Slice. |
| NF Load information | Analytics ID: NF load information | Load statistics or predictions information for specific NF(s). |
| Network Performance information | Analytics ID: Network Performance | Statistics or predictions on the load in an Area of Interest; in addition, statistics or predictions on the number of UEs that are located in that Area of Interest. |
| UE mobility information | Analytics ID: UE Mobility | Statistics or predictions on UE mobility. When visited AOI(s) is included in the Analytics Filter information, only statistics on UE mobility can be provided. |
| UE Communication information | Analytics ID: UE Communication | Statistics or predictions on UE communication. |
| Expected UE behavioural parameters | Analytics ID: UE Mobility and/or UE Communication | Analytics on UE Mobility and/or UE Communication. |
| UE Abnormal behaviour information | Analytics ID: Abnormal behaviour | List of observed or expected exceptions, with Exception ID, Exception Level and other information, depending on the observed or expected exceptions. |
| End-to-end data volume transfer time | Analytics ID: E2E data volume transfer time | Analytics on E2E data volume transfer time. |
| User Data Congestion information | Analytics ID: User Data Congestion | Statistics or predictions on the user data congestion for transfer over the user plane, for transfer over the control plane, or for both. |
| QoS Sustainability | Analytics ID: QoS Sustainability | For statistics, the information on the location and the time for the QoS change and the threshold(s) that were crossed; or, for predictions, the information on the location and the time when a potential QoS change may occur and what threshold(s) may be crossed. |
| Session Management Congestion Control Experience | Analytics ID: Session Management Congestion Control Experience | Statistics on session management congestion control experience for specific DNN and/or S-NSSAI. |
| Redundant Transmission Experience | Analytics ID: Redundant Transmission Experience | Statistics or predictions aimed at supporting redundant transmission decisions for URLLC services. |
| WLAN performance | Analytics ID: WLAN performance | Statistics or predictions on WLAN performance of UE. |
| Dispersion | Analytics ID: UE Dispersion | Statistics or predictions that identify the location (i.e. areas of interest) or network slice(s) where a UE, or a group of UEs disperse their data volume, or disperse mobility or session management transactions or both. |
| DN Performance | Analytics ID: DN Performance | Statistics or predictions on user plane performance for a specific Edge Computing application. |
| PFD Determination | Analytics ID: PFD Determination | Statistics on PFD information for a known application identifier(s). |
| Movement Behaviour | Analytics ID: Movement Behaviour | Statistics or predictions on movement behaviour for an applicable area. |
| Location Accuracy | Analytics ID: Location Accuracy | Predictions on Location Accuracy. |
| Relative Proximity | Analytics ID: Relative Proximity | Statistics or predictions on Relative Proximity among UEs. |
| PDU Session traffic | Analytics ID: PDU Session traffic | Statistics on whether traffic of UEs via one or multiple PDU sessions is according to the information provided by the service consumer. |
| User Plane Traffic Pattern | Analytics ID: User Plane Traffic Pattern | Analytics on statistics and predictions on the characteristics of user plane traffic pattern. |

* * *End of Changes * * *