3GPP TSG-WG SA2#173	S2-260xxxx

# Goa, India, 9 – 13 February, 2026

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | xxxx | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New analytics ID on Abnormal User Plane Traffic |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN_Ph2 |  |  |  |  |  | Date: |  |  | 2026-01-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per conclusions for KI#2 UC#1 in TR 23.700-04 (Section 8.1.2): - A new Analytics ID "Abnormal user plane traffic" will be introduced for UC#1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define Abnormal User Plane Traffic analytics |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Abnormal User Plane Traffic analytics is not defined |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

## 6.24	Abnormal User Plane Traffic Analytics

### 6.24.1	General

This clause specifies analytics for abnormal user plane traffic.

The abnormal user plane traffic analytics provides analytics information (statistics or predictions) regarding abnormal user plane traffic. Abnormal user plane traffic refers to traffic patterns that significantly deviate from expected or baseline behavior. This can include, for example, unexpected traffic volume and/or rate, traffic anomalies associated with security threats (such as DDoS), unexpected traffic bursts, unexpected UE wake-up, and an unexpected number of malformed, duplicate, retransmitted or fragmented packets.

The output of the analytics of abnormal user plane traffic is basically statistics or predictions of (observed or predicted) abnormal traffic pattern(s) and associated information (e.g. IP packet filter(s) and/or Application ID(s) as information on the traffic source; UE(s), PDU Session(s) and/or UPF(s) affected on a per observed or predicted abnormal traffic pattern).

The NWDAF provides statistics and/or prediction analytics to consumer NFs, e.g. SMF, PCF, UPF and AF.

The consumer of these analytics indicates in the request or subscription:

-	Analytics ID = "Abnormal User Plane Traffic";

-	Target of Analytics Reporting as defined in clause 6.1.3;

-	An Analytics target period indicates the time period over which the statistics or predictions are requested;

-	Analytics Filter Information optionally including:

-	list of abnormal traffic pattern ID, where each abnormal traffic pattern ID refers to a user plane traffic abnormality of interest, e.g. unexpected traffic volume and/or rate, traffic anomalies associated with security threats (such as DDoS), unexpected traffic bursts, unexpected UE wake-up, and an unexpected number of malformed, duplicate, retransmitted or fragmented packets;

-	Area of interest;

-	Application ID;

-	DNN;

-	S-NSSAI.

### 6.24.2	Input data

The NWDAF supporting abnormal user plane traffic analytics shall be able to collect information from UPF, SMF and optionally from AF, OAM and NRF.

NWDAF triggers data collection from UPF (and SMF) to retrieve user plane traffic related parameters and abnormalities. The detailed input data is described in Table 6.24.2-1.

Table 6.24.2-1: Input data

| Information | Source | Description |
| --- | --- | --- |
| UE ID (NOTE 1) | SMF, UPF | Identifier of the UE (e.g. SUPI) |
| UE IP Address | SMF, UPF | UE IP address |
| S-NSSAI | SMF, UPF | Network Slice |
| DNN | SMF, UPF | Data Network Name |
| PDU Session ID | SMF | PDU Session identifier |
| numSessRep | SMF | Number of received Session Report from UPF triggered by DL packet in case of PDU Session is in 5GCM-idle state |
| UPF ID | UPF | IP address or FQDN of the UPF |
| Application ID | UPF | Identifies the application |
| Traffic Abnormality ID (1..max) | UPF | Identifies the traffic abnormality |
| Application Traffic Flow Information (1..max) (NOTE 2) | UPF | Per Application Traffic flow related information |
| > IP 5-tuple | UPF | Identifies a service flow of the UE that uses the application |
| > Start time | UPF | Start time of traffic detection for the flow |
| > End time | UPF | End time of traffic detection for the flow |
| > UL Data volume | UPF | Measured UL data traffic volume for the flow |
| > DL Data volume | UPF | Measured DL data traffic volume for the flow |
| > UL Data Rate | UPF | Measured UL data rate for the flow |
| > DL Data Rate | UPF | Measured DL data rate for the flow |
| > URL list | UPF | List of URLs extracted from the inspected user plane packets in the flow |
| > Domain Name list | UPF | List of domain names extracted from the inspected user plane packets in the flow |
| > UL Traffic Bursts | UPF | Number of detected UL traffic bursts in the flow |
| > DL Traffic Bursts | UPF | Number of detected DL traffic bursts in the flow |
| > UL Malformed packets | UPF | Number of UL malformed packets in the flow. |
| > DL Malformed packets | UPF | Number of DL malformed packets in the flow |
| > UL Duplicate packets | UPF | Number of UL duplicate packets in the flow |
| > DL Duplicate packets | UPF | Number of DL duplicate packets in the flow |
| > UL Retransmitted packets | UPF | Number of UL retransmitted packets in the flow |
| > DL Retransmitted packets | UPF | Number of DL retransmitted packets in the flow |
| > UL Fragmented packets | UPF | Number of UL fragmented packets in the flow |
| > DL Fragmented packets | UPF | Number of DL fragmented packets in the flow |
| > Per flow stats | UPF | Statistics e.g. average packet size, average packet inter-arrival, etc |
| NOTE 1: The event subscription by NWDAF to UPF can target any UE or specific UEs and provides information about targeted traffic abnormalities and associated thresholds. When the NWDAF requests to report traffic abnormalities, then the UPF only reports when the corresponding traffic abnormalities are detected and the associated threshold(s) are exceeded. NOTE 2: Application Traffic Flow Information is only reported by UPF for those flows where some abnormality has been detected (e.g. based on thresholds provided by consumer or configured by operator e.g. at UPF). |  |  |

NWDAF might also trigger data collection (e.g. from OAM, NRF, UPF) to retrieve information relative to UPF load via existing mechanisms.

NWDAF might also trigger data collection from AF to retrieve traffic characteristics of normal traffic (this can be used by the NWDAF to determine whether the input traffic is abnormal traffic by comparing the characteristics of input traffic with the characteristics of normal traffic). Alternatively, this might be configured by the operator (e.g. in NWDAF).

### 6.24.3	Output Analytics

The abnormal user plane traffic analytics result provided by the NWDAF is defined in Table 6.24.3-1 and Table 6.24.3-2.

Abnormal user plane traffic statistics information is defined in Table 6.24.3-1.

Table 6.24.3-1: Abnormal user plane traffic statistics

| Information | Description |
| --- | --- |
| Abnormalities (1..max) | List of observed abnormalities (abnormal user plane traffic) |
| > Abnormal traffic pattern ID | The identifier of the observed abnormality |
| > Severity level | Scalar value indicating the severity of the observed abnormality |
| > Trend | Measured trend (up/down/unknown/stable) of the observed abnormality |
| > S-NSSAI | Network Slice associated to the observed abnormality |
| > DNN | Data Network Name associated to the observed abnormality |
| > UE list (1..max) | List of UEs (SUPIs) affected by the observed abnormality. When the consumer requests the analytics for “any UE”, this information is not present. |
| > Ratio | Estimated percentage of UEs affected by the observed abnormality within the Target of Analytics Reporting |
| > List of UPF Instance ID | List of UPFs affected by the observed abnormality |
| > Application traffic info (1..max) | Application traffic related information associated to the observed abnormality |
| >> Application ID list (0..max) | (If available) Application ID(s) affected by the observed abnormality |
| >> IP packet filter set (1..max) | List of IP packet filters (e.g. 3-tuples) affected by the observed abnormality |
| > Time slot information (1..max) | List of time slots associated to the observed abnormality |
| >> Time slot start | Time slot start |
| >> Duration | Duration of the time slot |

Abnormal user plane traffic predictions information is defined in Table 6.24.3-2.

Table 6.24.3-2: Abnormal user plane traffic predictions

| Information | Description |
| --- | --- |
| Abnormalities (1..max) | List of predicted abnormalities (abnormal user plane traffic) |
| > Abnormal traffic pattern ID | The identifier of the predicted abnormality |
| > Severity level | Scalar value indicating the severity of the predicted abnormality |
| > Trend | Measured trend (up/down/unknown/stable) of the predicted abnormality |
| > S-NSSAI | Network Slice associated to the predicted abnormality |
| > DNN | Data Network Name associated to the predicted abnormality |
| > UE list (1..max) | List of UEs (SUPIs) affected by the predicted abnormality. When the consumer requests the analytics for “any UE”, this information is not present. |
| > Ratio | Estimated percentage of UEs affected by the predicted abnormality within the Target of Analytics Reporting |
| > List of UPF Instance ID | List of UPFs affected by the predicted abnormality |
| > Application traffic info (1..max) | Application traffic related information associated to the predicted abnormality |
| >> Application ID list (0..max) | (If available) Application ID(s) affected by the predicted abnormality |
| >> IP packet filter set (1..max) | List of IP packet filters (e.g. 3-tuples) affected by the predicted abnormality |
| > Time slot information (1..max) | List of time slots associated to the predicted abnormality |
| >> Time slot start | Time slot start |
| >> Duration | Duration of the time slot |
| > Confidence | Confidence of this prediction |

Abnormal traffic pattern ID can be unexpected traffic volume and/or rate, traffic anomalies associated with security threats (such as DDoS), unexpected traffic bursts, unexpected UE wake-up, and an unexpected number of malformed, duplicate, retransmitted or fragmented packets, or operator specific abnormal traffic pattern ID(s).

### 6.24.4	Procedures

Figure 6.24.4-1 shows the procedure for deriving and providing the abnormal user plane traffic analytics to a consumer (e.g. SMF).

Figure 6.24.4-1: Procedure for analytics on abnormal user plane traffic

1	Consumer (e.g. PCF, SMF) subscribes to NWDAF analytics on abnormal user plane traffic, by triggering Nnwdaf_AnalyticsSubscription_Subscribe request message including as analytics identifier ("Abnormal User Plane Traffic"), including as analytic filter a list of user plane traffic data abnormalities of interest (abnormal traffic pattern ID) e.g. unexpected traffic volume and/or rate, traffic anomalies associated with security threats (such as DDoS), unexpected traffic bursts, unexpected UE wake-up, and an unexpected number of malformed, duplicate, retransmitted or fragmented packets, the DNN, S-NSSAI and/or Area of Interest, as analytic target a list of SUPIs, a group of SUPIs or any UE, and this either for the traffic of the whole PDU session or for certain application identifier(s).

2	NWDAF triggers data collection, e.g. from SMF to retrieve session parameters, and e.g. from OAM, NRF, UPF to retrieve information relative to UPF load via existing mechanisms.

3	NWDAF triggers data collection from UPF to retrieve user plane traffic parameters and abnormalities, by triggering a Nupf_EventExposure_Subscribe request message, including e.g. as eventType="USER_DATA_ABNORMALITIES", and a list of user plane traffic data abnormalities of interest (traffic abnormality ID), and for each optionally a threshold, e.g. to report only when the percentage of fragmented packets goes over a certain threshold (when the threshold is included it is assumed to be locally configured at NWDAF; when not included, it is assumed the threshold to be locally configured at UPF), and the target SUPI(s) and application identifier(s).

NOTE: To reduce the reporting load from UPF, the NF event consumer (NWDAF) might support the provisioning of the skip reporting instruction information in the subscription request, then the UPF does not send the event reports if they meet the skip reporting instruction, or UPF may bundle event reports of multiple subscriptions in a same Notify request, see TS 29.564.

4	UPF collects data for the above event, specifically the user plane traffic related parameters and abnormalities of interest: metrics or KPIs like volume, data rate, etc and/or packet parameters like 5-tuple(s), SNI(s), domain name(s), etc and/or classification related information, if available, like application identifier(s), traffic burst(s), malformed, duplicate, retransmitted and/or fragmented packets.

5	UPF notifies NWDAF by triggering a Nupf_EventExposure_Notify request message, including the eventType=" USER_DATA_ABNORMALITIES " and the event information, specifically the user plane traffic parameters and abnormalities indicated above (when the corresponding thresholds are surpassed).

6	NWDAF runs analytics based on collected data and derives the analytics result, specifically a list of Abnormal traffic pattern ID corresponding to observed or predicted abnormal user plane traffic, e.g. when the number and/or percentage of fragmented packets is over the threshold, indicating for which UE(s), application identifier(s) and/or server IP addresses it applies to. NWDAF can also determine and indicate to the consumer if and how the abnormal user plane traffic has any impact on the UPF load.

7	NWDAF notifies the analytic consumer by triggering Nnwdaf_AnalyticsSubscription_Notify request message including the analytics identifier ("Abnormal User Plane Traffic") and the analytics result. Based on the analytics result, the consumer can apply actions to improve user plane performance, e.g. to avoid abnormal traffic.

Table 6.24.4-1: Example action of the consumer

| Consumer | Example of actions |
| --- | --- |
| UPF | - Downlink traffic suppression (e.g., selective packet dropping or rate limitations). - Adjust the packet processing resources. NOTE 1 |
| SMF | - UPF reselection to distribute the load across UPF instances - Configuring UPF to enforce the downlink traffic suppression (e.g., selective packet dropping or packet per second limitations) or shape abnormal traffic, to enforce bandwidth limitations. For each observed traffic anomaly that can be reported by the NWDAF, corresponding mitigation actions are configured in the SMF. Then, the SMF provides them to the UPF. The SMF may use actions on non-PDU session level applicable to any PDU session, via node rules in a N4 Node level procedure where SMF instructs UPF to apply actions (e.g. block or shape) on data flows that match the provided packet filters. NOTE 2 |
| PCF | - Policy creation or update and provisioning to SMF, e.g. executing traffic gating or shaping, enforcing bandwidth parameters (e.g. rate limiting) or adjusting QoS parameters. |
| AF | - Correct abnormal traffic. |
| NOTE 1: When UPF is consumer, the subscription initiation follows the "Subscribe-Notify" as in clause 7.1.2 of TS 23.501 [2] (Figure 7.1.2-3). In this case, the SMF subscribes to NWDAF services on the new Analytics ID on abnormal data packets patterns on behalf of the UPF and indicates that reduced output is requested, and the NWDAF can send directly the analytics to the UPF for activation and deactivation of the related rules in the UPF for the abnormal data packet handling, such as enforcing the downlink traffic suppression (e.g., selective packet dropping or packet per second limitations) or adjust the packet processing resources.UPF will only receive Analytics with reduced Output (i.e. type of abnormal traffic and traffic descriptor). UPF has operator-configured policies corresponding to the types of abnormal traffic, that can be activated for the received traffic descriptor. This does not apply on PDU-session level. NOTE 2: Any actions that are provisioned via the node level procedure shall be taken before the normal PDR packet handling. |  |

UPF can detect abnormal traffic. When the abnormal traffic is detected, the UPF can take actions to improve user plane performance using existing PDRs, and associated N4 rules such as QER or FAR, this is independent on the use of analytics. This can be done either by detection of abnormal traffic at UPF with reporting to SMF that results in related N4 rules or directly detection and enforcement of N4 rules at the UPF.

#### 6.24.4.1	Provisioning of N4 rules to take mitigation actions on node level

Figure 6.24.4.1-1 depicts the procedure for the example Use Case of mitigation actions on a per node level basis, which applies to the case when the consumer (i.e. SMF) subscribes/requests the AnalyticsID for any UE, and e.g. when an Abnormal traffic pattern ID is reported with Severity level indicating high severity, and the Ratio (as estimated percentage of UEs affected by the Exception within the Target of Analytics Reporting) is high.

NOTE:	Not depicted in Figure 6.24.4.1-1, but in the case that the consumer (SMF) subscribes/requests the AnalyticsID for a list of SUPIs or for a group of SUPIs, it is assumed the mitigation actions are to be applied on a per individual SUPI basis (not per node), e.g. when an Abnormal traffic pattern ID is reported with Severity level indicating high severity, and for the SUPIs included in the PDU Session list in the analytics output, as such the procedure in clause 6.24.4 applies.

Figure 6.24.4.1-1: Procedure for mitigation actions on a per node level basis

1	Consumer (SMF) subscribes to NWDAF on AnalyticsID related to "Abnormal User Plane Traffic" for any UE. This step corresponds to steps 1-6 in Figure 6.24.4.1-1.

2	NWDAF sends the notification report(s) according to the subscription. The report includes (as analytic output) the Abnormal traffic pattern ID(s) and the (if available) application ID(s) and/or packet filter(s) of the source of the traffic causing the exception(s). This step corresponds to step 7 in Figure 6.24.4-1.

3	Based on analytics output received from NWDAF, SMF decides to trigger mitigation actions towards UPF via a new PFCP per Node procedure.

4	SMF triggers a N4 PFCP Node Traffic Control Request message including the following information as node rules:

-	Application ID/IP Packet Filter Set: Indicates the traffic for which to apply the actions on a per N4 level (i.e. for all UE´s PFCP sessions).

-	Action (drop, throttle, inhibit DLDR): Indicates the action(s) to be applied to the traffic matching the rules above.

5	UE #1 transmits/receives user plane traffic passing through UPF.

6	Based on the information (node rules) received in step 4 above, UPF matches traffic (irrespective of the UE) based on Application ID/IP Packet Filter Set and applies the corresponding action(s):

-	Drop: Block traffic matching Application ID/IP Packet Filter Set.

-	Throttle: Apply bandwidth limitation (e.g. to a pre-configured value in Mbps) to the traffic matching Application ID/IP Packet Filter Set.

-	Inhibit DLDR: When UE is in IDLE mode and receives (DL) traffic matching Application ID/IP Packet Filter Set, UPF does not trigger N4 DLDR notification towards SMF, thus avoiding waking up UE unnecessarily (due to abnormal traffic).

7	UE #2 transmits/receives user plane traffic passing through UPF.

8	Based on the information received in step 4 above (node rules), UPF matches traffic (irrespective of the UE) against the node rules based on Application ID/IP Packet Filter Set Traffic and applies the corresponding action(s), same as above.

Finally, another alternative of this solution when no NWDAF is deployed, the cNF (i.e. SMF) can directly subscribe to Nupf_EventExposure (i.e. steps 3 to 5 in the sequence diagram in Figure 6.24.4-1) with the event extensions proposed. The thresholds are set to differentiate data flows with abnormal behaviour(s), and cNF can take actions to mitigate the impact of abnormal traffic based on UPF reporting. Alternatively, when the UPF has the N4 rules to be applied to mitigate the abnormal traffic, actions can be taken locally at UPF without reporting to SMF.

**** End of First Change ****

**** Second Change ****

# 7	Nnwdaf Services Description

## 7.1	General

Table 7.1-1 illustrates the NWDAF Services.

Table 7.1-1: NF services provided by NWDAF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnwdaf_AnalyticsSubscription | Subscribe | Subscribe / Notify | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, LMF, NRF, SCP, ADRF |
|  | Unsubscribe |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, LMF, NRF, SCP, ADRF |
|  | Notify |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, MFAF, LMF, NRF, SCP, ADRF |
|  | Transfer | Request / Response | NWDAF |
| Nnwdaf_AnalyticsInfo | Request | Request / Response | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, LMF, NRF, SCP, ADRF |
|  | ContextTransfer | Request / Response | NWDAF |
| Nnwdaf_DataManagement | Subscribe | Subscribe / Notify | NWDAF, DCCF |
|  | Notify |  | NWDAF, DCCF, MFAF, ADRF |
|  | Fetch | Request / Response | NWDAF, DCCF, MFAF, ADRF |
| Nnwdaf_MLModelProvision | Subscribe | Subscribe / Notify | NWDAF, LMF |
|  | Unsubscribe |  | NWDAF, LMF |
|  | Notify |  | NWDAF, LMF |
| Nnwdaf_MLModelInfo | Request | Request / Response | NWDAF, LMF |
| Nnwdaf_MLModelMonitor | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
|  | Register | Request / Response | NWDAF |
|  | Deregister |  | NWDAF |
| Nnwdaf_MLModelTraining | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
| Nnwdaf_MLModelTrainingInfo | Request | Request / Response | NWDAF |
| Nnwdaf_RoamingAnalytics | Subscribe | Subscribe / Notify | H-RE-NWDAF, V-RE-NWDAF |
|  | Unsubscribe |  | H-RE-NWDAF, V-RE-NWDAF |
|  | Notify |  | H-RE-NWDAF, V-RE-NWDAF |
|  | Request | Request / Response | H-RE-NWDAF, V-RE-NWDAF |
| Nnwdaf_RoamingData | Subscribe | Subscribe / Notify | H-RE-NWDAF, V-RE-NWDAF |
|  | Unsubscribe |  | H-RE-NWDAF, V-RE-NWDAF |
|  | Notify |  | H-RE-NWDAF, V-RE-NWDAF |
| Nnwdaf_VFLTraining | Subscribe | Subscribe / Notify | NWDAF, AF, NEF |
|  | Unsubscribe |  | NWDAF, AF, NEF |
|  | Notify |  | NWDAF, AF, NEF |
|  | Request | Request / Response | NWDAF, AF, NEF |
| Nnwdaf_VFLInference | Subscribe | Subscribe / Notify | NWDAF, AF, NEF |
|  | Unsubscribe |  | NWDAF, AF, NEF |
|  | Notify |  | NWDAF, AF, NEF |
|  | Request | Request / Response | NWDAF, AF, NEF |
| NOTE 1: How OAM consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.550 [7] Annex H and out of the scope of this TS. NOTE 2: How CEF consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.201 [21] and out of the scope of this TS. NOTE 3: The Nnwdaf_MLModelProvision service and the Nnwdaf_MLModelInfo service are provided by an NWDAF containing MTLF and consumed by an NWDAF containing AnLF or LMF. The services are also provided by an NWDAF containing MTLF supporting FL as a server and consumed by an NWDAF containing MTLF. |  |  |  |

Table 7.1-2 shows the analytics information provided by NWDAF service.

Table 7.1-2: Analytics information provided by NWDAF

| Analytics Information | Request Description | Response Description |
| --- | --- | --- |
| Slice Load level information | Analytics ID: load level information | Load level provided as number of UE registrations and number of PDU sessions for a Network Slice and Network Slice instances as well as resource utilization for Network Slice instances. |
| Observed Service experience information | Analytics ID: Service Experience | Observed Service experience statistics or predictions may be provided for a Network Slice or an Application. They may be derived from an individual UE, a group of UEs or any UE. For slice service experience, they may be derived from an Application, a set of Applications or all Applications on the Network Slice. |
| NF Load information | Analytics ID: NF load information | Load statistics or predictions information for specific NF(s). |
| Network Performance information | Analytics ID: Network Performance | Statistics or predictions on the load in an Area of Interest; in addition, statistics or predictions on the number of UEs that are located in that Area of Interest. |
| UE mobility information | Analytics ID: UE Mobility | Statistics or predictions on UE mobility. When visited AOI(s) is included in the Analytics Filter information, only statistics on UE mobility can be provided. |
| UE Communication information | Analytics ID: UE Communication | Statistics or predictions on UE communication. |
| Expected UE behavioural parameters | Analytics ID: UE Mobility and/or UE Communication | Analytics on UE Mobility and/or UE Communication. |
| UE Abnormal behaviour information | Analytics ID: Abnormal behaviour | List of observed or expected exceptions, with Exception ID, Exception Level and other information, depending on the observed or expected exceptions. |
| End-to-end data volume transfer time | Analytics ID: E2E data volume transfer time | Analytics on E2E data volume transfer time. |
| User Data Congestion information | Analytics ID: User Data Congestion | Statistics or predictions on the user data congestion for transfer over the user plane, for transfer over the control plane, or for both. |
| QoS Sustainability | Analytics ID: QoS Sustainability | For statistics, the information on the location and the time for the QoS change and the threshold(s) that were crossed; or, for predictions, the information on the location and the time when a potential QoS change may occur and what threshold(s) may be crossed. |
| Session Management Congestion Control Experience | Analytics ID: Session Management Congestion Control Experience | Statistics on session management congestion control experience for specific DNN and/or S-NSSAI. |
| Redundant Transmission Experience | Analytics ID: Redundant Transmission Experience | Statistics or predictions aimed at supporting redundant transmission decisions for URLLC services. |
| WLAN performance | Analytics ID: WLAN performance | Statistics or predictions on WLAN performance of UE. |
| Dispersion | Analytics ID: UE Dispersion | Statistics or predictions that identify the location (i.e. areas of interest) or network slice(s) where a UE, or a group of UEs disperse their data volume, or disperse mobility or session management transactions or both. |
| DN Performance | Analytics ID: DN Performance | Statistics or predictions on user plane performance for a specific Edge Computing application. |
| PFD Determination | Analytics ID: PFD Determination | Statistics on PFD information for a known application identifier(s). |
| Movement Behaviour | Analytics ID: Movement Behaviour | Statistics or predictions on movement behaviour for an applicable area. |
| Location Accuracy | Analytics ID: Location Accuracy | Predictions on Location Accuracy. |
| Relative Proximity | Analytics ID: Relative Proximity | Statistics or predictions on Relative Proximity among UEs. |
| PDU Session traffic | Analytics ID: PDU Session traffic | Statistics on whether traffic of UEs via one or multiple PDU sessions is according to the information provided by the service consumer. |
| Signalling Storm | Analytics ID: Signalling storm | Statistics or predictions for signalling storm mitigation or prevention. |
| QoS and Policy Assistance | Analytics ID: QoS and Policy Assistance | Analytics on predicted QoE for QoS parameter set(s) and candidate QoS parameter sets associated to the corresponding predicted QoE. |
| Abnormal User Plane Traffic | Analytics ID: Abnormal User Plane Traffic | Statistics or predictions on abnormal user plane traffic |

**** End of Changes ****