SA WG2 Meeting #S2-173	S2-2600742

09 - 13 Feb, 2026, Goa, India

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1553 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedure for SMF subscribe abnormal traffic analytic for UPF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN_Ph2-ARC |  |  |  |  |  | Date: |  |  | 2026-01-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Conclusion for KI#2 use case#1 has agreed to add a new analytic for abnormal traffic. This contribution proposed to add a new clause for this analytic for the procedure when UPF act as analytic consumer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add procedures when analytic consumer is UPF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The analytic and handling for abnormal user plane traffic is not supported when UPF is analytic consumer. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X.4.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.X.4.2	Procedure for SMF subscribe abnormal traffic analytic for UPF

UPF

SMF

1. N6 rule configuration (for abnormal traffic handling)

NWDAF

2. Nnwdaf_AnalyticsSubscription_subscribe request (analytic ID = Abnormal user plane traffic, Notification Target Address = UPF address)

AF

3. Data collection from UPF, SMF and AF

4. NWDAF derives analytic output

6. Map N6 rule and notification based on abnormal type and handle the abnormal traffic

5. Nnwdaf_AnalyticsSubscription_Notify (information to activate N6 rule)

Figure 6.X.4.2-1: SMF subscribes abnormal user plane traffic analytic for UPF

When UPF is the analytic consumer, the SMF could subscribe the abnormal traffic analytic to NWDAF for the UPF and the UPF is configured with one or several N6 rules to suppress the abnormal traffic for a UPF node (i.e. traffic from N6 side) based on the analytic output.

- The SMF configures one or several N6 rules (as described in table 6.X.4.2-1) for UPF node level abnormal traffic handling on UPF as described in clause 4.4 of TS 23.502 [3]. The N6 rule is generated by SMF based on local configuration or previous subscription for “Abnormal user plane traffic” from the NWDAF as described in clause 6.X.4.1

The format of N6 rule sent from SMF to UPF is as follows:

Table 6.X.4.2-1: N6 rule configured on UPF by SMF

| Attribute | Description |
| --- | --- |
| N6 rule ID | Unique identifier to identify this rule. |
| Type of Abnormal traffic | Contains one of the type of abnormal traffic as described in clause 6.X.3. |
| Action | Identifies the action to apply to the packet corresponding to the Packet detection information or Abnormal type. Contains one of the following actions: packet drop, redirection, data rate limitation. |

The N6 rule includes the mapping table between abnormal type and corresponding action.

- The SMF subscribes to NWDAF for the "Abnormal user plane traffic" analytics and set the Notification target Address to the address of UPF.

NOTE : The Notification target Address can also include SMF address to subscribe same analytic as described  	 in clause 6.X.4.1. The SMF may use the analytic output to configure the N6 rule in step 1 (i.e. in this 	 case, step 1 is performed after step 2).

- The NWDAF collects input data from UPF, SMF and AF as described in clause 6.X.4.1.

- The NWDAF derives the analytic output for the UPF node as described in clause 6.X.4.1.

- The NWDAF notifies the analytic output information as described in clause 6.X.3 directly to the UPF.

6.	 Based on the analytic output from NWDAF, the UPF maps the type of abnormal type in the analytic output (received from NWDAF) with the one from the N6 rule. If they are the same, the UPF handles the packets identified by the Identification of the traffic flows information in the analytic output based on the corresponding action in the N6 rule.

# * * * * End of changes * * * *