SA WG2 Meeting #S2-173	S2-2600743

# 09 - 13 Feb, 2026, Goa, India

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1554 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New analytic for normal traffic analytic for QoS adjustment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, CATT?, Samsung?, Vivo? |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN_Ph2-ARC |  |  |  |  |  | Date: |  |  | 2026-01-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Conclusion for KI#2 use case#2 has agreed to add a new analytic for QoS adjustment of normal traffic. This contribution proposed to add a new clause for this analytic and give examples how PCF adjust QoS parameters based on the analytic output. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Add input and output of the traffic pattern for QoS adjustment analytic. 2. Add procedure for the normal traffic analytic. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The analytic and handling for traffic pattern for QoS adjustment is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change  * * * *

6.X	Traffic pattern analytics

6.X.1	General

With traffic pattern Analytics, NWDAF could provide statistics or predictions of the traffic pattern for a traffic flow based on the collected user plane traffic information. The NF consumer (i.e. PCF) can perform QoS adjustment based on the traffic pattern.

The service consumer is PCF.

The consumer of these analytics may indicate in the request:

-	Analytics ID = "Traffic pattern ";

-	Target of Analytics Reporting as defined in clause 6.1.3;

-	Analytics Filter Information:

-	Application ID or Packet Filter set;

-	S-NSSAI;

-	DNN;

-  	UE address.

-	an optional list of analytics subsets that are requested (see clause 6.x.3).

-	Optionally, preferred level of accuracy of the analytics.

-	Optionally, preferred level of accuracy per analytics subset (see clause 6.x.3);

-	An Analytics target period indicates the time period over which the analytics are requested.

-	Reporting Thresholds, which apply only for subscriptions and indicate conditions on the levels to be reached for the respective analytics subsets (see clause 6.x.3).

-	Optionally, maximum number of objects.

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

6.X.2	Input data

NWDAF collects input data from the SMF and/or UPF. The detailed data are described in Table 6.yy.2-1. The NWDAF may also collect the packet drop and/or packet delay measurement data from the OAM and UPF as described in Table 6.13.2-1 of clause 6.13.2.

Table 6.X.2-1: Traffic characteristic Information collected by NWDAF

| Information | Source | Description |
| --- | --- | --- |
| Application/Service data flow(s) identifier | SMF, UPF | Application ID or Packet Filter set |
| S-NSSAI | SMF | Network Slice of the traffic flow. |
| DNN | SMF | Data Network Name of the traffic flow. |
| UE ID | SMF | Any UE identifier (associated with DNN, S-NSSAI) or Identifier of the UE(s) (e.g. SUPI) or group ID associated for the traffic flow. |
| UE IP Address | SMF | UE IP address associated for the traffic flow. |
| Data flow characteristic information | SMF, UPF | The characteristic of the service data flow. |
| > QoS flow Packet Delay | SMF, UPF | The observed Packet delay for UL/DL/round trip directions between UE and PSA_UPF. |
| > N6 Delay (NOTE 1) | UPF | The observed N6 delay between the UPF and measurement endpoint in the DN, as defined in clause 5.8.2.23 of TS 23.501 [2]. |
| > UL/DL data rates | SMF, UPF | The measured average UL/DL data rate during the measurement duration as defined in clause 5.45 of TS 23.501 [2]. |
| > packet interval | SMF, UPF | The interval value (e.g. minimum, maximum, average and/or variance value) between packets with the traffic flow. This can be used to provide periodicity and burst analysis. |
| >Congestion information | SMF, UPF | A percentage of congestion level as defined in clause 5.45 of TS 23.501 [2]. |
| > periodicity | SMF | Periodicity as defined in clause 5.27 of TS 23.501 [2] of the traffic for target application(s). |
| NOTE 1: Whether N6 delay (independent of QoS Flow) is considered to derive the analytics in comparison to delay between UE and UPF (dependent on the QoS Flow) is left to implementation based on the operator's policy. How the NWDAF obtains N6 delay measurement from UPF is left to implementation in this Release. |  |  |

The NWDAF collects input data from the UPF either indirectly via the SMF, or directly from the UPF using the "UserDataUsageMeasures" and "QoS Monitoring" event exposure events as described in clause 4.15.4.5 of TS 23.502 [3]. Further details about input parameters are described in Table 4.15.4.5.1 of TS 23.502 [3].

The NWDAF notifies/responses the Output Analytics to the consumer NF as listed in Tables 6.X.3-1 and 6.X.3-2.

6.X.3	Output analytics

Table 6.X.3-1: Output Analytics of traffic pattern statistics

| Information | Description |
| --- | --- |
| Traffic flow identifier | Information identifying the traffic flow, e.g. the Application ID and/or Packet Filter Set, identifier of the service data flow(s) of the target application(s). |
| Identifier of the affected UE(s) | Identifier of the UE(s) affected by the traffic pattern, e.g. UE ID(s). |
| Traffic pattern information | The traffic pattern statistic of the traffic flow. |
| > Maximum Burst Size | Maximum Burst Size statistic as defined in clause 6.1.3.22 of TS 23.503 [4] of the target application(s). |
| > Maximum Bitrate | Maximum Bitrate statistic (UL, DL) as defined in clause 6.1.3.22 of TS 23.503 [4] of the target application(s). |
| > Guaranteed Bitrate | Guaranteed Bitrate statistic (UL, DL) as defined in clause 6.1.3.22 of TS 23.503 [4] of the target application(s). |
| > 5GS delay | 5GS delay statistic (UL, DL) as defined in clause 6.1.3.22 of TS 23.503 [4] of the target application(s). |
| > Packet Error Rate | Packet Error Rate statistic (UL, DL) as defined in clause 6.1.3.22 of TS 23.503 [4] of the target application(s). |
| > Periodicity | Periodicity statistic as defined in clause 5.27 of TS 23.501 [2] of the traffic for target application(s) if applicable. |

Table 6.X.3-2: Output Analytics of traffic pattern predictions

| Information | Description |
| --- | --- |
| Traffic flow identifier | Information identifying the traffic flow, e.g. the Application ID and/or Packet Filter Set, identifier of the service data flow(s) of the target application(s). |
| Identifier of the affected UE(s) | Identifier of the UE(s) affected by the traffic pattern, e.g. UE ID(s). |
| Traffic pattern information | The predicted traffic pattern of the traffic flow. |
| > Maximum Burst Size | Predicted Maximum Burst Size as defined in clause 6.1.3.22 of TS 23.503 [4] of the target application(s). |
| > Maximum Bitrate | Predicted Maximum Bitrate (UL, DL) as defined in clause 6.1.3.22 of TS 23.503 [4] of the target application(s). |
| > Guaranteed Bitrate | Predicted Guaranteed Bitrate (UL, DL) as defined in clause 6.1.3.22 of TS 23.503 [4] of the target application(s). |
| > 5GS delay | Predicted 5GS delay (UL, DL) as defined in clause 6.1.3.22 of TS 23.503 [4] of the target application(s). |
| > Packet Error Rate | Predicted Packet Error Rate (UL, DL) as defined in clause 6.1.3.22 of TS 23.503 [4] of the target application(s). |
| > Periodicity | Predicted Periodicity as defined in clause 5.27 of TS 23.501 [2] of the traffic for target application(s) if applicable. |

Table 6.X.3-3: Example action of the consumer

| Consumer | Example of actions |
| --- | --- |
| PCF | Take the output Analytics into account, e.g. to generate or update PCC rules by adjusting the QoS. |

6.X.4	Procedures

UPF

SMF

PCF

NWDAF

1 Nnwdaf_AnalyticsSubscription_Subscribe

RAN

UE

3. Nnwdaf_AnalyticsSubscription_Notify

4. Npcf_SMPolicyAssociation_Update

2. Data collection from SMF, UPF

5. PDU Session Modification procedure

Figure 6.X.4.1-1: Procedure for NWDAF assisted traffic pattern generation for QoS adjustment

1.

based on the subscription retrieved from UDR or internal logic, the PCF subscribes to NWDAF for the “traffic pattern” analytic and includes the Analytics Filter Information in the message.

2.	Based on the subscription from PCF, the NWDAF collects input data from SMF and/or UPF as described in table 6.X.2-1. The NWDAF can discover the SMF serving the PDU session based on the DNN, S-NSSAI of the PDU session and UE ID provided from PCF as described in clause 6.2.8.

3.	The NWDAF provides the analytic output as described in Table 6.25.3-2 to PCF.

4.	PCF creates/updates PCC rule for target application(s) based on the traffic pattern provided from NWDAF as described in clause 5.27.3 of TS 23.501 [2] and 6.1.3.22 of TS 23.503[4].

5.	SMF triggers PDU Session Modification Procedure as described in TS 23.502 [3] based on the updated PCC rule.

# * * * * Next change  * * * *

7.1	General

Table 7.1-1 illustrates the NWDAF Services.

Table 7.1-1: NF services provided by NWDAF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnwdaf_AnalyticsSubscription | Subscribe | Subscribe / Notify | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, LMF, NRF, SCP, ADRF |
|  | Unsubscribe |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, LMF, NRF, SCP, ADRF |
|  | Notify |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, MFAF, LMF, NRF, SCP, ADRF |
|  | Transfer | Request / Response | NWDAF |
| Nnwdaf_AnalyticsInfo | Request | Request / Response | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, LMF, NRF, SCP, ADRF |
|  | ContextTransfer | Request / Response | NWDAF |
| Nnwdaf_DataManagement | Subscribe | Subscribe / Notify | NWDAF, DCCF |
|  | Notify |  | NWDAF, DCCF, MFAF, ADRF |
|  | Fetch | Request / Response | NWDAF, DCCF, MFAF, ADRF |
| Nnwdaf_MLModelProvision | Subscribe | Subscribe / Notify | NWDAF, LMF |
|  | Unsubscribe |  | NWDAF, LMF |
|  | Notify |  | NWDAF, LMF |
| Nnwdaf_MLModelInfo | Request | Request / Response | NWDAF, LMF |
| Nnwdaf_MLModelMonitor | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
|  | Register | Request / Response | NWDAF |
|  | Deregister |  | NWDAF |
| Nnwdaf_MLModelTraining | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
| Nnwdaf_MLModelTrainingInfo | Request | Request / Response | NWDAF |
| Nnwdaf_RoamingAnalytics | Subscribe | Subscribe / Notify | H-RE-NWDAF, V-RE-NWDAF |
|  | Unsubscribe |  | H-RE-NWDAF, V-RE-NWDAF |
|  | Notify |  | H-RE-NWDAF, V-RE-NWDAF |
|  | Request | Request / Response | H-RE-NWDAF, V-RE-NWDAF |
| Nnwdaf_RoamingData | Subscribe | Subscribe / Notify | H-RE-NWDAF, V-RE-NWDAF |
|  | Unsubscribe |  | H-RE-NWDAF, V-RE-NWDAF |
|  | Notify |  | H-RE-NWDAF, V-RE-NWDAF |
| Nnwdaf_VFLTraining | Subscribe | Subscribe / Notify | NWDAF, AF, NEF |
|  | Unsubscribe |  | NWDAF, AF, NEF |
|  | Notify |  | NWDAF, AF, NEF |
|  | Request | Request / Response | NWDAF, AF, NEF |
| Nnwdaf_VFLInference | Subscribe | Subscribe / Notify | NWDAF, AF, NEF |
|  | Unsubscribe |  | NWDAF, AF, NEF |
|  | Notify |  | NWDAF, AF, NEF |
|  | Request | Request / Response | NWDAF, AF, NEF |
| NOTE 1: How OAM consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.550 [7] Annex H and out of the scope of this TS. NOTE 2: How CEF consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.201 [21] and out of the scope of this TS. NOTE 3: The Nnwdaf_MLModelProvision service and the Nnwdaf_MLModelInfo service are provided by an NWDAF containing MTLF and consumed by an NWDAF containing AnLF or LMF. The services are also provided by an NWDAF containing MTLF supporting FL as a server and consumed by an NWDAF containing MTLF. |  |  |  |

Table 7.1-2 shows the analytics information provided by NWDAF service.

Table 7.1-2: Analytics information provided by NWDAF

| Analytics Information | Request Description | Response Description |
| --- | --- | --- |
| Slice Load level information | Analytics ID: load level information | Load level provided as number of UE registrations and number of PDU sessions for a Network Slice and Network Slice instances as well as resource utilization for Network Slice instances. |
| Observed Service experience information | Analytics ID: Service Experience | Observed Service experience statistics or predictions may be provided for a Network Slice or an Application. They may be derived from an individual UE, a group of UEs or any UE. For slice service experience, they may be derived from an Application, a set of Applications or all Applications on the Network Slice. |
| NF Load information | Analytics ID: NF load information | Load statistics or predictions information for specific NF(s). |
| Network Performance information | Analytics ID: Network Performance | Statistics or predictions on the load in an Area of Interest; in addition, statistics or predictions on the number of UEs that are located in that Area of Interest. |
| UE mobility information | Analytics ID: UE Mobility | Statistics or predictions on UE mobility. When visited AOI(s) is included in the Analytics Filter information, only statistics on UE mobility can be provided. |
| UE Communication information | Analytics ID: UE Communication | Statistics or predictions on UE communication. |
| Expected UE behavioural parameters | Analytics ID: UE Mobility and/or UE Communication | Analytics on UE Mobility and/or UE Communication. |
| UE Abnormal behaviour information | Analytics ID: Abnormal behaviour | List of observed or expected exceptions, with Exception ID, Exception Level and other information, depending on the observed or expected exceptions. |
| End-to-end data volume transfer time | Analytics ID: E2E data volume transfer time | Analytics on E2E data volume transfer time. |
| User Data Congestion information | Analytics ID: User Data Congestion | Statistics or predictions on the user data congestion for transfer over the user plane, for transfer over the control plane, or for both. |
| QoS Sustainability | Analytics ID: QoS Sustainability | For statistics, the information on the location and the time for the QoS change and the threshold(s) that were crossed; or, for predictions, the information on the location and the time when a potential QoS change may occur and what threshold(s) may be crossed. |
| Session Management Congestion Control Experience | Analytics ID: Session Management Congestion Control Experience | Statistics on session management congestion control experience for specific DNN and/or S-NSSAI. |
| Redundant Transmission Experience | Analytics ID: Redundant Transmission Experience | Statistics or predictions aimed at supporting redundant transmission decisions for URLLC services. |
| WLAN performance | Analytics ID: WLAN performance | Statistics or predictions on WLAN performance of UE. |
| Dispersion | Analytics ID: UE Dispersion | Statistics or predictions that identify the location (i.e. areas of interest) or network slice(s) where a UE, or a group of UEs disperse their data volume, or disperse mobility or session management transactions or both. |
| DN Performance | Analytics ID: DN Performance | Statistics or predictions on user plane performance for a specific Edge Computing application. |
| PFD Determination | Analytics ID: PFD Determination | Statistics on PFD information for a known application identifier(s). |
| Movement Behaviour | Analytics ID: Movement Behaviour | Statistics or predictions on movement behaviour for an applicable area. |
| Location Accuracy | Analytics ID: Location Accuracy | Predictions on Location Accuracy. |
| Relative Proximity | Analytics ID: Relative Proximity | Statistics or predictions on Relative Proximity among UEs. |
| PDU Session traffic | Analytics ID: PDU Session traffic | Statistics on whether traffic of UEs via one or multiple PDU sessions is according to the information provided by the service consumer. |
| Signalling Storm | Analytics ID: Signalling storm | Statistics or predictions for signalling storm mitigation or prevention. |
| QoS and Policy Assistance | Analytics ID: QoS and Policy Assistance | Analytics on predicted QoE for QoS parameter set(s) and candidate QoS parameter sets associated to the corresponding predicted QoE. |
| Traffic pattern | Analytics ID: Traffic pattern | Statistics or predictions on traffic patterns of the service data flow(s). |

# * * * * End of changes * * * *