3GPP TSG- SA WG2 Meeting #173	S2-2601281

# Goa, India, 9 – 13 February, 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6521 | rev | 1 | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on architecture and EIF function to support policy control for network energy saving |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys_Ph2-ARC |  |  |  |  |  | Date: |  |  | 2026-01-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | Based on the conclution for KI#2, the PCF may get the energy related information from OAM and EIF. The related interface and function need to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Based on the conclution for KI#2, it is proposed to add the interface between the PCF and OAM/EIF in the architecture. The EIF function is updated to include the notification to the PCF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The related interface between the PCF and OAM/EIF is missing. The EIF can not notify energy related information to the PCF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.18.1, 5.51.2.1, 6.2.34 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### ==============First change==============

4.2.18.1	Non-roaming architecture

Figure 4.2.18-1 depicts the non-roaming reference architecture using service-based interfaces. The figure depicts only the NFs enhanced to support the Energy Efficiency and Energy Saving functionality.

Figure 4.2.18-2 depicts the non-roaming reference architecture, using the reference point representation showing how various network functions interact with each other. The figure depicts the direct interface of the Energy Information Function (EIF) with other NFs only.

Figure 4.2.18.1-1: Non-roaming architecture for Energy Efficiency and Energy Saving

Figure 4.2.18.1-2: Non-roaming architecture for Energy Efficiency and Energy Saving in reference point representation

NOTE 1:	The reference points between NF and NRF is not shown in the figure 4.2.18.1-2.

NOTE 2:	The interface between EIF and OAM are not shown. The EIF requests energy-related information from OAM by creating management objects defined in TS 28.622 [218] using procedures defined in TS 28.532 [219].

NOTE 3:	The interface between PCF and OAM are not shown. The PCF requests energy-related information from OAM by creating management objects defined in TS 28.622 [218] using procedures defined in TS 28.532 [219].

In this Release, the roaming architecture for Energy Efficiency and Energy Saving is not supported.

### ============Second change==============

5.51.2.1	General

The Energy Information Function (EIF) is defined to:

-	collect the UE related Energy Consumption information;

-	calculate the Energy Consumption information at UE, S-NSSAI, PDU Session and Service Data Flow (e.g. per UE per application) granularity; and

-	expose the Energy Consumption information to the authorized consumer NF(s) (AF/NEF, PCF).

Procedures for energy consumption information collection and exposure are defined in clause 4.29 of TS 23.502 [3].

In this Release of the specification, collecting and calculating of UE related Energy Consumption information is supported only for the UP resources of the 3GPP access type serving the UE in the 5GS.

### ============Third change==============

6.2.34	EIF

The Energy Information Function (EIF) includes support for the following functionalities:

-	Collect data from OAM and 5GC NF(s) to assist the calculation of energy related information.

-	Calculate the energy related information (including energy consumption information and renewable energy information) of user plane communication.

-	Expose the calculated energy related information to authorized consumers, i.e., AF/NEF, PCF.

==============End of change==============