3GPP TSG-WG SA2#173	S2-2601147

# Goa, India, 9 – 13 February, 2026

Source:	Samsung, Nokia, Deutsche Telekom (Pen-holder(s))

Title:	KI#1.2:Control signaling for 6G System: generic mechanism for operator services – penholders input for SA2#173

Document for:	Approval

Agenda item:	20.6.2

Work Item:	FS_6G_ARC

Abstract of the contribution: The contribution provides solution variants for KI#1.2.

1.	Discussion

The contribution is a pen-holder input for KI#1.2 of KI#1 and generated based on the submitted input solutions listed in Annex X.

Key Issue #1.2 1. Study the support for control signalling for 6G System, including at least the following: a) Whether and how to support generic mechanisms (e.g. service discovery, service authorization, transport mechanism) for UE to Core Network interaction to support operator services. NOTE 5: This KI#1.2 bullet 1a can include any transport mechanism such as using NAS or UP or new plane. KI#1.2 bullet 1a can have dependency on other key issues defined for e.g. KI#1.1 bullet 1a, KI#18, KI#19, KI#20, KI#21, KI#22. NOTE 6: 6G System Procedures will include interaction between RAN-CN, interaction within CN NFs. The signalling interaction between RAN-CN need coordination and alignment with RAN WG3. NOTE 7: Overall Interworking and migration aspects are covered by key issues for WT#2 (KI#17). Impact to IWK due to features introduced by this key issue will be covered by this key issue in alignment with WT#2 (KI#17). NOTE 8: This KI#1 covers operator services.

This pCR proposes to capture

-	5 solutions variants based on selected input solutions for KI#1.2.

-	list of the submitted input solutions in Annex X

The solution variants are:

- Solution variant #1.2.1: Non-backward compatible NAS-based control signaling for operator services over control plane

- Solution variant #1.2.2: Backward compatible NAS-based control signaling for operator services over control plane

- Solution variant #1.2.3: NAS-based control signaling for operator services over user plane

- Solution variant #1.2.4: New control framework for operator services

- Solution variant #1.2.5: RAN-CN interactions for operator services

2.	Text proposal

It is proposed to agree the following changes vs. TR 23.801-01:

* * * First Change * * * *

6.1.2	Solutions to KI#1.2

6.1.2.0	Solution variant #1.1.0: High Level Summary of Solution variants

6.1.2.0	Topics addressed and High-level Solution Principles

This clause shows the summary of solution variants for KI#1.2.

Table 6.X.Y.0-1: Summary of solution variants for KI#1.2

| List of Solution Variants |  |  | Summary | Operator services to be covered |
| --- | --- | --- | --- | --- |
| 1 | Non-backward compatible NAS-based control signaling for operator services over control plane | Solution #1, Solution #2, Solution #3, Solution #22 | New 6G NAS as a common control siganling for legacy and new operator services Enhancements to avoid impact on 6G MM NAS supports generic mechanism including transport of operator service signaling for operator services over control plane | Legacy and new operator services supported as NAS functionalities |
| 2 | Backward compatible NAS-based control signaling for operator services over control plane | Solution #4, Solution #5 | Backward compatible NAS as a common control siganling for legacy operator services Backward compatible NAS NAS supports generic mechanism including transport mechanism for operator services over NAS | Legacy operator services supported as NAS functionalities |
| 3 | NAS-based control signaling for operator services over user plane | Solution #1, Solution #2, Solution #3, Solution #7, Solution #8 | NAS as a common control siganling for new operator services over user plane NAS supports service capability registration, service discovery, service authorization for operator services Trasnport mechanism supported over user plane is supported - NAS supports the control the establishment of user plane path - NAS supports the control the establishment of connection between UE and the NF for operator service | Legacy and new operator services being transported over user plane |
| 4 | New control framework for operator services | Solution #6, Solution #10, Solution #11, Solution #12, Solution #13, Solution #14, Solution #16, Solution #17, Solution #18, Solution #19, Solution #20, Solution #21 | NAS for basic operator services (at least including MM/SM) A new control framework for new operator services (e.g., sensing, AI/ML, data) It is assumed that for new operator services using the framework, may not need to introduce new NAS functionality per operator service. The new framework supports generic mechanism including transport of operator service signaling for operator services On the new control framework, there are different proposals - RAN supports the routing of the control siganling between the UE and the NF via new interface other than N2 (e.g., SBI) - UPF supports the routing the control siganling between the UPF and the NF using SBI - Control signaling is exchanged over N6 (i.e., PDU Session) - An intermediate NF between the UE and the NF for operator services to manage the control signlaing connection with the UE and maintains the general UE context for operator services | New operator services (e.g., Sensing, AI, Computing) being transported over user plane or new plane |
| 5 | RAN-CN interactions for operator services | Solution #11, Solution #17, Solution #21 | -A new interface between 6G RAN and NF other than 6G N2 for new operator service (e.g., SBI or P2P) | New operator services that requires RAN-CN interactions (e.g., sNB-assisted/based sensing, data collection from sNB) |
| Other related but specific areas |  |  |  |  |

6.1.2.1.1	Description

6.1.2.1.2	Procedures

6.1.2.1.3	Services, Entities and Interfaces

6.1.2.1.4	Issues

This section highlights the open issues and variants for key technical areas proposed by the solutions for KI#1.2:

- Common NAS framework for all operator services vs. new control framework for new operator services

- Considerations of operator services that require UE-RAN interactions (e.g., data framework, Sensing)

- What operator services to be considered by the new control framework

- An operator service that can use more than one transport mechanisms

- * * * Next Change * * * *

6.1.2.1	Solution variant #1.2.1: Non-backward compatible NAS-based control signaling for operator services over control plane

6.1.2.1.0	Topics addressed and High-level Solution Principles

This solution variant addresses KI#1.2 and is extracted from Solution#1, Solution#2, Solution#3, Solution #22.

The high-level principles of the solution are listed as below.

-    NAS signaling is used as a common control signaling for operator services to support service discovery, connection management, and transportation of service specific signaling (e.g., UE service capability, service request)

-	NAS transport mechanism is enhanced used to support transport of 6G control signalling for operator services without impacting 6G Mobility Management (MM)

-    A new new NAS terminating function is introduced to perform following handling between the UE and 6G service NFs

-    NF discovery/selection

-	Connection management

-	UL/DL control signaling distribution for operator services

Editor's note:	Any aspect dependent on design of NAS will be aligned with KI#1.1.

6.1.2.1.1	Description

6.1.2.1.2	Procedures

6.1.2.1.3	Services, Entities and Interfaces

6.1.2.1.4	Issues

* * * Next Change * * * *

6.1.2.2	Solution variant #1.2.2: Backward compatible NAS-based control signaling for operator services over control plane

6.1.2.2.0	Topics addressed and High-level Solution Principles

This solution addresses KI#1.2 and is extracted from Soltuion #4, Solution #5

This solution proposes the following principles:

- 	The proposed solution is based on a CN architecture evolved from 5GC (Solution #4)

-     Direct NAS transport and container based NAS transport supported by the evolved AMF can be used as a transport mechanism between UE and NF for operator service

- 	This solution assumes NAS that supports backward compatibility with 5G (Solution #5)

-    The NAS transport can be enhanced so that addition of services does not impact NAS-MM/AMF.

Editor's note:	Any aspect dependent on design of NAS will be aligned with KI#1.1.

6.1.2.2.1	Description

6.1.2.2.2	Procedures

6.1.2.2.3	Services, Entities and Interfaces

6.1.2.2.4	Issues

* * * Next Change * * * *

6.1.2.3	Solution variant #1.2.3: NAS-based control signaling for operator services over user plane

6.1.2.3.0	Topics addressed and High-level Solution Principles

This solution addresses KI#1.2 and is extracted from (Solution #1, Solution #2, Solution #3, Solution #7, Solution #8, Solution #9, Solution #15)

This solution proposes the following principles:

- The UE uses the NAS signalling to provide service request and a service specific capability for the operator service to the 6G NF (e.g., new NF (i.e., Connection Management Function), AMF, or SMF)

- The NAS supports functionality to handle authorization and discovery of the requested operator service.

- The NAS terminating NF uses the NAS signalling to provide to the UP address information of the NF to the UE

- The UE transmit signalling and/or data exchange with the NF that provides the operator service, via the user plane.

- Mechanisms to authenticate/authorize the application for operator service in the UE (Solution #7)

6.1.2.3.1	Description

6.1.2.3.2	Procedures

6.1.2.3.3	Services, Entities and Interfaces

6.1.2.3.4	Issues

* * * Next Change * * * *

6.1.2.4	Solution variant #1.2.4: New control framework for operator services

6.1.2.4.0	Topics addressed and High-level Solution Principles

This solution addresses KI#1.2 and is extracted from Solution #6, Solution #10, Solution #11, Solution #12, Solution #13, Solution #14, Solution #16, Solution #17, Solution #18, Solution #19, Solution #20, Solution #21

This solution proposes the following principles:

The high level summary is:

-  A new control framework for new operator services is introduced to minimize the impact on NAS

-  The control framework supports generic mechanisms for operator services such as transport of operator service signaling, service capability registration, service discovery, service authentication

-  A generic control signaling for operator services is defined to support interactions between the UE and NF for operator service

NOTE X:	Coordination with CT1 is required

Editor’s Note: What transport path the new control signaling for operator services is FFS (e.g., based on 6G N6, UPF SBI, RAN routing, etc.).

6.1.2.4.1	Description

6.1.2.4.2	Procedures

6.1.2.4.3	Services, Entities and Interfaces

6.1.2.4.4	Issues

* * * Next Change * * * *

6.1.2.5	Solution variant #1.2.5: RAN-CN interactions for operator services

6.1.2.5.0	Topics addressed and High-level Solution Principles

This solution addresses KI#1.2 and is extracted from Solution #11, Solution #17, Solution #21

This solution proposes the following principles:

- The solution considers new operator services in 6G that require interactions between RAN and CN

- A new interface between 6G RAN and NF for operator service other than 6G N2 is supported to support generic mechanism for operator services such as transport of operator service signaling

6.1.2.5.1	Description

6.1.2.5.2	Procedures

6.1.2.5.3	Services, Entities and Interfaces

6.1.2.5.4	Issues

* * * Next Change * * * *

Annex X: Submitted solutions

X.Y	List of submitted solutions for KI#Y

Table X.Y: List of submitted solutions

| Meeting | Solution# | TDoc | Companies | Subject/Comment |
| --- | --- | --- | --- | --- |
| SA2#173 | 1 | S2-2600150 | Vivo | [KI#1.2] Solution proposal on NAS-based UE-CN interaction mechanism |
| SA2#173 | 2 | S2-2600100 | China Mobile | [KI#1, all bullets] Solution for KI#1 of intent-based signalling framework |
| SA2#173 | 3 | S2-2600434 | Huawei, HiSilicon | [KI#1] Solution #1.X: Solution for E2E control signalling in 6G system |
| SA2#173 | 4 | S2-2600249 | Ericsson | [KI#1, bullet #1.1 and #1.2] New Solution: UE and CN control signalling based on evolvement of 5G NAS |
| SA2#173 | 5 | S2-2600594 | Qualcomm | [KI#1.2] Solution for Support for operator services – a framework |
| SA2#173 | 6 | S2-2600266 | LG Electronics | [KI#1] Distributed NAS signalling support for new operator services with extended NAS architecture |
| SA2#173 | 7 | S2-2600092 | Futurewei | [KI#1, Bullet 2] Operator Service over User Plane Transport |
| SA2#173 | 8 | S2-2600160 | OPPO | [KI#1.2, bullet #1a] control signalling over UP |
| SA2#173 | 9 | S2-2600236 | Lenovo | [KI#1.2, bullet #1] Solution for transport control signalling for operator services over user plane |
| SA2#173 | 10 | S2-2600216 | Lenovo | [KI#1.2, bullet 1] Support control plane signalling for over the top operator services over user plane |
| SA2#173 | 11 | S2-2600219 | Google | KI #1: Generic Service Stratum Framework via User-Plane Tunnels |
| SA2#173 | 12 | S2-2600308 | NEC | [KI#1.1, 2a] Procedure to support generic mechanisms for UE to Core Network interaction to support operator services. |
| SA2#173 | 13 | S2-2600388 | Rakuten Mobile, China Mobile | [KI#1, bullet #2] Solution for UE–Core Interaction in 6G for control signalling to support operator services |
| SA2#173 | 14 | S2-2600397 | TCL | [KI#1] Solutions for generic UE-CN interaction mechanism |
| SA2#173 | 15 | S2-2600472 | ZTE | [KI#1.2] Service request procedure for operator service |
| SA2#173 | 16 | S2-2600489 | CATT | [KI#1.2] Support for operator services based on RAN SBI interface |
| SA2#173 | 17 | S2-2600495 | Samsung | [KI#1.2] Introduction of service plane for operator services |
| SA2#173 | 18 | S2-2600524 | Xiaomi | [KI#1, bullet #2] Solution for generic transport mechanism for UE-CN control signalling |
| SA2#173 | 19 | S2-2600532 | Apple | [KI#1.1, KI#1.2] New solution on Evolved Control Signalling for 6G |
| SA2#173 | 20 | S2-2600572 | InterDigital | [KI#2, KI#1.2] A Generic Framework to Support Communication with Operator Services |
| SA2#173 | 21 | S2-2600198 | NTT DOCOMO | [KI#1] Re-use of 5GS RAN-CN interfaces |
| SA2#173 | 22 | S2-2600311 | CATT | [KI#1] Generic mechanism for Feature discovery and Parameter authorization via NAS signalling |

* * * End of Changes * * * *