3GPP TSG-WG SA2#173	S2-2601376

# Goa, India, 9 – 13 February, 2026

Source:	Samsung, Nokia, Deutsche Telekom (Pen-holder(s))

Title:	KI#1.2:Control signaling for 6G System: generic mechanism for operator services – penholders input for SA2#173

Document for:	Approval

Agenda item:	20.6.2

Work Item:	FS_6G_ARC

Abstract of the contribution: The contribution provides solution variants for KI#1.2.

1.	Discussion

The contribution is a pen-holder input for KI#1.2 of KI#1 and generated based on the submitted input solutions listed in Annex X.

Key Issue #1.2 1. Study the support for control signalling for 6G System, including at least the following: a) Whether and how to support generic mechanisms (e.g. service discovery, service authorization, transport mechanism) for UE to Core Network interaction to support operator services. NOTE 5: This KI#1.2 bullet 1a can include any transport mechanism such as using NAS or UP or new plane. KI#1.2 bullet 1a can have dependency on other key issues defined for e.g. KI#1.1 bullet 1a, KI#18, KI#19, KI#20, KI#21, KI#22. NOTE 6: 6G System Procedures will include interaction between RAN-CN, interaction within CN NFs. The signalling interaction between RAN-CN need coordination and alignment with RAN WG3. NOTE 7: Overall Interworking and migration aspects are covered by key issues for WT#2 (KI#17). Impact to IWK due to features introduced by this key issue will be covered by this key issue in alignment with WT#2 (KI#17). NOTE 8: This KI#1 covers operator services.

This pCR proposes to capture

-	4 solutions variants based on selected input solutions for KI#1.2.

-	list of the submitted input solutions in Annex X

The solution variants are:

- Solution variant #1.2.1: NAS-assisted mechanism to transport control signaling and operator service signaling over control plane

- Solution variant #1.2.2: NAS-assisted mechanism to transport control signaling but operator service signaling over user plane

- Solution variant #1.2.3: New mechanism to transport control signaling and operator service signaling over user plane

- Solution variant #1.2.4: New mechanism to transport control signaling and operator service signaling over new plane

2.	Text proposal

It is proposed to agree the following changes vs. TR 23.801-01:

* * * First Change * * * *

6.1.2	Solutions to KI#1.2

6.1.2.0	Solution variant overview#1.1.0: High Level Summary of Solution variants

6.1.2.1.0	Topics addressed and High-level Solution Principles

This clause shows the summary of solution variants for KI#1.2.

Table 6.1.2.1.0-1: Summary of solution variants for KI#1.2

| List of Solution Variants |  |  | Summary | Operator services to be covered |
| --- | --- | --- | --- | --- |
| 1 | NAS-assisted mechanism to transport control signaling and operator service signaling over control plane | Solution #1, Solution #2, Solution #3, Solution #4, Solution #6, Solution #22, Solution #26 | NAS as control siganling for operator services over control plane NAS signaling connection supports generic mechanisms (e.g., service capability registration, service discovery, service authorization) for operator services A NAS signaling connection is used to exchange signaling for operator services Editor’s Note: MM/SM aspect needs to be aligned with KI#1.1. Whether the termination point of NAS is single or multiple needs to be aligned with KI#1.1. | Operator services transported over control plane |
| 2 | NAS-assisted mechanism to transport control signaling but operator service signaling over user plane | Solution #7, Solution #8, Solution #9, Solution #15, Solution #18, Solution #19, Solution #21 | NAS as general control siganling for operator services NAS signaling connection supports generic mechanisms (e.g., service capability registration, service discovery, service authorization) for operator services Each operator service that requires to exchange signaling over user plane uses NAS signaling for - The establishment of the PDU session and the connection between the UE and the NF over user plane Editor’s Note: MM/SM aspect needs to be aligned with KI#1.1. Whether the termination point of NAS is single or multiple needs to be aligned with KI#1.1. | Operator services transported over user plane |
| 3 | New mechanism to transport control signaling and operator service signaling over user plane | Solution #5, Solution #10, Solution #11, Solution #12, | Control signaling for generic mechanisms (e.g., service capability registration, service discovery, service authorization) and general control signaling for operator services are supported over the user plane. Termination point of the generic signaling connection is located in - Data Network (Solution #5) - 6G CN (Solution #10, Solution #11, Solution #12) The transport path for generic signaling connection is supported based on the PDU session - N6 (Solution #5) - UPF SBI (Solution #10, Solution #11, Solution #12) Service request, service capability registration signaling, transport of operator service signaling is supported over the user plane Editor’s Note: UE-CN interface for the control signaling requires coordination with CT1. Editor’s Note: Security aspect for the control signaling over user plane needs to be considered | Operator services transported over user plane |
| 4 | New mechanism to transport control signaling and operator service signaling over new plane | Solution #13, Solution #14, Solution #16, Solution #17, Solution #20, Solution #23, Solution #24, Solution #25 | Control signaling for generic mechanisms (e.g., service capability registration, service discovery, service authorization) and general control signaling for operator services are supported over the new plane. Termination point of the generic signaling connection is located in 6G CN The transport path for generic signaling connection is supported based on the new RAN-CN interface (in addition to 6G N2 and 6G N3). Service request, service capability registration signaling, transport of operator service signaling is supported over the control signaling connection over the new plane Editor’s Note: UE-CN interface for the control signaling requires coordination with CT1. Editor’s Note: Security aspect for the control signaling over new plane needs to be considered Editor’s Note: RAN-CN interface requires coordination with RAN3. | Operator services transported over new plane |
| Other related but specific areas |  |  |  |  |

6.1.2.1.1	Description

6.1.2.1.2	Procedures

6.1.2.1.3	Services, Entities and Interfaces

6.1.2.1.4	Issues

* * * Next Change * * * *

6.1.2.1	Solution variant #1.2.1: NAS-assisted mechanism to transport control signaling and operator service signaling over control plane

6.1.2.1.0	Topics addressed and High-level Solution Principles

This solution variant addresses KI#1.2 and is extracted from Solution #1, Solution #2, Solution #3, Solution #4, Solution #6, Solution #22, Solution #26

The high-level principles of the solution are listed as below.

- NAS as control siganling for operator services over control plane

- NAS signaling connection supports generic mechanisms (e.g., service capability registration, service discovery, service authorization) for operator services

- A NAS signaling connection is used to exchange signaling for operator services

Editor’s Note: MM/SM aspect needs to be aligned with KI#1.1. Whether the termination point of NAS is single or multiple needs to be aligned with KI#1.1.

6.1.2.1.1	Description

6.1.2.1.2	Procedures

6.1.2.1.3	Services, Entities and Interfaces

6.1.2.1.4	Issues

* * * Next Change * * * *

6.1.2.3	Solution variant #1.2.2: NAS-assisted mechanism to transport control signaling but operator service signaling over user plane

6.1.2.3.0	Topics addressed and High-level Solution Principles

This solution addresses KI#1.2 and is extracted from Solution #7, Solution #8, Solution #9, Solution #15, Solution #18, Solution #19, Solution #21

This solution proposes the following principles:

- NAS as general control siganling for operator services

- NAS signaling connection supports generic mechanisms (e.g., service capability registration, service discovery, service authorization) for operator services

- Each operator service that requires to exchange signaling over user plane uses NAS signaling for

  - The establishment of the PDU session and the connection between the UE and the NF over user plane

Editor’s Note: MM/SM aspect needs to be aligned with KI#1.1. Whether the termination point of NAS is single or multiple needs to be aligned with KI#1.1.

6.1.2.3.1	Description

6.1.2.3.2	Procedures

6.1.2.3.3	Services, Entities and Interfaces

6.1.2.3.4	Issues

* * * Next Change * * * *

6.1.2.4	Solution variant #1.2.3: New mechanism to transport control signaling and operator services over user plane

6.1.2.4.0	Topics addressed and High-level Solution Principles

This solution addresses KI#1.2 and is extracted from Solution #5, Solution #10, Solution #11, Solution #12,

This solution proposes the following principles:

The high level summary is:

- The NAS signaling connection is used for basic operator services (at least including MM/SM)

- The operator services using the new control signaling does not need to introduce new NAS functionality

- Transport of generic control signaling and control of the generic control signaling are supported

- Termination point of the generic signaling connection is located in

  - Data Network (e.g., analogous to IMS) (Solution #5)

  - 6G CN (Solution #10, Solution #11, Solution #12)

- The transport path for generic signaling connection is supported based on the PDU session

  - N6 (Solution #5)

  - UPF SBI (Solution #10, Solution #11, Solution #12)

- Service request, service capability registration signaling, transport of operator service signaling is supported over the control signaling connection over the user plane

Transport of generic control signaling and control of the generic control signaling are supported

- Termination point of the generic signaling connection is located in

  - Data Network (Solution #5)

  - 6G CN (Solution #10, Solution #11, Solution #12)

- The transport path for generic signaling connection is supported based on the PDU session

  - N6 (Solution #5)

  - UPF SBI (Solution #10, Solution #11, Solution #12)

- Service request, service capability registration signaling, transport of operator service signaling is supported over the user plane

Editor’s Note: UE-CN interface for the  control signaling requires coordination with CT1.

Editor’s Note: Security aspect for the control signaling over user plane needs to be considered

6.1.2.4.1	Description

6.1.2.4.2	Procedures

6.1.2.4.3	Services, Entities and Interfaces

6.1.2.4.4	Issues

* * * Next Change * * * *

6.1.2.4	Solution variant #1.2.4: New mechanism to transport control signaling and operator services over new plane

6.1.2.4.0	Topics addressed and High-level Solution Principles

This solution addresses KI#1.2 and is extracted from Solution #13, Solution #14, Solution #16, Solution #17, Solution #20, Solution #23, Solution #24, Solution #25

This solution proposes the following principles:

The high level summary is:

- Transport of generic control signaling and control of the generic control signaling are supported

- Termination point of the generic signaling connection is located in 6G CN

- The transport path for generic signaling connection is supported based on the new RAN-CN interface (in addition to 6G N2 and 6G N3).

- Service request, service capability registration signaling, transport of operator service signaling is supported over the control signaling connection over the new plane

Editor’s Note: UE-CN interface for the  control signaling requires coordination with CT1.

Editor’s Note: Security aspect for the control signaling over new plane needs to be considered

Editor’s Note: RAN-CN interface requires coordination with RAN3.

6.1.2.4.1	Description

6.1.2.4.2	Procedures

6.1.2.4.3	Services, Entities and Interfaces

6.1.2.4.4	Issues

* * * Next Change * * * *

Annex X: Submitted solutions

X.Y	List of submitted solutions for KI#Y

Table X.Y: List of submitted solutions

| Meeting | Solution# | TDoc | Companies | Subject/Comment |
| --- | --- | --- | --- | --- |
| SA2#173 | 1 | S2-2600150 | Vivo | [KI#1.2] Solution proposal on NAS-based UE-CN interaction mechanism |
| SA2#173 | 2 | S2-2600100 | China Mobile | [KI#1, all bullets] Solution for KI#1 of intent-based signalling framework |
| SA2#173 | 3 | S2-2600434 | Huawei, HiSilicon | [KI#1] Solution #1.X: Solution for E2E control signalling in 6G system |
| SA2#173 | 4 | S2-2600249 | Ericsson | [KI#1, bullet #1.1 and #1.2] New Solution: UE and CN control signalling based on evolvement of 5G NAS |
| SA2#173 | 5 | S2-2600594 | Qualcomm | [KI#1.2] Solution for Support for operator services – a framework |
| SA2#173 | 6 | S2-2600266 | LG Electronics | [KI#1] Distributed NAS signalling support for new operator services with extended NAS architecture |
| SA2#173 | 7 | S2-2600092 | Futurewei | [KI#1, Bullet 2] Operator Service over User Plane Transport |
| SA2#173 | 8 | S2-2600160 | OPPO | [KI#1.2, bullet #1a] control signalling over UP |
| SA2#173 | 9 | S2-2600236 | Lenovo | [KI#1.2, bullet #1] Solution for transport control signalling for operator services over user plane |
| SA2#173 | 10 | S2-2600216 | Lenovo | [KI#1.2, bullet 1] Support control plane signalling for over the top operator services over user plane |
| SA2#173 | 11 | S2-2600219 | Google | KI #1: Generic Service Stratum Framework via User-Plane Tunnels |
| SA2#173 | 12 | S2-2600308 | NEC | [KI#1.1, 2a] Procedure to support generic mechanisms for UE to Core Network interaction to support operator services. |
| SA2#173 | 13 | S2-2600388 | Rakuten Mobile, China Mobile | [KI#1, bullet #2] Solution for UE–Core Interaction in 6G for control signalling to support operator services |
| SA2#173 | 14 | S2-2600397 | TCL | [KI#1] Solutions for generic UE-CN interaction mechanism |
| SA2#173 | 15 | S2-2600472 | ZTE | [KI#1.2] Service request procedure for operator service |
| SA2#173 | 16 | S2-2600489 | CATT | [KI#1.2] Support for operator services based on RAN SBI interface |
| SA2#173 | 17 | S2-2600495 | Samsung | [KI#1.2] Introduction of service plane for operator services |
| SA2#173 | 18 | S2-2600524 | Xiaomi | [KI#1, bullet #2] Solution for generic transport mechanism for UE-CN control signalling |
| SA2#173 | 19 | S2-2600532 | Apple | [KI#1.1, KI#1.2] New solution on Evolved Control Signalling for 6G |
| SA2#173 | 20 | S2-2600572 | InterDigital | [KI#2, KI#1.2] A Generic Framework to Support Communication with Operator Services |
| SA2#173 | 21 | S2-2600198 | NTT DOCOMO | [KI#1] Re-use of 5GS RAN-CN interfaces |
| SA2#173 | 22 | S2-2600311 | CATT | [KI#1] Generic mechanism for Feature discovery and Parameter authorization via NAS signalling |
| SA2#173 | 23 | S2-2600547 | Jio platforms Ltd. | [KI#1.2] Solution to support generic mechanisms for UE-CN interaction for operator services |
| SA2#173 | 24 | S2-2600383 | IIT Bombay | [KI#22, bullet #2.a, KI#1.2 bullet #a)] A service-plane-based architectural solution to support computing services in 6G systems |
| SA2#173 | 25 | S2-2600541 | Ofinno | [KI#1.2] Solution to support generic mechanisms for UE-CN interaction for operator services |
| SA2#173 | 26 | S2-2600592 | Qualcomm | 23.801-01: [KI#1.1] Solution for NAS Control signalling for 6G System. |

* * * End of Changes * * * *