3GPP TSG-WG SA2#173	S2-2601182

# Goa, India, 9 – 13 February, 2026

Source:	ZTE, Apple (Pen-holder(s))

Title:	KI#11: Solution variants

Document for:	Approval

Agenda item:	20.6.11

Work Item:	FS_6G_ARC

Abstract of the contribution: This contribution includes the merged solutions for Key issue#11, support of non-3GPP access to 6G.

1.	Discussion

This pCR proposes to capture:

-	4 solutions variants based on selected input solutions for KI#11 (Support of non-3GPP access)

-	list of the submitted input solutions in Annex X

Table: List of submitted solutions and high level principles for this KI not considered in this paper

| Company | Tdoc | High level principles | Comments to the listed principles |
| --- | --- | --- | --- |
| NEC | S2-2600085 | - This solution aims to optimize the untrusted non-3GPP access to 6G for IMS voice by reducing 2 IPSec tunnels to 1 IPSec tunnel from UE perspective. - It suggests studying the removal of the IPSec tunnel between the UE and the untrusted non-3GPP Gateway, or the removal of the IPSec tunnel between the UE and P-CSCF. | This is a topic for SA3 to study. |
| Lenovo | S2-2600085 | - Cross-Access data session establishment via 3GPP Access: The UE requests a special type of data session over 3GPP access, referred to as "cross-access" data session. The characteristic of this type of data session is that it uses 3GPP access for control-plane signaling but non-3GPP access for user-plane communication. | This solution does not fulfill the requirements for this KI, i.e. the UE can access the 6G CN via non-3GPP access even if there is no 3GPP access available. |

2.	Text proposal

It is proposed to agree the following changes vs. TR 23.801-01:

* * * First Change * * * *

6.11	Solutions to KI#11

6.11.1	Solution variant #11.1: NAS-enabled UE

6.11.1.0	Topics addressed and High-level Solution Principles

This solution addresses the following bullet of KI#11:

For non-3GPP access:

1.	Study how to support untrusted non-3GPP access in 6G System architecture.

This solution merges the technical essence of solutions #3, #4, #5, #6, #7, and #8 of Annex X.11. The high-level solution principles:

- 	NAS Support in UE: The UE shall support the 6G NAS protocol stack to maintain consistency between 3GPP and non-3GPP access regarding registration, mobility management, session management.

-	In a solution variant (solution #7), the 6G-N3IWF performs the NAS Registration procedure on behalf of the 6G UE but subsequent NAS procedures are UE-initiated.

- 	Introduce a gateway similar to N3IWF: The 6G Non-3GPP InterWorking Function (6G-N3IWF) acts as a transparent relay for NAS signaling and EAP authentication, ensuring the core network remains access-agnostic. The 6G-N3IWF is similar to the N3IWF in 5G.

- 	Common Security Framework: Mutual authentication is performed using EAP-AKA' or 5G-AKA relayed to the 6G AUSF/UDM over a secure transport layer.

6.11.1.1	Description

The architecture in Figure 6.11.1.1-1 shows how to support the untrusted non-3GPP access in 6G system. The specific NFs are defined in solutions for other KIs, e.g., in KI#1, KI#2, KI#4, KI#18. Therefore, the 6G CN is simply illustrated by grouping the NFs into Control Plane functions (6G CP) and User Plane functions (6G UP).

Figure 6.11.1.1-1: Architecture for non-3GPP access

The interfaces related to the untrusted non-3GPP access in 6G architecture are defined as below:

Y1'	Reference point between the UE and the untrusted non-3GPP access (e.g. WLAN). This depends on the non-3GPP access technology and is outside the scope of 3GPP.

Y2'	Reference point between the untrusted non-3GPP access and the 6G-N3IWF for the transport of NWu traffic.

NWu'	Reference point between the UE and 6G-N3IWF for establishing secure tunnel(s) between the UE and 6G-N3IWF so that control-plane and user-plane exchanged between the UE and the 6G CN is transferred securely over untrusted non-3GPP access.

6G N1	Reference point between the UE and CP NFs for 6G NAS signalling exchange. The 6G NAS signalling exchange over 6G N1 interface is securely transmitted over the untrusted non-3GPP access.

6G N2	In addition to the functionality defined in the overall architecture, the 6G N2 is used to transport signalling between 6G UE and CP NFs for 6G UE authentication to enable the CP NFs to perform 6G-N3IWF configuration.

6G N3	The functionality defined in the overall architecture is reused.

NOTE 1:	The 6G interfaces will be determined as part of KI#1.

Editor’s Note: The roaming architecture for untrusted non-3GPP access is FFS.

The key properties of the architecture are the following:

- 	A unified 6G CN for both 3GPP and non-3GPP access includes the 6G CP and 6G UP. The UE and the 6G CP NFs maintain the UE context independently for each access. The UE and the 6G CP NFs perform the NAS procedures independently for each access.

- 	A new NF 6G-N3IWF is introduced, which is similar to the N3IWF in 5G. The 6G-N3IWF connects to the 6G CP via 6G-N2 interface and connects to the 6G UP via 6G-N3 interface.

- 	The 6G UE discovers the 6G-N3IWF by enhancing the ePDG/N3IWF discovery procedures.

The key properties of the signalling protocol between UE and 6G-N3IWF are the following:

- 	Most of solutions (solutions #4, #5, #6, #7, #8) use IKEv2/IPsec between UE and 6G-N3IWF.

- 	When IKEv2 is applied, the first child SA, i.e. the "signaling IPsec SA", is used for the secure transport of 6G NAS messages (solutions #4, #8).

- 	A solution variant (solution #3) use NAS over QUIC instead of NAS over IKEv2/IPsec between UE and 6G-N3IWF to improve performance and firewall traversal. The 6G-N3IWF acts as a transparent forwarder for NAS messages between the UE and the CP Functions during the initial phase.

For UE registration and authentication procedures, there are different approaches proposed:

- 	Decoupled Authentication and Registration (solutions #4, #8). The secure transport tunnel is fully established before the initiation of the 6G NAS registration procedures. There are two phases:

- 	In phase 1, establishment of IKEv2 signalling and authentication. The UE selects the 6G-N3IWF to establish the IPSec tunnel. The UE source IP address is allocated by 6G-N3IWF. The EAP authentication payload is encapsulated in the IKEv2 message, similar as the EAP authentication over IKEv2 between the UE and ePDG. The UE is authenticated by the 6GC based on the EAP procedures.

- 	In phase 2, the NAS messages are transported between the UE and the CN over the IPSec tunnel established in phase 1.

- 	Coupled Authentication and Registration (solution #5). Initial NAS PDUs (e.g., Registration Request) are encapsulated within the initial transport handshake (e.g., IKE_AUTH) to reduce connection establishment latency.

- 	Proxy-Initial Registration (solution #7). Upon receiving the initial IKEv2 signalling (e.g. IKE-AUTH Req) from the 6G UE, the 6G-N3IWF initiates the Registration request towards the 6G CN on behalf of the 6G UE. The Registration Request contains information extracted from the initial IKEv2 signalling. After registration, a secure connection (i.e. IPSec tunnel) is established between 6G UE and 6G-N3IWF via which 6G NAS signalling can be exchanged between 6G UE and the CP NFs.

- 	NAS over QUIC, Two-Phase Bootstrapping (solution #3). Phase 1 establishes an initial certificate-based QUIC connection to perform NAS authentication and derive a session key. Phase 2 establishes a final, fully encrypted QUIC tunnel using that key as a TLS-PSK.

The key properties of the Protocol and User Plane Mapping and Traffic Differentiation are the following:

- 	Two-Level User Plane: to enhance transmission efficiency, the protocol stack is optimized to two IP layers, eliminating the 5G "Inner IP" layer (solution #8).

- 	The signalling IPsec SA is used to exchange the NAS message. All PDU sessions share the same IKE SA, i.e. use the different child SA (IPsec SA) for different PDU session (solutions #7, #8). It is also possible to establish a new IKE SA for PDU session (solution #8).

- 	SPI-based Mapping: The gateway identifies user-plane tunnels using the Security Parameter Index (SPI) in the ESP header, allowing direct mapping to 6G-N3 tunnels (solution #8)

The control plane and user plane protocol stacks are represented on the following figures (based on solution #8):

- IPSec protocol stack in phase 1

(b) NAS protocol stack in phase 2

Figure 6.11.1.1-2: Control plane protocol stack between UE and CN

Figure 6.11.1.1-3: User plane protocol stack between UE and CN

Editor's note: It is FFS if the GRE layer is needed, depending on whether the QoS related information needs to be transported between the UE and Gateway.

NOTE 2:	How NAS is terminated in the 6G CN depends on the KI#1 conclusions.

6.11.1.2	Procedures

Guidance:

- 	include in this clause: description of the procedures and information flows for the solution.

- 	this clause is not a priority in the February meeting and can be added in a later meeting, but the penholders can make a judgement whether it can be proposed already in February (e.g. if it is straightforward and aligned in this solution variant)

6.11.1.3	Services, Entities and Interfaces

Guidance:

-	include in this clause: description of the Services, Entities and Interfaces assumed by the solution. If existing Services, Entities and/or Interfaces are impacted (e.g. 5G), describe the impacts.

- 	this clause is not a priority in the February meeting and can be added in a later meeting, but the penholders can make a judgement whether it can be proposed already in February (e.g. if it is straightforward and aligned in this solution variant)

6.11.1.4	Issues

Guidance:

- 	include in this clause: description of the open issues related to the solution variants. Open issues may e.g be due to a solution variant based on individual solution proposals that have large commonalities but differ in specific aspects, or due to aspects brought up during the discussion of solutions. The description of the open issues can e.g. consist of a set of Editor’s Notes with FFS statements.

* * * Next Change * * * *

6.11.2	Solution variant #11.2: NAS-GW initiated NAS procedures on behalf of the UE

6.11.2.0	Topics addressed and High-level Solution Principles

This solution addresses the following bullet of KI#11:

For non-3GPP access:

1.	Study how to support untrusted non-3GPP access in 6G System architecture.

This solution is based on solution #13 of Annex X.11. The high-level solution principles are the following:

- 	No NAS support in UE: The solution avoids mandating NAS signalling over non-3GPP access and is based on widely supported UE capabilities such as QUIC and EAP-AKA’.

- 	Introduce a gateway: The 6G Non-3GPP NAS Gateway (6G NAS-GW) acts as a Gateway. It performs registration, session management, policy enforcement and maintains the security context on behalf of the UE.

- 	Access-independent registration: The architecture decouples 3GPP and non-3GPP access procedures while maintaining a single core network session anchor.

6.11.2.1	Description

The architecture in Figure 6.11.2.1-1 shows how to support the untrusted non-3GPP access in 6G system. The specific NFs are defined in solutions for other KIs, e.g., in KI#1, KI#2, KI#4, KI#18. Therefore, the 6G CN is simply illustrated by grouping the NFs into Control Plane functions (6G CP) and User Plane functions (6G UP).

Figure 6.11.2.1-1: Architecture for non-3GPP access

The proposed solution is based on the principle that integration of untrusted non-3GPP access in 6G should be achieved without imposing unnecessary control-plane requirements on the UE. In particular, the solution avoids requiring the UE to support NAS signalling over non-3GPP access, and instead relies on widely deployed security and transport mechanisms such as QUIC and EAP-AKA’.

The solution further adopts a network-assisted integration model, where a dedicated interworking function (6G NAS-GW) performs 6G CN registration, session management, and policy enforcement on behalf of the UE for non-3GPP access. This enables the UE to obtain equivalent 6G core services over non-3GPP access as over 3GPP access, while keeping the access-specific procedures decoupled from core network control-plane procedures.

Another key principle is the decoupling of 3GPP and non-3GPP access registration. The solution allows a UE to be registered over either access independently, while maintaining a single PDU session anchor in the 6G Core. This enables seamless mobility, handover, and multi-access operation, including support for traffic steering and access selection under core network control.

Finally, the solution is aligned with 6G core network capabilities, including support for network slicing, policy-based QoS control, and ATSSS. These capabilities are applied consistently across 3GPP and non-3GPP access without introducing tight coupling between the access technologies.

QUIC-based secure transport between UE and 6G NAS-GW:

The use of QUIC as a secure transport protocol between the UE and the 6G NAS-GW is proposed in this solution. This approach aims to provide confidentiality, integrity, and robustness over untrusted non-3GPP access while enabling enhanced mobility capabilities aligned with 6G requirements.

In this approach, the UE establishes a QUIC connection towards the 6G NAS-GW over untrusted non-3GPP access (e.g. WLAN). The 6G NAS-GW terminates the QUIC connection and provides a QUIC proxy function, such as a MASQUE-based proxy, to tunnel service-related signaling and user-plane traffic between the UE and the 6G CN. The QUIC connection provides transport-layer security based on TLS 1.3 and supports operation in environments with NATs and middleboxes.

The QUIC-based transport is intended to provide functionality equivalent to the IPsec-based tunnel in terms of secure connectivity and reachability of 6G core services, while offering additional flexibility at the transport layer. The use of QUIC between the UE and the 6G NAS-GW inherently addresses non-3GPP access path switching, allowing the UE to seamlessly change IP addresses or network attachments (like switching Wi-Fi).

The following aspects require further study for the QUIC-based transport:

- mechanisms for binding QUIC connection to the UE NAS context in the 6G NAS-GW;

- APIs, procedures and information elements for conveying session-related and policy-related information (e.g. PDU session identifiers, network slice selection, QoS and traffic steering policies) over the QUIC connection between the UE and 6G NAS-GW;

- mapping of 6G QoS and network slicing semantics to QUIC streams, datagrams, or proxy tunnels.

Architecture assumptions:

The proposed solution is based on the following architectural assumptions:

- A new or enhanced interworking function, referred to as the 6G Non-3GPP Interworking Function (6G NAS-GW), is available to terminate secure UE connections over non-3GPP access and to interface with the 6G CN.

- The UE is capable of establishing secure connections to the 6G NAS-GW using QUIC and EAP-AKA’, but is not required to support NAS signalling over non-3GPP access.

- The 6G NAS-GW is able to perform registration, authentication, and PDU session management procedures with the 6G Core Network on behalf of the UE, using existing or evolved core network interfaces.

- PDU sessions are anchored in the 6G Core Network and can be associated with multiple access types, enabling mobility, handover, and multi-access connectivity without changing the UE’s IP address.

- Policy control, network slicing, and traffic steering decisions are provided by the 6G CN and enforced consistently across 3GPP and non-3GPP access.

6.11.2.2	Procedures

Guidance:

- 	include in this clause: description of the procedures and information flows for the solution.

- 	this clause is not a priority in the February meeting and can be added in a later meeting, but the penholders can make a judgement whether it can be proposed already in February (e.g. if it is straightforward and aligned in this solution variant)

6.11.2.3	Services, Entities and Interfaces

Guidance:

-	include in this clause: description of the Services, Entities and Interfaces assumed by the solution. If existing Services, Entities and/or Interfaces are impacted (e.g. 5G), describe the impacts.

- 	this clause is not a priority in the February meeting and can be added in a later meeting, but the penholders can make a judgement whether it can be proposed already in February (e.g. if it is straightforward and aligned in this solution variant)

6.11.2.4	Issues

Guidance:

- 	include in this clause: description of the open issues related to the solution variants. Open issues may e.g be due to a solution variant based on individual solution proposals that have large commonalities but differ in specific aspects, or due to aspects brought up during the discussion of solutions. The description of the open issues can e.g. consist of a set of Editor’s Notes with FFS statements.

6.11.3	Solution variant #11.3: No NAS support

6.11.3.0	Topics addressed and High-level Solution Principles

This solution addresses the following bullet of KI#11:

1.	Study how to support untrusted non-3GPP access in 6G System architecture.

This solution merges the technical essence of solutions #1, #2, #9, #10, #11, #12, #14 and #15 of Annex X.11. The high-level solution principles are the following:

- 	No NAS support: The NAS protocol is not used over non-3GPP access.

- 	Introduce a gateway similar to ePDG: The Non-3GPP access Gateway Function (N3GW) is similar to the ePDG in the EPC. It does not support NAS signalling between the 6G CN and the UE.

- 	Common Security Framework: Mutual authentication is performed using EAP-AKA' or 5G-AKA relayed to the 6G AUSF/UDM over a secure transport layer. The architecture eliminates legacy Diameter-based interfaces.

6.11.3.1	Description

The architecture shown in Figure 6.11.2.1-1 shows how to the support of untrusted non-3GPP access in 6G system. The specific NFs are defined in solutions for other KIs, e.g., in KI#1, KI#2, KI#4, KI#18. Therefore, the 6G CN is simply illustrated by grouping the NFs into Control Plane functions (6G CP) and User Plane functions (6G UP).

Figure 6.11.2.1-1: Architecture for non-3GPP access

Figure 6.11.2.1-2: Reference point Architecture for non-3GPP access

Sx1: derived from SWu: to communicate with the UE over untrusted Wi-Fi. It is based on IKEv2/IPsec for tunnel establishment and encryption (see TS 33.402).

Sx2: derived from SWn: to communicate with the untrusted Non-3GPP access (e.g., Wi-Fi Access Point).

The key properties of the architecture are the following:

- 	No NAS support: The NAS protocol is not used over non-3GPP access.

- 	The N3GW has signalling interfaces with multiple NFs in 6G CP and connects to the 6G UP via 6G-N3 interface.

- 	The N3GW terminates secure transport from the UE. The N3GW does not support NAS signalling between the 6G CN and the UE.

- 	The 6G UE discovers the N3GW by enhancing the ePDG/N3IWF discovery procedures.

The N3GW interfaces with the 6G CN via

SBI-interfaces (solutions #1, #2, #9, #10, #11, #12, #14, #15): The N3GW translates access-specific transport signaling (IKEv2 or QUIC) directly into Service-Based Interface (SBI) operations. It can interact with UDM, AUSF, NRF, SMF directly.

- 	In solution #15, the N3GW has the S2b-C interface with SMF and S2b-U interface with UPF.

The key properties of the signalling protocol between UE and N3GW are the following:

While the UE operates without a NAS stack, the solution supports two primary transport options:

- 	IKEv2/IPsec variant (solutions #1, #2, #9, #11, #12, #15): similar with 4G and 5G, the UE uses standard IKEv2/IPsec to establish a secure tunnel and uses it for authentication. It reuses the ePDG-style secure tunnel where connectivity parameters are carried in IKEv2 payloads (e.g., IDr or Configuration payloads) instead of NAS containers.

- 	QUIC variant (solutions #2, #10, #14): The UE uses QUIC to establish a secure tunnel with N3GW. In order to transfer the signalling, there are two options:

1) 	HTTP/3-based control plane protocol over QUIC (solution #10).

2)	MASQUE over HTTP/3 over QUIC (solution #14).

For both options, the N3GW extracts parameters from transport-layer handshakes (IKEv2 or QUIC) and maps them directly to 6GC service operations (solutions #1, #2, #9, #10, #11, #12, #14, #15).

For UE authentication procedures, there are different approaches proposed:

- 	Variant A (solutions #1, #2, #9): SBI-based mutual authentication is executed between the UE and 6G AUSF/UDM (e.g., Nx1, Ny), e.g. utilizing EAP-AKA'.

- 	Variant B (solution #12): SBI interface is used to connect N3GW with 6G AAA.

- 	Variant C (solution #14): Temporary ID Management. The UDM de-conceals the UE's concealed identity (SUCI) and generates a Temporary ID linked to the UE's permanent identifier, which the N3GW stores and uses it for subsequent session management.

The key properties of the Protocol and User Plane Mapping and Traffic Differentiation are the following:

IKEv2/IPsec variant:

- 	Two-Level User Plane: To enhance transmission efficiency, the protocol stack is optimized to two IP layers, eliminating the 5G "Inner IP" layer (solutions #2, #9, #12).

- 	For each UE, there is only one IKEv2 SA between UE and N3GW. All the PDU session are carried in the IPsec Child SAs within a single IKEv2 SA. Similarly to 4G, there is one IKEv2 SA between UE and N3GW for each PDU session. All the QoS flows of this PDU session are carried in the IPsec Child SAs within the IKEv2 SA.

In the QUIC variant, the PDU is conveyed over the stream in the QUIC connection between the UE and N3GW. Each stream is associated with a specific QoS flow. Multiple PDU sessions share the same QUIC connection.

The control plane and user plane protocol stacks for the IKEv2 variant are represented on the following figures (based on solution #2):

Figure 6.11.3.1-4. Control and User Plane Protocols for N3GW based non-3GPP access

6.11.3.2	Procedures

Guidance:

- 	include in this clause: description of the procedures and information flows for the solution.

- 	this clause is not a priority in the February meeting and can be added in a later meeting, but the penholders can make a judgement whether it can be proposed already in February (e.g. if it is straightforward and aligned in this solution variant)

6.11.3.3	Services, Entities and Interfaces

Guidance:

-	include in this clause: description of the Services, Entities and Interfaces assumed by the solution. If existing Services, Entities and/or Interfaces are impacted (e.g. 5G), describe the impacts.

- 	this clause is not a priority in the February meeting and can be added in a later meeting, but the penholders can make a judgement whether it can be proposed already in February (e.g. if it is straightforward and aligned in this solution variant)

6.11.3.4	Issues

Guidance:

- 	include in this clause: description of the open issues related to the solution variants. Open issues may e.g be due to a solution variant based on individual solution proposals that have large commonalities but differ in specific aspects, or due to aspects brought up during the discussion of solutions. The description of the open issues can e.g. consist of a set of Editor’s Notes with FFS statements.

* * * Next Change * * * *

6.11.4	Solution variant #11.4: Service continuity and 5GS interworking

6.11.4.0	Topics addressed and High-level Solution Principles

This solution addresses the following bullet of KI#11:

2.	Study how to support service continuity between 3GPP access and non-3GPP access in the above bullet(s).

3.	Study how to support Interworking aspects between 5GS and 6GS related to non-3GPP access.

This solution merges the technical essence of solutions #2, #5, #6, #9, #10 and #13 of Annex X.11. The high-level solution principles are the following:

- 	Unified User plane Anchoring: Service continuity and 5GS interworking are achieved by anchoring the PDU Session in a combined UPF. By maintaining the same anchor point in the CN, the network ensures UE IP address preservation when the UE moves between 3GPP and non-3GPP access paths, or between 5G and 6G.

- 	The UE in the target CN re-establishes the PDU session with "existing PDU session" flag.

NOTE:	The architecture for 5GS interworking aspects between 5GS and 6GS related to non-3GPP access depend on the output of KI#17. In this solution, it assumes that the 6G UDM and 5G UDM are co-located, the 6G SMF and 5G SMF are co-located, and the 6G UPF and 5G UPF are co-located.

6.11.4.1	Description

Guidance – include in this clause:

- 	description of the solution principles and architecture assumptions for corresponding key issue(s). Further (sub-) clause(s) may be added as needed.

6.11.4.2	Procedures

Guidance:

- 	include in this clause: description of the procedures and information flows for the solution.

- 	this clause is not a priority in the February meeting and can be added in a later meeting, but the penholders can make a judgement whether it can be proposed already in February (e.g. if it is straightforward and aligned in this solution variant)

6.11.4.3	Services, Entities and Interfaces

Guidance:

-	include in this clause: description of the Services, Entities and Interfaces assumed by the solution. If existing Services, Entities and/or Interfaces are impacted (e.g. 5G), describe the impacts.

- 	this clause is not a priority in the February meeting and can be added in a later meeting, but the penholders can make a judgement whether it can be proposed already in February (e.g. if it is straightforward and aligned in this solution variant)

6.11.4.4	Issues

Guidance:

- 	include in this clause: description of the open issues related to the solution variants. Open issues may e.g be due to a solution variant based on individual solution proposals that have large commonalities but differ in specific aspects, or due to aspects brought up during the discussion of solutions. The description of the open issues can e.g. consist of a set of Editor’s Notes with FFS statements.

* * * Next Change * * * *

Annex X: Submitted solutions

X.11	List of submitted solutions for KI#11

Table X.11: List of submitted solutions

| Meeting | Solution# | Company | TDoc | Subject/Comment |
| --- | --- | --- | --- | --- |
| SA2#173 | 1 | Qualcomm | S2-2600227 | [KI#11] Solution based on Non-3GPP access gateway Function (N3F) |
| SA2#173 | 2 | Ericsson | S2-2600253 | [KI#11, bullets #1 and #2] Non-3GPP access connectivity over N3GW for 6G CN |
| SA2#173 | 3 | Lenovo | S2-2600256 | [KI#11, bullet#1] QUIC-based Non-3GPP Access with 6G-N3IWF |
| SA2#173 | 4 | LG Electronics | S2-2600279 | [KI#11, bullet #1] Decoupling NAS and IKE signalling over untrusted non-3GPP access |
| SA2#173 | 5 | MediaTek Inc. | S2-2600282 | [KI#11, bullet #1-3]: Support of non-3GPP access in 6G |
| SA2#173 | 6 | Samsung | S2-2600313 | [KI#11, bullet 1,2,4] Method for supporting Non-3GPP |
| SA2#173 | 7 | Xiaomi | S2-2600329 | [KI#11, bullet#1] Solution on supporting of non-3GPP access |
| SA2#173 | 8 | Huawei, HiSilicon | S2-2600335 | [KI#11, bullet#1] Solution for bullet#1 of KI#11: Architecture and procedures for N3G access |
| SA2#173 | 9 | ZTE | S2-2600339 | [KI#11] A N3PDG based solution to support non-3GPP access |
| SA2#173 | 10 | ZTE | S2-2600340 | [KI#11] QUIC-based protocol for access to core network via non-3GPP access in 6G |
| SA2#173 | 11 | China Telecom | S2-2600354 | [KI#11, bullet #1] Solution for untrusted Non-3GPP access in 6G |
| SA2#173 | 12 | OPPO | S2-2600395 | KI#11: New solution on un-trusted non-3GPP access support |
| SA2#173 | 13 | CableLabs, Comcast, Rogers | S2-2600554 | [KI#11, bullet #1, #2] Untrusted non-3GPP using NAS-terminated IWF |
| SA2#173 | 14 | Charter, CableLabs, Comcast, Rogers | S2-2600558 | [KI#11, bullet #1] 6G non-3GPP Architecture leveraging QUIC |
| SA2#173 | 15 | Apple | S2-2600606 | [KI#11, bullet #1] Support of Untrusted Non-3GPP Access in 6G via enhanced ePDG architecture |

* * * End of Changes * * * *