3GPP TSG-WG SA2#173	S2-2601163r02

# Goa, India, 9 – 13 February, 2026

Source:	China Mobile, Apple, Samsung (Pen-holders)

Title:	KI#19: Solution variants

Document for:	Approval

Agenda item:	20.6.19

Work Item:	FS_6G_ARC

Abstract of the contribution: This paper proposes the solutions for the KI#19 of Network for AI.

1.	Discussion

2.	Text proposal

It is proposed to agree the following changes vs. TR 23.801-01:

* * * First Change * * * *

6.19	Solutions to KI#19

Table 6.19-1: Mapping of Solution variants to KI#19 bullets

|  | Key Issues # 19 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Solutions | Bullet#1 | Bullet#2 | Bullet#3 | Bullet#4 | Bullet#5 |
| #19.1 | X | X |  |  |  |
| #19.2 | X | X |  |  |  |
| #19.3 |  |  | X |  |  |
| #19.4 |  |  |  | X |  |
| #19.5 |  |  |  |  | X |

6.19.1	Solution variant #19.1: 6G CN support the UE AI agent registration, discovery and communication

6.19.1.0	Topics addressed and High-level Solution Principles

This solution variant addresses KI#19, bullet 1 and bullet 2.

1.	Study whether and how to support an AI agent on a UE to discover another AI agent on a different UE via the 6G network(s).

2.	Study whether and how to enable communication for AI agents on different UEs via the 6G network(s) e.g. identification and authorization of an AI agent on a UE.

This solution variant is extracted from Solutions#1,2,3,4,5,6,8,9,10,11,12,13,14,15,16,17,18,19, 20 (19 solutions) of Annex X.19.1.

The followings are high-level solution principles:

(1) UE AI agent ID assignment/generation

-	AI agent Identification Functionality in 6G CN is responsible for the AI agent ID assignment/generation for specific UE AI agent.

NOTE: This AI agent ID assignment/generation is not applied to all solutions.

-	UE AI agent request the AI agent Identification Functionality to assignment AI agent ID via NAS message, and the AI agent ID may be associated with UE ID.

-	On behalf of UE AI agent, the AF may send the UE AI agent registration request to the AI agent Identification Management Functionality, which may trigger the AI agent ID generation.

(2) UE AI agent registration, authentication and authorization.

-	UE AI agent registers in the 6G AI agent Management Functionality via NAS signalling, with providing AI agent ID and related attributes/capabilities. The 6G AI agent Management Functionality can authenticate and authorize the registration.

-	UE AI agent registration, authentication and authorization with AI agent Management Functionality via user plane

-	During registration, the UE AI agent may register the Unified Agentic Skill Profile to AI agent Management Functionality.

-	AF, on behalf of UE, trigger the UE AI agent registration, authentication and authorization.

(3) UE AI agent discovery

-	UE AI agent sends discovery request via NAS signalling to AI agent Management Functionality, with providing e.g., target AI agent identifier(s), the AI agent type, AI agent capabilities/AI agent profiles/ attributes/ skills.

-	UE AI agent discovery can be via user plane.

-	The AI agent Management Functionality discover the target UE AI agent, and response the target UE AI agent related information to source UE AI agent e.g., the AI agent ID, IP address, FQDN, AI Agent profile or the skills of AI agent.

(5) UE AI agent communication

-	One-to-one communication

-	Based on the above UE AI agent discovery, the source UE AI agent send traffics to the target UE AI agents via 6G user plane or 6G control plane;

-	For via 6G user plane, may direct communication e.g., LAN, or indirect communication via AGW/AI agent proxy in network.

-	During the communication, the distributed authorization may happen between UE AI agents.

-	Group communication,

-	The 6G UE AI agent may request the group communication via NAS signalling to AI agent group management functionality.

-	AF may request the group communication to AI agent group management functionality.

Editor’s NOTE: whether the AI agents communication can via data framework is FFS.

6.19.1.1	Description

6.19.1.1.1 UE AI agent ID assignment/generated by 6G CN

AI agent Identification Functionality in 6G CN is responsible for the AI agent ID assignment/generation for specific UE AI agent.

(1) UE AI agent requests the AI agent Identification Functionality to assignment AI agent ID via NAS message, and the AI agent ID may be associated with UE ID.

(2) On behalf of UE AI agent, the AF may send the UE AI agent registration request to the AI agent Identification Management Functionality, which may trigger the AI agent ID generation.

6.19.1.1.2  UE AI agent registration, authentication and authorization

(1) UE AI agent registers in the 6G AI agent Management Functionality via NAS signalling.

(2) UE AI agent registration, authentication and authorization with AI agent Management Functionality via user plane.

(3) AF, on behalf of UE, trigger the UE AI agent registration, authentication and authorization.

6.19.1.1.3  UE AI agent discovery

(1) UE AI agent sends discovery request via NAS signalling to AI agent Management Functionality, with providing e.g., target AI agent identifier(s), the AI agent type, AI agent capabilities/AI agent profiles/ attributes/ skills.

(2) UE AI agent discovery can be via user plane.

6.19.1.1.4  UE AI agent(s) communication

(1) One-to-one communication

Via 6G user plane.

-	For via 6G user plane, may direct communication e.g., LAN, or indirect communication via AGW/AI agent proxy in network.

Via 6G control plane;

(2) Group communication

UE AI agent requests the group communication

AF request the group communication.

6.19.1.2	Procedures

6.19.1.2.1 UE AI agent ID assignment/generated by 6G CN

6.19.1.2.2 UE AI agent registration, authentication and authorization

6.19.1.2.3 UE AI agent discovery

6.19.1.2.4 UE AI agent(s) communication

6.19.1.3	Services, Entities and Interfaces

6.19.1.4	Issues

6.19.2	Solution variant #19.2: UE AI agent registration, discovery and communication within application layer of the operator control domain

6.19.2.0	Topics addressed and High-level Solution Principles

This solution variant addresses KI#19, bullet 1 and bullet 2.

1.	Study whether and how to support an AI agent on a UE to discover another AI agent on a different UE via the 6G network(s).

2.	Study whether and how to enable communication for AI agents on different UEs via the 6G network(s) e.g. identification and authorization of an AI agent on a UE.

This solution variant is extracted from Solution#7 ,18 (2 solutions) of Annex X.19.1.

The followings are high-level solution principles:

-	The AI Agent Identification and Authorization and AI Agent Discovery is performed using application layer signalling that is transported over user plane, as such transparent to the 6G CN or any other CN such as 5G and not impacting NAS. And the application layer is in the operator control domain

-	AI agents can exchange application-layer agent registration/discovery messages with the server over the UP connection

6.X.Y.1	Description

This solution is proposed the UE AI agent registration/authentication/authorization by application enablement layer, and the communication between UE AI agents is over UP. There is no 6G CN enhancement is needed.

- AI Agent Identification & Authorization at the Agentic AI functions in the app enablement layer.

- AI Agent registration at the Agentic AI functions in the app enablement layer.

- AI Agent Discovery at the Agentic AI functions in the app enablement layer.

- AI Traffic Management, for example QoS, PDU Session management or support for group communication supported by the AF.

NOTE: Application layer is under operator control domain

6.X.Y.2	Procedures

6.X.Y.3	Services, Entities and Interfaces

6.X.Y.4	Issues

6.19.3	Solution variant #19.3: Network Capability Exposure Functionalities to AI agent on AF(s)

6.19.3.0	Topics addressed and High-level Solution Principles

This solution variant focuses on KI#19: 6G Network for AI, and the topic addressed is the following:

3.	Study whether and how to enhance network capability exposure functionalities to AI agent on AF(s).

The following are high-level solution principles captured by this solution variant based on the proposals in Solutions #1, #2 , #3 and #4 in Annex X.19.3, with further description available in clause 6.19.3.1:

1.	The network capability exposure interactions between 6G CN and external AI agents on AF(s) can be realized via agentic protocols [Solutions #1, #2 and #3 in Annex X.19.3].

2.	A translation function in the CN is required to enable such interactions, enabling combination of CN-level services into abstractions understandable by AI agents on AF(s) [Solutions #1, #2 in Annex X.19.3]. The translation function may reside in NEF, AF or in a new NF.

3.	The CN is largely agnostic to any agentic protocol used for exposure towards external AI agents [Solutions #1, #2, #4 in Annex X.19.3].

4.	The network capability exposure architecture should support intent-based requests [Solutions #1, #3, #4 in Annex X.19.3]. The intent-based request may be received either from an external UE AI agent or from external AI agents on AF(s) [Solution #4 in Annex X.19.3].

5.	External AI agents on AF(s) may interact with an AI Agent Proxy in the 6G CNwith  [Solution #3 in Annex X.19.3].

NOTE 1:	 The processing of intents within the CN and potential interactions with CN AI Agents are in scope of KI#18.

NOTE 2:	Not all solutions impact the network capability exposure when processing intent.

NOTE 3:	The high-level principles above represent solution options for network capability exposure functionalities.

6.19.3.1	Description

This solution variant is characterized by the following architecture principles, mainly extracted from Solutions #1, #2 in Annex X.19.3:

-	Adopt MCP (Model Context Protocol), A2A (Agent-to-Agent) or a similar agentic protocol as AI-agent-native exposure mechanism towards AI agents on AF(s) that complements existing NEF northbound APIs. MCP provides a common framework that avoids the need for custom-built integrations for each new service.

NOTE 1:	MCP and A2A are illustrative example of agentic protocols. Other agentic protocols may also be applicable.

-	Support a translation function which, acting as MCP server in case MCP is selected, maps MCP messages into CN interactions that are understandable by 6G NFs, and vice versa. The translation function may reside in different entities, namely i) NEF, ii) AF, and/or iii) a new Agent Interface Function (AIF).

Figure 6.19.3.1-1:  NEF as the intermediary between the AI agent on the AF and the core network [Solution #2]

Figure 6.19.3.1-2:  AF as the intermediary between the AI agent on the AF and the core network [Solution #2]

Figure 6.19.3.1-3: High-level system architecture with AIF and intent support [Solution #1]

-	The translation function may combine multiple CN requests/services into a single high-level “tool” or abstraction, which can then be invoked by or provided to an external AI agent. The translation function may also decompose a request by an AI agent into multiple SBI services actionable by the CN.

-	Regardless of where the translation occurs, CN NFs (other than NEF) remain fully agnostic to MCP or any other agentic protocol. This separation ensures that new agent interaction mechanisms can be introduced without modifying CN NF behavior or SBI procedures.

-	The AI agent on the AF discovers the available 6GC tools through mechanisms that depend on whether the AF is classified as untrusted or trusted under operator policy. For an untrusted AF, discovery is achieved through operator‑provisioned information or CAPIF‑based discovery. For a trusted AF, discovery may rely on operator‑provisioned information, CAPIF‑based discovery, or when authorized by the operator NRF‑based discovery.

NOTE 2:	CAPIF behaviour is defined in SA6. Coordination with WT#1.2 (Network Exposure) and SA6 is required for CAPIF-based discovery of available 6GC tools.

-	The network exposure services architecture may support intents, where instead of making multiple API requests, a 3rd party AI app/service would send a single, high-level intent to the network. The Core Network would then be responsible for autonomously interpreting this intent, act on it, and provide a consolidated response to the AI app/service leveraging the MCP framework.

In addition, as part of a broader proposal on the AI Agent 6G architecture in Solution #3 in Annex X.19.3, external AI Agents may communicate with the 6G CN via an AI Agent Proxy in the AI Agent Domain. The AI Agent domain enables an operator to host Network AI Agents, which make 3GPP functionalities available as agentic skills to external AI Agents, and by this, address intent-based requests from UE AI agents and external AI agents. Operators offer (External AI) agentic skills to UE AI Agents, also when the UE is roaming based on a home-routed PDU Session. The AI Agent Proxy authenticates and authorizes external AI Agents and maintains a catalogue of their skills. It also terminates the secure communication with UE AI Agents and external AI Agents and holds the information of which Network AI Agents the external AI Agents may access.

6.19.3.2	Procedures

6.19.3.3	Services, Entities and Interfaces

6.19.3.4	Issues

6.19.4	Solution variant #19.4: 6G CN can provide AI services (i.e. AI inferencing and AI training) to applications (AF or in UE)

6.19.4.0	Topics addressed and High-level Solution Principles

his solution variant addresses Bullet#4 of KI#19:

4.	Study whether and how the 6G CN can provide AI services (i.e. AI inferencing and AI training) to applications (AF or in UE).

Based on the submitted solutions, the following architectural and high-level principles are extracted to enable the 6G CN to provide network AI services to the application in UE or AF.

Common high-level principles for both AI inference and AI training services:

- Enable in 6G core to fully support network AI services for applications (including AI model training and/or inference operation).

- Enable network exposure of AI service for AI model training service and/or inference service to AF.

NOTE:	Network capability exposure will be coordinated and addressed under KI#7 Network Exposure.

- Enable the 6G NF(s) that supports network AI services to register its AI capabilities to the network.

- Enable the 6G CN to authorise or reject the AI service request from AF and/or UE.

High-level principles for network AI model training service:

- Enable the application in UE and/or AF to request the network to provide AI training service.

- Enable a UE to send request/subscription to the core network to request network AI service for AI model training via e.g., CP, UP

- Enable an AF to send request/subscription to the core network to request network AI service for AI model training.

- Enable the AI service NF to collect data from different sources for AI model training.

- Enable the AI service NF to send the AI service results to AI service consumer.

High-level principles for network inference service:

- Enable the application in UE and/or AF to request the network to provide inference service.

- Enable a UE to send request/subscription to the core network to request network AI service for AI inference, e.g. via CP, UP.

- Enable an AF to send request/subscription to the core network to request network AI service for AI model inference.

- Enable the AI service NF to collect data from different sources for AI model inference, e.g. via UP or data framework.

- Enable the AI service NF to send the AI service results to AI service consumer.

Connection aspects:

- A connection is established between UE and AI service NF which executes the AI service (e.g., AI inferencing or AI training).

NOTE x:	If the request to the core network is transferred via control plane, coordination with KI#1 might be needed.

NOTE y:	if data framework aspects will be deployed for data collection, coordination with KI# 21 might be needed.

6.19.4.1	Description

In this section, we list proposals addressing the high-level principles and other technical proposals covering the major concepts and procedures enabling the application of network AI services in UE or AF.

6.19.4.1.0 Scenarios and uses cases for network AI services.

Providing network AI services to applications in UE or AF can maximum the utilisation of network resource and information (Sol#3). Additionally, the network AI service training service could significantly reduce the load of UE or AF and lowers the capabilities requirements for UE to participate AI services (Sol#7). During the AI service operation, the network could consider the network side information (e.g. latency, PER, etc) for AI service coordination to guarantee the service performance, particularly for the AI Inference services requiring low E2E latency requirement (Sol#5).

The potential use cases for network AI model training services may include online training that requires low latency and off-line training that is less delay sensitive (Sol#6). The AI inference service normally requires low-latency. The potential use cases for network AI model training and inference may include: real-time traffic predication and navigation for smart vehicle, smartphones or low capabilities devices to participate AI service, home robots and other smart home user-centric AI service, etc. (Sol#3,4,5,6,7,8).

6.19.4.1.1 High-level principles

- Enable in 6G core to fully support network AI services for applications (including AI model training and/or inference operation).

Different proposals include:

- One AI service NF to be in charge all AI service related functionalities, e.g. AI service operation (e.g. AI training and inference) and other management: Sol#1 S2-2600063 CATT, Sol#2 S2-2600079 ZTE, Sol#7 S2-2600414 Samsung

- The NFs with MTLF and/or AnLF to perform network AI model training and inference, correspondingly: Sol#8 S2-2600454 Nokia

- Different NFs to perform AI operation management (e.g. authorisation, AI operation NF selection, AI model selection, etc.) and AI service operations: Sol#3 S2-2600096 China Mobile, Sol#4 S2-2600158 vivo, Sol#5 S2-2600171 Huawei, Sol#6 S2-2600191 Huawei & China Mobile.

-	Sol#3: introduces an AI Management Function and an AI Processing Function for AI training and inference.

-	Sol#4: introduces control functions for AI operation and computing node.

-	Sol#5, #6: introduces a new CP function (i.e. ASCF) for AI management, a new NF (AIPF) for AI Inference Processing, and a new NF (ATEF) for AI training.

- Enable network exposure of AI service for AI model training service and/or inference service to AF.  Proposed by Sol#5 S2-2600171 Huawei, Sol#7 S2-2600414 Samsung, Sol#8 S2-2600454 Nokia.

NOTE:	Network capability exposure will be coordinated and addressed under KI#7 Network Exposure.

- Enable the 6G NF(s) that supports network AI services to register its AI capabilities to the network.

Different proposals include:

- The NF supports AI service operation (e.g. AI model training and inference) register to the AI management NF: Sol#3 S2-2600096 China Mobile, Sol#5 S2-2600171 Huawei, Sol#6 S2-2600191 Huawei & China Mobile.

- The NF supports network AI service register to NRF: Sol#1 S2-2600063 CATT, Sol#7 S2-2600414 Samsung

- Enable the 6G CN to authorise or reject the AI service request from AF and/or UE.

- For the AI service request from AF, the request could be authorised by:

-	NEF: Sol#7 S2-2600414 Samsung, Sol#8 S2-2600454 Nokia.

-	The dedicated the AI service/ management NF: AI management NF: Sol#3 S2-2600096 China Mobile, Sol#5 S2-2600171 Huawei. Or AI service NF: Sol#7 S2-2600414 Samsung

- For the AI service request from UE, the request could be authorised by the AI management NF: Sol#3 S2-2600096 China Mobile, Sol#5 S2-2600171 Huawei.

For network AI model training service:

- Enable the application in UE and/or AF to request the network to provide AI training service.

Different proposals include:

- Enable UE and AF to request AI model training service from network: Sol#6 S2-2600191 Huawei, Sol#7 S2-2600414 Samsung

- Only enable UE to request AI model training service from network: Sol#1 S2-2600063 CATT, Sol#2 S2-2600079 ZTE, Sol#4 S2-2600158 vivo

- Only enable AF to request AI model training service from network: Sol#3 S2-2600096 China Mobile, Sol#8 S2-2600454 Nokia

- Enable a UE to send request/subscription to the core network to request network AI service for AI model training.

Different proposals on the signalling path include:

- Via CP: Sol#1 S2-2600063 CATT, Sol#6 S2-2600191 Huawei & China Mobile, Sol#7 S2-2600414 Samsung

- Via Data framework: Sol#1 S2-2600063 CATT

- Via UP, the UP is established using CP (NAS) signalling: Sol#2 S2-2600079 ZTE (named as AI connection)

- Enable an AF to send request/subscription to the core network to request network AI service for AI model training.

Different proposals on the signalling path include:

- Untrusted AF via NEF, other AF send request directly: Sol#3 S2-2600096 China Mobile, Sol#7 S2-2600414 Samsung

- Always via NEF: Sol#8 S2-2600454 Nokia, Sol#6 S2-2600191 Huawei & China Mobile (invoking API related service)

- Enable the AI service NF to collect data from different sources for AI model training.

- Data collection for AI model training:

-	UP: Sol#2 S2-2600079 ZTE (named as data connection)

- Data framework: Sol#1 S2-2600063 CATT, Sol#3 S2-2600096 China Mobile, Sol#6 S2-2600191 Huawei & China Mobile, Sol#7 S2-2600414 Samsung

NOTE:	Data framework aspects will be coordinated with KI# 21.

- Enable the AI service NF that generates the AI operation results to send the AI service results to AI service consumer.

- Results notification for AI model training to UE:

-	via UP: Sol#2 S2-2600079 ZTE (named as AI connection)

-	via data framework: Sol#6 S2-2600191 Huawei & China Mobile

-	via CP: Sol#6 S2-2600191 Huawei & China Mobile

- Results notification for AI model training to AF:

-	Network exposure: Sol#6 S2-2600191 Huawei & China Mobile

For network AI inference service:

- Enable the application in UE and/or AF to request the network to provide inference service.

Different proposals include:

- Enable UE and AF to request inference service from the network: Sol#5 S2-2600171 Huawei, Sol#7 S2-2600414 Samsung

- Only enable UE to request inference service from the network: Sol#1 S2-2600063 CATT, Sol#2 S2-2600079 ZTE, Sol#3 S2-2600096 China Mobile, Sol#4 S2-2600158 vivo

- Only enable AF to request inference service from the network: Sol#8 S2-2600454 Nokia

- Enable a UE to send request/subscription to the core network to request network AI service for AI inference.

Different from proposals on the signalling path include:

- Via CP: Sol#1 S2-2600063 CATT

- Via Data framework: Sol#1 S2-2600063 CATT

- Via UP, the UP is established using CP (NAS) signalling: Sol#1 S2-2600063 CATT, Sol#2 S2-2600079 ZTE (named as AI connection), Sol#3 S2-2600096 China Mobile, Sol#5 S2-2600171 Huawei

- Enable an AF to send request/subscription to the core network to request network AI service for AI model inference.

- Untrusted AF sends request via NEF, other AF send request directly: Sol#7 S2-2600414 Samsung

- AF always sends requests via NEF: Sol#8 S2-2600454 Nokia

- AF sends request to Network AI Agent via exposure framework: Sol#5 S2-2600171 Huawei

- Enable the AI service NF to collect data from different sources for AI model inference, e.g. via.

  - UP: Sol#2 S2-2600079 ZTE (named as data connection),

  - Data framework: Sol#1 S2-2600063 CATT, Sol#3 S2-2600096 China Mobile, Sol#7 S2-2600414 Samsung

NOTE:	Data framework aspects will be coordinated with KI# 21.

- Enable the AI service NF that generates the AI operation results to send the AI service results to AI service consumer.

- Results notification for inference to UE:

  - via UP: Sol#2 S2-2600079 ZTE (named as AI connection), Sol#3 S2-2600096 China Mobile, Sol#5 S2-2600171 Huawei

NOTE y:	If the request to the core network is transferred vis NAS, coordination with KI#1 might be needed.

NOTE z:	if data framework will be deployed for data collection, coordination with KI# 21 might be needed.

6.19.4.1.2 Other aspects

Principles 15 – 17 are derived from multiple company proposals listed in points 1) – 3).

Points 4) – 10) are related to the detailed information for UE/AF and 6G CN interaction to support network AI services, to be discussed later.

Points 11) – 13) cover aspects proposed by individual companies.

- Enable flexible AI operation mode between 6G CN and application in UE/AF.

As proposed by Sol#5 S2-2600171 Huawei and Sol#7 S2-2600414 Samsung, different operation modes of AI model training and inference between 6G CN and application in UE/AF could be supported, include:

- The UE/AF and the 6G CN split the AI task in federated approach; and,

- The UE/AF requests the 6G CN to perform the entire AI task.

For the split AI service, the 6G CN and UE/AF can take multiple factors (e.g., transmission latencies, the AI inference processing capabilities, network and UE load, etc.) into account to achieve the optimal AI operation performance.

- Different operation modes of AI model training and inference between 6G CN and application in UE/AF that are provided, include:

- The UE/AF and the 6G CN split the AI task in federated approach; and,

- The UE/AF requests the 6G CN to perform the entire AI task.

- enable flexible configuration of AI service consumer

As proposed by Sol#5 S2-2600171 Huawei and Sol#7 S2-2600414 Samsung, to support various scenarios, the AI service consumer can be an entity different from the UE/ AF that sends the AI service request. For example, a UE may require the network to perform the AI inference and send the inference results to another UE.

- The AI service NF that generates the AI operation results may send the AI service results to UE/AF that requests the AI service or to other given UEs/AFs.

- enable E2E session between AI service consumer and AI service NF

- Sol#2 S2-2600079 ZTE: A user plane connection named AI connection is established between the beyond connectivity service provider and the UE application. It transmits subscriptions/notifications of AI inference and training services, ML model management information (e.g. ML Model ID) and training/inference parameter (e.g. epoch).A user plane connection named data connection is established between the beyond connectivity service provider in the CN and the UE application. It handles transmission of training datasets, ML models, trained ML models, and inference input/output data.

- Sol#4 S2-2600158 vivo: the NET4AI Control Function can invoke the service provided by Computing Control Function to select computing resource for 6G AI service and establish the computing bearer/session between UE and selected computing resource to perform task for UE AI service.

- Sol#5 S2-2600171 Huawei: AI Inference session, which refers to a service session comprising a set of entities participating together with different roles in handling the data traffic associated with an AI Inferencing process. The data traffic managed in an AI Inference session includes the data stream generated by the source entity, and processed data stream based on the original data. ASCF supports dynamic control/management of entities participating in the AI inference session to adjust their execution behaviour(s)/rule(s) during the execution of AI inference processing (e.g., activate/deactivate split/distributed inference, how to aggregate distributed AI inference results).

- An AI service session is established between UE and AI service NF which executes the AI service (e.g., AI inferencing or AI training).

Points 4) – 10) to be discussed later

- Configuration information in AI model training request

- Sol#2 S2-2600079 ZTE: training task type (e.g. initial training, fine-tuning), training hyper-parameters such as number of epochs, batch size, learning rate and optimizer type, QoS requirements such as training completion time and priority level and a training task correlation ID.

- Sol#3 S2-2600096 China Mobile: model profile (e.g., model structure, size, number of layers), and may include the required model performance (e.g. inference accuracy). The request may also include intent information, e.g., “an AI model for navigation, which has capabilities of shortest path calculation, traffic congestion analysis, and dynamic travel time prediction.

- Sol#6 S2-2600191 Huawei & China Mobile:

-	Alt 1 to require model training: Required input parameter: information of the AI model to be trained (e.g., whether consumer-provided model or network owned model, model ID/name, model structure/type/size when network owned model). Optional input parameter: rule(s)/requirement(s) for AI model training (e.g., training method to be used, minimal training data volume, maximal number of epoch/iterations or training time).

-	Alt2 to require model training data: Required input parameters: information of generated training data type to be used for AI training. Optional input parameters: rule(s)/requirement(s) for the data for AI training (e.g., data volume needed, data performance such as accuracy and resolution, geo-location context of the data).

- Sol#7 S2-2600414 Samsung: request type of AI service (e.g. AI model training inference), execution approach (e.g. full offload or collaboratively split between network and UE/AF), and other associated information of the AI operation, e.g. the purpose/event of the AI operation, the type/ID of AI model, metadata, data sensitivity flags, etc.

- Sol#8 S2-2600454 Nokia: the application context (e.g., traffic prediction), training requirements (e.g., model type, performance metrics, delivery preferences) and data requirements (e.g., application data, network data).

- Configuration information in data collection request for AI model training

- Sol#2 S2-2600079 ZTE: the task correlation ID for tracking the training request, data pre-processing requirements (e.g. tensor dimensions and data structure, data size range, labelling accuracy expectations, training/validation split ratios), ML model upload requirements including model format, maximum model size and model architecture requirements.

- Detailed information of AI model training results. The AI service results for AI model training, include:

- Trained model (included in almost all solutions): Sol#6 S2-2600191 Huawei & China Mobile, Sol#7 S2-2600414 Samsung, Sol#8 S2-2600454 Nokia

- AI training data exposure: Sol#6 S2-2600191 Huawei & China Mobile

- Additional info,

-	performance metrics: Sol#6 S2-2600191 Huawei & China Mobile, Sol#8  S2-2600454 Nokia

-	training status information, model metadata, training status, progress updates: Sol#8 S2-2600454 Nokia

-	the task correlation ID for correlation, training performance summary, ML Model metric and/or loss values, total training time and/or epochs completed, trained model size: Sol#2 S2-2600079 ZTE

- AI model storage:

- stored in a new NF ATEF: Sol#6 S2-2600191 Huawei & China Mobile

- Detailed information/requirements in inference request:

- Sol#2 S2-2600079 ZTE: inference task correlation ID, ML model ID and/or ML Model version information, AI inference configuration parameters such as batch size, inference input data specification, times of AI inference, quality requirement for inference, etc., and QoS requirements such as inference completion time and priority level.

- Sol#3 S2-2600096 China Mobile: The request may include the requirements for the inference, e.g. required model structure, size, number of layers, complexity, inference accuracy, inference delay. For example, the robot requests for AI inference performed by YOLO for obstacle avoidance.

- Sol#5 S2-2600171 Huawei: Service ID: the identifier used to identify an AI inferencing service.  Task ID: the identifier used to identify each AI inferencing task within an AI inferencing service. Requested E2E latency: the requested time it takes from sending the prompt from UE to the AI inference processing function to receiving the AI inference results from the AI inference processing function. Requested first response latency: the time it takes from sending the prompt from UE to receiving the first response if the response is not empty. Required AI Inference speed per task: the time taken to generate each subsequent response.	Timestamp of the generation time of the prompt corresponding to the current task. Prompt size. Resolution. Frames Per Second (FPS).

- Sol#7 S2-2600414 Samsung: request type of AI service (e.g. AI inference), execution approach (e.g. full offload or collaboratively split between network and UE/AF), and other associated information of the AI operation, e.g. the purpose/event of the AI operation, the type/ID of AI model, metadata, data sensitivity flags, etc.

- Sol#8 S2-2600454 Nokia: The request may include the application context (e.g., traffic prediction), inference requirements (e.g., model type, performance metrics), data requirements (e.g., application data, network data), delivery preferences (e.g., in real-time, batch results) and expected time.

- Configuration information in data collection request for inference training

- Sol#2 S2-2600079 ZTE: required data format, acceptable data size range, and any pre-processing requirements that the UE should apply before transmission.  The request also includes transfer protocol specifications, and the task correlation ID to correlate this data collection with the original inference subscription.

- Detailed information in AI inference results.

- Sol#2 S2-2600079 ZTE: classification/regression results, detected objects, or other inference outcomes depending on the task type. The transfer may also include the task identifier for correlation, confidence scores or probability distributions associated with the predictions, etc.

- Sol#8 S2-2600454 Nokia: inference results or inference status information include model metadata, performance metrics, or training status or progress updates, depending on what was requested.

Points 11) – 13) proposed by individual companies

- AI service policy related proposals:

- Sol#9 S2-2600540 Lenovo proposes to leverage URSP rules for the AI agentic traffic, in particular for AI agent in UEs. As the AI agentic traffic may need to be classified and routed differently by the UE towards the network. The PCF that provides URSP rules shall be able to analyse the application and supporting AI agent connectivity requirements considering the respective AF guidance (i.e., the AF associated with the application and supporting AI agent), which indirectly influence the creation of URSP rules. The solution proposes the AF to provide application guidance when the service requirements change, e.g., when an AI agent evolves toe PCF. Based on this information, the PCF may determine and dynamically refine the URSP rules and provide corresponding URSP updates as needed to the AI agent in UEs.

- Sol#1 S2-2600063 CATT mentions that the AIMF queries PCF for user AI service policies to determine whether the AI services requested by the user are within the scope of the user's subscription. No more details are given.

- How to guarantee the desired KPI(s) for AI inference service

- Sol#5 S2-2600171 Huawei proposes to estimate the inferencing processing latency based on the AI inferencing requirements and processing resource, to further determine the transmission delay budget and resources in order to fulfil the desired KPI(s). The 6G AI management NF dynamically sends control messages/signalling to UE and/or AI inference processing function to adjust the AI inference processing. Meanwhile, UE can dynamically exchange the AI inference data parameters required for AI inference processing (e.g., task ID, prompt size, desired KPI(s)) to the AI inference processing function via the AI inference coordination layer between the UE and AI inference processing function over UP, to estimate the AI inference processing time and transmission delay budget in real-time to adjust the radio resource allocation or AI inference processing scheduling to fulfil the desired KPI(s). During the AI inferencing processing procedure, AI inference processing resource status and transmission status can be dynamically detected and collected by 6G CN so that coordination between transmission and AI inferencing processing can be performed to guarantee the desired KPI(s).

- Other procedures not covered by high-level principles

- Sol#2 S2-2600079 ZTE proposes the UE and the 6G NF that provide the AI services to exchange ack. information to ensure the UE is informed of the status of the AI operation at network side.

- Sol#4 S2-2600158 vivo correlates AI services for application in UE or AF tightly with computing services in KI #22. The contribution proposes that the AI service in KI#19 and the computing service in KI#22 to share the same framework and network functions to support the operation. The contributions proposes detailed procedures for establishment, modification and release of computing Bearer/Session Establishment for AI tasks.

6.19.4.2	Procedures

6.19.4.3	Services, Entities and Interfaces

6.19.4.4	Issues

6.19.5	Solution variant #19.5: 6G System impacts based on the characteristics of AI traffic

Editor's note:		This clause is void until characteristics of AI traffic are clarified.

* * * Next Change * * * *

Annex X: Submitted solutions

X.Y	List of submitted solutions for KI#Y

Table X.Y: List of submitted solutions

Annex X.19: 6G Network for AI

Annex X.19.1: KI#19 bullet 1&2

Input papers at SA2#173 for KI#19 bullet 1&2:

| Meeting | Solution# | Tdoc | Source | Subject/Comment |
| --- | --- | --- | --- | --- |
| SA2#173 | 1 | S2-2600061 | CATT | 23.801-01: [KI#19, bullet1] new solution for AI Agent discovery via the 6G networks(s) |
| SA2#173 | 2 | S2-2600062 | CATT | 23.801-01: [KI#19, bullet2] new solution for AI Agent communication via the 6G networks(s) |
| SA2#173 | 3 | S2-2600078 | ZTE | 23.801-01: [KI#19 bullets 1, 2] New solution on digital identity distribution, registration, authentication and session establishment for UE AI Agent |
| SA2#173 | 4 | S2-2600094 | China Mobile, Rakuten, CATT, Huawei | 23.801-01: [KI#19 bullet 1&2] Solution for Key Issue #19: 6G Network for AI-Enable AI agent communication |
| SA2#173 | 5 | S2-2600186 | OPPO | 23.801-01: [KI#19, bullet#1, bullet#2] New Solution on 6G network support UE agent identification, authorization and discovery |
| SA2#173 | 6 | S2-2600223 | Google | 23.801-01: KI#19-Skill-Based Distributed Al Agents Coordination Framework |
| SA2#173 | 7 | S2-2600245 | Ericsson, T-Mobile, Verizon | 23.801-01: [KI#19, bullet #1, bullet#2] Support for communication between AI Agents on the application layer via 6G CN |
| SA2#173 | 8 | S2-2600356 | LG Electronics | 23.801-01: [KI#19, bullet#1&bullet#2] New Solution for AI agent registration and discovery |
| SA2#173 | 9 | S2-2600360 | ETRI | 23.801-01: [KI#19, bullet#1] AI Agent Discovery via Agent Repository Function (ARF) |
| SA2#173 | 10 | S2-2600381 | IPLOOK | 23.801-01: [KI#19, bullet #1] New Solution for AI Agent Discovery via the AI Agent Exposure Function in 6G |
| SA2#173 | 11 | S2-2600424 | Xiaomi | 23.801-01: [KI#19, bullet #1 & bullet#2] New solution: Group-based AI agent communication |
| SA2#173 | 12 | S2-2600425 | Xiaomi | 23.801-01: [KI#19, bullet #1 & bullet#2] New solution: AI agent addressing and direct one- to-one communication |
| SA2#173 | 13 | S2-2600432 | Huawei, HiSilicon | 23.801-01: [KI#19, bullet#1,2] Solution for AI agent Communication Network |
| SA2#173 | 14 | S2-2600520 | TOYOTA MOTOR CORPORATION | 23.801-01: [KI#19, bullet #1] UE AI Agent Discovery using UE Agent Capability Profiles |
| SA2#173 | 15 | S2-2600530 | Boost Mobile Network | 23.801-01: [KI#19, bullet 2] Solution for KI#19: distributed authorization for AI agents' communications |
| SA2#173 | 16 | S2-2600536 | Mediatek Inc. | 23.801-01: [KI#19] 6G Network for AI Solution |
| SA2#173 | 17 | S2-2600546 | Ofinno | 23.801-01: [KI#19, bullet #1, bullet#2] UE AI agent Discovery and UE AI agent Session Establishment via 6G CN |
| SA2#173 | 18 | S2-2600561 | InterDigital Inc. | 23.801-01: [KI#19] AI agent Collaboration based on AI Agent Registration/Discovery and AI Agent Group Communication |
| SA2#173 | 19 | S2-2600369 | CSCN | 23.801-01: [KI#18, KI#19] New Solution - Discovery of Al agents via Al agent Repository Function |
| SA2#173 | 20 | S2-2600573 | Qualcomm | 23.801-01: [KI#18/KI#19] Native AI and agentic AI support for 6G |
| SA2#173 | 21 | S2-2600629 | Orange | 23.801-01: [KI#19] Solution for NAS-based LLM Service Commanding in 6G Core Network |

Annex X.19.2: Void

Annex X.19.3: KI#19 bullet 3

Input papers at SA2#173 for KI#19 bullet 3:

| Meeting | Solution# | Tdoc | Source | Subject/Comment |
| --- | --- | --- | --- | --- |
| SA2#173 | 1 | S2-2600224 | Lenovo | [KI#19, bullet 3] Supporting intent-based exposure from AI agent in an AF |
| SA2#173 | 2 | S2-2600452 | Nokia, AT&T | [KI#19, bullet#3] New solution to enhance network capability exposure functionalities to AI agent on AFs |
| SA2#173 | 3 | S2-2600573 | Qualcomm | 23.801-01: [KI#18/KI#19] Native AI and agentic AI support for 6G. |
| SA2#173 | 4 | S2-2600244 | Ericsson, AT&T, T-Mobile USA, Verizon | [KI#18, bullet#1a and bullet#1b] How to fulfil requests from the UE or the AF that includes intents. |

Annex X.19.4: KI#19 bullet 4

Input papers at SA2#173 for KI#19 bullet 4:

| Meeting |  | Solution# | Tdoc | Source | Subject/Comment |
| --- | --- | --- | --- | --- | --- |
| SA2#173 | 1 |  | S2-2600063 | CATT | [KI#19, bullet4] new solution for 6G CN provides AI services to applications(AF or in UE) |
| SA2#173 | 2 |  | S2-2600079 | ZTE | [KI#19 bullet 4] New solution on UE access AI services |
| SA2#173 | 3 |  | S2-2600096 | China Mobile | [KI#9 bullet 4] Solution for KI#19: AI model training and inference performed by 6GC |
| SA2#173 | 4 |  | S2-2600158 | vivo | [KI#19, bullet#4] 6G AI service to UE |
| SA2#173 | 5 |  | S2-2600171 | Huawei, HiSilicon | [KI#19, bullet #4] New Solution for 6G CN Providing AI Inferencing Services |
| SA2#173 | 6 |  | S2-2600191 | Huawei, HiSilicon, China Mobile | [KI#19, bullet #9] new solution: Support AI Training Service in 6G core network |
| SA2#173 | 7 |  | S2-2600414 | Samsung | [KI#19, bullet#4] 6G Network for AI services |
| SA2#173 | 8 |  | S2-2600454 | Nokia | [KI#19, bullet#4] New solution to enable 6G CN to support AI services to applications (UE or in AF) |
| SA2#173 | 9 |  | S2-2600540 | Motorola Mobility UK Ltd., Lenovo | [KI#19] Enabling UE application-based AI agent interactions with the network. |

Annex X.19.5: KI#19 bullet 5

Input papers at SA2#173 for KI#19 bullet 5:

| Meeting | Solution# | Tdoc | Source | Subject/Comment |
| --- | --- | --- | --- | --- |
| SA2#173 | 1 | S2-2600095 | China Mobile | [KI#19, bullet #5] Solution for KI#19 on Network Enhancement based on the characteristics of AI traffic |
| SA2#173 | 2 | S2-2600173 | LG Electronics | [KI#19, bullet#5] New Sol#X: “Traffic Detection based on AI-related traffic type” |
| SA2#173 | 3 | S2-2600393 | vivo | [KI#19, bullet#7] AI traffic characteristics for Generative AI |
| SA2#173 | 4 | S2-2600394 | vivo | [KI#19, bullet#7] Token related AI traffic |
| SA2#173 | 5 | S2-2600403 | OPPO | [KI#19] Discussion and proposal for traffic of AI service and AI codec |
| SA2#173 | 6 | S2-2600467 | Huawei, HiSilicon | [KI#19, bullet#5] [KI#19, bullet#5] AI/ML traffic characteristics |
| SA2#173 | 7 | S2-2600537 | Apple | [KI#19, bullet#5] Solution to Support GenAI Traffic |
| SA2#173 | 8 | S2-2600540 | Lenovo | 23.801-01: [KI#19] Enabling UE application-based AI agent interactions with the network. |

* * * End of Changes * * * *

For Part 1, the following papers are considered to be merged.

|  | Number | Title | Source Companies | Main part |
| --- | --- | --- | --- | --- |
| 1 | S2-2600061 | 23.801-01: [KI#19, bullet1] new solution for AI Agent discovery via the 6G networks(s) | CATT | Impact NF: AIMF, NRF, 1. UE sends the AI Agent discover request to the AIMF, containing the information about AI Agent(s) of other UE(s)\intent\the information of sensing equipment from UE\the information of AI Agent of UEs 2. When AIMF receives the request from UE, it performs request parse and sends the AI Agent query request to the NRF+ 3.Based on the AI Agent query request, NRF+ decides the AI Agent information and sends them to the AIMF. 4.AIMF sends the AI Agent(s) discovery request to other UE(s) and other UE(s) decides which the AI Agent is allowed to be discovered. 5.AIMF sends the AI Agent(s) information of other UE(s) to UE, UE selects the AI Agent of other UE(s) and s ends the AI Agent(s) to 6G network(s) for AI communication. |
| 2 | S2-2600062 | 23.801-01: [KI#19, bullet2] new solution for AI Agent communication via the 6G networks(s) | CATT | Impact NF: AIMF, RMF, - UE sends the AI Agent registration request to the AIMF, containing the information about AI Agent(s) of other UE(s), such as AI Agent ID. - When AIMF receives the request from UE, it performs request parse and sends the AI Agent registration request to the RMF. - Based on the AI Agent query request, RMF completes the identification and authorization of AI agents and finish the registration procedure. - When AIMF receives the AI Agent group communication request from users (e.g., UE), AIMF parses the request and decides the AI Agent which can satisfy the task requirements based on the AI Agent information of UEs. - AIMF sends the task to the different AI Agent(s) and establishes the group communication path for the AI Agents to exchange information and execute the task. - SM is responsible for session management and UPF+ selection to establish the user plane path for AI Agent group communication. |
| 3 | S2-2600078 | 23.801-01: [KI#19 bullets 1, 2] New solution on digital identity distribution, registration, authentication and session establishment for UE AI Agent | ZTE | Impact NF: ACF, DIM, DIR The solution covers two types of AI Agents: Embodied intelligence – AI agents with physical form (e.g. AI robots, smart vehicles) equipped with SIM cards or eSIM cards. Application-level agents – Virtual digital entities such as virtual digital humans running on UEs equipped with SIM cards or eSIM cards. The UE and CN perform digital identity capability negotiation during the initial registration (the registration using SUCI). The CN allocates digital identity to the AI Agent based on the request. The UE and CN perform registration and authentication using the allocated digital identity for agent. The CN may assist the AF (e.g. Agent server) establish collaborative session among UE Agents. A new NF named DIM (Digital Identity Management) is introduced to the CN. This NF provides digital identity allocation service to the UE and perform UE agent authentication in collaboration with DIM and ACF. A new NF named DIR (Digital Identity repository) is introduced to the CN. This NF stores digital identities generated by DIM and perform UE agent authentication in collaboration with DIM and ACF.  A new NF named ACF (Agent Control Function) is introduced to the CN. This NF process the registration request from UE Agent, perform UE agent authentication in collaboration with DIM/DIR and assist the AF (e.g., agent server) for collaborative session establishment among UE Agents. |
| 4 | S2-2600094 | 23.801-01: [KI#19 bullet 1&2] Solution for Key Issue #19: 6G Network for AI-Enable AI agent communication | China Mobile, Rakuten, CATT, Huawei | Impact NF: ACF, IDM, AGW IDM (Identification Management): - Support to store the AI agent subscription information and the association information of user and AI agent. - Support to store AI agent attributes information (e.g., type, environment awareness, executive capability, permission) and update. - Support to update/change AI agent attributes information (e.g., type, environment awareness, executive capability, permission) per request from NEF/AF. ACF (Agent Control Function): - Discovery and selection of AI agents. - Support dynamic group management, including creation, modification and deletion of communication group based on specific task requirements. AGW (AI Agent Gateway): - Support routing and forwarding function for dynamic group communication of AI agents; |
| 5 | S2-2600186 | 23.801-01: [KI#19, bullet#1, bullet#2] New Solution on 6G network support UE agent identification, authorization and discovery | OPPO | Impact NF: MAF The high-level solution principles are: - The agent ID on the UE is defined to support the agent identification. - The agent related subscription is introduced to support the agent authentication and authorization. - The agent profile is introduced to describe the agent related information. - A new NF named MAF (Management agent function) is introduced to support the agent registration. - The MAF stores the agent profile and perform the agent discovery based on the stored agent profile and the required agent discovery criteria. - Two types of the agent ID are defined to support the UE agent identification including the AGPU (Agent Public Identity) and AGPI (Agent private Identity). Each AGPI can correspond to the multiple AGPU. AGPU is used to identify the agents and can be included in the message interact with the network. - The agent related subscription is introduced to support the agent authentication and authorization. The 6G-UDM will store the agent related subscription, the data key can be the AGPI/SUPI. |
| 6 | S2-2600223 | 23.801-01: KI#19-Skill-Based Distributed Al Agents Coordination Framework | Google | Impact NF: ACRF, AAIHF • Phase 1: Unified Agentic Skill Registration (Abstraction & Foundation): o Individual Capability Exposure: Agentic UEs utilize the ACF to perform introspection and transmit their capabilities via NAS signaling within a UE_Agentic_Capability_Container. o Unified Profiling: The message encapsulates the Unified Agentic Skill Profile, a polymorphic data structure that normalizes capabilities across domains using a common Skill URI. o Repository Indexing: The ACRF or Enhanced NRF indexes these skills based on the Unified Agentic Skill Profile, establishing a semantic identity for the agent. • Phase 2: Cluster Formation (Initiation Mechanisms and and Semantic Discovery): During this phase, abstract user intents are decomposed into executable network "missions". o Intent Decomposition: The AAIHF (Agentic AI Host Function) or NWDAF receives an abstract intent (e.g., "Coordinate synchronized sensor fusion") and decomposes high-level intents into required Skill IDs represented by standardized URIs (e.g., mcp://skill/motion/sync-control). o Semantic Matching Engine: The ACRF utilizes a matching engine to calculate the similarity between the "Abstract Intent" and the "Registered Skills" to identify capable agents, even if exact keywords do not match. o Topological Matching: To ensure deterministic performance and members are physically positioned for low-latency collaboration, candidate selection is prioritized based on Network Proximity. The ACRF filters candidates by ensuring they share physical network anchors, such as:  Shared UPF anchors: Ensures low-latency data exchange without traversing the deep core.  RAN node anchors (gNB): Ensures agents are under the same or neighboring radio nodes for coordinated scheduling. o Contextual Binding: Once the optimal candidates are identified, the network generates a unique Task_Cluster_ID. This ID binds independent PDU sessions into a single logical unit, allowing the network to manage them as a synchronized swarm rather than isolated devices. o Mode Transition: The AAIHF issues a Mode_Change_Directive, commanding the selected Agentic UEs to transition from "Individual Mode" to "Cluster Mode," where they subordinate individual logic to network-anchored group logic. • Phase 3: Coordinated Execution (Synchronization & Supporting Mechanisms): o Consensus Arbitration: When members identify a logic conflict, they transmit an AI-Consensus-Request. The AAIHF/NWDAF evaluates real-time metadata (battery, trajectory) from the ACRF/Enhanced NRF to determine a Winning Directive. o Synchronized Mobility Control: To maintain swarm integrity, the network calculates T_sync and distributes a Synchronized_HO_Command to all members. o Simultaneous Trigger: At the precise T_sync timestamp, all members simultaneously execute the handover (CHO) or reconfiguration, ensuring connectivity remains stable. |
| 7 | S2-2600245 | 23.801-01: [KI#19, bullet #1, bullet#2] Support for communication between AI Agents on the application layer via 6G CN | Ericsson, T-Mobile, Verizon | Impact NF: NO To support communication between AI Agents in the UE over the 6G CN, the following functionality are needed: - AI Agent Identification & Authorization at the Agentic AI functions in the app enablement layer. - AI Agent registration at the Agentic AI functions in the app enablement layer. - AI Agent Discovery at the Agentic AI functions in the app enablement layer. - AI Traffic Management, for example QoS, PDU Session management or support for group communication supported by the AF. 6G CN support for AI Agent Discovery and AI Agent communication Steps 1 to step 3 enable the UE AI Agent to discover an AI Agent in another UE with certain characteristics, those steps are performed over user plane. Step 4 enables the UE AI Agent to request to set up resources for the communication over 6G CN, those steps are performed over user plane. Step 5 enables the AF to optionally request QoS for both source and target GPSIs from the 6G CN, i.e. PCF. Step 7 describes how the UE maps application traffic from the UE AI Agent into the PDU Session and QoS flows. The steps related to the AI Agent signalling are steps are only examples as those are transparent to the 6G Core Network. |
| 8 | S2-2600356 | 23.801-01: [KI#19, bullet#1&bullet#2] New Solution for AI agent registration and discovery | LG Electronics | Impact NF: AIMF The high-level solution principles are: - A new network function, AIMF (AI agent Management Function) is introduced to support AI agent management. - The AI agent-related procedure (e.g. AI agent registration, AI agent discovery) may be initiated by UE or AF. - The AIMF receives AI agent-related requests and determines whether the AI agent is authorized for the request. - The AIMF assigns an internal AI agent identifier and may request an external AI agent identifier to UE or AF. - The AIMF discovers AI agents on different UEs and sends discovered list of AI agents to UE or AF. |
| 9 | S2-2600360 | 23.801-01: [KI#19, bullet#1] AI Agent Discovery via Agent Repository Function (ARF) | ETRI | Impact NF: ARF UE send request to AF, and AF interact with the ARF. UE hosts an AI agent(s) and initiates AI agent registration/update and discovery requests via an AF. - AF acts as an application-level entry point for AI agent registration and discovery. The AF forwards registration and discovery requests to the ARF and returns corresponding responses to the UE. - ARF acts as a logical function responsible for managing registered AI agent information for registration and discovery purposes. The ARF stores AI agent metadata, maintains internal associations with corresponding UE context, and processes AI agent discovery requests. - NRF may be consulted by the AF to discover a network function capable of supporting AI agent management functionalities, where the ARF is considered as a logical function. - UDM provides subscription-related information (e.g. via SDM) for authorization purposes and UE context information (e.g. via UECM) to support AI agent registration and context synchronization. |
| 10 | S2-2600381 | 23.801-01: [KI#19, bullet #1] New Solution for AI Agent Discovery via the AI Agent Exposure Function in 6G | IPLOOK | Impact NF:AI agent exposure Function (AAEF) The high-level solution principles are as follows: - the AI Agent Exposure Function (AAEF) is responsible for the registration, discovery, management, and policy-controlled exposure of AI Agent capabilities. This function complements the existing NEF by specializing in AI Agent entities. - The solution integrates with UE registration procedures to establish early awareness of a UE’s capability to host AI Agents and its authorization to use discovery services. |
| 11 | S2-2600424 | 23.801-01: [KI#19, bullet #1 & bullet#2] New solution: Group-based AI agent communication | Xiaomi | Impact NF: ARF, GMF This solution follows the following principles: - A new function, i.e., ARF (AI Agent Repository Function), is introduced. ARF supports registration of AI agent profile and discovery of capable AI Agent(s) to complete a task. - A new function, i.e., GMF (Group Management Function), is introduced. GMF supports to discover capable AI Agent(s) based on task intent and create a dynamic group with allocation of group Id. - 6G SMF supports to authorize PDU session establishment with group Id. - 6G UPF supports to perform local switch by considering group Id. |
| 12 | S2-2600425 | 23.801-01: [KI#19, bullet #1 & bullet#2] New solution: AI agent addressing and direct one- to-one communication | Xiaomi | Impact NF: AARF - A new function, i.e., AARF (Agent Address Repository Function), is introduced to store the association between ID and address of an AI agent. - A source AI agent constructs a FQDN of the target AI agent based on local configuration or configuration info provisioned by the network. The source AI agent send a DNS message with the FQDN via user plane when direct one-to-one communication is required. - SMF configures a DNS handling rule at the 6G UPF to redirects the DNS message to AARF. AARF resolves the DNS message by returning the address of target AI agent. The 6G UPF returns the address to the source AI agent. |
| 13 | S2-2600432 | 23.801-01: [KI#19, bullet#1,2] Solution for AI agent Communication Network | Huawei, HiSilicon | Impact NF: IDM, ARDF, SCF 1. AI agent on UE is an autonomous entity requests/accesses network resource and should be identified by 6GC so that the 6GC can enable the communication for AI agents on different UEs. 2.The identity generation for the AI agent on UE could be controlled by 6GC, where the generated identity will be associated with the UE the AI Agent is hosted. 3. AI agent on UE can register its attributes to 6GC. The attributes can be used for AI agent authorization or AI agent discovery. 4. AI agent on UE is able to discover other AI agents on UEs via 6GC. 5. AI agent on UE is able to establish/join a communication group for cooperation task. 6. 6GC notifies AI agents on UE to join the communication group. 7. 6GC user plane support efficient and secure traffic/message distribution among/between AI agents and enable sync/async task cooperation. 8. AI agents on UEs can be served by different SCFs and UP NFs. |
| 14 | S2-2600520 | 23.801-01: [KI#19, bullet #1] UE AI Agent Discovery using UE Agent Capability Profiles | TOYOTA MOTOR CORPORATION | Impact NF: NRF or UDM or new NF UE AI agent discovery is enabled by defining a UE AI agent capability/attribute profile that can be advertised to the 6G system. A UE AI agent can issue a discovery query that includes required capability information and optional constraints (e.g., area/time/availability), and receive a list of candidate UE AI agents on other UEs. The UE AI agent capability/attribute profile supports dynamic update to reflect changes in capability, state, or constraints, to improve discovery accuracy over time. |
| 15 | S2-2600530 | 23.801-01: [KI#19, bullet 2] Solution for KI#19: distributed authorization for AI agents' communications | Boost Mobile Network | Impact NF: AIDF AIDF: AI Database Function. A function that maintains and has access to all AI agents’ information and their profiles. The AI Producer (AIP), e.g., front desk AI agent, is required to register its profile with the AI Database Function (AIDF) where the AIP profile includes the services the AIP offers, AIP Type, AIP ID, AIP owner ID, etc. 1. The AI agent producer shall be able to authenticate the AI agent consumer first and save the authenticated identity of the AI agent consumer. 2. The AI agent producer shall have a secure access to its own profile information which include, producers offered services, allowed AI agent consumers, Allowed AI Agent consumers groups, Allowed AI agent consumer ownerships, Allowed AI Agent consumer types, etc. 3. The AI agent producer should have access to its operator authorization policy. 4. The AI agent producer shall have access to the AI agent consumer information in a secure manner, e.g., information included in the AI agent consumer certificate, etc. 5. The AI agent producer shall be able to issue an access token that the AI agent consumer can use in future requests. 6. The AI agent producer shall be able to set the access token lifetime based on the AI agent consumer type and the level of security risk associated with the AI agent consumer, etc. 7. The AI agent producer shall have a secure access to its access token signing public/private keys pair. 8. The AI agent is either configured with its access token signing public/private key pair or it can generate the pair and securely protect them. |
| 16 | S2-2600536 | 23.801-01: [KI#19] 6G Network for AI Solution | Mediatek Inc. | Impact NF: not clear, maybe MAF? This solution proposes using the MAF to efficiently manage discovery and communication channels between AI agents on different UEs. These channels are organized according to specific topics, allowing AI agents to communicate and exchange information based on shared interests or functions. The organization of topics works like hashtags, making it easier for agents to discover relevant channels and participate in targeted discussions. |
| 17 | S2-2600546 | 23.801-01: [KI#19, bullet #1, bullet#2] UE AI agent Discovery and UE AI agent Session Establishment via 6G CN | Ofinno | Impact NF: 6G CN - AI agent discovery support is enabled by the 6G CN using UE-AI-agent-related parameters (e.g., AI agent identifier(s), AI agent capabilities) provided by UEs. - Capability exchange is used as an operation gate such that AI-agent-related procedures are performed only when both the UE and the 6G CN indicate support for AI agent communications during registration. - AI agent discovery is based on functional requirements (e.g., AI agent capabilities / task type), rather than relying only on UE identifiers, in order to enable a UE to identify candidate AI agents/UEs that can support the required AI-agent-related tasks and communications. - Discovery and session establishment are treated as separate functions, where discovery provides information to select a candidate UE/AI agent and the communication is enabled via a session establishment procedure. |
| 18 | S2-2600561 | 23.801-01: [KI#19] AI agent Collaboration based on AI Agent Registration/Discovery and AI Agent Group Communication | InterDigital Inc. | Impact NF: ACOF, ARF The ARF or ARS provides Agent Registration Service and Agent Discovery Service. In Alt.1, the AI agents can access the ARF services through an intermediary NF such as a CP signaling anchor function (e.g., AMF). In Alt 2, the AI agents can exchange application-layer agent registration/discovery messages with the ARS over the UP connection. For Alt. 2, the AI agent may need to discover the ARS address (e.g., URL) before invoking the agent registration/discovery services. An Agent Communication Orchestration Function (ACOF) may be introduced in 6G CN to facilitate the communication among the AI agent task group. The ACOF handles the task request from the task group members (i.e., AI agents), analyses and decomposes the task, assign the subtasks to the task group members, determines the UP communication configuration (network slices, QoS profiles, etc.) and triggers the task group members to establish the communication links with the ACOF. The task group members may then exchange the task-related data/message with each other through the ACOF. Task-related data/messages over CP may use “agent layer” protocol messages (e.g. A2A, MCP) that can be piggybacked over UE NAS messages; task-related data/messages over UP is supported by the UP communication links (i.e. PDU Sessions) between the AI agents and the ACOF. |

20.6.18

| 19 | S2-2600369 | 23.801-01: [KI#18, KI#19] New Solution - Discovery of Al agents via Al agent Repository Function | CSCN | Impact NF: ARF - The AI Agent Repository Function manages AI agents both within and outside the 6G CN, such as AI agents on 6G CN entities and AI agents on UEs. - The AI Agent Repository Function can identify the AI agent to be discovered based on the types of tasks the AI agent is capable of performing. - Before providing AI agent information to the AI agent discovery requester, the AI Agent Repository Function shall verify whether the requester is allowed to discover the target AI agent. - The AI Agent Repository Function receives AI agent profile information through AI agent registration procedures and maintains this information dynamically. |
| --- | --- | --- | --- | --- |
| 20 | S2-2600573 | 23.801-01: [KI#18/KI#19] Native AI and agentic AI support for 6G | Qualcomm | Impact NF: AI Agent Proxy, Subscription database, AI agent domain AI Agent Proxy acts as a proxy for interactions between UE AI Agents and NW AI Agents, specifically: - Authenticates and authorizes UE AI Agents and external AI Agents (based on the subscription database) and terminates the secure communication with UE AI Agents and external AI Agents; - Maintains a catalogue of the skills of all NW AI Agents, external AI agents and UE AI Agents; - Enables a UE AI Agent to discover and access skills of NW AI Agents and external AI Agents that the UE AI Agent is authorized to access (according to the UE AI Agent's subscription); - the other UE AI Agents that the UE AI Agent is associated with (according to the UE AI Agent's subscription). - Routes messages between UE AI Agents and Network / external AI Agents; Subscription database holds the following information: - For each subscribed UE AI Agent: - information to authenticate and authorize the UE AI Agent; - information indicating which Network AI Agents and external AI Agents the UE AI Agent may access; - list of associated UE AI Agents; - For each allowed external AI Agent: - information to authenticate and authorize external AI Agents; - information indicating which Network AI Agents the external AI Agent may access. NOTE: How to authenticate and authorize UE AI Agents and external AI Agents is assumed to be addressed by SA3. - UE AI Agents access skills of Network AI Agents, external AI Agents and skills of other UE AI Agents (if authorized by subscription). UE AI Agents may also offer skills to Network AI Agents. |