3GPP TSG-WG SA2#173	S2-2601625

# Goa, India, 9 – 13 February, 2026

Source:	China Mobile, Apple, Samsung (Pen-holders)

Title:	KI#19: Solution variants

Document for:	Approval

Agenda item:	20.6.19

Work Item:	FS_6G_ARC

Abstract of the contribution: This paper proposes the solutions for the KI#19 of Network for AI.

1.	Discussion

2.	Text proposal

It is proposed to agree the following changes vs. TR 23.801-01:

* * * First Change * * * *

6.19	Solutions to KI#19

Table 6.19-1: Mapping of Solution variants to KI#19 bullets

|  | Key Issues # 19 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Solutions | Bullet#1 | Bullet#2 | Bullet#3 | Bullet#4 | Bullet#5 |
| #19.1 | X | X |  |  |  |
| #19.2 | X | X |  |  |  |
| #19.3 |  |  | X |  |  |
| #19.4 |  |  |  | X |  |
| #19.5 |  |  |  |  | X |

6.19.1	Solution variant #19.1: 6G CN support the UE AI agent registration, discovery and communication

6.19.1.0	Topics addressed and High-level Solution Principles

This solution variant addresses KI#19, bullet 1 and bullet 2.

1.	Study whether and how to support an AI agent on a UE to discover another AI agent on a different UE via the 6G network(s).

2.	Study whether and how to enable communication for AI agents on different UEs via the 6G network(s) e.g. identification and authorization of an AI agent on a UE.

This solution variant is extracted from Solutions#1,2,3,4,5,6,8,9,10,11,12,13,14,15,16,17,18,19, 20, 21 (20 solutions) of Annex X.19.1.

The followings are high-level solution principles:

The AI Agent Identification, Authorization, AI Agent Discovery and communication are under the control of 6G CN.

(1) UE AI agent ID assignment/generation (Optional)

NOTE: This AI agent ID assignment/generation is not applicable to all solutions.

AI agent Identification Allocation Functionality in 6G CN is responsible for the AI agent ID assignment/generation for specific UE AI agent.

-	Option 1: UE AI agent requests the AI agent Identification Allocation Functionality to assignment AI agent ID via NAS message. (Sol#2, #3, #8, #13, #18,)

-	Option 2: On behalf of UE AI agent, the AF may send the UE AI agent registration request to the AI agent Identification Management Functionality, which may trigger the AI agent ID assignment/generation. (Sol#9)

(2) UE AI agent registration, authentication and authorization.

-	Option1: UE AI agent registers in the 6G AI agent Management Functionality via NAS signalling, with providing e.g., AI agent ID, skill, related attributes/capabilities. The 6G AI agent Management Functionality can authenticate and authorize the registration. (Sol#2, #3, #4, #5, #6, #8, #11, #13, #14, #15, #18, #19)

-	Option2: UE AI agent registration, authentication and authorization with AI agent Management Functionality via user plane (Sol#20, Sol#18)

-	Option3: AF, on behalf of UE, trigger the UE AI agent registration, authentication and authorization. (Sol#8, #9)

(3) UE AI agent discovery

UE AI agent sends discovery request to AI agent Discovery Control Functionality, with providing e.g., target AI agent identifier(s), skills, related attributes/capabilities.

-	Option1: via NAS signalling. (Sol#1, #4, #5, #10, #11, #13, #8, #17, #18, #14, #19, #20, #21)

Editor’s NOTE: How the NAS solutions work in the case of UE fallback to 5G network is FFS.

-	Option2: via user plane. (Sol#20)

-	Option3: AF triggered discovery (Sol#8, 9)

(4) UE AI agent communication

-	One-to-one communication

-	Option1: The source UE AI agent send traffics to the target UE AI agents via 6G user plane; (Sol#15, #12);

-	UE AI agent to UE AI agent communication via 6G UP/AI agent proxy in network. (Sol#11, #20, #15)

-	During the communication, authorization between UE AI agents is performed and the distributed authorization can be used (Sol#15).

-	Option2: The source UE AI agent send message to the target UE AI agents via 6G CP NF; (Sol#2)

-	Group communication

The group communication can be supported via user plane.

Editor’s NOTE: What group communication refers to is FFS.

-	Option1: The 6G UE AI agent may request the group communication, e.g., for task and logical binding, via NAS signalling to AI agent group management functionality. (Sol#2, #4, #11, #13, #18, #21).

-	Option2: AF may request the group communication to AI agent communication management functionality. (Sol#3, #18).

-	Option3: Agent skill discovery can be used to support the group communication. (Sol#20, #16)

Editor’s NOTE: The definition of the UE AI Agent is FFS.

Editor’s NOTE: Inter-PLMN roaming for UE AI agents is FFS.

Editor’s NOTE: AI agent Management Functionality needs to coordinate with KI#18

Editor’s NOTE: Whether the above functionalities are separate or combined is FFS.

6.19.1.1	Description

6.19.1.2	Procedures

6.19.1.3	Services, Entities and Interfaces

6.19.1.4	Issues

6.19.2	Solution variant #19.2: UE AI agent registration, discovery and communication within application layer of the operator control domain

6.19.2.0	Topics addressed and High-level Solution Principles

This solution variant addresses KI#19, bullet 1 and bullet 2.

1.	Study whether and how to support an AI agent on a UE to discover another AI agent on a different UE via the 6G network(s).

2.	Study whether and how to enable communication for AI agents on different UEs via the 6G network(s) e.g. identification and authorization of an AI agent on a UE.

This solution variant is extracted from Solution#7 ,18 (2 solutions) of Annex X.19.1.

The followings are high-level solution principles:

-	The AI Agent Identification and Authorization and AI Agent Discovery is performed using application layer signalling that is transported over user plane, as such transparent to the 6G CN or any other CN such as 5G and not impacting NAS. And the application layer is in the operator control domain

-	AI agents can exchange application-layer agent registration/discovery messages with the server over the UP connection

NOTE: Application layer is under operator control domain

Editor’s NOTE: The impact for 6GC is FFS and which part will be discussed in SA2 is FFS.

6.X.Y.1	Description

6.X.Y.2	Procedures

6.X.Y.3	Services, Entities and Interfaces

6.X.Y.4	Issues

6.19.3	Solution variant #19.3: Network Capability Exposure Functionalities to AI agent on AF(s)

6.19.3.0	Topics addressed and High-level Solution Principles

This solution variant focuses on KI#19: 6G Network for AI, and the topic addressed is the following:

3.	Study whether and how to enhance network capability exposure functionalities to AI agent on AF(s).

The following are high-level solution principles captured by this solution variant based on the proposals in Solutions #1, #2 , #3 and #4 in Annex X.19.3, with further description available in clause 6.19.3.1:

1.	The network capability exposure interactions between 6G CN and external AI agents on AF(s) can be realized via agentic protocols [Solutions #1, #2 and #3 in Annex X.19.3].

2.	A translation function in the CN is required to enable such interactions, enabling combination of CN-level services into abstractions understandable by AI agents on AF(s) [Solutions #1, #2 in Annex X.19.3]. The translation function may reside in NEF, AF or in a new NF.

3.	The CN is largely agnostic to any agentic protocol used for exposure towards external AI agents [Solutions #1, #2, #4 in Annex X.19.3].

4.	The network capability exposure architecture should support intent-based requests e.g. from external AI agents on AFs [Solutions #1, #3, #4 in Annex X.19.3]..

5.	External AI agents on AF(s) may interact with an AI Agent Proxy in the 6G CNwith  [Solution #3 in Annex X.19.3].

NOTE 1:	 The processing of intents within the CN and potential interactions with CN AI Agents are in scope of KI#18.

NOTE 2:	Not all solutions impact the network capability exposure when processing intent.

6.19.3.1	Description

6.19.3.2	Procedures

6.19.3.3	Services, Entities and Interfaces

6.19.3.4	Issues

6.19.4	Solution variant #19.4: 6G CN can provide AI services (i.e. AI inferencing and AI training) to applications (AF or in UE)

6.19.4.0	Topics addressed and High-level Solution Principles

This solution variant addresses Bullet#4 of KI#19:

4.	Study whether and how the 6G CN can provide AI services (i.e. AI inferencing and AI training) to applications (AF or in UE).

Based on the submitted solutions, the following architectural and high-level principles are extracted to enable the 6G CN to provide network AI services to the application in UE or AF.

Common high-level principles for both AI inference and AI training services:

- Enable in 6G CN to fully support network AI services for applications (including AI model training and/or inference operation).

- Enable network exposure of AI service for AI model training service and/or inference service to AF.

NOTE:	Network capability exposure will be coordinated and addressed under KI#7 Network Exposure.

- Enable the 6G NF(s) that supports network AI services to register its AI capabilities to the network.

- Enable the 6G core to authorise or reject the AI service request from AF and/or UE.

High-level principles for network AI model training service:

- Enable the application in UE and/or AF to request the network to provide AI training service.

- Enable a UE to send request/subscription to the core network to request network AI service for AI model training via e.g. CP, UP

- Enable an AF to send request/subscription to the core network to request network AI service for AI model training.

- Enable data collection from different sources for AI model training.

- Enable 6G CN to send the AI service results to AI service consumer.

High-level principles for network inference service:

- Enable the application in UE and/or AF to request the network to provide inference service.

- Enable a UE to send request/subscription to the core network to request network AI service for AI inference, e.g. via CP, UP.

- Enable an AF to send request/subscription to the core network to request network AI service for AI model inference.

- Enable data collection from different sources for AI model inference, e.g. via UP or data framework.

- Enable 6G CN to send the AI service results to AI service consumer.

Connection aspects:

Principle 15: A connection is established between UE and 6G CN which executes the AI service (e.g., AI inferencing or AI training)

Editor’s Note: whether the connection is via UP or via the application layer is FFS.

NOTE x:	If the request to the core network is transferred via NAS, coordination with KI#1 might be needed.

NOTE y:	if data framework aspects will be deployed for data collection, coordination with KI# 21 might be needed.

6.19.4.1	Description

6.19.4.2	Procedures

6.19.4.3	Services, Entities and Interfaces

6.19.4.4	Issues

6.19.5	Solution variant #19.5: 6G System impacts based on the characteristics of AI traffic

Editor's note:		Which working groups will determine the characteristics of AI traffic is subject to TSG SA plenary decision.

Editor's note:		This clause will be filled out once characteristics of AI traffic are clarified.

* * * Next Change * * * *

Annex X: Submitted solutions

X.Y	List of submitted solutions for KI#Y

Table X.Y: List of submitted solutions

Annex X.19: 6G Network for AI

Annex X.19.1: KI#19 bullet 1&2

Input papers at SA2#173 for KI#19 bullet 1&2:

| Meeting | Solution# | Tdoc | Subject/Comment |
| --- | --- | --- | --- |
| SA2#173 | 1 | S2-2600061 | 23.801-01: [KI#19, bullet1] new solution for AI Agent discovery via the 6G networks(s) |
| SA2#173 | 2 | S2-2600062 | 23.801-01: [KI#19, bullet2] new solution for AI Agent communication via the 6G networks(s) |
| SA2#173 | 3 | S2-2600078 | 23.801-01: [KI#19 bullets 1, 2] New solution on digital identity distribution, registration, authentication and session establishment for UE AI Agent |
| SA2#173 | 4 | S2-2600094 | 23.801-01: [KI#19 bullet 1&2] Solution for Key Issue #19: 6G Network for AI-Enable AI agent communication |
| SA2#173 | 5 | S2-2600186 | 23.801-01: [KI#19, bullet#1, bullet#2] New Solution on 6G network support UE agent identification, authorization and discovery |
| SA2#173 | 6 | S2-2600223 | 23.801-01: KI#19-Skill-Based Distributed Al Agents Coordination Framework |
| SA2#173 | 7 | S2-2600245 | 23.801-01: [KI#19, bullet #1, bullet#2] Support for communication between AI Agents on the application layer via 6G CN |
| SA2#173 | 8 | S2-2600356 | 23.801-01: [KI#19, bullet#1&bullet#2] New Solution for AI agent registration and discovery |
| SA2#173 | 9 | S2-2600360 | 23.801-01: [KI#19, bullet#1] AI Agent Discovery via Agent Repository Function (ARF) |
| SA2#173 | 10 | S2-2600381 | 23.801-01: [KI#19, bullet #1] New Solution for AI Agent Discovery via the AI Agent Exposure Function in 6G |
| SA2#173 | 11 | S2-2600424 | 23.801-01: [KI#19, bullet #1 & bullet#2] New solution: Group-based AI agent communication |
| SA2#173 | 12 | S2-2600425 | 23.801-01: [KI#19, bullet #1 & bullet#2] New solution: AI agent addressing and direct one- to-one communication |
| SA2#173 | 13 | S2-2600432 | 23.801-01: [KI#19, bullet#1,2] Solution for AI agent Communication Network |
| SA2#173 | 14 | S2-2600520 | 23.801-01: [KI#19, bullet #1] UE AI Agent Discovery using UE Agent Capability Profiles |
| SA2#173 | 15 | S2-2600530 | 23.801-01: [KI#19, bullet 2] Solution for KI#19: distributed authorization for AI agents' communications |
| SA2#173 | 16 | S2-2600536 | 23.801-01: [KI#19] 6G Network for AI Solution |
| SA2#173 | 17 | S2-2600546 | 23.801-01: [KI#19, bullet #1, bullet#2] UE AI agent Discovery and UE AI agent Session Establishment via 6G CN |
| SA2#173 | 18 | S2-2600561 | 23.801-01: [KI#19] AI agent Collaboration based on AI Agent Registration/Discovery and AI Agent Group Communication |
| SA2#173 | 19 | S2-2600369 | 23.801-01: [KI#18, KI#19] New Solution - Discovery of Al agents via Al agent Repository Function |
| SA2#173 | 20 | S2-2600573 | 23.801-01: [KI#18/KI#19] Native AI and agentic AI support for 6G |
| SA2#173 | 21 | S2-2600629 | 23.801-01: [KI#19] Solution for NAS-based LLM Service Commanding in 6G Core Network |

Annex X.19.2: Void

Annex X.19.3: KI#19 bullet 3

Input papers at SA2#173 for KI#19 bullet 3:

| Meeting | Solution# | Tdoc | Subject/Comment |
| --- | --- | --- | --- |
| SA2#173 | 1 | S2-2600224 | [KI#19, bullet 3] Supporting intent-based exposure from AI agent in an AF |
| SA2#173 | 2 | S2-2600452 | [KI#19, bullet#3] New solution to enhance network capability exposure functionalities to AI agent on AFs |
| SA2#173 | 3 | S2-2600573 | 23.801-01: [KI#18/KI#19] Native AI and agentic AI support for 6G. |
| SA2#173 | 4 | S2-2600244 | [KI#18, bullet#1a and bullet#1b] How to fulfil requests from the UE or the AF that includes intents. |

Annex X.19.4: KI#19 bullet 4

Input papers at SA2#173 for KI#19 bullet 4:

| Meeting |  | Solution# | Tdoc | Subject/Comment |
| --- | --- | --- | --- | --- |
| SA2#173 | 1 |  | S2-2600063 | [KI#19, bullet4] new solution for 6G CN provides AI services to applications(AF or in UE) |
| SA2#173 | 2 |  | S2-2600079 | [KI#19 bullet 4] New solution on UE access AI services |
| SA2#173 | 3 |  | S2-2600096 | [KI#9 bullet 4] Solution for KI#19: AI model training and inference performed by 6GC |
| SA2#173 | 4 |  | S2-2600158 | [KI#19, bullet#4] 6G AI service to UE |
| SA2#173 | 5 |  | S2-2600171 | [KI#19, bullet #4] New Solution for 6G CN Providing AI Inferencing Services |
| SA2#173 | 6 |  | S2-2600191 | [KI#19, bullet #9] new solution: Support AI Training Service in 6G core network |
| SA2#173 | 7 |  | S2-2600414 | [KI#19, bullet#4] 6G Network for AI services |
| SA2#173 | 8 |  | S2-2600454 | [KI#19, bullet#4] New solution to enable 6G CN to support AI services to applications (UE or in AF) |
| SA2#173 | 9 |  | S2-2600540 | [KI#19] Enabling UE application-based AI agent interactions with the network. |

Annex X.19.5: KI#19 bullet 5

Input papers at SA2#173 for KI#19 bullet 5:

| Meeting | Solution# | Tdoc | Subject/Comment |
| --- | --- | --- | --- |
| SA2#173 | 1 | S2-2600095 | [KI#19, bullet #5] Solution for KI#19 on Network Enhancement based on the characteristics of AI traffic |
| SA2#173 | 2 | S2-2600173 | [KI#19, bullet#5] New Sol#X: “Traffic Detection based on AI-related traffic type” |
| SA2#173 | 3 | S2-2600393 | [KI#19, bullet#7] AI traffic characteristics for Generative AI |
| SA2#173 | 4 | S2-2600394 | [KI#19, bullet#7] Token related AI traffic |
| SA2#173 | 5 | S2-2600403 | [KI#19] Discussion and proposal for traffic of AI service and AI codec |
| SA2#173 | 6 | S2-2600467 | [KI#19, bullet#5] [KI#19, bullet#5] AI/ML traffic characteristics |
| SA2#173 | 7 | S2-2600537 | [KI#19, bullet#5] Solution to Support GenAI Traffic |
| SA2#173 | 8 | S2-2600540 | 23.801-01: [KI#19] Enabling UE application-based AI agent interactions with the network. |

* * * End of Changes * * * *