3GPP TSG-WG SA2#173	S2-2601385 (was 1164)

# Goa, India, 9 – 13 February, 2026

Source:	Penholders (vivo, Nokia, Samsung)

Title:	[KI#21] Penholders PCR for 6G data framework in SA2

Document for:	Approval

Agenda item:	20.6.21

Work Item:	FS_6G_ARC / R20

Abstract of the contribution: this PCR covers solutions for 6G data framework in SA2.

1.	Introduction

This solution addresses KI#21.

Suggestion for work flow in the SA2#173 meeting：

As the work is just starting, and to keep discussion at this meeting focused, not all aspects of all input Tdocs has been included. Further details not taken into account by this already too long Tdoc will of course be considered during newt SA2 meetings.

This Tdoc should be discussed step by step (solution by solution)

  - In the first round of discussion discuss only the clause solution 0  (e.g., vocabulary, use-case, requirements, architecture, high level open questions)

  - In the second round of discussion each solution will be discussed individually in a step by step (i.e. solution by solutions) manner, keeping in mind that only essential aspects of the solutions are captured and need to be discussed

If time is too limited we could for example

    - discuss first solutions tagged as “architecture” (A to E)

    - Then discuss other solutions group by group where a group focuses on solutions target the same functionality

2.	Text proposal

It is proposed to agree the following changes vs. TS 23.801-01:

* * * First Change * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | #21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.0 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.A | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.B | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.C | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.D | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.E | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.F | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.G | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.H | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.I | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.J | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.K | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.L | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.M | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.N | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.O | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.P | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.Q | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.R | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.S | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * Next Change * * * * topic common aspects

6.21.0	Solution #21.0: Common aspects (Laurent+Vivian+Hyesung)

6.21.0.0	Topics addressed and High-level Solution Principles

This pseudo solution addresses KI#21. It

- collects terminology that can be referred to by other solutions (e.g. potentially common definitions, common names for functionality or NF etc..)

- describes at high level the points of agreements and the points where agreements need to be reached.

6.21.0.0.1		Definitions  (Laurent)

NOTE: 	defining a term in this clause does not mean that it is agreed that the corresponding feature or functionality is in scope of 6G specifications; it is possible that conclusions will mention that the functionality corresponding to some of the terms defined in this clause is not subject to specifications

Data collection: the process of requesting a 6GS entity to provide some data (or events).

Data transfer: the process of sending data (or events) from an entity that has these data (or that has detected the event).

6.21.0.4		high level questions  (Laurent)

Editor’s note: This sub-clause lists the VERY HIGH level questions to be addressed to reach conclusions and can be considered as a BIG Editor’s NOTE; there are of course other extra points requiring an Editor’s Note

- Define the use cases at both functional level (does data framework apply to AIML inference, sensing, etc…) but also at domain level (for the different functional use cases, does data framework apply to collecting from UE domain, RAN domain, OAM domain,…)

  - For example, can 6G CN collect data from RAN, or does it collect RAN data via the OAM domain

- Define technically the involved functionalities but also which data framework functionality is to apply for which use case.

- Define the involved network functions (where functionalities could be defined as standalone NFs , collocated with a NF not part of data framework or merges in a data framework NF)

- How to transfer data (especially when data is transferred from UE and/or RAN)? using Control Plane, User Plane or a new “data plane” (which would need to be defined)

  - (for collection from UE)whether a dedicated Data Session distinct from the PDU session is established for data transfer purposes is FFS.

- How 6G data framework uses or does not use 5G data framework as a starting point

- Whether to introduce Data Agent in 6G for data framework

- Whether there may be common functionalities between SA5 and SA2 (e.g. DRF, DPF, DEF)

- For data collection from UE, which entity sends the data collection request, and which entity selects the UEs (when target UEs need to be selected)

- Is Data source entity information/capability registration a functionality needed, If yes, which NF supports it, which entities (among UE, RAN, and NF) can register.

- Should the data framework follow the Publish/Subscribe paradigm

- Should the data framework support new data transfer modes beyond HTTP (streaming, file transfer)

- The details for user consent management are FFS and subject to coordination with SA3

- Whether the data framework needs to define metadata distinct from the data themselves, what these metadata would contain (e.g. data producer info, data collection time) and how metadata would be used e.g. whether metadata would be used when data producer would register on a data registry; whether we need to define a metadata handler

6.21.0.0.2	Use cases ( Laurent)

Potential  use cases:

Editor’s Note: some further definition of each use case is needed.

- AIML Model training with data collection for 6G CN NF from a 6G CN NF

There is no strong requirement on the delay to transfer data for this AIML Model training use case

- data collection for AIML inference

There is requirement on the delay to transfer data for this AIML inference use case

- data collection for ground truth retrieval (model quality monitoring) for 6G CN NF from a 6G CN NF

- AIML Model transfer between 6G CN NFs

Editor’s Note: Whether “AIML Model transfer between 6G CN NFs” is a use case for data framework depends on KI#18.

- AIML Model training with data collection for UE sided model training (radio related)

Editor’s Note: Whether “AIML Model training with data collection for UE sided model training (radio related)” is a use case depends on RAN requirements

- AIML Model training with data collection by a   NF (for transfer) from a non 6G CN entity (e.g. (R)AN, UE) (e.g. for UE-sided radio related model training) (justification?)

Editor’s Note: Whether “)” is a use case

- Sensing: data collection by the 6G CN NF responsible of sensing from RAN when using RAN as a Sensing Entity,

- Sensing: data collection by the 6G CN NF responsible of sensing from RAN when using UE as a Sensing Entity,

- Sensing: data collection by the 6G CN NF responsible of sensing from UE when using UE as a Sensing Entity

Editor’s Note: Whether “sensing” related use cases are applicable for KI#21 is FFS and needs to be coordinated with KI#20

- data collection by the 6G CN NF responsible of Positioning from RAN

- data collection by the 6G CN NF responsible of Positioning from a UE

- Data collection for Exposure of operator data assets to 3rd party or UE:

The operator may decide to expose some of its data to third parties or UE. Different sets of data assets may be exposed to different sets of third-party entities or UE, hence the operators need granular control over data exposure/ This use case requires tight operator control and support of user consent and privacy

- Network Digital Twin : providing UE /PDU Session related data from 6G CN NF to the OAM entities supporting a Digital Twin

Editor’s Note: Whether this “Network Digital Twin” related use cases are applicable for KI#21 and needs to be coordinated with SA5

  - ?

  - N

6.21.0.0.3	Architectural aspect including list of topics (Vivian)

NOTE 1: 	Defining a term xxxF or NF xxxNF in this clause does not mean that it is agreed that the corresponding feature or functionality is in scope of 6G specifications; it is possible that conclusions will mention that a functionality corresponding to some of the terms defined in this clause is not subject to specifications. Furthermore, final conclusions can determine that some of these functionalities are always co-located.

NOTE 2: 	Several options exist for each NF’s or functionality name, , the final name for NF or functionality  will be determined at a later stage.

NOTE 3: 	it is FFS whether NFs and functionalities listed in this clause will be specified as evolutions of similar NFs and functionalities defined in 5GC.

Functionalities potentially needed

(they are further defined below but their need is FFS)

-   	Data Control Functionality

-	Data Transfer Functionality

-	Data Repository Functionality

-	Data processing Functionality

-	Data Exposure Functionality

-	Data (capability) registration Functionality

-	Data Agent Functionality

Editor’s Note: Which functionality is appliable for which use case or all use cases is FFS.

Editor’s Note: 	Whether any of the functionalities above can be common between SA2 and SA5 data frameworks is FFS

- Data Control Functionality is to control data handling. It performs:

- handling a data request from data consumer

- authentication and authorization for data request from data consumers

- data source selection and data discovery

- configuration of data source for data collection and transfer related parameters

- enforcing of user consent checking: If target data is associated with UE, the Data Control Functionality enforces user consent checking for data collection from data consumer

- control of data distribution/providing to data consumers

- control of data storage and retrieval

- control of data processing

- A Data Transfer Functionality is to support:

- data receiving from data sources

- data distribution/ to data consumers based on the control by the Data Control Functionality

- optionally data processing, e.g.  anonymization, desensitization, aggregation, labelling for data, etc.

Editor’s Note: Whether the data transfer is over CP, UP or new plane is FFS.

- A Data Processing Functionality is defined to provide data processing such as:

- data aggregation (e.g.  multi-source data fusion)

- data anonymization

- data pseudonymization

- data labelling

- formatting before delivering data to data consumer,

- data analysis

- data cleaning

- dataset creation (specifying size, samples, format)

- data alignment

- enforcing quality

- metadata handling

NOTE X: The details for data anonymization and data pseudonymization mechanism need coordination with SA3.

- A Data Repository Functionality is used for data storage and retrieval, it may perform:

- storing the collected data under the control of the Data Control Functionality

- retrieving the data under the control of the Data Control Functionality

-  	 optionally generating metadata of the stored data, or labelling different data types (e.g., AI data, sensing data, structured data, unstructured data)

-	Optionally DRF could do some

- A Data Exposure Functionality supports data exposure to the third-party AF with consideration of service authorization, privacy protection.

- Access control (e.g. authentication and authorization) for data request from the third party AF

NOTE x: Access control (e.g. authentication and authorization) for data request from UE is performed by the Data Control Functionality.

- A Data Capability Registration Functionality supports:

- registration of data production capability (e.g. supported data type, data format) of data source

- discovering data source based on registered data production capability

- A Data Agent Functionality may support:

- Based on agentic technology, parsing data related requests (e.g., intent-based request, data with quality requirement, customized data) from UE, AF and other network entities

- Sending data request to the Data Control Functionality as a data consumer

NOTE x: whether Data Agent Functionality is included and how does it work on parsing data related requests depend on KI#18.

Common Principles for data collection/transfer

- The data requester (e.g. Data Control Functionality or  possibly other functionalities / NF) in 6G CN sends a data collection/transfer request to the data source(s), the data collection/transfer request contains the configuration about which data to collect and how to transfer the collected data (e.g. addressing information of where to transfer the data). The data requester may also stop data collection/transfer procedure.

- The data collection/transfer request may also include some data collection termination criteria.

- The data source transfers the collected data according to the configuration negotiated with the data requester.

Editor’s Note: how to transfer the collected data to 6G CN, e.g. via CP, UP, new data plane or tunnel is FFS which may be coordinated with RAN WGs when RAN is involved.

- A data source(e.g. 6G UE, 6G RAN, and 6G NF) can reject or cancel the data collection or data transfer anytime with consideration of its related status (e.g., for UE: battery status, for RAN or 6G CN NF: load status, this may also be based on a change of subscriber permission status)

NOTE: Any impacts of the data framework to RAN will be subject to co-ordination with TSG RAN WGs

Principles for UE data collection/transfer

- Dedicated path for UE data transfer

- A dedicated path is established for data transfer between UE and the 6G NF collecting data from the UE (containing the Data TansferFunctionality).

Editor’s note: The transport mechanism to be used for the dedicated path (e.g. CP, UP or new Data Plane) is FFS. Co-ordination with K1.2 may be needed.

- The UE receives in the data collection request the address of the 6G NF (containing the Data Tansfer Functionality) collecting data from UE.

3.	Data transfer procedure:

-	A request for collection/transfer with requested data information is sent to the target UE(s).

-	The UE reports the requested data via the established dedicated path to the 6G NF (containing the Data Transfer Functionality) collecting data from UE.

4.	Traffic Differentiation:

Editor’s note: how to differentiate the traffic of the collected data for 6G data framework from the UE regular traffic, e,g, based a new type of data session distinct from PDU session , or just based on DNN, S-NSSAI is FFS.

Items requiring further work

Editor’s note: Comparing with existing control plane and user plane, whether a new data plane needs to be introduced in 6G for data framework is FFS. Editor’s note: What is the precise definition of the data plane concept is FFS.  Does it refer to a new generalized plane distinct from the CP or UP, or does it just denote a data transmission channel?

Editor’s note: Whether and how to define QoS model for data plane is FFS.

Editor’s note: Whether new common interface (e.g. DBI) should be introduced for data framework is FFS. How SBI would differ from DBI (data bus interface) is also FFS

Editor’s note: (for data collection/transfer from UE) whether a dedicated Data Session distinct from the PDU session is established for data transfer purposes is FFS.

Editor’s note: Whether introduce Data Agent in 6G for data framework is FFS.

6.21.0.0.4	Requirements (Hyesung)

Potential requirements:

- The data framework supports full lifecycle management of data, including collection, transfer, processing, storage, and exposure, while taking into account user consent and privacy.

- A common set of interfaces and APIs can be used for the data framework for a range of use cases.

- The 6G data framework shall avoid control plane overload.

- (Operator controllability) MNO has full control of the data collection, data transfer process and can manage data access and exposure. This includes initiating, terminating and fully managing data transfer.

- (Operator controllability) The 6G data framework enable MNO controlled granular data access for exposure and security, including the scope of access to the data framework for 3rd party entities.

Editor’s Notes: It is FFS whether to support controllability on the scope of access to the data framework for network internal entities

- Operator visibility: MNO has full visibility for standardized data e.g. for standardized data and when applicable, the 6G CN verifies/matches the requested data to be transferred and the data that is being reported.

- The data transfer for data collection in 6G data framework shall be differentiated from the user traffic (also no charging to the subscriber for the data collection).

- The 6G data framework shall allow flexible deployment (e.g., deployed as a single instance or multiple instance)

- Data source entity (or data provider entity) shall be able to reject or cancel the data collection or data transfer.

- User consent (or subscriber permission) shall be respected.

Editor’s note: The details for user consent management are FFS and subject to coordination with SA3.

- The 6G data framework shall provide privacy protection mechanism (e.g., anonymization and pseudonymization).

Editor’s note: The details for privacy protection mechanism are FFS and subject to coordination with SA3.

- The 6G data framework shall support data transfer mechanisms (potentially event-based, batch access/file-based and streaming-based data transfer ) depending on data characteristic (e.g., real-time vs. non-real time, data volume)

- The 6G data framework shall support bulky or highly frequent data transfer mechanisms.

- Principles for quality of Data (QoD): Data framework supports data quality aspects as per the requirement from Data Consumers

.Editor’s note: What are the criteria of data quality (e.g., confidence interval, data freshness information, data accuracy)and how to enforce the quality of data is FFS. It is FFS whether Quality of data can include  KPI for each phase, e.g, data collection (e.g., fulfilling the requirements for Data collection latency), data processing (e.g., the requirements for data Completeness and/or Data processing latency) and data transmission (e.g., guaranteeing latency considering Data transmission).

- The 6G data framework shall consider whether to support roaming scenario based on SLA and regulation.

- The 6G data framework shall support to manage historical data.

- The 6G data framework should be futureproof.

- The data framework supports data storage based on operator policies, for short and long term.

- The data framework should avoid duplicate collection, storage and processing. If multiple data consumers request the same data from the same data provider or the same processing functionality, the data framework should aggregate these requests and prevent duplicate subscription.

- Data source entity (or data provider entity) shall be able to reject or cancel the data collection or data transfer and may stop data transferring based on internal determination (i.e., based on implementation).

- The 6G data framework should avoid impact on user plane and minimize impact on control plane.

- Operator’s partial visibility: MNO has partial visibility for vendor-specific data.

- The 6G system has a single framework to support collection and transfer for both data and events.

- The followings are out of scope of 6G data framework: Internal state of NFs such as context data (e.g., AMF context, SMF context), UDM/UDR data (e.g., subscription data), LI data,.

- A common solution is applicable to support data collection from UE, 6G RAN, 6G CN and OAM, and data consuming by UE, 6G RAN, 6G CN and the third AF based on operator policy.

Editor’s note: Whether data can be collected via data framework from OAM is FFS.

6.21.0.1	Procedures

Editor’s Note: For further Study

6.21.0.2	Services, Entities and Interfaces

Editor’s Note: For further Study

* * * End of Change * * *

To be discussed in next meeting

* * * Next Change * * * * topic data framework architecture (Vivian)

Variant A: data framework without introducing new plane

Variant B: data framework with introducing new plane or new interface

Variant E: Architecture with data islands

Variant F: Architecture reusing 5GC as baseline

6.21.A	Solution #21.A: Architecture Variant: data framework without introducing new plane

6.21.A.0	Topics addressed and High-level Solution Principles

This solution addresses KI#21

Editor’s Note: the terminology used in this solution should be aligned with the terminology in solution “0”

- Functionality and interface

- one or moreNF (i.e. includes Data Control Functionality) control data handling for data lifecycle. E.g data discovery, collection, transmission, processing, storage, and exposure.

- This NF(s) also includes Data Transfer Functionality to support as data anchor, to receive data from data source(s) and to distributing data to data consumer(s). This NF may also support Data Processing Functionality if needed.

- A NF containing Data Repository Functionality;

- [optional] A NF containing Data Processing Functionality;

- [optional] A NF containing Data Exposure Functionality.

This architecture does not introduce a new plane for data transfer but leverages Control plane within the network and User plane (PDU session) for the interface with the UE

6.21.A.1	Description

Editor’s Note: For further Study

6.21.A.2	Procedures

Editor’s Note: For further Study

6.21.A.3	Services, Entities and Interfaces

Editor’s Note: For further Study

* * * Next Change * * * *

6.21.B	Solution #21.B: Architecture variant: data framework with introducing new plane or new interface

6.21.B.0	Topics addressed and High-level Solution Principles

This solution addresses KI#21.

Editor’s Note: the terminology used in this solution should be aligned with the terminology in solution “0”

- Functionality and interface

- A centralized  NF (i.e. includes Data Control Functionality) is defined in 6G CN to control data handling for data lifecycle. E.g data discovery, collection, transmission, processing, storage, and exposure. Control signaling for related data handling is based on e.g., :

- SBI for control plane;

- or new DBI for new plane (e.g. Data plane)

- There may be also dedicated NF as the data anchor in CN (i.e. includes Data Transfer Functionality ), to receive data from data source(s) and to distributing data to data consumer(s). The transferring of data is based on new interface e.g. DBI (e.g. called Data Based Interface, or Data Bus Interface) for new plane (e.g. Data plane or service plane).  This NF may also contain Data Processing Functionality as defined in clause 6.21.0.0.3.

- A NF containing Data Repository Functionality;

- [optional] A NF containing Data Processing Functionality;

- [optional] A NF containing Data Agent Functionality;

- [optional] A NF containing Data Exposure Functionality.

6.21.B.1	Description

Editor’s Note: For further Study

6.21.B.2	Procedures

Editor’s Note: For further Study

6.21.B.3	Services, Entities and Interfaces

Editor’s Note: For further Study

* * * Next Change * * * *

6.21.D	Solution #21.D: Architecture variant with data islands

6.21.D.0	Topics addressed and High-level Solution Principles

This solution addresses KI#21.

A Data framework can be instantiated separately in so-called data islands. Each data island is responsible for a set of data objects, based on operator configuration. The data islands may be deployed separately and may be realized by different vendors. The data islands all use the same, standardized interfaces or APIs, but they may be optimized in their implementation or deployment for specific characteristics. The operator should be able to control which entities have access to a given data island.

The figure below illustrates in an example that the data framework’s infrastructure is instantiated into Data island A, which may e.g., hold AI/ML data and is volume optimized, and into Data island B, which may contain e.g., analytics results and is optimized for high reliability. The NFs use the same interfaces or APIs to connect to either Data island A or Data island B, or both.

Figure 6.21.D.0-1: Example deployment for architecture variant with multiple data islands

As another example, it may be possible for the operator to deploy e.g., a data island in the CN domain and another data island in the OAM domain. Both the data island in the CN and the data island in the OAM use the same 6G data framework technology and are based on the same interfaces and APIs, but the deployments may be optimized for the specific needs and characteristics of each domain. The figure below shows an example use case where a Network Digital Twin can use the common set of interfaces and APIs of the data framework to access data both in the OAM and CN domains.

Figure 6.21.D.0-2: Example deployment for architecture variant with separate data islands in CN and OAM domains

The use of multiple data islands is optional at the discretion of the operator: in some networks, the operator may decide to deploy multiple data islands, while in other networks, the operator may choose to deploy a single data island only. In the example below, a single data island is used for all data in the operator’s network. The same data island can be used in multiple domains, e.g. in CN and in OAM.

Figure 6.21.D.0-3: Example deployment with single data island

The operator has control over which entities have access to a given set of data objects within the data island. Subject to operator policies, 3rd party access to a data island is possible.

The solution focuses on the external interfaces and APIs of the data island, while allowing the internals of the data islands to be optimized for the specific requirements and characteristics of the given deployment.

6.21.D.1	Description

Editor’s Note: For further Study

6.21.D.2	Procedures

Editor’s Note: For further Study

6.21.D.3	Services, Entities and Interfaces

Editor’s Note: For further Study

* * * Next Change * * * *

6.21.E	Solution #21.E: Architecture variant reusing 5GC as baseline

6.21.E.0	Topics addressed and High-level Solution Principles

This solution addresses KI#21.

- This architecture focuses on data collection from CN, OAM and AF to support feature of AI/ML in 6G CN + UE data collection for UE-sided model training (no support of data collection from UE for AIML operation in 6G CN)

- A Model Training functionality and / or Inference functionality may collect data from data sources directly, or collect data via the DCCF 	acting as DCF defined in solution 0

- There is no direct data collection / reporting between RAN and a data consumer for AIML in CN. The data consumer can subscribe to OAM to get MDT / SON related data,

- The architecture aims at reusing the DCCF defined in TS 23.288

- The ADRF is reused for AIML in CN for 6G as well, to store the historic information and register the stored

Figure XX: Data framework for AIML in 6G CN

Figure XX:UE data collection for UE-sided model training architecture

Editor’s Note: the terminology used in this solution should be aligned with the terminology in solution “0”

6.21.E.1	Description

Editor’s Note: For further Study

6.21.E.2	Procedures

Editor’s Note: For further Study

6.21.E.3	Services, Entities and Interfaces

Editor’s Note: For further Study

* * * Next Change * * * * (Topic for Data registration and discovery) ( Hyesung)

Subtopic  Data source entity information/capability registration

6.21.F	Solution #21.F: Topic: Data registration/discovery (Data source entity information/capability registration to a dedicated NF)

6.21.F.0	Topics addressed and High-level Solution Principles

This solution addresses KI#21, the topic of Data Registration / discovery.

This variant of topic for data registration/discovery is based on the following principles:

-	A dedicated data management NF provides data source entity registration service.

-	Data information (e.g., supported data type, meta data) and data capabilities of data source entity are registered to the data management NF.

As a variant the Data Producer/source may register the metadata of the data (and/or the actual data) to a Data capability Registry (DCRF); The data service consumer may be configured to query the DCRF using metadata to discover the requested data and/or the Data Producer.

In this variant the metadata of the data may include following information:

- data type

- data usage purpose

- data size

- data producer info

- data collection time

- data collection location

- data quality

- data sample number

Editor’s Note: Whether the data framework needs to define metadata distinct from the data themselves, what these metadata would contain (e.g. data producer info, data collection time) and how metadata would be used e.g. whether metadata would be used when data producer would register on a data registry

Discussion point: which entities (among UE, RAN, and NF) can register to the data management function supporting data registration.

6.21.F.1	Description

Editor’s Note: For further Study

6.21.F.2	Procedures

Editor’s Note: For further Study

6.21.F.3	Services, Entities and Interfaces

Editor’s Note: For further Study

* * * Next Change * * * *(Topic Common Data transfer aspects) ( Laurent)

6.21.G	Solution #21.G: Topic Common Data collection / transfer aspects

6.21.G.0	Topics addressed and High-level Solution Principles

This solution addresses KI#21. Its main aspects (data collection and data transfer are defined in solution 0) :

- There is a single framework to support data collection for both data and events.

- There is a single framework to support data transfer for both data and events.

- Data transfer may use one of the following delivery/transfer modes:

-	Notification (as defined for SBA based Event-based reporting),

-	Streaming (like what is defined in TS 28.532, TS 28.104), e.g.  for data collection for continuous, Real Time

-	File-based (like what is defined in TS 28.532, TS 28.104) e.g. for bulk non-Real Time data transfer

- The data / event information format is independent of the data transfer mode

- The data collection and transfer may involve a dedicated (to data framework) NF (from data framework) that supports both DCF and DTF functionality or another NF not part of the data framework such as the SENF (sensing).

- The data requester (e.g. DCF or  possibly other functionalities / NF) in 6G CN sends a data collection/transfer request to the data provider (e.g. data source(s)), the data collection/transfer request contains the configuration about which data to collect and how to transfer the collected data (e.g. addressing information of where to transfer the data). The data requester may also stop data collection/transfer procedure.

- The data collection/transfer request may also include some data collection termination criteria.

- The data provider transfers the collected data according to the configuration negotiated with the data requester.

- A data provider (e.g. 6G UE, 6G RAN, and 6G NF) can reject or cancel the data collection or data transfer anytime with consideration of its related status (e.g., for UE: battery status, for RAN or 6G CN NF: load status, this may also be based on a change of subscriber permission status)

NOTE 1: 	The content of this solution can apply to data collection from 6G CN, UE and RAN (where each of 6G CN, RAN and UE is a potential data provider)

NOTE 2: 	Data collection is specific to the application e.g. data retrieval for AIML, sensing etc.. Data transfer can be common for multiple applications, and multiple domains e.g. the same set of data transfer modes can apply within the Core and also within the OAM (SA5) domain

NOTE 3: 	The 3 mechanisms above refer to 3GPP SA5 mechanisms defined for 5GS. In 6G, protocols used for data transfer should be common between SA2 and SA5. These protocols are not in the scope of SA2 but SA2 should be able to set requirements for these 3 data transfer/delivery modes.

NOTE 4: 	the data collection control signalling is NOT meant to be handled by three data transfer modes above (notification, streaming, and file).

Editor’s Note: it is For further Study whether the Data transfer capabilities above need to be identified as a new Data bus (interface) (DBI)

6.21.G.1	Description

Editor’s Note: For further Study

6.21.G.2	Procedures

Editor’s Note: For further Study

6.21.G.3	Services, Entities and Interfaces

Editor’s Note: For further Study

* * * Next Change * * * * topic UE data collection/transfer (Vivian)

Subtopic 1 – how to design the mechanism for the data collection/transfer control signaling.

- Variant H: UE data collection not involve RAN (CN does UE selection)

- Variant I: UE data collection involve RAN (RAN does UE selection)

-Subtopic 2 – which path for UE data transfer.
- Variant J:  UE data collection and transfer variant using PDU Sessions

Variant K: UE data collection and transfer variant using Data Session

6.21.H	Solution #21.H: topic UE data collection and transfer variant : not involving RAN

6.21.H.0	Topics addressed and High-level Solution Principles

This solution addresses KI#21, the topic of UE data collection and transfer.

This variant proposes procedures when data collection does not require RAN involvement (RAN only forwards NAS messages), in this case

-	the data collection request message sent to the UE is a NAS message created by a 6G CN NF (e.g. DCF or SeNF for sensing) and transparently relayed by the Access network (e.g. RAN) to the UE

-	It is the 6G CN that selects the target UEs

Editor’s Note: it is For further Study whether for the solution a tunnel needs to be established between the PSA UPF and DTF.

Editor’s Note: it is For further Study whether 6G CN NF (e.g. DCF or SeNF for sensing) that sends the NAS request needs support from AMF

6.21.H.1	Description

Editor’s Note: For further Study

6.21.H.2	Procedures

Editor’s Note: For further Study

6.21.H.3	Services, Entities and Interfaces

Editor’s Note: For further Study

* * * Next Change * * * *

6.21.I	Solution #21.I: topic UE data collection and transfer variant:  with RAN involvement

6.21.I.0	Topics addressed and High-level Solution Principles

This solution addresses KI#21, the topic of UE data collection and transfer.

This variant proposes procedures when data collection requires RAN involvement, in this case:

-	The data collection request message sent to the UE is a RRC message created by the RAN but that may contain information like data transfer information (addressing, protocol and security information to transfer the data) determined by the 6G CN (when a 6G CN NF like the DCF or SeNF (sensing) is the actual recipient of the data transfer)

-	It is the RAN that selects the target UEs

Editor’s Note: FFS: The 6G RAN can be configured with the user consent related information for target UE selection.

Editor’s Note: it is For further Study whether for the solution a tunnel needs to be established between the PSA UPF and DTF.

6.21.I.1	Description

Editor’s Note: For further Study

6.21.I.2	Procedures

Editor’s Note: For further Study

6.21.I.3	Services, Entities and Interfaces

Editor’s Note: For further Study

* * * Next Change * * * *

6.21.J	Solution #21.J: topic UE data collection and transfer variant: using PDU Sessions

6.21.J.0	Topics addressed and High-level Solution Principles

This solution addresses KI#21, the topic of UE data collection and transfer.

This variant proposes procedures when data transfer from UE is via UP session (i.e. PDU session)

- Using URSP to determine which kind of PDU Session to use

- Establishing a PDU Session dedicated to data transfer and establishing a data transfer session.

Editor’s Note: it is For further Study whether for the solution a tunnel needs to be established between the PSA UPF and DTF.

6.21.J.1	Description

Editor’s Note: For further Study

6.21.J.2	Procedures

Editor’s Note: For further Study

6.21.J.3	Services, Entities and Interfaces

Editor’s Note: For further Study

* * * Next Change * * * *

6.21.K	Solution #21.K: topic UE data collection and transfer variant : using Data Session

6.21.K.0	Topics addressed and High-level Solution Principles

This solution addresses KI#21, the topic of UE data collection and transfer.

This variant proposes procedures when data transfer from UE is via a new type of data session

- A dedicated Data Session is established for UE data transfer purposes via Data Plane.

6.21.K.1	Description

Editor’s Note: For further Study

6.21.K.2	Procedures

Editor’s Note: For further Study

6.21.K.3	Services, Entities and Interfaces

Editor’s Note: For further Study

* * * Next Change * * * * (Topic: RAN data collection) Hyesung

Variant L: Dedicated connection/session between RAN and 6G CN

Variant M: Integrated collection of RAN data and RAN OAM data via RAN-CN interaction

Variant N: Integrated collection of RAN data and RAN OAM data via CN-OAM interaction

6.21.L	Solution #21.L: Topic: RAN data collection, variant: Dedicated connection/plane between RAN and CN

6.21.L.0	Topics addressed and High-level Solution Principles

This solution addresses KI#21, the topic of RAN data collection.

This variant of topic “dedicated connection/session” between RAN and 6G CN for RAN data collection is based on the following principles:

- For data collection (configuration of RAN acting as data source): Data Control signaling is exchanged between RAN and data collection NF (or via proxy or broker).

- For data transfer: The 6G system support the direct data connection and interface over new plane between RAN and the data collection NF.

Editor’s Note: the following are FFS: Whether RAN SBI is needed or not. What this new plane consists of

6.21.L.1	Description

Editor’s Note: For further Study

6.21.L.2	Procedures

Editor’s Note: For further Study

6.21.L.3	Services, Entities and Interfaces

Editor’s Note: For further Study

6.21.M	Solution #21.M: Topic: RAN data collection, variant : Integrated collection of RAN data and RAN OAM data via RAN-CN interaction

6.21.M.0	Topics addressed and High-level Solution Principles

This solution addresses KI#21, the topic of RAN data collection.

This variant of topic for RAN/RAN OAM data integrated collection is based on the following principles:

- For data collection (configuration of RAN acting as data source):

-	Configuration for RAN and RAN OAM data collection. RAN also collects the RAN OAM data and transfer RAN OAM data to the 6G CN

-	data collection subscription over NGAP

6.21.M.1	Description

Editor’s Note: For further Study

6.21.M.2	Procedures

Editor’s Note: For further Study

6.21.M.3	Services, Entities and Interfaces

Editor’s Note: For further Study

* * * Next Change * * * *

6.21.N	Solution #21.N: Topic: RAN data collection, variant : Integrated collection of RAN data and RAN OAM data via CN-OAM interaction

6.21.N.0	Topics addressed and High-level Solution Principles

This solution addresses KI#21, the topic of RAN data collection.

This variant of topic for RAN/RAN OAM data integrated collection is based on the following principles:

- For data collection (configuration of RAN acting as data source):

  - Configuration for RAN and RAN OAM data collection. RAN data is only collected via RAN OAM; 6G CN collects RAN data by acting as a 6G management data consumer.

6.21.N.1	Description

Editor’s Note: For further Study

6.21.N.2	Procedures

Editor’s Note: For further Study

6.21.N.3	Services, Entities and Interfaces

Editor’s Note: For further Study

* * * Next Change * * * * Hyesung

6.21.O: Solution #21.O Topic: RAN/CN data collection leveraging the Publish/Subscribe model

6.21.O.0	Topics addressed and High-level Solution Principles

This solution addresses KI#21, the topic of data collection and transfer.

This solution leverages the Publish/Subscribe model.

- data producers may be NFs and/or RAN nodes, while data consumers are NFs. The data broker is a CN NF that supports a publish/subscribe mechanism (e.g., DCCF),

- 2 alternatives

- data producer directly registers available data (stored data or collectable data) with the data broker.

- The data broker obtains the data producer information from a data discovery function

- The data producer(s) transfer the data to the data broker.

- The data broker distributes the received data to the data consumer(s).

6.21.O.1	Description

Editor’s Note: For further Study

6.21.O.2	Procedures

Editor’s Note: For further Study

6.21.O.3	Services, Entities and Interfaces

Editor’s Note: For further Study

* * * Next Change * * * * (Topic: Data storage) Hyesung

6.21.P	Solution #21.P: Topic: Data storage

6.21.P.0	Topics addressed and High-level Solution Principles

This solution addresses KI#21, the topic of data storage

This variant of topic for data registration/discovery is based on the following principles:

- 6G data framework has a dedicated data storage function

- Store the collected data to avoid duplicated collection

- Support efficient storage of data for different data types (e.g., sensing related data, AI/ML data), based on configuration of DCF.

- Performs unified and efficient storage, retrieval and access control according to policies configured by the Data Control Function

Discussion point:

- The scope of data to be stored in the data storage function,

- Which entity can access the storage function.

6.21.P.1	Description

Editor’s Note: For further Study

6.21.P.2	Procedures

Editor’s Note: For further Study

6.21.P.3	Services, Entities and Interfaces

Editor’s Note: For further Study

* * * Next Change * * * * topic data processing (Vivian)

6.21.Q	Solution #21.Q: Topic Data processing

6.21.Q.0	Topics addressed and High-level Solution Principles

This solution addresses KI#21 specifically for the topic of data processing.

- Potential data processing operations for the 6G data framework include: aggregation (e. g. multi-source data fusion), anonymization, pseudonymization, formatting, labelling, analysis, data cleaning, dataset creation (specifying size, samples, format), data alignment, enforcing quality, metadata handling.

- Data processing for 6G data framework includes the following solution (not mutually exclusive):

- Data source for data pre-processing e,g, anonymization, labelling, generating metadata, adding standardized identifiers, sampling, parameter translation, etc..

- A new 6G CN NF (e.g. DPNF) for dedicatedly data processing, e.g. data aggregation, data anonymization, data pseudonymization, data labelling, formatting, synthetic data generation, etc.

- Data processing may be carried out by the DTF (data Transfer Functionality) for in-path data processing, e.g. aggregation, anonymization, pseudonymization, formatting, labelling, etc..

- Data processing may be carried out by the DRF for the stored data processing, e.g. generating meta data or labelling the data, etc.

Editor’s Note: it is FFS whether each of these solutions is needed.

6.21.Q.1	Description

Editor’s Note: For further Study

6.21.Q.2	Procedures

Editor’s Note: For further Study

6.21.Q.3	Services, Entities and Interfaces

Editor’s Note: For further Study

* * * Next Change * * * * topic data exposure (Vivian)

6.21.R	Solution #21.R: Topic Data exposure

6.21.R.0	Topics addressed and High-level Solution Principles

This solution addresses KI#21 and data exposure topic.

There are following variant solutions for data exposing in 6G data framework.

- 6G NEF should be enhanced to support data exposure to the third party with consideration of service authorization, privacy protection, user consent and security.

- Data agent is used to control data exposure to authorized AF/UE based on intent.

- The DCF functionality for controlling of data handling also controls data exposure to 3rd AF or UE.

- A new NF DENF dedicated for data service exposure is introduced. DENF exposes requested data to 3rd AF or UE.

Editor’s Note: which entity will be chosen to do data exposure is FFS

6.21.R.1	Description

Editor’s Note: For further Study

6.21.R.2	Procedures

Editor’s Note: For further Study

6.21.R.3	Services, Entities and Interfaces

Editor’s Note: For further Study

* * * Next Change * * * * (Topic: User consent and privacy) (Hyesung)

6.21.S	Solution #21.S: Topic user consent/subscriber permission and privacy

6.21.S.0	Topics addressed and High-level Solution Principles

This solution addresses KI#21 and data exposure topic.

This variant proposes solution for user consent/subscriber permission and data privacy aspects.

- An NF in the CN manages user consent/subscriber permission information.
FFS: Whether to provision user consent/subscriber permission information into RAN.

- The user consent/subscriber permission information is provided from the UE and can be changed over conditions (e.g., temporally, spatially).

- The 6G data framework supports the enforcement of user consent/subscriber permission, i.e., authorize the data collection/transfer request for UE-related data.

- In the 6G data framework, an NF performs data privacy protection/preserving mechanisms (including anonymization, pseudonymization, differential privacy protection method such as noise addition) depending on the requested data type/service, data consumer, or request by data consumer.

- The 6G data framework support to store the data which is anonymized or pseudonymized.

6.21.S.1	Description

Editor’s Note: For further Study

6.21.S.2	Procedures

Editor’s Note: For further Study

6.21.S.3	Services, Entities and Interfaces

Editor’s Note: For further Study

* * * Next Changes * * * *

Annex X: Submitted solutions

X.1	List of submitted solutions

Guidance – include in this clause:

- 	a table containing references to the individual solution proposals that lead to the solution variants in this pCR.

-	The “Solution#” column is used to cross-reference between the solution variant description (clause 6.X.Y.0) and the input papers in this Table. Use sequential numbering (1,2,3,…). The rapporteurs will adjust the numbering when implementing the TR.

Table X.4: List of submitted solutions

| Meeting | Solution# | TDoc | Subject/Comment |
| --- | --- | --- | --- |
| SA2#173 |  | S2-2600080 | 23.801-01: [KI#21] New solution on 6G data framework. |
| SA2#173 |  | S2-2600088 | 23.801-01: [KI#21, Bullet #1, #2] 6G data framework Architecture and Procedures. |
| SA2#173 |  | S2-2600090 | 23.801-01: [KI#21] Solution about Data framework for 6G. |
| SA2#173 |  | S2-2600099 | 23.801-01: [KI#21, bullet #1&2] Solution for KI#21 about the introduction of Data Plane for 6G Data Framework Design. |
| SA2#173 |  | S2-2600103 | 23.801-01: [KI#21, bullet#2] New solution for data framework. |
| SA2#173 |  | S2-2600124 | 23.801-01: KI#21bullet#2: Solution#X Architecture for 6G data framework . |
| SA2#173 |  | S2-2600144 | 23.801-01: [KI#21] new solution of 6G Data Framework between 6G UE and 6G CN. |
| SA2#173 |  | S2-2600145 | 23.801-01: [KI#21] new solution of 6G Data Framework between 6G RAN and 6G CN. |
| SA2#173 |  | S2-2600152 | 23.801-01: [KI#21, bullet 2] New Solution for unified framework for data storage and exposure. |
| SA2#173 |  | S2-2600154 | 23.801-01: [KI#21, bullet #1,#2,#3] 6G data framework in SA WG2. |
| SA2#173 |  | S2-2600170 | 23.801-01: [KI#21] New solution on Data framework architecture and procedures. |
| SA2#173 |  | S2-2600172 | 23.801-01: [KI#21, bullet#1&bullet#2] New Sol#X: Generalized and common data framework . |
| SA2#173 |  | S2-2600187 | 23.801-01: [KI#21, bullet#1, bullet#2] New Solution on 6G network support UE data collection for UE-sided model training. |
| SA2#173 |  | S2-2600195 | [KI#21] High Level Principles for 6G data framework |
| SA2#173 |  | S2-2600201 | [KI#21, bullet #2] Functions and Procedure for Data Registration, Discovery and Distribution |
| SA2#173 |  | S2-2600226 | [KI#21, bullet 2] Data Collection Framework leveraging the Publish/Subscribe model |
| SA2#173 |  | S2-2600228 | [KI#21] new solution: Event and data collection and transfer requirements |
| SA2#173 |  | S2-2600229 | [KI#21] new solution: Data transfer from UE using PDU Sessions |
| SA2#173 |  | S2-2600239 | [KI#21, bullet 1] Use case for instantiation of the data framework based on volume, quality, cost or other optimization criteria |
| SA2#173 |  | S2-2600240 | [KI#21, bullet 1] Network Digital Twin use case for the data framework |
| SA2#173 |  | S2-2600241 | [KI#21, bullet 1] Operator controlled granular data access for exposure and security |
| SA2#173 |  | S2-2600242 | [KI#21, bullet 1] Network analytics use case for the data framework |
| SA2#173 |  | S2-2600243 | [KI#21] Principles of the data framework |
| SA2#173 |  | S2-2600263 | New Solution: Adapter-based 6G Data Framework Architecture for Sensing |
| SA2#173 |  | S2-2600288 | [KI#21] Use case (UE data collection/transfer for AI use) and solution proposal |
| SA2#173 |  | S2-2600289 | [KI#21] Use case (RAN data collection/transfer for AI use) and solution proposal |
| SA2#173 |  | S2-2600290 | [KI#21] Use case (CN NF data collection/transfer for AI use) and solution proposal |
| SA2#173 |  | S2-2600309 | KI#21, bullet#2] Metadata handling for data identification and exposure in 6G Core Network |
| SA2#173 |  | S2-2600310 | [KI#21, bullet#2] Metadata handling for data identification and exposure in 6G Core Network |
| SA2#173 |  | S2-2600362 | [KI#21, bullet#1&2] UE-side Model Training as a Use Case for the 6G Data Framework |
| SA2#173 |  | S2-2600365 | [KI#21] Solution to support data services based on atomic operations |
| SA2#173 |  | S2-2600372 | [KI#21, bullet #1] Use cases on data framework in 6G architecture |
| SA2#173 |  | S2-2600373 | [KI#21, bullet #2] Solution on data framework in 6G architecture |
| SA2#173 |  | S2-2600375 | [KI#21] New Solution - Cross-Domain Data Framework |
| SA2#173 |  | S2-2600385 | [KI#21] Architecture and procedure solution for Data framework |
| SA2#173 |  | S2-2600439 | [KI#21] New solution: Data anonymization |
| SA2#173 |  | S2-2600445 | [KI#21] New solution: Data collection from RAN node/CN NF to CN NF |
| SA2#173 |  | S2-2600447 | [KI#21] New solution: Metadata based data discovery |
| SA2#173 |  | S2-2600448 | [KI#21] New solution: UE data collection |
| SA2#173 |  | S2-2600462 | [KI#21, bullet #2] Management of data source capabilities and data labels |
| SA2#173 |  | S2-2600493 | [KI#21] Solution to support Data Set/Encoder Parameter Sharing |
| SA2#173 |  | S2-2600525 | KI#21, bullet #2] Solution for basic architecture for data framework. |
| SA2#173 |  | S2-2600526 | [KI#21, bullet #2] New solution for the unified 6G data framework design |
| SA2#173 |  | S2-2600608 | [KI#21] 6G data framework in SA2 |
| SA2#173 |  | S2-2600373 | [KI#21, bullet #2] Solution on data framework in 6G architecture |
| SA2#173 |  | S2-2600375 | [KI#21] New Solution - Cross-Domain Data Framework |
| SA2#173 |  | S2-2600385 | [KI#21] Architecture and procedure solution for Data framework |
| SA2#173 |  | S2-2600439 | [KI#21] New solution: Data anonymization |
| SA2#173 |  | S2-2600445 | [KI#21] New solution: Data collection from RAN node/CN NF to CN NF |
| SA2#173 |  | S2-2600447 | [KI#21] New solution: Metadata based data discovery |
| SA2#173 |  | S2-2600448 | [KI#21] New solution: UE data collection |
| SA2#173 |  | S2-2600462 | [KI#21, bullet #2] Management of data source capabilities and data labels |
| SA2#173 |  | S2-2600493 | [KI#21] Solution to support Data Set/Encoder Parameter Sharing |
| SA2#173 |  | S2-2600526 | [KI#21, bullet #2] New solution for the unified 6G data framework design |

NOTE 1: One TDoc contribution may be mapped to multiple solutions, and the “Solution#” column will be updated later.

* * * *