3GPP TSG-WG SA2#173	S2-2601164

# Goa, India, 9 – 13 February, 2026

Source:	Penholders (vivo, Nokia, Samsung)

Title:	[KI#21] Penholders PCR for 6G data framework in SA2

Document for:	Approval

Agenda item:	20.6.21

Work Item:	FS_6G_ARC / R20

Abstract of the contribution: this PCR covers solutions for 6G data framework in SA2.

# 1.	Introduction

This solution addresses KI#21.

## Suggestion for work flow in the SA2#173 meeting：

As the work is just starting, and to keep discussion at this meeting focused, not all aspects of all input Tdocs has been included. Further details not taken into account by this already too long Tdoc will of course be considered during newt SA2 meetings.

This Tdoc should be discussed step by step (solution by solution)

  - In the first round of discussion discuss only the clause solution 0  (e.g., vocabulary, use-case, requirements, architecture, high level open questions)

  - In the second round of discussion each solution will be discussed individually in a step by step (i.e. solution by solutions) manner, keeping in mind that only essential aspects of the solutions are captured and need to be discussed

If time is too limited we could for example

    - discuss first solutions tagged as “architecture” (A to E)

    - Then discuss other solutions group by group where a group focuses on solutions target the same functionality

All text with rev marks added by LTH after the CC (while other pen holders were sleeping°

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.801-01:

* * * First Change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | #21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.0 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.A | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.B | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.C | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.D | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.E | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.F | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.G | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.H | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.I | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.J | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.K | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.L | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.M | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.N | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.O | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.P | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.Q | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.R | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.S | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * Next Change * * * * topic common aspects

### 6.21.0	Solution #21.0: Common aspects (Laurent+Vivian+Hyesung)

#### 6.21.0.0	Topics addressed and High-level Solution Principles

This pseudo solution addresses KI#21. It

- collects terminology that can be referred to by other solutions (e.g. potentially common definitions, common names for functionality or NF etc..)

- describes at high level the points of agreements and the points where agreements need to be reached.

##### 6.21.0.0.1		Definitions  (Laurent)

NOTE: 	defining a term in this clause does not mean that it is agreed that the corresponding feature or functionality is in scope of 6G specifications; it is possible that conclusions will mention that the functionality corresponding to some of the terms defined in this clause is not subject to specifications

Data collection: the process of requesting a 6GS entity to provide some data (or events).

Data transfer: the process of sending data (or events) from an entity that has these data (or that has detected the event).

Data registration: the process of a data provider indicating which data types (and possibly data transfer modes) it can provide.

##### 6.21.0.0.5		list of high level questions  (Laurent)

Editor’s note: This sub-clause lists the VERY HIGH level questions to be addressed to reach conclusions and can be considered as a BIG Editor’s NOTE; there are of course other extra points requiring an Editor’s Note

- Define the use cases at both functional level (does data framework apply to AIML inference, sensing, etc…) but also at domain level (for the different functional use cases, does data framework apply to collecting from UE domain, RAN domain, OAM domain,…)

  - For example, can 6G CN collect data from RAN, or does it collect RAN data via the OAM domain

- Define technically the involved functionalities but also which data framework functionality is to apply for which use case.

- Define the involved network functions (where functionalities could be defined as standalone NFs , collocated with a NF not part of data framework or merges in a data framework NF)

- How to transfer data (especially when data is transferred from UE and/or RAN)? using Control Plane, User Plane or a new “data plane” (which would need to be defined)

  - whether a dedicated Data Session distinct from the PDU session is established for data transfer purposes is FFS.

- How 6G data framework uses or does not use 5G data framework as a starting point

- Whether to introduce Data Agent in 6G for data framework

- Whether there may be common functionalities between SA5 and SA2 (e.g. DRF, DPF, DEF)

- For data collection from UE, which entity sends the data collection request, and which entity selects the UEs (when target UEs need to be selected)

- Is Data source entity information/capability registration a functionality needed, If yes, which NF supports it, which entities (among UE, RAN, and NF) can register.

- Should the data framework follow the Publish/Subscribe paradigm

- Should the data framework support new data transfer modes beyond HTTP (streaming, file transfer)

- The details for user consent management are FFS and subject to coordination with SA3

- Whether the data framework needs to define metadata distinct from the data themselves, what these metadata would contain (e.g. data producer info, data collection time) and how metadata would be used e.g. whether metadata would be used when data producer would register on a data registry; whether we need to define a metadata handler

##### 6.21.0.0.2	Use cases ( Laurent)

- AIML Model training with data collection for 6G CN NF from a 6G CN NF

- AIML Model transfer between 6G CN NFs (KI#18?)

- AIML Model training with data collection for UE sided model training (radio related)  subjective RAN decision.

- data collection for AIML inference (merge #1)

- data collection for ground truth retrieval (model quality monitoring) (CN use case only?)

- AIML Model training with data collection by a   NF (for transfer) from a non 6G CN entity (e.g. (R)AN, UE) (e.g. for UE-sided radio related model training) (justification?)

- Sensing: data collection by SENF from RAN when using RAN as a Sensing Entity,

- Sensing: data collection by SENF from RAN when using UE as a Sensing Entity,

- Sensing: data collection by SENF from UE when using UE as a Sensing Entity

  - Need general wording agreeable for everyone.

- Positioning (more details)

- Data collection related with OAM (not a use case)

- Missing:

  - Exposure?

  - Network Digital twin related use case?

##### 6.21.0.0.3	Architectural aspect including list of topics (Vivian)

NOTE 1: 	Defining a term xxxF or NF xxxNF in this clause does not mean that it is agreed that the corresponding feature or functionality is in scope of 6G specifications; it is possible that conclusions will mention that a functionality corresponding to some of the terms defined in this clause is not subject to specifications. Furthermore, final conclusions can determine that some of these functionalities are always co-located.

NOTE 2: 	Several options exist for each NF’s or functionality name, , the final name for NF or functionality  will be determined at a later stage.

NOTE 3: 	it is FFS whether NFs and functionalities listed in this clause will be specified as evolutions of similar NFs and functionalities defined in 5GC.

Functionalities potentially needed

(they are further defined below but their need is FFS)

-   	Data Collection ( control?) Functionality DCF: coordinates data collection, transfer and storage (even if it does not itself handles transfer and storage)

-	Data Transfer Functionality DTF (not generic)

-	Data Repository Functionality DRF

-	Data processing Functionality DPF

-	Data Exposure Functionality DEF

-	Data (capability) registration Functionality DCRF,

Not ready to start to add Data agent Functionality.

- Which functionality is appliable for which use case or all use cases?

- It is not NF, it is a functionality.

- Functionality that handles a flexible request from data consumer?

Editor’s Note: 	whether any of the functionalities above (and if yes which ones) can be common between SA2 and SA5 data frameworks is FFS

- A new Data Control Functionality (called DCF is defined in 6G CN to control data handling for data lifecycle. E.g data discovery, collection, processing, storage, and exposure. It performs: (This is the possible functionality)?

- authentication and authorization for data collection request from data consumers

- data source selection and data discovery

- (Data Orchestration) configuration of data source for data collection and transfer related parameters

- enforcing of user consent checking: If target data is associated with UE, the DCF enforces user consent checking for data collection from data consumer

- (Data Orchestration) control of data distribution/providing to data consumers

- control of data storage and retrieval

- control of data processing

- A Data Transfer Functionality DTF is defined in 6G CN, supporting: (whether this Functionality is UP or CP or new plan?) Keep it generic as much as possible.

- establishment, modification, and deletion of data transfer path for data transfer; This is done under control of the DCF

- data receiving from data sources

- data distribution/ to data consumers based on the control by DCF

- optionally data processing, e.g.  anonymization, desensitization, aggregation, labelling for data, etc.

- (When UE supports Data Collection and Transfer) For standardized data and when applicable, the DTF verifies/matches the requested data to be transferred and the data that is being reported.

- Need to add Data agent functionality  Can be considered as solution?

Editor’s note: whether the DTF is needed is FFS, and whether is collocated with the DCF or separated is FFS

- A Data Processing Functionality DPF is defined to provide data processing such as:

- data aggregation,

- data anonymization,  further coordination with SA3

- data pseudonymization,

- data labelling,

- formatting before delivering data to data consumer,

- data analysis

- etc.

Editor’s note: it is FFS whether the DPF is a separate NF or it is collocated with other functionalities.

- A Data Repository Functionality DRF is used for data storage and retrieval, it may perform:

- storing the collected data under the control of DCF

- retrieving the data under the control of DCF

-  	 optionally generating metadata of the stored data, or labelling different data types (e.g., AI data, sensing data, structured data, unstructured data)

-	Optionally DRF could do some processing for the stored data, e.g. generating meta data or labelling the data

Can be moved to #6?

- A Data Exposure Functionality DEF supports data exposure to the third party or UE with consideration of service authorization, privacy protection.

  - Need further clarification.

Editor’s note: How to expose data to 3rd party AF, e.g.  via SBI or new interface, invoking new service is FFS  further simplication.

- A Data (capability) registration functionality DCRF may support registration of data production capability (e.g. supported data type, data format) by a NF or possibly by a non 6G CN entity (e.g. RAN).

Common Principles for data collection/transfer

- The data requester (e.g. DCF or  possibly other functionalities / NF) in 6G CN sends a data collection/transfer request to the data provider (e.g. data source(s)), the data collection/transfer request contains the configuration about which data to collect and how to transfer the collected data (e.g. addressing information of where to transfer the data). The data requester may also stop data collection/transfer procedure.

- The data collection/transfer request may also include some data collection termination criteria.

- The data provider transfers the collected data according to the configuration negotiated with the data requester.

Editor’s Note: how to transfer the collected data to 6G CN, e.g. via CP, UP, new data plane or tunnel is FFS which may be coordinated with RAN WGs when RAN is involved.

- A data provider (e.g. 6G UE, 6G RAN, and 6G NF) can reject or cancel the data collection or data transfer anytime with consideration of its related status (e.g., for UE: battery status, for RAN or 6G CN NF: load status, this may also be based on a change of subscriber permission status)

NOTE: Any impacts of the data framework to RAN will be subject to co-ordination with TSG RAN WGs

Principles for UE data collection/transfer

- Dedicated path for UE data transfer

- A dedicated path is established for data transfer between UE and the 6G NF collecting data from the UE (containing the DTF functionality).

Editor’s note: The transport mechanism to be used for the dedicated path (e.g. CP, UP or new Data Plane) is FFS. Co-ordination with K1.2 may be needed.

- The UE receives in the data collection request the address of the 6G NF (containing the DTF functionality) collecting data from UE.

3.	Data transfer procedure:

-	A request for collection/transfer with requested data information is sent to the target UE(s).

-	The UE reports the requested data via the established dedicated path to  the 6G NF (containing the DTF functionality) collecting data from UE.

4.	Traffic Differentiation:

Editor’s note: how to differentiate the traffic of the collected data for 6G data framework from the UE regular traffic, e,g, based a new type of data session distinct from PDU session , or just based on DNN, S-NSSAI is FFS.

###### Items requiring further work

Editor’s note: Comparing with existing control plane and user plane, whether a new data plane needs to be introduced in 6G for data framework is FFS. Editor’s note: What is the precise definition of the data plane concept is FFS.  Does it refer to a new generalized plane distinct from the CP or UP, or does it just denote a data transmission channel?

Editor’s note: Whether and how to define QoS model for data plane is FFS.

Editor’s note: Whether new common interface (e.g. DBI) should be introduced for data framework is FFS. How SBI would differ from DBI (data bus interface) is also FFS

Editor’s note: (for data collection/transfer from UE) whether a dedicated Data Session distinct from the PDU session is established for data transfer purposes is FFS.

Editor’s note: Whether introduce Data Agent in 6G for data framework is FFS.

##### 6.21.0.0.4	Requirements (Hyesung)

NOTE 1:	whether all the functionalities listed in this sub-clause will be subject to specifications will be determined when defining the study conclusions.

NOTE  2: whether all the functionalities listed in this sub-clause are covered by data framework for all use cases will be determined when defining the study conclusions.

Agreed requirements:

- The data framework supports full lifecycle management of data, including collection, transfer, processing, storage, and exposure, while taking into account user consent and privacy.

- A common set of interfaces and APIs can be used for the data framework for a range of use cases.

- The 6G data framework shall avoid control plane overload.

- (Operator controllability) MNO has full control of the data collection, data transfer process and can manage data access and exposure. This includes initiating, terminating and fully managing data transfer.

- (Operator controllability) The 6G data framework enable MNO controlled granular data access for exposure and security, including the scope of access to the data framework for 3rd party entities.

Editor’s Notes: It is FFS whether to support controllability on the scope of access to the data framework for network internal entities

- Operator visibility: MNO has full visibility for standardized data e.g. for standardized data and when applicable, the 6G CN verifies/matches the requested data to be transferred and the data that is being reported.

- The data transfer for data collection in 6G data framework shall be differentiated from the user traffic (also no charging to the subscriber for the data collection).

- The 6G data framework shall allow flexible deployment (e.g., deployed as a single instance or multiple instance)

- Data source entity (or data provider entity) shall be able to reject or cancel the data collection or data transfer.

- User consent (or subscriber permission) shall be respected.

Editor’s note: The details for user consent management are FFS and subject to coordination with SA3.

- The 6G data framework shall provide privacy protection mechanism (e.g., anonymization and pseudonymization).

Editor’s note: The details for privacy protection mechanism are FFS and subject to coordination with SA3.

- The 6G data framework shall support data transfer mechanisms (potentially event-based, batch access/file-based and streaming-based data transfer ) depending on data characteristic (e.g., real-time vs. non-real time, data volume)

- The 6G data framework shall support bulky or highly frequent data transfer mechanisms.

- Principles for quality of Data (QoD): Data framework supports data quality aspects as per the requirement from Data Consumers

.Editor’s note: What are the criteria of data quality (e.g., confidence interval, data freshness information, data accuracy)and how to enforce the quality of data is FFS. It is FFS whether Quality of data can include  KPI for each phase, e.g, data collection (e.g., fulfilling the requirements for Data collection latency), data processing (e.g., the requirements for data Completeness and/or Data processing latency) and data transmission (e.g., guaranteeing latency considering Data transmission).

- The 6G data framework shall consider whether to support roaming scenario based on SLA and regulation.

- The 6G data framework shall support to manage historical data.

- The 6G data framework should be futureproof.

- The data framework supports data storage based on operator policies, for short and long term.

- The data framework should avoid duplicate collection, storage and processing. If multiple data consumers request the same data from the same data provider or the same processing functionality, the data framework should aggregate these requests and prevent duplicate subscription.

- Data source entity (or data provider entity) shall be able to reject or cancel the data collection or data transfer and may stop data transferring based on internal determination (i.e., based on implementation).

Requirements under discussion (FFS):

- The 6G data framework should avoid impact on user plane and minimize impact on control plane.

- Operator’s partial visibility: MNO has partial visibility for vendor-specific data.

- The 6G system has a single framework to support collection and transfer for both data and events.

- The followings are out of scope of 6G data framework: Internal state of NFs such as context data (e.g., AMF context, SMF context), UDM/UDR data (e.g., subscription data), LI data,.

- A common solution is applicable to support data collection from UE, 6G RAN, 6G CN and OAM, and data consuming by UE, 6G RAN, 6G CN and the third AF based on operator policy.

Editor’s note: Whether data can be collected via data framework from OAM is FFS.

#### 6.21.0.1	Procedures

Editor’s Note: For further Study

#### 6.21.0.2	Services, Entities and Interfaces

Editor’s Note: For further Study

* * * Next Change * * * * topic data framework architecture (Vivian)

Variant A: control signaling and data transfer collocation architecture

Variant B: architecture with split between control signalling and data transfer

Variant C: No dedicated data distributing entity/proxy as the intermediate hop

Variant E: Architecture with data islands

Variant F: Architecture reusing 5GC as baseline

Variant T: Architecture with attachable data framework endpoint capability

### 6.21.A	Solution #21.A: Architecture Variant : control signalling and data transfer collocation architecture

#### 6.21.A.0	Topics addressed and High-level Solution Principles

This solution addresses KI#21

Editor’s Note: the terminology used in this solution should be aligned with the terminology in solution “0”

- A centralized coordination NF (e.g. called DCF) is defined in 6G CN to control data handling for data lifecycle. E.g data discovery, collection, transmission, processing, storage, and exposure.

- This new coordination NF also supports as data anchor, to receive data from data source(s) and to distributing data to data consumer(s)

#### 6.21.A.1	Description

Editor’s Note: For further Study

#### 6.21.A.2	Procedures

Editor’s Note: For further Study

#### 6.21.A.3	Services, Entities and Interfaces

Editor’s Note: For further Study

* * * Next Change * * * *

### 6.21.B	Solution #21.B: Architecture variant: architecture with split between control signalling and data transfer

#### 6.21.B.0	Topics addressed and High-level Solution Principles

This solution addresses KI#21.

Editor’s Note: the terminology used in this solution should be aligned with the terminology in solution “0”

- A centralized coordination NF (e.g. called DCF) is defined in 6G CN to control data handling for data lifecycle. E.g data discovery, collection, transmission, processing, storage, and exposure.

- There is also dedicated data anchor in CN (distributing entity/proxy), to receive data from data source(s) and to distributing data to data consumer(s)

Editor’s Note: it is For further Study whether the Data transfer Interface above need to be identified as a new Data bus (interface) (DBI) or Data Plane

#### 6.21.B.1	Description

Editor’s Note: For further Study

#### 6.21.B.2	Procedures

Editor’s Note: For further Study

#### 6.21.B.3	Services, Entities and Interfaces

Editor’s Note: For further Study

* * * Next Change * * * *

### 6.21.C	Solution #21.C: Architecture Variant: No dedicated data distributing entity/proxy as the intermediate hop

#### 6.21.C.0	Topics addressed and High-level Solution Principles

This solution addresses KI#21.

Editor’s Note: the terminology used in this solution should be aligned with the terminology in solution “0”

A centralized coordination NF (e.g. called DCF) is defined in 6G CN to control data handling for data lifecycle. E.g data discovery, collection, transmission, processing, storage, and exposure.

- There is no dedicated data distributing entity/proxy as the intermediate hop between data source and data consumer.

#### 6.21.C.1	Description

Editor’s Note: For further Study

#### 6.21.C.2	Procedures

Editor’s Note: For further Study

#### 6.21.C.3	Services, Entities and Interfaces

Editor’s Note: For further Study

* * * Next Change * * * *

### 6.21.D	Solution #21.D: Architecture variant with data islands

#### 6.21.D.0	Topics addressed and High-level Solution Principles

This solution addresses KI#21.

A Data framework can be instantiated separately in the CN domain and in the OAM domain in so-called data islands.

It should also be possible to have data islands that support multiple domains but this would be a matter for deployment and configuration

RAN data are only provided to the OAM domain.

What is to be standardized are only interfaces with the data islands. The Data framework defines only

-	the interface for data consumer to get data from the data framework.

-	the interface for data Framework to get data from the data provider.

#### 6.21.D.1	Description

Editor’s Note: For further Study

#### 6.21.D.2	Procedures

Editor’s Note: For further Study

#### 6.21.D.3	Services, Entities and Interfaces

Editor’s Note: For further Study

* * * Next Change * * * *

### 6.21.E	Solution #21.E: Architecture variant reusing 5GC as baseline

#### 6.21.E.0	Topics addressed and High-level Solution Principles

This solution addresses KI#21.

- This architecture focuses on data collection from CN, OAM and AF to support feature of AI/ML in 6G CN + UE data collection for UE-sided model training (no support of data collection from UE for AIML operation in 6G CN)

- A Model Training functionality and / or Inference functionality may collect data from data sources directly, or collect data via the DCCF 	acting as DCF defined in solution 0

- There is no direct data collection / reporting between RAN and a data consumer for AIML in CN. The data consumer can subscribe to OAM to get MDT / SON related data,

- The architecture aims at reusing the DCCF defined in TS 23.288

- The ADRF is reused for AIML in CN for 6G as well, to store the historic information and register the stored

Figure XX: Data framework for AIML in 6G CN

Figure XX:UE data collection for UE-sided model training architecture

Editor’s Note: the terminology used in this solution should be aligned with the terminology in solution “0”

#### 6.21.E.1	Description

Editor’s Note: For further Study

#### 6.21.E.2	Procedures

Editor’s Note: For further Study

#### 6.21.E.3	Services, Entities and Interfaces

Editor’s Note: For further Study

* * * Next Change * * * *

### 6.21.T	Solution #21.T Architecture variant: use of an attachable data framework endpoint capability for 6G CN NF data producer and/or data consumer

#### 6.21.T.0	Topics addressed and High-level Solution Principles

This solution addresses KI#21.

The solution introduces the DFAF functionality as

- an attachable data framework endpoint capability enabling an existing 6G CN NF to act as a data producer and/or data consumer.

#### 6.21.T.1	Description

Editor’s Note: For further Study

#### 6.21.T.2	Procedures

Editor’s Note: For further Study

#### 6.21.T.3	Services, Entities and Interfaces

Editor’s Note: For further Study

* * * Next Change * * * * (Topic for Data registration and discovery) ( Hyesung)

Subtopic 1 - Data source entity information/capability registration

Subtopic 2 - Data registration/discovery for UE data collection with RAN involvement
- Variant A: 6G CN to select the target UE
- Variant B: 6G RAN to select the target UE

### 6.21.F	Solution #21.F: Topic: Data registration/discovery (Data source entity information/capability registration to a dedicated NF)

#### 6.21.F.0	Topics addressed and High-level Solution Principles

This solution addresses KI#21, the topic of Data Registration / discovery.

This variant of topic for data registration/discovery is based on the following principles:

-	A dedicated data management NF provides data source entity registration service.

-	Data information (e.g., supported data type, meta data) and data capabilities of data source entity are registered to the data management NF.

As a variant the Data Producer/source may register the metadata of the data (and/or the actual data) to a Data capability Registry (DCRF); The data service consumer may be configured to query the DCRF using metadata to discover the requested data and/or the Data Producer.

In this variant the metadata of the data may include following information:

- data type

- data usage purpose

- data size

- data producer info

- data collection time

- data collection location

- data quality

- data sample number

Editor’s Note: Whether the data framework needs to define metadata distinct from the data themselves, what these metadata would contain (e.g. data producer info, data collection time) and how metadata would be used e.g. whether metadata would be used when data producer would register on a data registry

Discussion point: which entities (among UE, RAN, and NF) can register to the data management function supporting data registration.

#### 6.21.F.1	Description

Editor’s Note: For further Study

#### 6.21.F.2	Procedures

Editor’s Note: For further Study

#### 6.21.F.3	Services, Entities and Interfaces

Editor’s Note: For further Study

* * * Next Change * * * *(Topic Common Data transfer aspects) ( Laurent)

### 6.21.G	Solution #21.G: Topic Common Data collection / transfer aspects

#### 6.21.G.0	Topics addressed and High-level Solution Principles

This solution addresses KI#21. Its main aspects (data collection and data transfer are defined in solution 0) :

- There is a single framework to support data collection for both data and events.

- There is a single framework to support data transfer for both data and events.

- Data transfer may use one of the following delivery/transfer modes:

-	Notification (as defined for SBA based Event-based reporting),

-	Streaming (like what is defined in TS 28.532, TS 28.104), e.g.  for data collection for continuous, Real Time

-	File-based (like what is defined in TS 28.532, TS 28.104) e.g. for bulk non-Real Time data transfer

- The data / event information format is independent of the data transfer mode

- The data collection and transfer may involve a dedicated (to data framework) NF (from data framework) that supports both DCF and DTF functionality or another NF not part of the data framework such as the SENF (sensing).

NOTE 1: 	The content of this solution can apply to data collection from 6G CN, UE and RAN (where each of 6G CN, RAN and UE is a potential data provider)

NOTE 2: 	Data collection is specific to the application e.g. data retrieval for AIML, sensing etc.. Data transfer can be common for multiple applications, and multiple domains e.g. the same set of data transfer modes can apply within the Core and also within the OAM (SA5) domain

NOTE 3: 	The 3 mechanisms above refer to 3GPP SA5 mechanisms defined for 5GS. In 6G, protocols used for data transfer should be common between SA2 and SA5. These protocols are not in the scope of SA2 but SA2 should be able to set requirements for these 3 data transfer/delivery modes.

NOTE 4: 	the data collection control signalling is NOT meant to be handled by three data transfer modes above (notification, streaming, and file).

Editor’s Note: it is For further Study whether the Data transfer capabilities above need to be identified as a new Data bus (interface) (DBI)

#### 6.21.G.1	Description

Editor’s Note: For further Study

#### 6.21.G.2	Procedures

Editor’s Note: For further Study

#### 6.21.G.3	Services, Entities and Interfaces

Editor’s Note: For further Study

* * * Next Change * * * * topic UE data collection/transfer (Vivian)

Subtopic 1 – how to design the mechanism for the data collection/transfer control signaling.

- Variant H: UE data collection not involve RAN (CN does UE selection)

- Variant I: UE data collection involve RAN (RAN does UE selection)

-Subtopic 2 – which path for UE data transfer.
- Variant J:  UE data collection and transfer variant using PDU Sessions

Variant K: UE data collection and transfer variant using Data Session

### 6.21.H	Solution #21.H: topic UE data collection and transfer variant : not involving RAN

#### 6.21.H.0	Topics addressed and High-level Solution Principles

This solution addresses KI#21, the topic of UE data collection and transfer.

This variant proposes procedures when data collection does not require RAN involvement (RAN only forwards NAS messages), in this case

-	the data collection request message sent to the UE is a NAS message created by a 6G CN NF (e.g. DCF or SeNF for sensing) and transparently relayed by the Access network (e.g. RAN) to the UE

-	It is the 6G CN that selects the target UEs

Editor’s Note: it is For further Study whether for the solution a tunnel needs to be established between the PSA UPF and DTF.

Editor’s Note: it is For further Study whether 6G CN NF (e.g. DCF or SeNF for sensing) that sends the NAS request needs support from AMF

#### 6.21.H.1	Description

Editor’s Note: For further Study

#### 6.21.H.2	Procedures

Editor’s Note: For further Study

#### 6.21.H.3	Services, Entities and Interfaces

Editor’s Note: For further Study

* * * Next Change * * * *

### 6.21.I	Solution #21.I: topic UE data collection and transfer variant:  with RAN involvement

#### 6.21.I.0	Topics addressed and High-level Solution Principles

This solution addresses KI#21, the topic of UE data collection and transfer.

This variant proposes procedures when data collection requires RAN involvement, in this case:

-	The data collection request message sent to the UE is a RRC message created by the RAN but that may contain information like data transfer information (addressing, protocol and security information to transfer the data) determined by the 6G CN (when a 6G CN NF like the DCF or SeNF (sensing) is the actual recipient of the data transfer)

-	It is the RAN that selects the target UEs

Editor’s Note: FFS: The 6G RAN can be configured with the user consent related information for target UE selection.

Editor’s Note: it is For further Study whether for the solution a tunnel needs to be established between the PSA UPF and DTF.

#### 6.21.I.1	Description

Editor’s Note: For further Study

#### 6.21.I.2	Procedures

Editor’s Note: For further Study

#### 6.21.I.3	Services, Entities and Interfaces

Editor’s Note: For further Study

* * * Next Change * * * *

### 6.21.J	Solution #21.J: topic UE data collection and transfer variant: using PDU Sessions

#### 6.21.J.0	Topics addressed and High-level Solution Principles

This solution addresses KI#21, the topic of UE data collection and transfer.

This variant proposes procedures when data transfer from UE is via UP session (i.e. PDU session)

- Using URSP to determine which kind of PDU Session to use

- Establishing a PDU Session dedicated to data transfer and establishing a data transfer session.

Editor’s Note: it is For further Study whether for the solution a tunnel needs to be established between the PSA UPF and DTF.

#### 6.21.J.1	Description

Editor’s Note: For further Study

#### 6.21.J.2	Procedures

Editor’s Note: For further Study

#### 6.21.J.3	Services, Entities and Interfaces

Editor’s Note: For further Study

* * * Next Change * * * *

### 6.21.K	Solution #21.K: topic UE data collection and transfer variant : using Data Session

#### 6.21.K.0	Topics addressed and High-level Solution Principles

This solution addresses KI#21, the topic of UE data collection and transfer.

This variant proposes procedures when data transfer from UE is via a new type of data session

- A dedicated Data Session is established for UE data transfer purposes via Data Plane.

#### 6.21.K.1	Description

Editor’s Note: For further Study

#### 6.21.K.2	Procedures

Editor’s Note: For further Study

#### 6.21.K.3	Services, Entities and Interfaces

Editor’s Note: For further Study

* * * Next Change * * * * (Topic: RAN data collection) Hyesung

Variant L: Dedicated connection/session between RAN and 6G CN

Variant M: Integrated collection of RAN data and RAN OAM data

Variant N: RAN data collection, variant : Integrated collection of RAN data and RAN OAM data

### 6.21.L	Solution #21.L: Topic: RAN data collection, variant: Dedicated connection/plane between RAN and CN

#### 6.21.L.0	Topics addressed and High-level Solution Principles

This solution addresses KI#21, the topic of RAN data collection.

This variant of topic “dedicated connection/session” between RAN and 6G CN for RAN data collection is based on the following principles:

- For data collection (configuration of RAN acting as data source): Data Control signaling is exchanged between RAN and data collection NF (or via proxy or broker).

- For data transfer: The 6G system support the direct data connection and interface over new plane between RAN and the data collection NF.

Editor’s Note: the following are FFS: Whether RAN SBI is needed or not. What this new plane consists of

#### 6.21.L.1	Description

Editor’s Note: For further Study

#### 6.21.L.2	Procedures

Editor’s Note: For further Study

#### 6.21.L.3	Services, Entities and Interfaces

Editor’s Note: For further Study

### 6.21.M	Solution #21.M: Topic: RAN data collection, variant : Integrated collection of RAN data and RAN OAM data

#### 6.21.M.0	Topics addressed and High-level Solution Principles

This solution addresses KI#21, the topic of RAN data collection.

This variant of topic for RAN/RAN OAM data integrated collection is based on the following principles:

- For data collection (configuration of RAN acting as data source):

-	Configuration for RAN and RAN OAM data collection. RAN also collects the RAN OAM data and transfer RAN OAM data to the 6G CN

-	data collection subscription over NGAP

- For data transfer: The 6G RAN supports SBI for RAN data transfer to the 6G CN

#### 6.21.M.1	Description

Editor’s Note: For further Study

#### 6.21.M.2	Procedures

Editor’s Note: For further Study

#### 6.21.M.3	Services, Entities and Interfaces

Editor’s Note: For further Study

* * * Next Change * * * *

### 6.21.N	Solution #21.N: Topic: RAN data collection, variant : Integrated collection of RAN data and RAN OAM data

#### 6.21.N.0	Topics addressed and High-level Solution Principles

This solution addresses KI#21, the topic of RAN data collection.

This variant of topic for RAN/RAN OAM data integrated collection is based on the following principles:

- For data collection (configuration of RAN acting as data source):

  - Configuration for RAN and RAN OAM data collection. RAN data is only collected via RAN OAM; 6G CN collects RAN data only acting as a 6G management data consumer.

#### 6.21.N.1	Description

Editor’s Note: For further Study

#### 6.21.N.2	Procedures

Editor’s Note: For further Study

#### 6.21.N.3	Services, Entities and Interfaces

Editor’s Note: For further Study

* * * Next Change * * * * Hyesung

### 6.21.O: Solution #21.O Topic: RAN/CN data collection leveraging the Publish/Subscribe model

#### 6.21.O.0	Topics addressed and High-level Solution Principles

This solution addresses KI#21, the topic of data collection and transfer.

This solution leverages the Publish/Subscribe model.

- data producers may be NFs and/or RAN nodes, while data consumers are NFs. The data broker is a CN NF that supports a publish/subscribe mechanism (e.g., DCCF),

- 2 alternatives

- data producer directly registers available data (stored data or collectable data) with the data broker.

- The data broker obtains the data producer information from a data discovery function

- The data producer(s) transfer the data to the data broker.

- The data broker distributes the received data to the data consumer(s).

#### 6.21.O.1	Description

Editor’s Note: For further Study

#### 6.21.O.2	Procedures

Editor’s Note: For further Study

#### 6.21.O.3	Services, Entities and Interfaces

Editor’s Note: For further Study

* * * Next Change * * * * (Topic: Data storage) Hyesung

### 6.21.P	Solution #21.P: Topic: Data storage

#### 6.21.P.0	Topics addressed and High-level Solution Principles

This solution addresses KI#21, the topic of data storage

This variant of topic for data registration/discovery is based on the following principles:

- 6G data framework has a dedicated data storage function

- Store the collected data to avoid duplicated collection

- Support efficient storage of data for different data types (e.g., sensing related data, AI/ML data), based on configuration of DCF.

- Performs unified and efficient storage, retrieval and access control according to policies configured by the Data Control Function

Discussion point:

- The scope of data to be stored in the data storage function,

- Which entity can access the storage function.

#### 6.21.P.1	Description

Editor’s Note: For further Study

#### 6.21.P.2	Procedures

Editor’s Note: For further Study

#### 6.21.P.3	Services, Entities and Interfaces

Editor’s Note: For further Study

* * * Next Change * * * * topic data processing (Vivian)

### 6.21.Q	Solution #21.Q: Topic Data processing

#### 6.21.Q.0	Topics addressed and High-level Solution Principles

This solution addresses KI#21 specifically for the topic of data processing.

- Potential data processing operations for the 6G data framework include: aggregation (e. g. multi-source data fusion), anonymization, pseudonymization, formatting, labelling, analysis, data cleaning, dataset creation (specifying size, samples, format), data alignment, enforcing quality, metadata handling.

- Data processing for 6G data framework includes the following solution (not mutually exclusive):

- Data source for data pre-processing e,g, anonymization, labelling, generating metadata, adding standardized identifiers, sampling, parameter translation, etc..

- A new 6G CN NF (e.g. DPNF) for dedicatedly data processing, e.g. data aggregation, data anonymization, data pseudonymization, data labelling, formatting, synthetic data generation, etc.

- Data processing may be carried out by the DTF (data Transfer Functionality) for in-path data processing, e.g. aggregation, anonymization, pseudonymization, formatting, labelling, etc..

- Data processing may be carried out by the DRF for the stored data processing, e.g. generating meta data or labelling the data, etc.

Editor’s Note: it is FFS whether each of these solutions is needed.

#### 6.21.Q.1	Description

Editor’s Note: For further Study

#### 6.21.Q.2	Procedures

Editor’s Note: For further Study

#### 6.21.Q.3	Services, Entities and Interfaces

Editor’s Note: For further Study

* * * Next Change * * * * topic data exposure (Vivian)

### 6.21.R	Solution #21.R: Topic Data exposure

#### 6.21.R.0	Topics addressed and High-level Solution Principles

This solution addresses KI#21 and data exposure topic.

There are following variant solutions for data exposing in 6G data framework.

- 6G NEF should be enhanced to support data exposure to the third party with consideration of service authorization, privacy protection, user consent and security.

- Data agent is used to control data exposure to authorized AF/UE based on intent.

- The DCF functionality for controlling of data handling also controls data exposure to 3rd AF or UE.

- A new NF DENF dedicated for data service exposure is introduced. DENF exposes requested data to 3rd AF or UE.

Editor’s Note: which entity will be chosen to do data exposure is FFS

#### 6.21.R.1	Description

Editor’s Note: For further Study

#### 6.21.R.2	Procedures

Editor’s Note: For further Study

#### 6.21.R.3	Services, Entities and Interfaces

Editor’s Note: For further Study

* * * Next Change * * * * (Topic: User consent and privacy) (Hyesung)

### 6.21.S	Solution #21.S: Topic user consent/subscriber permission and privacy

#### 6.21.S.0	Topics addressed and High-level Solution Principles

This solution addresses KI#21 and data exposure topic.

This variant proposes solution for user consent/subscriber permission and data privacy aspects.

- An NF in the CN manages user consent/subscriber permission information.
FFS: Whether to provision user consent/subscriber permission information into RAN.

- The user consent/subscriber permission information is provided from the UE and can be changed over conditions (e.g., temporally, spatially).

- The 6G data framework supports the enforcement of user consent/subscriber permission, i.e., authorize the data collection/transfer request for UE-related data.

- In the 6G data framework, an NF performs data privacy protection/preserving mechanisms (including anonymization, pseudonymization, differential privacy protection method such as noise addition) depending on the requested data type/service, data consumer, or request by data consumer.

- The 6G data framework support to store the data which is anonymized or pseudonymized.

#### 6.21.S.1	Description

Editor’s Note: For further Study

#### 6.21.S.2	Procedures

Editor’s Note: For further Study

#### 6.21.S.3	Services, Entities and Interfaces

Editor’s Note: For further Study

* * * Next Changes * * * *

Annex X: Submitted solutions

## X.1	List of submitted solutions

Guidance – include in this clause:

- 	a table containing references to the individual solution proposals that lead to the solution variants in this pCR.

-	The “Solution#” column is used to cross-reference between the solution variant description (clause 6.X.Y.0) and the input papers in this Table. Use sequential numbering (1,2,3,…). The rapporteurs will adjust the numbering when implementing the TR.

Table X.4: List of submitted solutions

| Meeting | Solution# | TDoc | Subject/Comment |
| --- | --- | --- | --- |
| SA2#173 |  | S2-2600080 | 23.801-01: [KI#21] New solution on 6G data framework. |
| SA2#173 |  | S2-2600088 | 23.801-01: [KI#21, Bullet #1, #2] 6G data framework Architecture and Procedures. |
| SA2#173 |  | S2-2600090 | 23.801-01: [KI#21] Solution about Data framework for 6G. |
| SA2#173 |  | S2-2600099 | 23.801-01: [KI#21, bullet #1&2] Solution for KI#21 about the introduction of Data Plane for 6G Data Framework Design. |
| SA2#173 |  | S2-2600103 | 23.801-01: [KI#21, bullet#2] New solution for data framework. |
| SA2#173 |  | S2-2600124 | 23.801-01: KI#21bullet#2: Solution#X Architecture for 6G data framework . |
| SA2#173 |  | S2-2600144 | 23.801-01: [KI#21] new solution of 6G Data Framework between 6G UE and 6G CN. |
| SA2#173 |  | S2-2600145 | 23.801-01: [KI#21] new solution of 6G Data Framework between 6G RAN and 6G CN. |
| SA2#173 |  | S2-2600152 | 23.801-01: [KI#21, bullet 2] New Solution for unified framework for data storage and exposure. |
| SA2#173 |  | S2-2600154 | 23.801-01: [KI#21, bullet #1,#2,#3] 6G data framework in SA WG2. |
| SA2#173 |  | S2-2600170 | 23.801-01: [KI#21] New solution on Data framework architecture and procedures. |
| SA2#173 |  | S2-2600172 | 23.801-01: [KI#21, bullet#1&bullet#2] New Sol#X: Generalized and common data framework . |
| SA2#173 |  | S2-2600187 | 23.801-01: [KI#21, bullet#1, bullet#2] New Solution on 6G network support UE data collection for UE-sided model training. |
| SA2#173 |  | S2-2600195 | [KI#21] High Level Principles for 6G data framework |
| SA2#173 |  | S2-2600201 | [KI#21, bullet #2] Functions and Procedure for Data Registration, Discovery and Distribution |
| SA2#173 |  | S2-2600226 | [KI#21, bullet 2] Data Collection Framework leveraging the Publish/Subscribe model |
| SA2#173 |  | S2-2600228 | [KI#21] new solution: Event and data collection and transfer requirements |
| SA2#173 |  | S2-2600229 | [KI#21] new solution: Data transfer from UE using PDU Sessions |
| SA2#173 |  | S2-2600239 | [KI#21, bullet 1] Use case for instantiation of the data framework based on volume, quality, cost or other optimization criteria |
| SA2#173 |  | S2-2600240 | [KI#21, bullet 1] Network Digital Twin use case for the data framework |
| SA2#173 |  | S2-2600241 | [KI#21, bullet 1] Operator controlled granular data access for exposure and security |
| SA2#173 |  | S2-2600242 | [KI#21, bullet 1] Network analytics use case for the data framework |
| SA2#173 |  | S2-2600243 | [KI#21] Principles of the data framework |
| SA2#173 |  | S2-2600263 | New Solution: Adapter-based 6G Data Framework Architecture for Sensing |
| SA2#173 |  | S2-2600288 | [KI#21] Use case (UE data collection/transfer for AI use) and solution proposal |
| SA2#173 |  | S2-2600289 | [KI#21] Use case (RAN data collection/transfer for AI use) and solution proposal |
| SA2#173 |  | S2-2600290 | [KI#21] Use case (CN NF data collection/transfer for AI use) and solution proposal |
| SA2#173 |  | S2-2600309 | KI#21, bullet#2] Metadata handling for data identification and exposure in 6G Core Network |
| SA2#173 |  | S2-2600310 | [KI#21, bullet#2] Metadata handling for data identification and exposure in 6G Core Network |
| SA2#173 |  | S2-2600362 | [KI#21, bullet#1&2] UE-side Model Training as a Use Case for the 6G Data Framework |
| SA2#173 |  | S2-2600365 | [KI#21] Solution to support data services based on atomic operations |
| SA2#173 |  | S2-2600372 | [KI#21, bullet #1] Use cases on data framework in 6G architecture |
| SA2#173 |  | S2-2600373 | [KI#21, bullet #2] Solution on data framework in 6G architecture |
| SA2#173 |  | S2-2600375 | [KI#21] New Solution - Cross-Domain Data Framework |
| SA2#173 |  | S2-2600385 | [KI#21] Architecture and procedure solution for Data framework |
| SA2#173 |  | S2-2600439 | [KI#21] New solution: Data anonymization |
| SA2#173 |  | S2-2600445 | [KI#21] New solution: Data collection from RAN node/CN NF to CN NF |
| SA2#173 |  | S2-2600447 | [KI#21] New solution: Metadata based data discovery |
| SA2#173 |  | S2-2600448 | [KI#21] New solution: UE data collection |
| SA2#173 |  | S2-2600462 | [KI#21, bullet #2] Management of data source capabilities and data labels |
| SA2#173 |  | S2-2600493 | [KI#21] Solution to support Data Set/Encoder Parameter Sharing |
| SA2#173 |  | S2-2600525 | KI#21, bullet #2] Solution for basic architecture for data framework. |
| SA2#173 |  | S2-2600526 | [KI#21, bullet #2] New solution for the unified 6G data framework design |
| SA2#173 |  | S2-2600608 | [KI#21] 6G data framework in SA2 |
| SA2#173 |  | S2-2600373 | [KI#21, bullet #2] Solution on data framework in 6G architecture |
| SA2#173 |  | S2-2600375 | [KI#21] New Solution - Cross-Domain Data Framework |
| SA2#173 |  | S2-2600385 | [KI#21] Architecture and procedure solution for Data framework |
| SA2#173 |  | S2-2600439 | [KI#21] New solution: Data anonymization |
| SA2#173 |  | S2-2600445 | [KI#21] New solution: Data collection from RAN node/CN NF to CN NF |
| SA2#173 |  | S2-2600447 | [KI#21] New solution: Metadata based data discovery |
| SA2#173 |  | S2-2600448 | [KI#21] New solution: UE data collection |
| SA2#173 |  | S2-2600462 | [KI#21, bullet #2] Management of data source capabilities and data labels |
| SA2#173 |  | S2-2600493 | [KI#21] Solution to support Data Set/Encoder Parameter Sharing |
| SA2#173 |  | S2-2600526 | [KI#21, bullet #2] New solution for the unified 6G data framework design |

NOTE 1: One TDoc contribution may be mapped to multiple solutions, and the “Solution#” column will be updated later.

* * * End of Changes * * * *