3GPP TSG-WG SA2#173	S2-2601616 (was 1385)

# Goa, India, 9 – 13 February, 2026

Source:	Penholders (vivo, Nokia, Samsung)

Title:	[KI#21] Penholders PCR for 6G data framework in SA2

Document for:	Approval

Agenda item:	20.6.21

Work Item:	FS_6G_ARC / R20

Abstract of the contribution: this PCR covers solutions for 6G data framework in SA2.

# 1.	Introduction

This solution addresses KI#21.

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.801-01:

* * * First Change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | #21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.0 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.A | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.B | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.C | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.D | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.E | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.F | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.G | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.H | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.I | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.J | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.K | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.L | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.M | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.N | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.O | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.P | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.Q | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.R | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #21.S | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * Next Change * * * * topic common aspects

### 6.21.0	Solution #21.0: Common aspects

#### 6.21.0.0	Topics addressed and High-level Solution Principles

This pseudo solution addresses KI#21. It

- collects terminology that can be referred to by other solutions (e.g. potentially common definitions, common names for functionality  etc..)

##### 6.21.0.0.1		Definitions

NOTE: 	defining a term in this clause does not mean that it is agreed that the corresponding feature or functionality is in scope of 6G specifications; it is possible that conclusions will mention that the functionality corresponding to some of the terms defined in this clause is not subject to specifications

Data source triggering: the process of requesting a 6GS entity to provide some data (or events).

Data transfer: the process of sending data (or events) from an entity that has these data (or that has detected the event).

Data distribution: the process of transferring data from the data framework to data consumer(s).

##### 6.21.0.0.2	Use cases

Potential  use cases:

Editor’s Note: some further definition of each use case is needed.

- Data handling for AIML Model training for a 6G CN NF

- Data handling for AIML inference by a 6G CN NF

- Data handling for model performance monitoring by a 6G CN NF

NOTE: data sources for these three use cases can be captured in solutions

- AIML Model training with data collection for UE sided model training outside the CN (e.g. 3rd  party AF)( (radio related)

Editor’s Note: Whether “AIML Model training with data collection for UE sided model training (radio related)” is a use case depends on RAN requirements

- AIML Model training with data collection for two-sided model training outside the CN (e.g. 3rd  party AF)(radio related)

NOTE: This use case needs to be coordinated with RAN WGs.

- Data handling to support Positioning

Editor’s Note: Whether “Data handling to support Positioning” is a use case needs to be co-ordinated with3GPP RAN WGs

- Data Exposure of operator data assets to 3rd party :

The operator may decide to expose some of its data to third parties. Different sets of data assets may be exposed to different third-party entities, hence the operators need granular control over data exposure. This use case requires tight operator control and support of user consent and privacy

Example of such data:  Operator exposes data  (e.g., period of specific traffic flow the UE is visiting, group information for UE to UE communication via 6G UP ) or analytics results about selected users to third parties subject to operator business agreements and user consent.

- UE as a data consumer of the data framework.

Editor’s Note: Example of such data should be provided in this clause

- Digital Twin based on data collected from CN NFs providing data defined by SA2 e.g., UE /PDU Session related data and data collected from OAM entities.

Editor’s Note: Whether the “Network Digital Twin” related use case is applicable for KI#21 needs to be coordinated with SA5

Use cases still requiring further discussion

- Data handling for Sensing: ,

Editor’s Note: Whether “sensing” related use cases are applicable for KI#21 is FFS and needs to be coordinated with KI#20

Editor’s Note: “Sensing” related use cases require co-ordination with RAN

- AIML Model transfer between 6G CN NFs

Editor’s Note: Whether “AIML Model transfer between 6G CN NFs” is a use case needs to be coordinated with KI#18.

- Data handling to support Positioning

Editor’s Note: Whether “Data handling to support Positioning” is a use case needs to be co-ordinated with3GPP RAN WGs

- Enhance CN AIML  with OAM data

Editor’s Note: How to access OAM data is to be defined by SA5

- .

- Planning the way of fulfilling flexible request, e.g., intent-based request.

##### 6.21.0.0.3	Data framework related functionalities

NOTE 1: 	Defining functionality in this clause does not mean that it is agreed that the corresponding feature or functionality is in scope of 6G specifications; it is possible that conclusions will mention that a functionality in this clause is not subject to specifications.

NOTE 2: 	 Some use cases can possibly need only a subset of the functionalities listed in this clause.

Functionalities potentially needed

(they are further defined below but their need is FFS)

-   	Data Control Functionality

-	Data Transfer Functionality

-	Data Repository Functionality

-	Data processing Functionality

-	Data Exposure Functionality

-	Data Production Capability Registration Functionality

-	Data Agent Functionality

Editor’s Note: Which functionality is appliable for which use case is FFS (e.g., for only a subset of user case or all use cases)

Editor’s Note: 	Coordination between SA2 and SA5 is needed for overlapping functionality.

- Data Control Functionality is to control data handling. It supports:

- handling a data request from data consumer

- authentication and authorization for data consumers and producers

- data source selection and data discovery

- triggering data source to provide data

- enforcing of user consent checking: when applicable, the data framework enforces user consent checking for data collection from data consumer

- control of data distribution to data consumers

- control of data storage and retrieval

- control of data processing

- A Data Transfer Functionality is to support:

- Transferring data from data sources

- data transfer/distribution to data consumer(s)

- Data duplication when the same data needs to be distributed to multiple consumers.

- A Data Processing Functionality is defined to provide data processing such as:

- data aggregation (e.g.  multi-source data fusion)

- data anonymization

- data pseudonymization

- data labelling

- formatting before delivering data to data consumer,

- enforcing quality requirement (e.g. completeness) from Data Control Functionality

- metadata handling

NOTE X: The details for data anonymization and data pseudonymization mechanism need coordination with SA3.

- A Data Repository Functionality is used for data storage and retrieval, and may support:

- storing the collected data

- retrieving the data under request

- A Data Exposure Functionality supports the control of data exposure to the third-party AF or UE with consideration of service authorization, privacy protection and operator policy.

- Access control (e.g. authentication and authorization) for data request from the third party AF

NOTE x:	Access control (e.g. authentication and authorization) for data request from UE is performed by the Data Control Functionality when applicable.

NOTE x:	Further details on technical difference between exposure to AF and UE will discussed in each solution variant.

- A Data Production Capability Registration Functionality may support:

- registration of data production capability (e.g. supported/stored data type, data format, number of stored data sample) of data source and/or Data Repository Functionality

- discovering and selecting the data source and/or Data Repository Functionality based on registered data production capability

- A Data Agent Functionality may support to:

- comprehend the data related requests (e.g., intent-based request, data with quality requirement, customized data) from UE, AF and other network entities.

- Based on the request, identify the required Data/Quality, etc,

- leverage corresponding functionality (e.g. Data control functionality) to trigger the data source triggering / transfer / processing to fulfil the request.

Editor’s note: Whether Agentic capability is applicable to this KI is FFS and subject to alignment with KI#18.

#### 6.21.0.1	Procedures

Editor’s Note: For further Study

#### 6.21.0.2	Services, Entities and Interfaces

Editor’s Note: For further Study

##### 6.21.0.4		list of high level questions

Editor’s note: This sub-clause will list the VERY HIGH level questions to be addressed to reach conclusions and can be considered as a BIG Editor’s NOTE; there are of course other extra points requiring an Editor’s Note

* * * End of Change * * *