3GPP TSG-WG SA2#173	S2-2601183

# Goa, India, 9 – 13 February, 2026                                                        (Revision of S2-2601166)

Source:	Sony, AT&T

Title:	KI#24: Analyse of 5GS IoT features and its applicability in 6GS

Document for:	Approval

Agenda item:	20.6.24

Work Item:	FS_6G_ARC

Abstract of the contribution: This pCR proposes, based on penholders analysis of the input papers, to capture two solutions variants for KI#24 (Analyse 5GS IoT features and solutions).

# 1.	Discussion

This pCR proposes to capture

-	One solution variant based on selected input solutions for KI#24 (Analyse 5GS IoT features and solutions)

-	list of the submitted input solutions in Annex X

The table 1 below provides high-level overview of the applicability of each 5GS IoT feature in TS 23.501 clause 5.31 captured from the analysis provided in each input paper (last three digits of the tdoc number are captured). The table shall not be considered and used to see any majority view. It was a tool used when analysing the input papers.

Table 1: High-level overview of the applicability of each 5GS IoT feature in TS 23.501 clause 5.31 captured from the analysis provided in each input paper.

| Clause | 121 | 246 | 316 | 332 | 380 | 396 | 401 | 444 | 529 | 590 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.31.1 General |  |  |  |  |  |  |  |  |  |  |
| 5.31.2 Preferred and Supported Network Behaviour |  |  |  |  | Yes |  |  | Yes | Yes |  |
| 5.31.3 Selection, steering and redirection between EPS and 5GS |  |  |  |  | No |  |  | No | No |  |
| 5.31.4 Control Plane CIoT 5GS Optimisation | Yes |  | No | No | Yes | Yes | No | Yes | Yes |  |
| 5.31.5 Non-IP Data Delivery (NIDD) | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| 5.31.6 Reliable Data Service | Yes |  | Yes | Yes | No |  | Yes | Yes | No |  |
| 5.31.7 Power Saving Enhancements | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |
| 5.31.8 High latency communication | Yes |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |
| 5.31.9 Support for Monitoring Events | Yes |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |
| 5.31.10 NB-IoT UE Radio Capability Handling |  |  | No |  | No |  | No | No | No |  |
| 5.31.11 Inter-RAT idle mode mobility to and from NB-IoT |  |  | No | No | No |  | No | No | No |  |
| 5.31.12 Restriction of use of Enhanced Coverage | Yes |  | No | No | Yes |  | Yes | Yes | depend on RAN |  |
| 5.31.13 Paging for Enhanced Coverage | Yes |  | No |  | Yes |  | Yes | Yes | depend on RAN |  |
| 5.31.14 Support of rate control of user data | Yes |  | No | No | No |  | No | Yes | No |  |
| 5.31.15 Control Plane Data Transfer Congestion Control | Yes |  | No |  | No |  | No | Yes | No |  |
| 5.31.16 Service Gap Control | Yes |  | Yes | Yes | No |  | No | Yes | No |  |
| 5.31.17 Inter-UE QoS for NB-IoT |  |  | No |  | No |  | No | No | No |  |
| 5.31.18 User Plane CIoT 5GS Optimisation | No | Yes | Yes | No | Yes |  | Yes | Yes | Yes |  |
| 5.31.19 QoS model for NB-IoT |  |  | No |  | No |  | No | No | No |  |
| 5.31.20 Category M UEs differentiation | Yes |  | No |  | Yes |  | No | Yes | No |  |
| NR RedCap and NR eRedCap UEs differentiation |  |  | Yes |  |  |  |  |  |  |  |

Additional notes from input papers not strictly connected to the TS 23.501 clauses in the above table.

121: 6G RAN should have native capability to support NB-IOT and LTE-M. Support EDT and Header Compression for CP CIoT optimization.

590: Roaming (global), efficient MT data, efficient small data, integrity protection, Ethernet PDN type.

# 2.	Text proposal

It is proposed to agree the following changes vs. TR 23.801-01:

* * * First Change * * * *

## 6.24	Solutions to KI#24

### 6.24.1	Solution variant #24.1: Features applicable for 6G IoT solution.

#### 6.24.1.0	Topics addressed and High-level Solution Principles

This solution addresses KI#24 and specifically provides a list of 5GS IoT features that should be further studied and possible enhanced to support 6G IoT.

#### 6.24.1.1	Description

The 5GS CIoT features can be treated as the start point for the 6G IoT study.

Features driven by this key issue are expected to be generic and may apply to any UEs in 6G.

Based on clause 5.31 “Support for cellular IoT” in TS 23.501 [2], this solution has the following analysis and observations.

Following features are necessary for the 6G IoT , so SA2 should study these features in this key issue. Enhancements on these features may be needed.

-	5.31.5	Non-IP Data Delivery (NIDD), including e.g. Ethernet PDN type.

-	5.31.7	Power Saving Enhancements

NOTE:	Some of 5G IoT power saving features has high dependence on RAN WGs and coordination with RAN WGs is needed.

-	5.31.8	High latency communication

Editor’s Note:	High latency communication has high dependence on whether data is routed via CP or UP. Whether data routing is supported via CP, UP or both is FFS.

-	5.31.9	Support for Monitoring Events

-	Efficient data communication. Both MO and MT data communication is included and should consider small data and larger data communication aspects.

Editor’s Note:	Whether data routing is supported via CP, UP or both is FFS. This feature may also have dependence on which power saving features to be supported.

- Roaming

Editor’s Note:	Whether support of roaming is a generic 6G feature and studied in other KI or this KI is FFS.

The following features have clear RAN dependency, and should be led by RAN WGs. Alignment work may be needed in SA2 based on RAN progress.

-	5.31.12 Restriction of use of Enhanced Coverage

-	5.31.13 Paging for Enhanced Coverage

Features in clause 5.31 “Support for cellular IoT” in TS 23.501 [2] that are listed below are considered not applicable for 6G IoT.

- 	5.31.3		Selection, steering and redirection between EPS and 5GS

- 	5.31.10	NB-IoT UE Radio Capability Handling

- 	5.31.11	Inter-RAT idle mode mobility to and from NB-IoT

- 	5.31.17	Inter-UE QoS for NB-IoT

- 	5.31.19	QoS model for NB-IoT

#### 6.24.1.2	Procedures

Guidance:

- 	include in this clause: description of the procedures and information flows for the solution.

- 	this clause is not a priority in the February meeting and can be added in a later meeting, but the penholders can make a judgement whether it can be proposed already in February (e.g. if it is straightforward and aligned in this solution variant)

#### 6.24.1.3	Services, Entities and Interfaces

Guidance:

-	include in this clause: description of the Services, Entities and Interfaces assumed by the solution. If existing Services, Entities and/or Interfaces are impacted (e.g. 5G), describe the impacts.

- 	this clause is not a priority in the February meeting and can be added in a later meeting, but the penholders can make a judgement whether it can be proposed already in February (e.g. if it is straightforward and aligned in this solution variant)

#### 6.24.1.4	Issues

The following features need more discussion before agreeing on the applicability for 6G IoT.

-	5.31.4		Control Plane CIoT 5GS Optimisation

-	5.31.6		Reliable Data Service

-	5.31.14	Support of rate control of user data

-	5.31.15 	Control Plane Data Transfer Congestion Control

-	5.31.16	Service Gap Control

-	5.31.18 	User Plane CIoT 5GS Optimisation

-	Reduced Capability signalling.

* * * Next Change * * * *

Annex X: Submitted solutions

## X.24	List of submitted solutions for KI#24

Table X.24: List of submitted solutions

| Meeting | Solution# | TDoc | Subject/Comment |
| --- | --- | --- | --- |
| SA2#173 | 1 | S2-2600246 | Only user plane based features |
| SA2#173 | 1 | S2-2600316 | Only user plane based features |
| SA2#173 | 1 | S2-2600332 | Support user plane solution, but not the UP CIoT optimization feature in 5GS. |
| SA2#173 | 1 | S2-2600380 | Include support for both Control plane and User plane based features. |
| SA2#173 | 1 | S2-2600401 | Only user plane based features |
| SA2#173 | 1 | S2-2600444 | Include support for both Control plane and User plane based features. |
| SA2#173 | 1 | S2-2600529 | Include support for both Control plane and User plane based features. |
| SA2#173 | 1 | S2-2600590 | Includes important features for 6G IoT, but not connected to CP or UP solution |
| SA2#173 | 1 | S2-2600121 | Only control plane based features |
| SA2#173 | 1 | S2-2600396 | Only control plane based features |

* * * End of Changes * * * *