3GPP TSG-WG SA2#173	S2-26xxxxx

# Goa, India, 9 – 13 February, 2026

Source:	Apple

# Title:	KI#3: Conclusions

# Document for:	Approval

Agenda Item:	20.8.1

Work Item / Release:	FS_SMS2EC_ARC / Rel-20

Abstract of the contribution: This contribution proposes conclusions for KI#3.

# 1. Discussion

This contribution proposes conclusions for KI#3: SMS over IP Routing to Appropriate PSAP.

# 2. Text Proposal

It is proposed to add this solution to TR 23.700-65.

# * * * * First change * * * *

# 8	Conclusions

Editor's note: This clause will list conclusions that have been agreed during the course of the study item activities.

## 8.x	Conclusions for KI#3

-	The serving PLMN sends an indication towards the UE during IP-CAN registration, e.g., during the 5GS Registration procedure or during E-UTRAN Attach/TAU procedure, to indicate support for SM to Emergency Response Centre over IMS.

-	When UE detects the SM to Emergency Response Centre based on the used emergency numbers, the UE performs emergency IMS registration, unless the UE already has a valid emergency IMS registration.

-	The UE sends a SIP MESSAGE request to the P-CSCF containing a Request-URI set to SOS URN or an emergency number. The SIP MESSAGE contains the SM to the Emergency Response Centre.

-	The P-CSCF forwards the SIP MESSAGE to an E-CSCF that is capable of handling the SM to the Emergency Response Centre.

-	The E-CSCF optionally queries the LRF to obtain user location information and determines a suitable PSAP address. The E-CSCF includes the PSAP address in the Request-URI in the SIP MESSAGE request and forwards it to the IP-SM-GW configured in the E-CSCF.

-	The IP-SM-GW selects a local SMSC and forwards the SM to the Emergency Response Centre via MAP-MO to the SMSC including the PSAP address.

-	The SMSC selects an appropriate Emergency Response Centre and sends the SM to the Emergency Response Centre.

-	When the UE receives the indication that SM to Emergency Response Centre over IMS is supported in the serving PLMN but it fails to send the SM to Emergency Response Centre over 3GPP access, the UE attempts to send the SM to Emergency Response Centre over non-3GPP access, if available.

-	When the UE does not receive the indication that SM to Emergency Response Centre over IMS is supported in the serving PLMN, it attempts to send the SM to Emergency Response Centre over NAS, if configured in the UE and supported by the network.

-	When the UE does not receive the indication that neither SM to Emergency Response Centre over NAS, nor over IMS is supported in the serving PLMN, the UE can attempt to select another PLMN in order to send a SM to Emergency Response Centre.

# * * * * End of change * * * *