3GPP TSG-WG SA2 Meeting #173	S2-260xxxx

Goa, India, 09-13 February, 2026

Source:	Ericsson

Title:	KI#2 conclusion for SMS to Emergency Response Centre via NAS

Document for:	Approval

Agenda Item:	20.8.1

Work Item / Release:	FS_SMS2EC_ARC / Rel-20

Abstract: This contribution proposes conclusions for KI#2 in TR 23.700-65, v1.0.0.

# Discussion

For the conclusion of the SMS2EC deliver via NAS, the following is proposed:

- Clarify the formation of the RP-DA/TP-DA at UE side (in synch with KI#1).

- Clarify the SMS2EC identification in EPC/5GC for the SMS to SMS-C delivery using the RP-DA.

- Clarify the inclusion of the UE ULI and MSISDN when MME/SMSF forward the SMS to SMS-C (via SMS-IWMSC) based on existing parameters already specified in diameter message/SBI service operations.

- Clarify that SMS-C is responsible for proper PSAP selection based on emergency service types/categories and UE ULI.

* * * Start of change * * * *

## 8.2	Conclusions for KI#2

1.	For sending a SMS to Emergency Response Centre, the UE provides:

a) In TP-DA with the local emergency number.

b) In RP-DA with the local emergency number or SMSC address of the serving PLMN.

2.	MME/SMSF identifies the SMS to Emergency Response Center based RP-DA as described in the conclusion of KI#1 and deliver the SMS to the SMSC in the serving PLMN (via SMS-IWMSC).

a) For the non-roaming case, the MME/SMSF just sends all the SMS (normal SMS and SMS to Emergency Response Centre) to SMSC of the serving PLMN (e.g. no need of checking RP-DA).

b) For the roaming case, the MME/SMSF needs to select which SMSC, in HPLMN or in VPLMN, to send the SMS based on RP-DA but without the need to identify the emergency service type/category.

3.	MME and AMF/SMSF include UE MSISDN and may include UE ULI (e.g. Cell-ID)/when sending the SMS to the SMSC in serving PLMN.

4.	SMSC is responsible for the identification of emergency service type/category included in the emergency number, selection of proper PSAPs for the delivery of SMS with different emergency service type/category (as described in conclusion for KI#1), considering also the UE location (e.g. UE ULI).

* * * End of change * * * *