3GPP TSG-WG SA2 Meeting #173	S2-260xxxx

Goa, India, 09-13 February, 2026

Source:	Ericsson

Title:	KI#3 conclusion for SMSoIP to Emergency Response Centre

Document for:	Approval

Agenda Item:	20.8.1

Work Item / Release:	FS_SMS2EC_ARC / Rel-20

Abstract: This contribution proposes conclusions for KI#3 in TR 23.700-65, v.1.0.0

# Discussion

The proposed Conclusions for KI#3 focus on maximizing the reuse of the already specified IMS Emergency Services, as outline in the following principles Architectural Assumption described in Section 4.1 of the TR 23.700-65, v.1.0.0

- “The architectures for SMS over NAS (see TS 23.272 [7], TS 23.401 [5], TS 23.501 [2], TS 23.502 [3]) and SMS over IP (see TS 23.204 [8]) and the architectural principles of Emergency Services (see TS 23.167 [9]) will be used as a basis.”

* * * Start of change (all new text)* * * *

## 8.3	Conclusions for KI#3

- For SMS over IP delivery to Emergency Response Centre, the IMS Core NFs, except for IP-SM-GW, do not need to decode the SMS included in the SIP MESSAGE body.

- In case of UE detectable Emergency Session:

  - Prior to initiating the SMS over IP delivery to Emergency Response Centre, the UE establishes an emergency PDU session and performs emergency IMS registration.

  - The UE sets the SIP MESSAGE R-URI to the SOS URN to indicate it represents an SMS over IP to be delivered to Emergency Centre. Additionally, the UE sets the TP-DA and RP-DA of the encapsulates SMS to the values agreed in the conclusions to KI#1

  - P-CSCF forwards the SIP MESSAGE to an E-CSCF that can handle SMS over IP to Emergency Response Centre.

  - E-CSCF optionally queries the LRF to get user location information.

  - E-CSCF sends the SIP MESSAGE to the IP-SM-GW. The SIP MESSAGE may include the LRF network provided location.

  - The IP-SM-GW in the serving PLMN is responsible to deliver the SMS to the SMS service centre based on the emergency number inserted in RP-DA. If the LRF network-provided is received from E-CSCF, IP-SM-GW is included in the message sent to SMS-C.

* * * End of change * * * *