SA WG2 Meeting #173	S2-2600820

9 - 13 February 2026, Goa, India	(was S2-251XXXX)

Source: 	Nokia

Title: 	Conclusions on KI#3 Support of SMS Delivery to Emergency Response Centre via IP

Document for: 	Approval

Agenda Item: 	20.8.1

Work Item / Release:	FS_SMS2EC_ARC / Rel-20

Abstract: This contribution provides a conclusion of KI#3 Support of SMS Delivery to the Emergency Response Centre via IP.

# 1	Discussion

At the SA2#171 meeting, 8 solutions were proposed for Key Issue #3, forming a comprehensive set of “SMS over IP” routing options for SMS to EC.

Given the common framework proposed in all the solution papers, this paper proposes Conclusions for KI#3

# 2 	Proposal

It is proposed to capture the following contents into TR 23.700-65 v1.0.0.

# * * * * First change* * * *

# 8	Conclusions

Editor's note: This clause will list conclusions that have been agreed during the course of the study item activities.

## 8.1	Conclusions for Key Issue #3

The following conclusions for Key Issue #3 Support of SMS Delivery to Emergency Response Centre via IP are made:

-

-	When UE detects an emergency number, and it does not have emergency IMS registration, then UE shall, in the case that bearer resources for the transport of the IMS-related signalling are required to be reserved in the IP-CAN, reserve the resources in the IP-CAN. The IP-CAN may support a UE indication that this request is for an emergency service, and then complete emergency IMS registration before sending the SMS over IP to EC. The IMS emergency registration is as per UE detectable emergency session procedure as described in TS 23.167 [X].

-	After completing the emergency IMS registration procedure as per clause 7.2 of 3GPP TS 23.167, the UE sends an SMS as a SIP MESSAGE containing an SMS in the payload to the P-CSCF in the serving PLMN. The P-CSCF forwards the SIP MESSAGE to an E-CSCF. The E-CSCF may query the LRF to obtain the UE’s location information and routing information. The E-CSCF can also take the UE provided location information via P-CSCF for determining SMS routing.

-	The E-CSCF forwards the SIP MESSAGE, including the short message and location information to an IP-SM-GW capable of handling emergency SMS. The IP-SM-GW selects an appropriate SMS-SC and forwards the emergency SMS to the SMS-SC. The SMS-SC selects an appropriate PSAP and delivers the SMS to the selected PSAP based on the emergency service type and location information. The IP-SM-GW can forward the SMS directly to an appropriate PSAP without including SMS-SC (e.g. in the deployment, where SMSC is not there in serving PLMN) in the path based on routing and location information received from E-CSCF and based on local configuration.

-	Any SMS sent by the PSAP to the UE is handled as a regular MT SMS. i.e. either via SMS over NAS or SMS over IP using regular IMS registration leg.

# * * * * End change* * * *