3GPP TSG-WG SA2 Meeting #173	S2-2600655r1

Goa, India, 09-13 February, 2026                             (revision of S2-2600655)

Source:	Ericsson

Title:	KI#2, Conclusion for SMS to Emergency Response Centre via NAS

Document for:	Approval

Agenda Item:	20.8.1

Work Item / Release:	FS_SMS2EC_ARC / Rel-20

Abstract: This contribution proposes conclusions for KI#2 in TR 23.700-65, v1.0.0.

# Discussion

For the conclusion of the SMS2EC deliver via NAS, the following is proposed:

- Clarify the formation of the RP-DA/TP-DA at UE side (in synch with KI#1).

- Clarify the SMS2EC identification in EPC/5GC for the SMS to SMS-C delivery using the RP-DA.

- Clarify the inclusion of the UE ULI and MSISDN when MME/SMSF forward the SMS to SMS-C (via SMS-IWMSC) based on existing parameters already specified in diameter message/SBI service operations.

- Clarify that SMS-C is responsible for proper PSAP selection based on emergency service types/categories and UE ULI.

* * * Start of change * * * *

## 8.2	Conclusions for KI#2

1.	The UE constructs an SMS to be routed to the Emergency Response Centre, in accordance with the conclusion of KI#1.

2.	MME/SMSF identifies the SMS to Emergency Response Center based on RP-DA (or a specific NAS “SMS2EC Indication”) and deliver the SMS to the SMSC in the serving PLMN (via SMS-IWMSC).

a) For the non-roaming case, the MME/SMSF sends all the SMS (normal SMS and SMS to Emergency Response Centre) to SMSC of the serving PLMN. The checking of RP-DA may be needed if a specific SMSC (e.g. different from the SMSC handling normal SMS) is deployed to handle SMS2EC in the PLMN.

b) For the roaming case, the MME/SMSF needs to select which SMSC, in HPLMN (for normal SMS) or in VPLMN (for SMS2EC), to send the SMS based on RP-DA, but without the need to identify the emergency service type/category.

NOTE: AMF delivers the MO SMS always to SMSF transparently for a registered UE with SMS service.

3.	MME and AMF/SMSF include UE MSISDN and UE ULI (e.g. Cell-ID) when sending the SMS to the SMSC in serving PLMN.

4.	SMSC is responsible for the identification of emergency service type/category included in the emergency number, selection of proper PSAPs for the delivery of SMS with different emergency service type/category (as described in conclusion for KI#1), considering also the UE location (e.g. UE ULI).

5.	The SMS response from PSAP to the UE is handled as a regular SMS. The response message is submitted to the SMSC in the MO UE’s serving PLMN (i.e., the HPLMN of the PSAP), and processed as a regular MT SMS.

* * * End of change * * * *