3GPP TSG-WG SA2 Meeting #173	S2-260xxxx

Goa, IN, 9th Feb – 13th Feb, 2026	(revision of S2-260xxxx)

Source:	Huawei, HiSilicon

Title:	Sensing Release Procedure

Document for:	Approval

Agenda Item:	20.2.2

Work Item / Release:	Sensing_ARC / Rel-20

Abstract: it captures Sensing Release procedure in the potential new TS for sensing.

1. Introduction

Sensing release procedure is pointing at the procedure of releasing the resources for a certain sensing service request. This can be triggered by the following reasons as per the conclusion of TR 23.700-14:

-	Sensing Service Consumer (i.e., AF) determines to release the sensing resources for the previous request. This can be due to e.g. the sensing result is no longer needed by the consumer;

-	Sensing (Control) Function determines/NEF to release the sensing resources of the on-going sensing service request. This can be due to e.g. the subscription of the Sensing Service Consumer changes and authorization criteria is no longer met.

This document proposes the sensing release procedure as per the above-mentioned aspect.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.xxx.

# * * * * First change * * * *

7	Sensing Procedures

7.2	Sensing Release Procedure

7.2.1	General

This procedure is used by the Sensing Service Consumer (i.e. AF) to cancel the on-going Sensing Service Request when required. The NEF or Sensing (Control) Function can also make use of this procedure to revoke the previously authorized Sensing Service Request when the authorization criteria are no longer met, see clause X.Y1.

7.2.2	Sensing Release procedure

1b. Nsf_SensingService_Delete Request/Response

6. Nnef_SensingService_TerminationNotify

5. Nsf_SensingService_TerminationNotify

2. Sensing Measurement Update Request /Sensing Measurement Release Request

1a. Nsf_SensingService_Delete Request/Response

1a. Nnef_SensingService_Delete Request

Sensing Service Consumer

NEF

Sensing Function (including control and processing)

Sensing Entity

1c. Determination the revocation of the sensing procedure

4. Removal of the Sensing Context

3. Release data connection towards Sensing Function (cf.7.2.3)

Figure 7.2.2-1: Sensing Release procedure

1.	The procedure may be triggered by following events:

1a.	(Sensing Service Consumer initiated release) The Sensing Service Consumer sends the Nnef_SensingService_Delete Request, to release the sensing resources for the previous request.

1b.	(NEF initiated release) The NEF can revoke the authorization of the AF for Sensing Service request.

1c.	(Sensing (Control) Function initiated release). The Sensing (Control) Function releases the on-going sensing service according to e.g., SLA change, latest sensing authorization information. The Sensing Control Function retrieves the sensing authorization information, and the sensing authorization information is as described in clause X.Y1

NOTE 1:	NEF may not be required if AF is in trusted domain.

2.	The Sensing (Control) Function determines whether any of the existing Sensing operation performing by RAN node(s) needs to be updated or release.

NOTE 2:	One example is if the Target Sensing area of the Sensing Service to be deleted overlaps with the Target Sensing area of other on-going service(s). In that case, the Sensing operation by RAN regarding that area might be maintained. If the removal of Target Sensing area does not have any overlaps, the Sensing Control Function may interact with the RAN nodes to update or release its Sensing operation.

The Sensing (Control) Function may send a Sensing Measurement Update/Release Request to RAN nodes.

Editor's Note: Messages involving RAN nodes and the parameters included are FFS, and need coordination with RAN WGs.

NOTE 3:	In case of the separate SCF and SPF deployment, the Sensing Control Function may further inform Sensing Processing Function to configure the processing.

3.	RAN node(s) may trigger the release of the path for 3GPP sensing data reporting towards the Sensing (Processing) Function as described in clause 7.2.3.

Editor's Note: Relationship between step 3 and 4 is FFS.

4.	The Sensing (Control) Function removes the Sensing Context related to the Sensing Service Delete request received or related to the internal decision.

5.	The Sensing (Control) Function sends Nsf_SensingService_TerminationNotify message to the NEF to indicate that the Sensing Service has been released.

6.	The NEF sends Nnef_SensingService_TerminationNotify message to the AF to indicate that the Sensing Service has been released.

Editor's Note: Whether RAN nodes can initiate the Sensing release/update procedure is FFS and depends by RAN WGs.

7.2.3	Release of data connection towards Sensing Function

Editor's Note: this procedure depends on RAN and needs further input from RAN WGs.

# * * * * End of changes * * * *