3GPP TSG-WG SA2 Meeting #173	S2-2601321

Goa, IN, 9th Feb – 13th Feb, 2026	(revision of S2-2601074)

Source:	Huawei, HiSilicon

Title:	Authorization and revocation for Sensing Service

Document for:	Approval

Agenda Item:	20.2.2

Work Item / Release:	Sensing_ARC / Rel-20

Abstract: it captures AF authorization for sensing service to the new TS.

# 1. Introduction

In clause 8.2 of TR 23.700-14, the Conclusion for Key Issue #2 Authorization and Revocation to Support Sensing Service is achieved, and so it shall be adopted into the normative work.

# 2. Text Proposal

It is proposed to capture the following changes vs. TS 23.xxx.

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[xx]	3GPP TS 33.501: " Security architecture and procedures for 5G system".

# * * * * Second change * * * *

## 5.1	Authorization and revocation of sensing service consumer

## 5.1.1	Authorization and revocation of sensing service consumer

If the AF as sensing service consumer is outside the trusted domain, authorization of the sensing service consumer is performed by the NEF as defined in clause 12 of TS 33.501 [xx]; If the AF is inside the trusted domain, authorization of the sensing service consumer is performed according to clause 13.3 as defined in TS 33.501 [xx].

The NEF shall revoke the previously authorized request of sensing servicewhen the subscription of AF is not authorized any more.

### 5.1.2	Authorization and revocation of sensing service request

The SenF authorizes the request of sensing service from the AF, considering the Sensing Authorization information described in Table 5.1.1-1.and

1)	proceeds the Sensing Service Request based on requested sensing requirements, if the request from the AF is authorized.

2)	rejects the Sensing Service Request, if request from the AF is not authorized.

The Sensing Function (SenF) performs authorization of the request of sensing service from AF	.

The Sensing Authorization information is pre-configured or configured by OAM, and stored in the SF for authorization of the sensing service request.

Table 5.1.1-1 below describes items stored as AF authorization data for the Sensing Service.

Table 5.1.1-1: Sensing Authorization for Sensing Service

| AF Authorization Data | Description |
| --- | --- |
| AF ID | Identifier used to identify the AF. |
| Allowed sensing service type | Indicate the allowed sensing service type (e.g. object tracking, detection) for the indicated AF to trigger the sensing services operations.. |
| Allowed/Not allowed area for sensing | Indicate the allowed/not allowed area for the indicated AF to request the sensing services operations (NOTE 1). |
| Allowed/Not allowed time period for sensing service | Indicate the allowed time period within which the AF can be authorized as a consumer of sensing service.. |
| NOTE 1: The Allowed/Not allowed area may be both present or only one of them is present, for example, if the AF is allowed to make a request for the majority of PLMN coverage excluding the not allowed area, only the not allowed area is present. If both are not present, all area of PLMN coverage is allowed. |  |

.

Editor’s note: Further details of the sensing authorization information is FFS

-	SF performs authorization of sensing service request, using the AF authorization data pre-configured locally in SF or configured by OAM as described in above Table 5.1.1-1.

Editor’s note: Whether and how to perform partial authorization of request of the sensing service is FFS.

NOTE:	In case of the SF contains SCF and SPF, the SCF performs the authorisation and revocation of Sensing Service Request.

The sensing service for an AF can be revoked by either the NEF or SF:

-	The NEF revokes the previously authorized Sensing Service Request when the subscription of AF is not authorized any more.

The SenF revokes the previously authorized request of sensing service when the sensing service authorization criteria  described in Table 5.1.1-1 are no longer met.

There is no need for authorization for the gNB as the Sensing Entity, as the gNB is deployed by operator.

Editor’s Note: Whether and how to pause the ongoing sensing service operation is FFS.

Upon revocation, the SenF may terminate or update the relevant sensing procedures related to the revoked sensing service as defined in clause 6.2.

The AF can cancel the on-going Sensing Service Request when required as defined in clause 6.2, the SF may terminate or update the on-going sensing service operation between the SenF and the gNB upon cancellation.

# * * * * End of changes * * * *