3GPP TSG-WG SA2 Meeting #173	S2-2601518

Goa, IN, 9th Feb – 13th Feb, 2026	(revision of S2-2601321)

Source:	Huawei, HiSilicon, ETRI, Vivo, NTT Docomo

Title:	Authorization and revocation for Sensing Service

Document for:	Approval

Agenda Item:	20.2.2

Work Item / Release:	Sensing_ARC / Rel-20

Abstract: it captures AF authorization for sensing service to the new TS.

# 1. Introduction

In clause 8.2 of TR 23.700-14, the Conclusion for Key Issue #2 Authorization and Revocation to Support Sensing Service is achieved, and so it shall be adopted into the normative work.

# 2. Text Proposal

It is proposed to capture the following changes vs. TS 23.xxx.

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[xx]	3GPP TS 33.501: " Security architecture and procedures for 5G system".

# * * * * Second change * * * *

## 5.1	Authorization and revocation of Sensing Service Consumer

### 5.1.1	Authorization and revocation of Sensing Service Consumers

If the AF as Sensing Service Consumer is outside the trusted domain, authorization of the sensing service consumer is performed by the NEF as defined in clause 12 of TS 33.501 [xx]; If the AF is inside the trusted domain, authorization of the Sensing Service Consumer is performed according to clause 13.3 as defined in TS 33.501 [xx].

The AF authorisation shall be revoked by NEF if the AF subscription is no more valid.

### 5.1.2	Authorization and revocation of Sensing Service Requests

The SenF authorizes the received Sensing Service Request, considering the Sensing Authorization information described in Table 5.1.1-1, and

1)	if the authorization is granted, SenF proceeds the Sensing Service Request, as specified in clause 6.2.

2)	if the authorization is not granted, the SenF rejects the Sensing Service Request.

The Sensing Authorization information is pre-configured or configured by OAM, and stored in the SF for authorization of the sensing service request.

The Table 5.1.1-1 below describes items stored as AF authorization data for the Sensing Service.

Table 5.1.1-1: Example of Sensing Service Authorization information

| AF Authorization Data | Description |
| --- | --- |
| AF ID | Identifier used to identify the AF. |
| Allowed sensing service type | Indicate the allowed sensing service type (e.g. object tracking, detection) for the indicated AF to trigger the sensing services operations.. |
| Allowed/Not allowed area for sensing | Indicate the allowed/not allowed area for the indicated AF to request the sensing services operations (NOTE 1). |
| Allowed/Not allowed time period for sensing service | Indicate the allowed time period within which the AF can be authorized as a consumer of sensing service.. |
| NOTE 1: For example, if the AF is allowed to make a request for the majority of PLMN coverage excluding the not allowed area, only the not allowed area is present. If both are not present, all area of PLMN coverage is allowed. |  |

.

Editor’s note: Details of the Sensing Service Authorization information and other attributes, if any, is FFS.

Editor’s note: Whether and how to perform partial authorization of the sensing service request is FFS.

Editor’s Note: Whether and how to pause the ongoing sensing service operation is FFS

NOTE:	In case of the SenF contains SenF-C and SenF-P, the SenF-C performs the authorisation and revocation of Sensing Service Request.

The SenF revokes the previously authorized Sensing Service Request when the sensing service authorization criteria  described in Table 5.1.1-1 are no longer met.

The authorisation of gNB as a Sensing Entity is not needed since the gNb is deployed by the operator

Upon revocation of AF authorization/sensing service authorization, the SenF may terminate the relevant sensing procedures related to the revoked Sensing Service as defined in clause 6.2.

The AF may cancel the on-going Sensing Service Request when required as defined in clause 6.2; in such case, the SenF may inform the SenE about the cancellation

# * * * * End of changes * * * *