3GPP TSG-WG SA2 Meeting #173	S2-260xxxx

Goa, IN, 9th Feb – 13th Feb, 2026	(revision of S2-260xxxx)

Source:	Huawei, HiSilicon

Title:	[KI#2 Conclusion] DO-A capable AIoT Device Power Saving

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract: Conclusion for power saving operations for AIoT Devices.

# 1. Introduction/Discussion

To optimize power efficiency, an AIoT Device may be configured to enter power-saving states where it is not required to monitor paging.

The existing power-saving mechanisms such as eDRX and PSM/MICO can be utilized as the baseline principle. Furthermore, it is noted that Solution 12 facilitates MT traffic delivery by employing a MICO-like mechanism.

For PSM/MICO operation, the AIoT Device is not required to monitor paging. The AIoT Device can initiate UL traffic, and when this occurs DL traffic can be sent to the AIoT Device. To ensure the AIoT Device remains reachable for DT/DO-DTT traffic, such as inventory or commands, an Active Time can be configured in the AIoT Device, to extend when DL traffic can be sent to the AIoT Device after it has send UL traffic. During this Active Time, the AIoT Device monitors paging to allow successful network-initiated reachability.

For eDRX operation the AIoT Device is configured with an eDRX cycle and parameters that are used to determine when it is required to monitor paging. When there is DL traffic for the AIoT Device the network can perform paging when the AIoT Device is monitoring paging and deliver DL traffic. The AIoT Device can initiate UL traffic at any time, and any DL traffic can be sent to AIoT Device at this time.

For group-based operations, the eDRX cycle and parameters should be aligned across a group of AIoT Devices. This alignment enables the network to efficiently reach and manage the entire group within a single window, minimizing the operational impact on the network.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-30.

# * * * * First change (All new) * * * *

### 7.1.2.z	Support of Power Saving for DO-A Capable AIoT Devices

To optimize power consumption for AIoT Devices, the principles of extended Discontinuous Reception (eDRX) and Power Saving Mode (PSM) are be supported. The network may provide negotiated power-saving parameters to the AIoT Device during the Registration procedure.

When PSM/MICO-like mechanism is utilized:

-	The AIoT Device is not required to monitoring paging when it has been configured with PSM/MICO, unless an Active Timer has been provided, and can initiates DO-A transmissions at any time.

-	An AIoT Device can remain reachable for paging and therefore downlink data transfer during an Active Time period, which is triggered by UL and DL traffic. Upon expiry of the Active Time, the AIoT Device enters the state where it can initiate DO-A transmissions at any time and does not monitor paging. When the Active Time period expires, the AIoT Device does not need to monitor paging.

When eDRX is utilized:

-	The AIOTF determines an eDRX cycle and related parameters for an AIoT Device or a group of AIoT Devices, and provides them to the AIoT Device(s) in the Registration procedure. The AIoT Device is only required to monitor paging in the periodic window defined by the eDRX cycle and parameters. The AIOTF performs paging when the AIoT Device(s) is monitoring paging. The power saving parameters are sent to NG-RAN as part of the Inventory procedure.

# * * * * End of changes * * * *