3GPP SA WG2#173	S2-260xxxx

Goa, India, Feb. 9th – 13th, 2026	(Revision of S2-250xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.369 | CR | xxxx | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on the inventory and command procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-ARC |  |  |  |  |  | Date: |  |  | 2026-01-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently the Inventory procedure and Command procedure are using Naiotf_AIoT_Inventory Notify and Naiotf_AIoT_Command Notify message in the steps and figures. While the Network Function Services Clause defined only generic notify service as Naiotf_AIoT_Notify service. So it is proposed to update current procedure clauses to align with the service operation. Transaction ID is allocated by the source entity which initiates the request and will be echoed as part of the response to that request by the receiving enity to indicate for which service the response corresponds. Current specification text does not contain the Transaction IDs in multiple request messages in the required Input list. AIoT procedures have been defined in a transaction basis that may involve requests, responses to the requests and multiple notifications in a period afterwards. However CT1 has decided that AIoT NAS does not define timer in NAS level, and there is no description in the specification how the period that the transaction will sustain is decided and how the transaction is terminated gracefully when the period ends. It is therefore proposed to define a transaction timer from the AF and is used by AIOTF and NG-RAN to determine their own timer and use timer to terminate the transaction accordingly. RAN can queue AIoT operation requests and process them in turn, saving turn-around time between AIoT operations maximising the use of the physical resources. It is also expected that the AIOTF may queue requests and provide them to RAN to process. This allows the AIOTF to control which RAN nodes requests to go and when, and if needed, allows the ATIOF to adjust the queue of requests, e.g., removing requests that have been completed by another RAN node (e.g., when sending a command to a single device, and that device is found). While none of this is standardised behaviour, it is expected an implementation may do this, and this needs to be acknowledged in the specification. The NG-RAN node is aware of the RAN Readers' status; however, this status is not synchronized with AIOTF. Thus it is proposed to add a NOTE in step 8 in clause 6.2.2 In the incoming SL from RAN3 (S2-2506146), NGAP: Command Request procedure is a per single device procedure. It is proposed to clarify that the Command request procedure is sent per AIoT Device. Command procedure is precedented by Inventory procedure and Temporary ID should be used if privacy protection is used. The temporary ID and related information retrieval is missing in Command procedure. The AIOTF provides information to NG-RAN including the size of the Command Response message from the AIoT Device and that is not an optional IE. The Command Response message size is derived by the AIOTF based on the command type specific parameters (e.g., the length of application data to read) provided by the AF.However, in the Command procedure, the AF sends an optional IE “approximate D2R message size” to the CN, and the CN sends an optional IE “approximate D2R message size” to the NG-RAN. The AF does not need to provide this information in addition to the command type specific parameters, and it is mandatory for the AIOTF to provide this information to the NG-RAN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 6.2.2 and 6.2.3 are updated according to the changes of S2-2510707, S2-2510322, S2-2510358, S2-2510661, S2-2510363, S2-2510099, S2-2510725, S2-2510628 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The messages and IEs are not aligned. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2, 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

### 6.2.2	Inventory Procedure

Figure 6.2.2-1 describes the inventory procedure.

The procedure focuses on the messages and parameters used for the communication between AIOTF and NG-RAN regardless of the path to access NG-RAN, see clause 4.2.2.1. The handling of the different communication paths is described in clause 6.2.4.

Figure 6.2.2-1: Inventory Procedure

1.	The AF invokes Nnef_AIoT_Inventory Request (AF ID, AF Transaction ID, [External Target Area information], Transaction duration time, [information about the target AIoT Device(s)], [Approximate number of AIoT Devices], [time interval], [location information requested]) to the NEF.

Information about the target AIoT Device(s) may include Filtering Information, as described in clause 5.8, or include complete AIoT Device Identifier(s). If Information about the target AIoT Device(s) is not included, the Inventory Request is for all AIoT Devices.

The approximate number of AIoT Devices, if provided, is used to determine the number of AIoT Devices expected to respond to this inventory AIoT service operation request, which is sent by AIOTF to the NG-RAN in the assistance information for NG-RAN in step 7 for proper radio resource allocation.

The time interval, if provided, is described in clause 5.9.

The location information requested parameter indicates whether the AF expects the network to provide the device location information.

Transaction duration time is provided by the AF to indicate the duration of the transaction for this request.

2.	The NEF may further authorize the AF request as specified in clause 5.6.

The NEF determines the Target Area information from the External Target Area information, and selects one or multiple AIOTF(s) to handle the request as specified in clause 5.3.1. If no AIOTF can be selected, the NEF rejects the Nnef_AIoT_Inventory request with an appropriate cause code and step 6 is performed before ending the procedure

3.	The NEF invokes the Naiotf_AIoT_Inventory Request (AF ID, Transaction ID, [Target Area information], Transaction duration time, [information about the target AIoT Device(s)], [Approximate number of AIoT Devices], [time interval], [location information requested]) towards each of the selected AIOTF(s).

4.	The AIOTF receives the Naiotf_AIoT_Inventory request and checks the parameters included in the request. The AIOTF may perform authorization as specified in clause 5.6. If the AIoT service operation request cannot be processed, the AIOTF rejects the AIoT service operation request with an appropriate cause code, and step 7 onwards are skipped.

The AIOTF generates a Correlation ID corresponding to this AIoT service operation request and the Correlation ID is used for the AIOTF to correlate the service operation responses received from NG-RAN to the AIOTF request. The AIOTF creates the AIoT Session for the AF service operation request, which is identified by the Correlation ID.

The AIOTF obtains security parameters from the ADM, as described in TS 33.369 [9].

The AIoT Identification Information to be provided to NG-RAN contains either Filtering Information as defined in clause 5.8, an AIoT Device Permanent Identifier as defined in clause 5.7.2 or an AIoT Device Temporary Identifier as defined in clause 5.7.4.

When the AIoT Identification Information includes an AIoT Device Temporary Identifier, the AIoT Device Temporary Identifier and related information is determined as specified in TS 33.369 [9].

If no AIoT Identification Information is provided to NG-RAN, then the NG-RAN inventories all AIoT Devices.

NOTE 1:	The concerns of paging all or paging a large group of AIoT devices are described in TS 33.369 [9].

The AIoT Identification Information and security parameters are provided to NG-RAN in step 7.

The AIOTF performs Reader Selection, see clause 5.3.3. If no NG-RAN or RAN Reader can be selected, the AIOTF rejects the AIoT service operation request with an appropriate cause code.

The AIOTF determines assistance information as described in clause 5.4, taking into account the parameters provided in the AIoT service operation request.

The AIOTF may perform AMF selection as described in clause 5.3.4.

The AIOTF sets a transaction timer based on the received Transaction duration time. If the timer expires, the AIOTF terminates the transaction and ignore any received response to the request afterwards.

5.	AIOTF sends the AIoT Inventory Service Response to the NEF containing the accept or reject result for the AIoT service operation request based on step 4.

6.	NEF sends the AIoT service operation response to the AF, containing the accept or reject result for the AIoT service operation request as specified in clause 8.3.

7.	The AIOTF sends the Inventory Request message including AIoT Identification Information and security parameters determined in step 4 to be included in the paging message, Correlation ID, Requested Service Area Information and assistance information to the selected NG-RAN node(s) as specified in TS 38.413 [10].

NOTE X:	The AIOTF can buffer and queue the Inventory Request before it is sent to an NG-RAN node, e.g., while waiting for responses from the same or other NG-RAN nodes.

8.	The NG-RAN sends an Inventory Response to the AIOTF with the Correlation ID indicating that the Inventory Request is received successfully and will perform the AIoT service operation accordingly as specified in TS 38.413 [10]. If the Inventory Request is not rejected, then an AIoT Session is created in the NG-RAN. After this step, the AIoT Session between the NG-RAN and AIOTF identified by correlation ID is established.

NOTE Y:	It is left to stage 3 to handle the case that the NG-RAN fails to deal with the inventory request due to the abnormal status of RAN Reader(s).

9.	Upon reception of the Inventory Request message from the AIOTF, the RAN Reader(s) will execute the inventory operation as specified in TS 38.300 [5] and TS 38.391 [11]. The RAN Reader(s) broadcast the paging message that includes the AIoT Identification Information and the security parameters.

If the received AIoT Identification Information contains:

-	Filtering Information, the AIoT Device determines whether it matches the AIoT Identification Information, as described in clause 5.8.

-	AIoT Device Temporary Identifier, the AIoT Device determines whether it matches the AIoT Identification Information, as described TS 33.369 [9].

-	AIoT Device Permanent Identifier, the AIoT Device determines whether it matches the AIoT Identification Information by comparing it with the stored AIoT Device Permanent Identifier.

The AIoT Device determines whether it matches the AIoT Identification Information, as described in clause 5.8.

If an AIoT device matches the AIoT Identification Information in the paging message, the AIoT Device responds to the paging message and sends an AIOT NAS message that includes its device specific authentication information as specified in TS 33.369 [9] and optionally the AIoT Device Permanent Identifier if privacy protection is not used.

10.	NG-RAN sends one or more Inventory Report messages to the AIOTF including the Correlation ID, Reader ID and the AIOT NAS message(s) from the AIoT Device(s) as specified in TS 38.413 [10]. The NG-RAN may aggregate multiple Inventory Report messages based on the assistance information before reporting the response to the AIOTF as described in clause 5.9. The AIOTF stores the mapping between the Reader ID and AIoT Device ID(s) as part of the AIoT Device context.

When the NG-RAN detects that no more AIoT Devices will respond to the inventory procedure, the NG-RAN informs the AIOTF by including the Inventory Complete Indication in the last Inventory Report message.

11.	The AIOTF authenticates the AIoT Device and retrieves the AIoT Device Permanent Identifier as specified in TS 33.369 [9]. The AIOTF may aggregate the results.

12.	If the NG-RAN has sent an inventory complete indication in step 10, the AIOTF triggers the AIoT Session Release procedure defined in clause 6.2.5 to release the AIoT Session between the NG-RAN and the AIOTF.

13.	The AIOTF reports the progress of the Naiotf_AIoT_Inventory request to the NEF by sending the Naiotf_AIoT_Notify message including a list of AIoT Device Permanent Identifier (s) and optionally location of each AIoT Device. The AIOTF may send multiple reports.

Based on operator policy, if the location information is requested by the AF and if the location of the reader is configured, the AIOTF uses the Reader ID reported from NG-RAN in step 10 to determine the AIoT Device Location.

The AIOTF in the final Naiotf_AIoT_Notify message indicates it is the last report for this operation. When the last report is sent, the AIOTF ends the AIoT Session. If multiple AIOTFs are involved in the procedure, the NEF may receive Naiotf_AIoT_Notify messages from multiple AIOTFs.

14.	When receiving the Naiotf_AIoT_Notify message from AIOTF, the NEF informs the AF of the outcome of the Nnef_AIoT_Inventory request by sending the Nnef_AIoT_Notify message(s) including the AIoT Device Permanent Identifier(s) and optionally location of each AIoT Device. The NEF in the final Nnef_AIoT_Notify message indicates that it is the last report for this operation.

* * * * Next Change * * * *

### 6.2.3	Command Procedure

Figure 6.2.3-1 depicts the command procedure.

The procedure focuses on the messages and parameters used for the communication between AIOTF and NG-RAN regardless of the path to access NG-RAN, see clause 4.2.2.1. The handling of the different communication paths is described in clause 6.2.4.

Figure 6.2.3-1: Command Procedure

1.	The AF sends the Nnef_AIoT_Command Request message (AF ID, AF Transaction ID, Command Type, Transaction duration time,[information about the target AIoT Device(s)], [External Target Area information], [Approximate number of AIoT Devices], [Command type specific parameters], [location information requested]) to NEF.

Information about the target AIoT Device(s) may include Filtering Information, as described in clause 5.8, or include complete AIoT Device Identifier(s).

The approximate number of AIoT Devices (see clause 5.4), if provided, is used to indicate the number of AIoT Devices expected to respond to this AIoT service operation request, which is sent by AIOTF to the NG-RAN in the assistance information as specified in clause 5.4.

Command Type provides the operation to be performed and the Command type specific parameters provides the required parameters for the operation. The service operations are described in clause 5.2.2.

Transaction duration time is provided by the AF to indicate the duration of the transaction for this request.

2.	Step 2 of the Inventory Procedure specified in clause 6.2.2 is performed for External Target Area information processing and AIOTF selection with the following clarifications:

-	If AIOTF selection fails, the NEF rejects the Nnef_AIoT_Command request and step 6 of this procedure is performed instead.

3.	The NEF sends Naiotf_AIoT_Command Request message (AF ID, Transaction ID, Command Type, Transaction duration time, [information about the target AIoT Device(s)], [Target area information], [Approximate number of AIoT Devices], [Command type specific parameters], [location information requested]) message to the selected AIOTF(s).

4.	The AIOTF receives the Naiotf_AIoT_Command Request and checks the parameters included in the request. The AIOTF performs Reader Selection as specified in clause 5.3.3. If no NG-RAN or RAN Reader can be selected, the AIOTF rejects the AIoT Command request with an appropriate cause code.

The AIOTF generates a Correlation ID corresponding to this AIoT service operation request and the Correlation ID is used for the AIOTF to correlate the service operation responses received from NG-RAN to the request. The AIOTF creates the AIoT Session for the AF service operation request, which is identified by the Correlation ID.

The AIOTF performs Reader Selection, see clause 5.3.3. If no NG-RAN or RAN Reader can be selected, the AIOTF rejects the AIoT service operation request with an appropriate cause code.

The AIOTF determines assistance information as described in clause 5.4, taking into account the parameters provided in the AIoT service operation request.

The AIOTF performs AF authorization for AIoT service operation request as described in clause 5.6.

If the information about the target AIoT device(s) received in Step 1 is Device Permanent Identifier(s), and privacy protection is used, the AIOTF retrieves the Device Temporary Identifier(s), temporary identifier related information and security parameters for the target AIoT device(s) from the ADM as described in TS 33.369 [9]. The Device Temporary Identifier(s) and related information is then to be used as the AIoT Identification Information in Step 7.

The AIOTF may perform AMF selection as described in clause 5.3.4.

The AIOTF sets a transaction timer based on the received Transaction duration time. If the timer expires, the AIOTF terminates the transaction and ignore any received response to the request afterwards.

5.	AIOTF sends the Naiotf_AIoT_Command Response message (accept or reject, [cause code]) to the NEF.

6.	NEF sends the Nnef_AIoT_Command Response message (accept or reject, [cause code]) to the AF. If the response was a reject the procedure stops here.

7.	Step 7 to step11 of procedure for Inventory specified in clause 6.2.2 are performed with the following clarifications:

-	In step 7, the AIOTF also includes follow on command indication in the Inventory Request message to inform the NG-RAN command delivery occurs after the inventory.

-	In step 10, the NG-RAN also includes the RAN AIoT Device NGAP ID for each AIoT Device in the Inventory Report as specified in TS 38.413 [10].

-	In step 11, the AIOTF authenticates the AIoT Devices as specified in TS 33.369 [9], and determines whether the command should be sent to an AIoT Device, e.g., by checking the Target AIoT device information. The AIOTF updates the corresponding AIoT device context in the AIOTF to include the RAN AIoT Device NGAP ID.

If none of successful Inventory response is received when the transaction timer set at step 4 expires, Step 8 -11 is not performed and the AIOTF sends a failure report to the NEF in step 12.

8.	For each successful Inventory response received, the AIOTF sends Command Request message (Correlation ID, [Reader ID], NAS Command Request, Estimated NAS Response message size, RAN AIoT Device NGAP ID) to the NG-RAN directly or as a NGAP AIoT information via an AMF as specified in clause 6.2.4 for each AIoT Device. The NAS Command Request message includes the AIoT data. The Correlation ID is as the same as the Correlation ID generated in step 4. The RAN AIoT Device NGAP ID is used by the NG-RAN to determine the AIoT device context in NG-RAN as specified in TS 38.413 [10].

The AIOTF uses the Command Type and Command type specific parameters received in Step 3 to determine the NAS Command Request to send to the AIoT Device, as described in clause 5.2.2.

The protection of the NAS Command Request message is specified in clause 5.3 of TS 33.369 [9].

NOTE 1:	Command Request(s) can be sent to NG-RAN when inventory procedure is ongoing.

9.	The NG-RAN sends the AS R2D message (NAS Command Request) to the AIoT Device as defined in TS 38.391 [11].

10.	The AIoT Device performs security check of the received NAS Command Request and sends the AS D2R message (including NAS Command Response) to the NG-RAN as defined in TS 38.391 [11]. The NAS Command Response message may include the AIoT data.

The protection of the NAS Command Response message is specified in clause 5.3 of TS 33.369 [9].

11.	The NG-RAN responds with a Command Response message (Correlation ID, Reader ID, NAS Command Response, RAN AIoT Device NGAP ID) to the AIOTF directly or as a NGAP AIoT information via an AMF as specified in clause 6.2.4. The AIOTF determines the AIoT device context by the RAN AIoT Device NGAP ID received.

If the NAS Command Response indicates the NAS Command type is not supported by the AIoT Device, the AIOTF reports the error result in step 13 and step 14.

NOTE 2:	The AIOTF can take this into account to avoid initiating further command procedures of the same command type towards the AIoT Device by implementation.

12.	After receiving the inventory complete indication in the last inventory report from the NG-RAN, when the AIOTF has completed sending Requests and all the Command Response have been received, the AIOTF initiates the AIoT Session Release Procedure in clause 6.2.5 to release the AIoT Session created during the Inventory Procedure.

13.	The AIOTF reports the result of the Naiotf_AIoT_Command request to the NEF by sending the Naiotf_AIoT_ Notify message (a list of AIoT Device(s) response information (AIoT Device ID(s), AIoT data and optionally location of each AIoT Device), Transaction ID, [Last Report Indication]). If multiple AIOTFs are involved in the procedure, the NEF may receive Naiotf_AIoT_Notify messages from multiple AIOTFs.

Based on operator policy, if the location information is requested by the AF and if the location of the reader is configured, the AIOTF uses the Reader ID reported from NG-RAN during inventory in step 7 to determine the AIoT Device Location.

When the last report is sent, the AIOTF ends the AIoT Session.

14.	The NEF informs the AF of the result of the Nnef_AIoT_Command request by sending the Nnef_AIoT_Notify message (a list of AIoT Device(s) response information (AIoT Device ID(s), AIoT data and optionally location of each AIoT Device), AF Transaction ID, [Last Report Indication]).

* * * * End of Changes * * * *