3GPP SA WG2#173	S2-2600412

Goa, India, 9-13 February, 2026	(Revision of S2-260xxxx)

Source:	OPPO, Huawei, NTT Docomo, NEC

Title:	Update to interim agreements for KI#1

Spec:	3GPP TR 23.700-30

Agenda item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Document for:	Approval

Abstract of the contribution: This contribution is to update the interim conclusion for KI#1.

1. Discussion

Based on the endorsed last page in S2-2511016 at SA2#172, it’s proposed to resolve the ENs for KI#1 interim conclusion.

Since in TS38.413, the AMF UE NGAP ID and RAN UE NGAP ID are included in the NG-AP message and used to identify a UE, on top of endorsed S2-2511016 (AMF UE NG-AP ID and AMF set ID is used to identify the UE reader between NG-RAN and AIOTF), the AMF UE NGAP and RAN UE NGAP ID is used to identify the UE reader to align with TS38.413.

2. Proposal

It is proposed to agree the following changes to 3GPP TR23.700-30.

* * * * First Change * * * *

# 7	Interim agreements

## 7.1	Agreed Principles

### 7.1.1	Agreed Principles for KI#1

'Figure 7.1.1-1 depicts the AIoT System Architecture for Topology 2.

Figure 7.1.1-1: AIoT System Architecture for Topology 2

Figure 7.1.1-2 depicts the AIoT system architecture, using the reference point representation.

Figure 7.1.1-2: AIoT System Architecture for Topology 2 in reference point representation

NOTE:	Whether the interface between AIOTF and UDM is needed will be checked in later phase.

Message and protocol stack

-	Messages between the UE Reader and the AIOTF are delivered using RRC between UE and NG-RAN and NGAP between NG-RAN and AMF and using an SBI interface between AMF and AIOTF. The related protocol stack is shown in Figure 7.1.1-3.

Figure 7.1.1-3: Protocol Stack for the RRC option

UE Reader 	Subscription aspects:

-	The UE subscription in the UDM will be extended with UE Reader subscription information, which consists of the following:

-	information indicating whether the UE is allowed to operate as a UE Reader.

-	UE Reader subscription information is available to AMF.

UE Reader Authorization:

-	If AMF receives the capability indication as UE Reader in the UE MM Core Network Capability from UE in the Registration Request and the indication that the UE is authorized to operate as a UE Reader in the subscription information from UDM:

a)	AMF sends an authorization indication as UE Reader to NG-RAN that the UE is authorized to operate as a UE Reader.

UE Reader Revocation:

-	If the subscription information has been revoked, the indication that the UE is not authorized to operate as a UE Reader from the UDM, then the AMF informs NG-RAN that the UE is not authorized to operate as a UE Reader.

Radio resource management for UE Reader operation:

-	If the NG-RAN has received the indication that a UE is authorized to operate as a UE Reader, then the NG-RAN may assign radio resources to the UE for UE Reader operation.

UE Reader ID information at AIOTF:

-	UE Reader subscription information is used by the AIOTF to select the UE reader. When receiving the AF Request, the AIOTF retrieves the UE reader ID information (AMF UE NGAP ID and RAN UE NGAP ID) from the serving AMF of the UE reader by sending SUPI to AMF. AMF UE NGAP ID and RAN UE NGAP ID are used to identify the UE reader during AIoT Service operation between AIOTF and NG-RAN.

AIOTF Discovery and Selection when AF provides UE reader ID:

-	When receiving the UE reader ID, the NEF queries the UDM to get the serving AMF of the UE reader and selects the AIOTF according to the local configuration.

NOTE:	NEF is configured with the mapping between the AMF ID and AIOTF ID(s).

UE reader selection part:

Two scenarios will be supported for UE reader selection:

-	For AF providing UE reader ID case:

-	If UE Reader ID(s) is provided by the AF via the NEF for the operation, then that is used as the selected Reader(s) if authorized as UE reader. The AIOTF provides the selected UE Reader(s) to the NG-RAN.

-	AMF UE NGAP ID and RAN UE NGAP ID are used to identify the UE reader during AIoT Service operation between AIOTF and NG-RAN.

-	The AIOTF receives the AMF UE NGAP ID and RAN UE NGAP ID from the AMF by providing SUPI to AMF.

-	For AF providing only target area case:

-	AIOTF selects the UE Reader based on its local configuration. After this step, the subsequent call flows are same as the case of AF providing UE reader ID

NOTE x:	The AIOTF is locally configured e.g., a mapping between the area information and UE reader information (SUPI and its serving AMF).

-	Or AIOTF selects the NG-RAN node and determines a Requested Service Area Information for each selected NG-RAN node using the existing procedures as specified in clause 5.3.3 in TS 23.369 [3] and it is up to NG-RAN to then select the UE reader to use. The discussion will be coordinated with RAN3.

* * * * End of Changes * * * *