SA WG2 Meeting #173	S2-2600738

Goa, India, February 9 – 13, 2026

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.369 | CR | 0144 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on interaction between NG-RAN and AIOTF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-ARC |  |  |  |  |  | Date: |  |  | 2026-1-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1. Some editorial changes are needed to align service operation and optimize word style for AIoT session release procedure. 2. How to handle the release request from AF is not clear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Update parameters of NGAP message for AIoT Session Release procedure 2. Add clarification that the AIOTF shall be able to trigger multiple session release procedures related to a transaction 3. editorial corrects |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is not clear |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.4.1, 6.2.4.2, 6.2.4.3, 6.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.2.4	Procedures between AIOTF and NG-RAN for Indirect Connectivity

#### 6.2.4.1	Overview

An AIOTF and NG-RAN can use an indirect interface via an AMF as described in clause 4.2.2.4. The procedure for NGAP message delivery from an AIOTF to NG-RAN via an AMF is defined in clause 6.2.4.2 and the procedure for message delivery from NG-RAN to an AIOTF via an AMF is defined in clause 6.2.4.3.

#### 6.2.4.2	Message delivery to NG-RAN

When the AIOTF invokes Namf_AIoT_MessageDelivery request with NGAP information towards an AMF, the AMF sends the corresponding NGAP message to NG-RAN. The additional steps used for indirect interface between AIOTF and NG-RAN are shown in Figure 6.2.4.2-1.

Figure 6.2.4.2-1: Procedure for NGAP message delivery to NG-RAN using indirect connectivity via an AMF

1.	The AIOTF invokes Namf_AIoT_MessageDelivery Request (NGAP AIoT information, NG-RAN ID, AIOTF ID, Correlation ID, AIoT NGAP Message Type, Notification endpoint, [RAN AIoT Device NGAP ID]) to the AMF.

The AIoT NGAP Message Type identifies the NGAP message to send to NG-RAN and NGAP AIoT information is provided to NG-RAN in the NGAP message.

If the AMF accepts the request, the AMF creates a context for the transaction and stores the NG-RAN ID, AIOTF ID, Correlation ID and Notification endpoint. The AIOTF is implicitly subscribed to Namf_AIoT_Notify events.

2.	The AMF responds to the AIOTF with a result indicating whether the AMF will handle the request. If a failure is indicated by the result indication, then the procedure stops and the remaining steps are skipped.

3.	The AMF sends an NGAP message (AIOTF ID, Correlation ID, NGAP AIoT information, [RAN AIoT Device NGAP ID]) to the target NG-RAN.

4.	The procedure for handling NGAP messages from NG-RAN is used to route NGAP messages from NG-RAN to the AIOTF, see clause 6.2.4.3.

#### 6.2.4.3	Message delivery to AIOTF

When the AMF receives an NGAP message from NG-RAN, for example, responses to a procedure or events like Inventory Reports, the AMF determines which AIOTF to send the information from NG-RAN and invokes Namf_AIoT_Notify towards that AIOTF. The additional steps used for indirect interface from NGAP towards AIOTF is shown in Figure 6.2.4.3-1.

Figure 6.2.4.3-1: Procedure for NG-RAN event handling using indirect connectivity via an AMF

1.	Before any NGAP message can be routed from NG-RAN to the AIOTF, the AMF needs routing information to be able to route the messages from NG-RAN to the AIOTF, see clause 6.2.4.2. The same context can be used route multiple NGAP messages from NG-RAN.

2.	NG-RAN sends an NGAP message (AIOTF ID, Correlation ID, NGAP AIoT information, [RAN AIoT Device NGAP ID]) to an AMF. NGAP AIoT information is determined by NG-RAN depending upon the operation the NGAP message relates to.

3.	AMF determines the Notification endpoint using the AIOTF ID and the Correlation ID received in step 2 from NG-RAN, and then invokes Namf_AIoT_Notify (AIoT NGAP Message Type, NGAP AIoT information) towards the AIOTF.

Step 2 and step 3 are repeated for each NGAP message received from NG-RAN.

If the message received from NG-RAN is an Inventory Failure indicating an error or an AIOT Session Release Request, then the AMF releases the context which is associated with the AIOTF ID and Correlation ID, after sending the Namf_AIoT_Notify message to the AIOTF.

### 6.2.5	AIoT Session Release Procedure

This procedure is used to release the AIoT Session between the NG-RAN and the AIOTF. The AIoT Session release procedure can be triggered by the AIOTF or the NG-RAN node and is specified in TS 38.413 [10].

The initiation of AIoT Session release may be:

-	NG-RAN-initiated e.g. if the NG-RAN detects no AIoT Devices responds to the inventory procedure or the command procedure; or

-	AIOTF-initiated, e.g. if the AIOTF fails validating the results of AIOT NAS Inventory Response as specified in TS 33.369 [9] in the command procedure.

Both NG-RAN-initiated and AIOTF-initiated AIoT Session Release procedures are shown in Figure 6.2.5-1.

Figure 6.2.5-1: AIoT Session Release Procedure

1.	NG-RAN may decide to initiate the AIoT Session release procedure. NG-RAN sends an AIoT Session Release Request message (Correlation ID, Cause) to the AIOTF directly (1a) or performs steps 2-3 for the AIoT Session Release Request specified in clause 6.2.4.3 (1b).

2.	If the AIOTF receives the AIoT Session Release request message or the AIOTF decides to terminate all activities related to the AIoT Session, the AIOTF sends an AIoT Session Release Command message (Correlation ID, Cause) to the NG-RAN directly (2a) or performs steps 1-3 for the AIoT Session Release Command specified in clause 6.2.4.2 (2b).

3.	The NG-RAN node releases the AIoT Session and radio resources related to the AIoT session identified by the Correlation ID provided by the AIOTF, as specified in TS 38.300 [5]. And the NG-RAN confirms the AIoT Session Release by returning an AIoT Session Release Complete message (Correlation ID) to the AIOTF directly (3a) or performs steps 2-3 for the AIoT Session Release Complete specified in clause 6.2.4.3 (3b). The AIOTF releases AIoT Session.

If a transaction involves multiple AIoT sessions between multiple NG-RAN nodes and the AIOTF, when the AIOTF determines to terminate the transaction, e.g., as requested by an AF, the AIOTF shall trigger AIoT Session Release Procedures for all the AIoT sessions identified by the Correlation IDs associated with the transaction.

# * * * * End of changes * * * *