3GPP TSG-WG SA2 Meeting #173	S2-2601305

Goa, IN, 9th Feb – 13th Feb, 2026	(revision of S2-2601012)

Source:	Huawei, HiSilicon

Title:	[KI#2 Conclusion] DO-A capable AIoT Device Power Saving

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract: Conclusion for power saving operations for AIoT Devices.

# 1. Introduction/Discussion

Key Issue 2 includes the following bullet:

-	Whether and how to consider power consumption of DO-A Capable AIoT Devices.

Solutions, such as solution #12, provide mechanisms to address this bullet.

To optimize power efficiency, an AIoT Device may be configured to enter power-saving states where it is not required to monitor paging.

The existing power-saving mechanisms such as eDRX and PSM/MICO can be utilized as the baseline principle. Furthermore, it is noted that Solution 12 facilitates MT traffic delivery by employing a MICO-like mechanism.

For PSM/MICO operation, the AIoT Device is not required to monitor paging. The AIoT Device can initiate UL traffic, and when this occurs DL traffic can be sent to the AIoT Device. To ensure the AIoT Device remains reachable for DT/DO-DTT traffic, such as inventory or commands for a period of time after the DO-A traffic, an Active Time can be configured in the AIoT Device by core network. During this Active Time, the AIoT Device monitors paging in the same way it would normally if PSM/MICO had not been configured to allow successful network-initiated reachability. Once the Active Time expires (e.g., after many minutes) the AIoT Device is no longer required to monitor paging.

How to perform paging during Active Time and when PSM/MICO is not used will be determined by RAN group.

Figure 1 PSM operation

For eDRX operation the AIoT Device is configured with an eDRX cycle and parameters that are used to determine when it is required to monitor paging, the PTW, and during the PTW the AIoT Device monitors paging in the same way it would normally if PSM/MICO had not been configured. When there is DL traffic for the AIoT Device the network can determine when the PTW is and request paging within the PTW (i.e., send the paging request to the Reader) When the AIoT Device responds to paging, DL traffic can be delivered normally. The AIoT Device can initiate UL traffic at any time, and any DL traffic can be sent to AIoT Device at this time. The core network can decide the length of Paging Time Window (PTW) and eDRX cycle (i.e. time between the PTWs).

How to perform paging during PTW and when eDRX is not used will be determined by RAN group.

Figure 2 eDRX operation

For group-based operations, the eDRX cycle and parameters should be aligned across a group of AIoT Devices. This alignment enables the network to efficiently reach and manage the entire group within a single window, minimizing the operational impact on the network.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-30.

# * * * * First change (All new texts) * * * *

### 7.1.2.z	Support of Power Saving for DO-A Capable AIoT Devices

To optimize power consumption for AIoT Devices, the principles of  Power Saving Mode (PSM)/MICO mode are be supported to enable an AIoT Device originated only traffic mode. The network may negotiated power-saving parameters to the AIoT Device during the Registration procedure.

NOTE:	Further power saving features e.g. eDRX could be considered in the normative phase if RAN WG decides to specify such power saving features.

When PSM/MICO-like mechanism is utilized to enable a device originated only traffic:

-	The AIoT Device is not required to monitoring paging when it has been configured with PSM/MICO. The AIoT Device can initiate DO-A transmissions at any time and stored DL data/command in the AIOTF can be transmitted to the AIoT Device before the session is released.

# * * * * End of changes * * * *